CADENCE INTERVIEW QUESTIONS

1.What are components of the Processor?

  Arithmetic Logic Unit (ALU): Performs arithmetic operations (add, subtract, multiply) and logical operations (AND, OR, compare).

  Control Unit (CU): Directs and coordinates all processor operations by fetching and decoding instructions.

  Registers: Very small and fast memory inside the processor used to store data, instructions, and results temporarily.

  Program Counter (PC): Holds the address of the next instruction to be executed.

  Instruction Register (IR): Stores the current instruction being executed.

  Cache Memory: High-speed memory that stores frequently used data and instructions to reduce access time.

  Bus System: Transfers data, addresses, and control signals between processor components.

2. What is the role of the Control Unit?

- Fetches instructions from memory.

- Decodes them to generate control signals.

- Directs ALU, registers, and buses for execution.

- Handles instruction sequencing and branching.

- Coordinates with input/output devices.

4.  D FF with feedback

A DFF with feedback means the output is connected back to the input (directly or through logic). This allows the flip-flop to store and update its previous value, making it useful for counters, registers, and state machines.

A D flip-flop with inverted feedback ( D = Q̅ ) acts as a toggle flip-flop.

5. Define Memory and Cache Memory

Memory is a storage unit that holds data, instructions, and results needed by the processor. It allows the system to store information temporarily or permanently so the CPU can access it during program execution.

Cache memory is a small, very fast memory placed close to the CPU. It stores frequently used data and instructions to reduce access time and improve overall system speed.

6. Wht are Interrupts

Interrupts are signals sent to the processor to temporarily stop the current task and execute a higher-priority task. After handling the interrupt, the processor resumes the previous program from where it stopped.

7. Difference between C, Python, and Java

- C: A procedural, low-level language that is fast and gives direct control over hardware and memory. It is mainly used in embedded systems, operating systems, and drivers.

- Python: A high-level, interpreted language that is easy to read and write. It is slower than C but widely used in automation, data science, AI, and scripting.

- Java: An object-oriented, platform-independent language that runs on the JVM. It offers good performance, security, and portability, and is used in enterprise applications and Android development.

8.Define Synchronous and Asynchronous Counter 
A synchronous counter is a counter in which all flip-flops are triggered by the same clock signal. Because all stages change state at the same time, it is faster and more reliable.

An asynchronous counter is a counter in which only the first flip-flop receives the clock, and the output of one flip-flop clocks the next. This causes delay (ripple effect) and makes it slower than a synchronous counter.

9. Important Blocks of a Processor:

- Fetch Unit:
Fetches instructions from memory.

- Instruction Decode Unit (IDU):
Decodes the instruction and decides what operation to perform.

- Control Unit (CU):
Generates control signals and coordinates all processor activities.

- Execution Unit (EU):
Executes instructions (arithmetic, logic, load/store, branch).

- Arithmetic Logic Unit (ALU):
Performs arithmetic and logical operations.

- Registers:
Fast internal storage for operands, addresses, and results.

- Program Counter (PC):
Holds the address of the next instruction.

- Instruction Register (IR):
Holds the current instruction being executed.

- Cache Memory (L1/L2/L3):
Stores frequently used data and instructions for fast access.

- Load/Store Unit:
Handles data transfer between registers and memory.

- Bus Interface Unit:
Manages data, address, and control signal transfer.

Interview-ready closing line:
👉 A processor works in stages: Fetch → Decode → Execute → Memory → Write-back.

10. Assembly language instructions

Assembly language is a low-level programming language that uses mnemonics (like MOV, ADD) to write instructions that are directly understood by the processor. It is hardware-dependent and gives very close control over CPU operations.

- Data Movement: MOV, LOAD, STORE

- Arithmetic: ADD, SUB, MUL

- Logical: AND, OR, XOR

- Control: JMP, CALL, RET

- Branching: BEQ, BNE

11. Mealy and Moore with overlapping and non-overlapping.

- Overlapping sequence detector: Can reuse part of the sequence for the next detection.

- Non-overlapping: Waits for a complete reset of the sequence.

12. Sensitivity List:
A sensitivity list is the list of signals in an always block that trigger its execution whenever any of those signals change.

Example (easy):
always @(a or b) → block runs when a or b changes

Used to model combinational and sequential logic correctly in Verilog.

13. Synchronous and Asynchronous Reset

A synchronous reset means the reset is applied only at the clock edge.

An asynchronous reset means the reset is applied immediately, without waiting for the clock

Synchronous Reset:
A synchronous reset is applied only on the clock edge. The reset works together with the clock, so the output changes only when the clock arrives.

Asynchronous Reset:
An asynchronous reset is applied immediately, independent of the clock. The output resets as soon as the reset signal is active.

(Reset is a control signal used to bring a digital system into a known initial state)

14. Build Gates using MUX

NOT= assign y = a ? 1'b0 : 1'b1;               (if a=1 than 0 or else 1)

And = assign y = a? b : 2’b0;                     (if a=1 than b or else 0..according to truthtable )

OR= assign y = a ? 1'b1 : b;

XOR = assign y = a ? ~b : b;

15. Define SRAM  and DRAM

Static RAM stores data using flip-flops and does not need refreshing. It is very fast, expensive, and mainly used as cache memory.

( Uses flip-flops (latches) ->  Data stays as long as power is ON ->  No refresh needed)

Dynamic RAM stores data using capacitors and needs periodic refreshing. It is slower than SRAM, cheaper, and mainly used as main memory.

( Uses capacitors to store charge -> Charge leaks over time ->Needs periodic refresh)

16. What is the difference between cache and RAM and why are they used?

Cache Memory:
Cache is very fast, small memory placed close to the CPU. It stores frequently used data and instructions so the processor doesn’t have to wait, which improves speed.

RAM:
RAM is larger but slower than cache. It stores programs and data currently in use so the CPU can access them during execution.

Why both are used:
Cache is used for speed, RAM is used for capacity. Using both gives a balance of high performance and large storage.

17. Define ALU and MMU

- ALU (Arithmetic Logic Unit): Performs arithmetic (add, sub, mul) and logical (AND, OR, XOR) operations.

- MMU (Memory Management Unit): Handles virtual memory, address translation, and memory protection.