LATTICE INTERVIEW QUESTIONS

1. –

2.-

3. Draw a D-latch using 2:1 Mux.

EN = 1 → MUX selects I1 = D → Q follows D (transparent)

EN = 0 → MUX selects I0 = Q → output holds previous value

(Verilog code : assign Q = EN ? D : Q; )

4. What is even and odd parity ?..define it

Even Parity
Even parity is a parity scheme where the total number of 1s (data bits + parity bit) is even.

Odd Parity
Odd parity is a parity scheme where the total number of 1s (data bits + parity bit) is odd.

5. Whole front-end VLSI flow

- Specification

- Architecture design

- RTL coding

- RTL simulation / Functional verification

- Synthesis (RTL → gate netlist)

- Gate-Level Simulation (GLS)

- Static Timing Analysis (STA)

- DFT insertion

- Equivalence checking

- Front-end sign-off

(1️⃣ Specification
Decide what the chip should do.

2️⃣ Architecture Design
Split the chip into small functional blocks.

3️⃣ RTL Coding
Write hardware code in Verilog/VHDL.

4️⃣ RTL Simulation
Check if RTL works correctly.

5️⃣ Synthesis yosys
Convert RTL into logic gates.

6️⃣ Gate-Level Simulation (GLS)
Verify gate-level design works same as RTL.

7️⃣ Static Timing Analysis (STA)
Check timing meets clock speed.

8️⃣ DFT Insertion
Add test logic to test chip after fabrication.

9️⃣ Equivalence Checking
Confirm RTL and gate-level logic are identical

10 Front-End Sign-off

Front-end sign-off means all front-end checks are passed and the design is ready to go to back-end )…. .)  …there small definations

6. Tell me what is clock tree synthesis and how is it done?

Clock Tree Synthesis is the process of distributing the clock signal evenly from the clock source to all flip-flops in the chip with minimum delay difference.

Clock Tree Synthesis (CTS) Flow

1️⃣ Clock defined
Clock source, period, uncertainty specified.

2️⃣ Pre-CTS timing check
Ensure logic is reasonable before CTS.

3️⃣ Clock buffer/inverter insertion
Buffers added to drive clock loads.

4️⃣ Clock tree construction
Clock network built (balanced tree / H-tree).

5️⃣ Skew optimization
Arrival times balanced at all flip-flops.

6️⃣ Post-CTS timing analysis
Check setup/hold after CTS.

7. What is the setup and hold time?

Setup time is the minimum time data must be stable before the clock edge so the flip-flop can capture it correctly.
Hold time is the minimum time data must remain stable after the clock edge so the flip-flop keeps the correct value.

8.\\\\\\\\

9. Why do we use chips and where do we use them?

A chip is used to process data, store data, and control electronic systems.

Used in : Computers & laptops ,  Mobile phones ,  Automobile, Home appliances ,  Medical devices ,  Industrial systems ,  Communication systems .

11. Steps after RTL Gate-level Netlist generation

1️⃣ Gate-Level Simulation (GLS)
Check that the gate-level netlist behaves same as RTL.

2️⃣ Static Timing Analysis (STA)
Verify setup and hold timing meets clock requirements.

3️⃣ DFT Insertion
Add scan chains and test logic for manufacturing test.

4️⃣ Equivalence Checking
Confirm RTL logic = gate-level logic (no mismatch).

5️⃣ Front-End Sign-off
Approve that netlist is correct and timing-clean.

6️⃣ Physical Design (Back-End starts)

- Floorplanning
Decide where major blocks are placed on the chip.

- Placement
Place standard cells (gates, flip-flops) inside the blocks.

- CTS (Clock Tree Synthesis)
Distribute the clock signal evenly to all flip-flops.

- Routing
Connect all cells using metal wires.

7️⃣ GDSII generation  (Graphic Data System ii)

8️⃣  Tape-out

- What is the difference between always @(*) and always @(clk) and what are they both used

always @(*)is used to describe combinational logic, where the output changes immediately when any input changes.

always @(posedge clk) is used to describe sequential (clocked) logic, where the output changes only on the active clock edge.

12. What is VLSI?
VLSI (Very Large Scale Integration) is the technology of integrating millions of transistors on a single chip to build complex circuits like processors and memory.

13.Difference between combinational and sequential circuits (in Verilog):

Combinational circuit: Output depends only on present inputs (written using assign or always @(*)).

Sequential circuit: Output depends on inputs and past values, controlled by a clock (written using always @(posedge clk)).

14.Difference between a latch and a flip-flop:

- Latch: Level-sensitive, output changes as long as enable is active.

- Flip-flop: Edge-triggered, output changes only at clock edge.

15.

16. What is a K-map and what is its use?

K-map (Karnaugh Map)
A K-map is a graphical method used to simplify Boolean logic expressions.

Use of K-map:
It helps to reduce the number of logic gates, making the circuit simpler, faster, and cheaper.

One-line interview answer:
K-map is used to minimize Boolean expressions and optimize digital circuits.

17. Explain the VLSI design flow and its various stages.

1️⃣ Specification

Defines what the chip should do (function, speed, power, area).

2️⃣ Architecture Design

Breaks the system into functional blocks and decides how they connect.

3️⃣ RTL Design

Hardware is described using Verilog/VHDL.

4️⃣ RTL Verification

Checks whether the RTL code works correctly using simulation.

5️⃣ Synthesis

Converts RTL code into gate-level netlist using standard cells.

6️⃣ Gate-Level Simulation (GLS)

Verifies that the gate-level netlist behaves same as RTL.

7️⃣ Static Timing Analysis (STA)

Checks setup and hold timing to meet clock requirements.

8️⃣ Physical Design (Back-End)

Includes:

- Floorplanning

- Placement

- Clock Tree Synthesis (CTS)

- Routing

9️⃣ Physical Verification

Ensures layout correctness using DRC and LVS checks.

🔟 GDSII Generation & Tape-out

Final layout (GDSII) is sent to the foundry for chip fabrication.

18. What is the output of a GDSII file?

The output of a GDSII file is the final physical layout of the chip, showing all geometric patterns of every layer (metal, diffusion, polysilicon, vias).

19. What are universal gates and what is their significance?

NAND and NOR are Universal gates

Any logic circuit can be implemented using only NAND or only NOR gates, which simplifies IC design and manufacturing and reduces overall cost and complexity.

20. Design a NOT gate using NAND and NOR gates….in vlsi interview questions

21. How do buffers work and how are they designed?

How buffers work:
A buffer is a circuit that copies the input to the output without changing the logic value, while increasing drive strength so the signal can drive larger loads.

How buffers are designed:
Buffers are designed by using two inverters connected back-to-back, so the output logic is the same as the input but with stronger driving capability.

(Since bigger transistors provide more current, a buffer uses larger transistors to increase current driving capability, thereby strengthening the signal and reducing delay.”

due to transistors in inverters the signal gets strengthen )

22. What is the always block in Verilog and how is it used?

The always block models combinational or sequential hardware based on its sensitivity list.

A sensitivity list is the list of signals that tell an always block when to run.

- always @(*)
Runs whenever any input changes (combinational logic)

- always @(posedge clk)
Runs only when the clock edge occurs (sequential logic)

23. Write Verilog code for a D latch and a D flip-flop.

24. What is FSM and what are its types?

Finite state machine is a mathematical computation model consisting of finite number of states, transition between those states and actions used to control the execution flow if a system .

A Mealy FSM is a type of FSM where the output depends on both the present state and the current input.
→ Output can change immediately with input change.

Interview keyword: Fast response

Moore Machine – Definition

A Moore FSM is a type of FSM where the output depends only on the present state.
→ Output changes only on clock edge.

Interview keyword: Stable output

25. Write Verilog code for a D latch and a D flip-flop. aslo define them

A latch is a level-sensitive memory element that stores 1 bit of data.
Its output changes whenever the enable signal is active

Code : assign Q = EN ? D : Q;

A flip-flop is an edge-triggered memory element that stores 1 bit of data.
Its output changes only at the clock edge

Code :always @(posedge clk) begin

if (rst)

Q <= 1'b0;   // reset

else

Q <= D;

2nd easy way

always @(posedge clk) begin

Q <= D;

26. Clock Skew
Clock skew is the difference in arrival time of the clock signal at different flip-flops in a circuit.

27. wire and reg

wire
A wire is used to connect signals and represents combinational connections.
It cannot store a value and must be continuously driven.

reg
A reg is used to store a value inside an always block.
It holds its value until it is updated.

28. What are the different types of processor architectures and where are they used

Different types of processor architectures & where they are used

- Von Neumann Architecture
Used in basic computers and simple systems.

- Harvard Architecture
Used in microcontrollers and DSP systems.

- Modified Harvard Architecture
Used in modern CPUs (ARM, Intel) for better performance.

- RISC Architecture
Used in mobile phones, embedded systems (ARM, RISC-V).

- CISC Architecture
Used in desktop and laptop processors (Intel x86).

29.What is the difference between Von Neumann and Harvard architecture

| Feature | Von Neumann Architecture | Harvard Architecture |
| --- | --- | --- |
| Memory | Single memory for data & instructions | Separate memory for data & instructions |
| Bus | One common bus | Separate buses |
| Speed | Slower (bus bottleneck) | Faster (parallel access) |
| Complexity | Simple design | More complex design |
| Cost | Low | Higher |
| Usage | Early computers, simple systems | Microcontrollers, DSPs |

30.What is the difference between RISC and CISC architectures

| Feature | RISC | CISC |
| --- | --- | --- |
| Full form | Reduced Instruction Set Computer | Complex Instruction Set Computer |
| Instruction set | Small & simple | Large & complex |
| Instruction length | Fixed | Variable |
| Execution | Most instructions execute in 1 clock cycle | Multiple clock cycles |
| Registers | Large number | Fewer |
| Pipelining | Easy | Difficult |
| Power | Low | Higher |
| Usage | ARM, RISC-V (mobiles, embedded) | Intel x86 (PCs, laptops) |

31. Write asynchronous code and synchronous code for any sequential circuit

Code for d flipflop

Synchronous :

always @(posedge clk) begin

if (rst)

Q <= 1'b0;   // reset happens on clock edge

else

Q <= D;

end

Asynchronous :

always @(posedge clk or posedge rst) begin

if (rst)

Q <= 1'b0;   // reset happens immediately

else

Q <= D;

End

( Reset puts the circuit in a known initial state, Essential for FSMs, counters, registers)

32. What is the always block and what is a sensitivity list in verilog

always block (Verilog):
The always block is used to describe hardware behavior that executes whenever a specified signal changes.

Sensitivity list:
A sensitivity list is the list of signals that trigger the execution of an always block.

Examples:

- always @(*) → runs when any input changes (combinational logic)

- always @(posedge clk) → runs on clock edge (sequential logic)

33. What flip-flops do?

Flip-flops are clock-controlled storage elements used to store and synchronize one bit of data

34. What is the verification process in verilog?

Verification process in Verilog:
The verification process in Verilog is used to check whether the design works correctly by simulating it with a testbench before hardware implementation.

Simple steps:

- Write RTL design (DUT)

- Create a testbench (no ports)

- Apply test inputs (stimulus)

- Simulate the design

- Observe and compare outputs with expected results

35. What is RLT design?

RTL design is the process of describing digital hardware in terms of registers and the transfer of data between them, using Verilog or VHDL.

36. Difference between RTL and Behavioral Model

RTL Model

- Describes how data moves between registers

- Uses clock, registers, and combinational logic

- Used for chip design

Behavioral Model

- Describes what the system does

- Uses high-level constructs (if, for, delays) …….(You write logic using if-else, loops, tasks, delays, without worrying about registers, clocks, or gates.)

- Used for simulation and verification

37 . What does the always block do in verilog?

The always block is used to define the behavior of hardware logic by executing a set of statements whenever signals in its sensitivity list change.

38. Implement different gates using 2:1 MUX.

1️⃣ NOT gate

assign Y = A ? 1'b0 : 1'b1;

2️⃣ AND gate

assign Y = A ? B : 1'b0;

3️⃣ OR gate

assign Y = A ? 1'b1 : B;

4️⃣ NAND gate

assign Y = A ? ~B : 1'b1;

5️⃣ NOR gate

assign Y = A ? 1'b0 : ~B;

6️⃣ XOR gate

assign Y = A ? ~B : B;

# Design a simple 32-bit ALU

- 00 → ADD

- 01 → SUB

- 10 → AND

- 11 → OR

Verilog Code: 32-bit ALU

module alu_32bit (

input  wire [31:0] A,

input  wire [31:0] B,

input  wire [1:0]  ALU_Sel,

output reg  [31:0] ALU_Out,

output wire        Zero

);

always @(*) begin

case (ALU_Sel)

2'b00: ALU_Out = A + B;   // Addition

2'b01: ALU_Out = A - B;   // Subtraction

2'b10: ALU_Out = A & B;   // AND

2'b11: ALU_Out = A | B;   // OR

default: ALU_Out = 32'b0;

endcase

end

# 2:1 Multiplexer truthtable

Inputs:

- I0 – input 0

- I1 – input 1

- S – select line

- Y – output

Simple rule (easy to remember):

- S = 0 → Y = I0

- S = 1 → Y = I1