CUBESAT FPGA CONVERGENCE

12-Subsystem Complete Mapping

Comprehensive Module Division, Requirements Traceability & Development Workflow

Project: Ani2204/FPGA-Convergence   |   Status: Ready for Implementation   |   Revision: 1.0   |   Date: 2026-04-02

# PART 1: Quick Reference Tables

## Table 1A — Subsystem Overview (12 Subsystems)

| ID | Subsystem Name | Category | Req Count | New RTL Files | Clock Domain | BRAM (B) | DSP48 | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS1 | IMU Sensor Interface (SPI) | Sensor | 1 | 3 | clk_100mhz | 256 | 0 | Design Phase |
| CS2 | Magnetometer Interface (I2C) | Sensor | 1 | 3 | sys_clk | 128 | 0 | Design Phase |
| CS3 | Sun Sensor ADC Interface | Sensor | 1 | 3 | clk_100mhz | 64 | 0 | Design Phase |
| CS4 | Quaternion Propagator | Estimation | 1 | 3 | sys_clk | 96 | 3 | Design Phase |
| CS5 | Extended Kalman Filter (7-State) | Estimation | 2 | 5 | sys_clk | 512 | 6 | Design Phase |
| CS6 | PD Attitude Control Law | Control | 1 | 3 | sys_clk | 64 | 2 | Design Phase |
| CS7 | RW & MTQ Drivers | Actuation | 2 | 4 | sys_clk | 128 | 1 | Design Phase |
| CS8 | ADCS Mode FSM & Health Monitor | Management | 3 | 6 | sys_clk | 3,072 | 0 | Design Phase |
| CS9 | Orbit Propagator (SGP4-Lite) | Estimation | 7 | 7 | sys_clk | 1,536 | 8 | Design Phase |
| CS10 | Laser Pointing FSM | Management | 10 | 10 | sys_clk | 512 | 2 | Design Phase |
| CS11 | Telemetry Encoder & Multiplexer | Communication | 8 | 9 | sys_clk | 768 | 0 | Design Phase |
| CS12 | System Integration & Clock Dist. | System | — | 7 | sys_clk / clk_100mhz | 256 | 0 | Design Phase |
| TOTAL |  |  | 37 | 63 | — | 7,392 | 22 | — |

## Table 1B — Existing RTL Files Allocation (20 Files)

| RTL File | Purpose | Primary Subsystem(s) | Secondary Users | Reuse Factor |
| --- | --- | --- | --- | --- |
| spi_master.sv | SPI protocol master | CS1, CS3, CS7 | IMU, sun sensor, RW | 3x |
| i2c_master.sv | I2C protocol master | CS2 | Magnetometer | 1x |
| adc_interface.sv | ADC controller | CS3 | Sun sensor | 1x |
| cordic.sv | CORDIC trigonometric | CS4, CS5, CS9 | Math library | 3x |
| sqrt.sv | Square root calculator | CS4, CS5, CS9 | Math library | 3x |
| fp_divider.sv | Fixed-point divider | CS4, CS5, CS9 | Math library | 3x |
| pid_controller.sv | PID control | CS6, CS10 | Attitude & laser | 2x |
| pwm_gen.sv | PWM generator | CS7, CS10 | MTQ & laser mod | 2x |
| stepper_driver.sv | Stepper motor | CS10 | Laser gimbal | 1x |
| lpf.sv | Low-pass filter | CS10 | Signal filtering | 1x |
| uart_controller.sv | UART interface | CS11 | Telemetry downlink | 1x |
| crc_calc.sv | CRC calculator | CS11 | Frame validation | 1x |
| mavlink_parser.sv | MAVLink protocol | CS11 | Command decoding | 1x |
| async_fifo.sv | Async FIFO buffer | CS11 | Clock domain bridging | 1x |
| clk_divider.sv | Clock divider | CS12 | System clock tree | 1x |
| tick_gen.sv | Tick/strobe generator | CS1, CS2, CS12 | CE generation | 3x |
| synchronizer.sv | CDC synchronizer | CS1, CS2, CS8, CS12 | Clock crossing | 4x |
| debouncer.sv | Input debouncer | CS2, CS8 | Signal conditioning | 2x |
| edge_detect.sv | Edge detector | CS1, CS3, CS8 | Event detection | 3x |
| kalman_1v.sv | Kalman filter template | CS5 | State estimation | 1x |

## Table 1C — Requirements-to-Subsystem Mapping (37 Requirements)

### ADCS Requirements (12 total) → CS1–CS8

| Req ID | Requirement Title | Subsystem | Acceptance Criteria (Brief) |
| --- | --- | --- | --- |
| CS-ADCS-001 | IMU Sensor Interface — SPI Read | CS1 | SPI @ 10MHz, 20µs transaction, ≥99.9% CRC pass, raw output LSB/°/s |
| CS-ADCS-002 | Magnetometer Interface — I2C Read | CS2 | I2C @ 400kHz, 50µs per axis, ACK received within 1 clock period |
| CS-ADCS-003 | Sun Sensor ADC Interface — SPI Read | CS3 | 4 channels within 10ms, ≥12 bits, <0.1% crosstalk, presence flag |
| CS-ADCS-004 | Attitude Quaternion Propagation | CS4 | q(k+1) = q(k) + 0.5×q(k)⊗ω×Δt, norm 1±0.001, drift <0.01° over 60s |
| CS-ADCS-005 | Extended Kalman Filter — State Update | CS5 | 7-state (q, bx, by, bz), predict+update within 8ms, error ≤0.5° (3σ) |
| CS-ADCS-006 | EKF Covariance Matrix Management | CS5 | 7×7 P matrix, Joseph-form update, positive-definite check |
| CS-ADCS-007 | PD Attitude Control Law | CS6 | τ = -Kp×q_err - Kd×ω @ 1kHz, saturation ±10 mNm, error <0.5° |
| CS-ADCS-008 | Reaction Wheel Motor Driver Command | CS7 | SPI commands (±6000 RPM), @ 1kHz, fault status read @ 10Hz |
| CS-ADCS-009 | Magnetorquer PWM Drive | CS7 | 3-axis PWM @ 10kHz, ≥12-bit resolution, <1% coupling, SAFE blanking |
| CS-ADCS-010 | ADCS Mode Management — State Machine | CS8 | 5-state FSM: BOOT→DETUMBLE→COARSE_POINT→FINE_POINT→SAFE |
| CS-ADCS-011 | ADCS Health Monitoring & Fault Detection | CS8 | SPI/I2C timeout >5ms, IMU overflow >500°/s, EKF divergence flags |
| CS-ADCS-012 | ADCS Data Logging to BRAM | CS8 | 256×96B circular buffer @ 100Hz, dual-port for telemetry |

### Orbit Requirements (7 total) → CS9

| Req ID | Requirement Title | Subsystem | Acceptance Criteria (Brief) |
| --- | --- | --- | --- |
| CS-ORB-001 | TLE Input & Storage | CS9 | 69-char TLE lines, checksum validation, survives soft reset |
| CS-ORB-002 | SGP4-Lite Propagation Engine | CS9 | ECI pos/vel, ±8000km / ±10km/s, ≤5km accuracy over 24h |
| CS-ORB-003 | Position Output Format & LVLH Conversion | CS9 | LVLH matrix from pos+vel, orbital elements (a,e,i,RAAN,ω,ν), altitude |
| CS-ORB-004 | Time Reference & Epoch Management | CS9 | MET counter @ 1Hz, 32-bit, loadable via AXI4-Lite, GPS sync |
| CS-ORB-005 | Inter-Satellite Relative Position | CS9 | ΔR_12, ΔR_13 (ECI), ΔV_12, ΔV_13, separation distance |
| CS-ORB-006 | Ground Track & Contact Window Prediction | CS9 | Geodetic lat/lon ±0.1°, AOS prediction ≤30s accuracy |
| CS-ORB-007 | Orbit Propagator Health & Validity | CS9 | PROPAGATOR_VALID flag, overflow detection, TLE age tracking |

### Laser FSM Requirements (10 total) → CS10

| Req ID | Requirement Title | Subsystem | Acceptance Criteria (Brief) |
| --- | --- | --- | --- |
| CS-LSR-001 | Laser Pointing FSM — State Definition | CS10 | 6 states: IDLE, SEARCH, ACQUIRE, TRACK, COMM, FAULT; no dead states |
| CS-LSR-002 | IDLE State Behavior | CS10 | Home position (0°,0°), modulator disabled, await START or trigger |
| CS-LSR-003 | SEARCH State Behavior | CS10 | Raster scan ±10°az×±5°el, boustrophedon pattern, ≤0.5° step |
| CS-LSR-004 | ACQUIRE State Behavior | CS10 | Spiral ±2° reducing 50%/pass, peak-hold, convergence <0.1° in ≤5s |
| CS-LSR-005 | TRACK State Behavior | CS10 | PD control @ 100Hz, RMS error <0.05°, transition when error <0.02° |
| CS-LSR-006 | COMM State Behavior | CS10 | Modulator enable, ISL 8-bit data @ 100MHz, maintain <0.05° RMS |
| CS-LSR-007 | FAULT State Behavior & Recovery | CS10 | Safe all outputs, log fault (8 FIFO), manual CLEAR command only |
| CS-LSR-008 | Gimbal Stepper Motor Control | CS10 | 2-axis (az/el), ≥2µs pulse, 200kHz max, 400 steps/°, position tracking |
| CS-LSR-009 | Gimbal Pointing Command Interface | CS10 | Absolute (Q8.8°) + relative (steps), command priority, ≤1ms latency |
| CS-LSR-010 | Optical Signal Strength Monitoring | CS10 | 10kHz sampling, ≥12-bit, configurable threshold, rolling average |

### Telemetry Requirements (8 total) → CS11

| Req ID | Requirement Title | Subsystem | Acceptance Criteria (Brief) |
| --- | --- | --- | --- |
| CS-TLM-001 | Telemetry Frame Format | CS11 | SYNC(4B) ¦ APID(2B) ¦ SEQ(2B) ¦ LEN(2B) ¦ TS(4B) ¦ PAYLOAD(≤128B) ¦ CRC16(2B) |
| CS-TLM-002 | Telemetry Encoding — UART Downlink | CS11 | Baud 9600–115200, 8N1, ≤frame_length×10/baud + 500µs latency |
| CS-TLM-003 | Telemetry Payload — ADCS Packet | CS11 | q[4], ω[3], mode, faults, τ[3], RW speeds[3], MTQ duty[3] = 44B |
| CS-TLM-004 | Telemetry Payload — Orbit Packet | CS11 | ECI pos[3], vel[3], alt, lat, lon, eclipse, contact, AOS = 47B |
| CS-TLM-005 | Telemetry Payload — Laser/ISL Packet | CS11 | FSM state, gimbal az/el, signal strength, pointing error, fault = 20B |
| CS-TLM-006 | Telemetry Multiplexer & Arbiter | CS11 | 1s superframe: HK@0ms, ADCS@100ms, ORBIT@200ms, LASER@300ms |
| CS-TLM-007 | Housekeeping Telemetry Packet | CS11 | uptime, FPGA temp (XADC), VCCINT, VCCO, subsystem health = 18B |
| CS-TLM-008 | Ground Command Reception & Decoding | CS11 | SYNC(4B) ¦ APID(2B) ¦ CMD_CODE(1B) ¦ DATA(≤16B) ¦ CRC16(2B) |

# PART 2: Detailed Subsystem Specifications

CS1: IMU Sensor Interface (SPI Read)   [Sensor]

Requirements: CS-ADCS-001  |  Clock Domain: clk_100mhz → sys_clk  |  BRAM: 256 B  |  DSP48: 0  |  Timing: 20 µs (SPI) + 2 µs (CDC)

Functional Description:

Reads 3-axis accelerometer, gyroscope, and magnetometer data from MPU-9250 over SPI @ 10 MHz SCLK. Implements CRC/parity checking (≥99.9% frame success). Transaction must complete within 20 µs. Raw units: gyro 131 LSB/°/s @ ±250°/s, accel 16384 LSB/g @ ±2g.

Existing RTL Files:

- spi_master.sv — SPI protocol engine

- tick_gen.sv — 100 Hz strobe generation

- synchronizer.sv — CDC from clk_100mhz to sys_clk

New RTL to Create:

- imu_controller.sv — IMU SPI transaction sequencer, state machine

- imu_data_handler.sv — Raw data extraction, 9-axis buffering

- spi_imu_wrapper.sv — Top-level wrapper

Interface Signals:

Input:  clk_100mhz, sys_clk, rst_n, spi_sclk, spi_mosi, spi_miso

Output: imu_data_valid, imu_accel[3], imu_gyro[3], imu_mag[3], crc_pass

CS2: Magnetometer Interface (I2C Read)   [Sensor]

Requirements: CS-ADCS-002  |  Clock Domain: sys_clk  |  BRAM: 128 B  |  DSP48: 0  |  Timing: 50 µs per axis × 3 = 150 µs

Functional Description:

Reads 3-axis magnetic field data from magnetometer via I2C @ 400 kHz Fast Mode. Implements ACK monitoring; NACK within 1 I2C clock period triggers FAULT_MAG flag. 3-byte per-axis read, ≤1 ms total per 100 Hz cycle. 13-bit ADC output scalable to µT via coefficient register.

Existing RTL Files:

- i2c_master.sv — I2C protocol engine, 400 kHz

- debouncer.sv — Input line debouncing

- synchronizer.sv — CDC for fault signals

New RTL to Create:

- i2c_mag_controller.sv — I2C read sequencing, address/data handling

- mag_fault_detector.sv — ACK/NACK timeout detection

- i2c_mag_wrapper.sv — Top-level wrapper

Interface Signals:

Input:  sys_clk, rst_n, i2c_sda, i2c_scl

Output: mag_data[3], mag_valid, mag_fault, mag_age_ms

CS3: Sun Sensor ADC Interface   [Sensor]

Requirements: CS-ADCS-003  |  Clock Domain: clk_100mhz → sys_clk  |  BRAM: 64 B  |  DSP48: 0  |  Timing: 10 ms for all 4 channels

Functional Description:

Reads 4-channel sun sensor photodiode signals via SPI-connected 12-bit ADC, sharing IMU SPI bus (multiplexed via CS_N). Resolution ≥12 bits, no missing codes across 0–3.3V. <0.1% FS crosstalk via independent CS_N sequencing. Sun-presence flag asserted when any channel > configurable threshold (default 10% FS = ~409 counts).

Existing RTL Files:

- spi_master.sv — Shared SPI, same 10 MHz as IMU

- adc_interface.sv — ADC read protocol, 12-bit

- edge_detect.sv — Channel change detection

New RTL to Create:

- sun_sensor_adc.sv — 4-channel sequencer, CS_N multiplexing

- adc_multiplexer.sv — Channel selection logic, timing

- sun_presence_detector.sv — Threshold compare, flag generation

Interface Signals:

Input:  clk_100mhz, sys_clk, rst_n, spi_mosi, spi_miso, adc_sclk, adc_cs_n

Output: sun_channel[4], sun_valid, sun_present, sun_presence_age_ms

CS4: Quaternion Propagator   [Estimation]

Requirements: CS-ADCS-004  |  Clock Domain: sys_clk  |  BRAM: 96 B  |  DSP48: 3  |  Timing: 5 ms execution

Functional Description:

Propagates spacecraft attitude quaternion using gyroscope angular rate integration. Implements q(k+1) = q(k) + 0.5 × q(k) ⊗ ω × Δt in Q2.46 fixed-point arithmetic. Norm constraint: |q| = 1 ± 0.001 maintained via normalization each cycle. Drift <0.01° over 60 seconds free-run (no EKF correction).

Existing RTL Files:

- cordic.sv — Quaternion magnitude computation

- sqrt.sv — Norm normalization via square root

- fp_divider.sv — Fixed-point division for normalization

New RTL to Create:

- quat_propagator.sv — Main propagation pipeline: multiply, accumulate

- quaternion_math.sv — Quaternion multiplication, conjugate operations

- norm_checker.sv — Magnitude check, out-of-range detection, flags

Interface Signals:

Input:  sys_clk, rst_n, ce_100hz, omega_x, omega_y, omega_z, gyro_valid

Output: q_out[4], q_norm, q_norm_valid, norm_err_flag, quat_ready

CS5: Extended Kalman Filter (7-State)   [Estimation]

Requirements: CS-ADCS-005, CS-ADCS-006  |  Clock Domain: sys_clk  |  BRAM: 512 B  |  DSP48: 6  |  Timing: 8 ms execution

Functional Description:

Implements 7-state EKF fusing IMU and magnetometer for attitude estimation. State vector: [q0, q1, q2, q3, bx, by, bz] where q = quaternion, b = gyro bias. Predict step: constant-velocity model. Update step via Joseph-form covariance update. Steady-state error ≤0.5° (3σ). Gyro bias convergence to ±0.01°/s within 60s. Divergence watchdog: resets to last-known-good on 3 consecutive 5σ innovations.

Existing RTL Files:

- kalman_1v.sv — Single-vehicle EKF template

- cordic.sv — Trigonometric functions for H matrix

- sqrt.sv — Covariance square root for Joseph form

- fp_divider.sv — Matrix inversion, Kalman gain division

New RTL to Create:

- ekf_core.sv — Main EKF orchestrator, cycle manager

- ekf_predict.sv — State + covariance prediction step

- ekf_update.sv — Measurement update, innovation calculation

- ekf_covariance.sv — Joseph-form covariance update, PD check

- ekf_matrix_ops.sv — Utility: matrix multiply, transpose, inversion

Interface Signals:

Input:  sys_clk, rst_n, ce_100hz, q_prior[4], gyro_bias[3], accel[3], mag[3], measurement_valid

Output: q_est[4], bias_est[3], P_diag[7], ekf_valid, divergence_flag, innovation[7]

CS6: PD Attitude Control Law   [Control]

Requirements: CS-ADCS-007  |  Clock Domain: sys_clk  |  BRAM: 64 B  |  DSP48: 2  |  Timing: 1 ms execution

Functional Description:

Implements PD control law: τ = −Kp × q_err(1:3) − Kd × ω, executing @ 1 kHz. Output torque saturated to ±10 mNm per axis. Steady-state pointing error <0.5° (3σ) under 1e-6 N·m disturbance. Kp and Kd gains loadable via AXI4-Lite; changes effective within 1 control cycle.

Existing RTL Files:

- pid_controller.sv — PID template, reusable for PD subset

New RTL to Create:

- pd_controller_quaternion.sv — Quaternion error computation, PD law

- torque_saturation.sv — ±10 mNm saturation + flag generation

- control_law_engine.sv — Integrates PD + saturation, output multiplexer

Interface Signals:

Input:  sys_clk, rst_n, ce_1khz, q_err[4], omega[3], Kp_coeff, Kd_coeff, axi_gain_write

Output: torque_cmd[3], saturation_flag, sat_count, ctrl_valid

CS7: Reaction Wheel & Magnetorquer Drivers   [Actuation]

Requirements: CS-ADCS-008, CS-ADCS-009  |  Clock Domain: sys_clk (1 kHz), clk_100mhz (PWM)  |  BRAM: 128 B  |  DSP48: 1  |  Timing: 1 ms (SPI RW + MTQ PWM update)

Functional Description:

RW SPI Driver: Translates PD control torque commands into 3 independent SPI motor driver commands @ 1 kHz. Speed range: ±6000 RPM (16-bit signed, 1 LSB = 0.2 RPM). MTQ PWM Drive: 3-axis PWM @ 10 kHz carrier, ≥12-bit resolution. Cross-axis coupling <1%. SAFE mode blanking within 1 control cycle.

Existing RTL Files:

- spi_master.sv — SPI for 3× RW drivers

- pwm_gen.sv — PWM generators, 3× instances

New RTL to Create:

- rw_spi_driver.sv — 3× SPI RW sequencer, CS_N multiplexing

- magnetorquer_pwm.sv — 3× PWM instances, 10 kHz carrier

- actuator_command_arbiter.sv — Routes τ_cmd → RW speed + MTQ duty

- fault_status_monitor.sv — RW fault polling, MTQ current monitoring

Interface Signals:

Input:  sys_clk, clk_100mhz, rst_n, ce_1khz, torque_cmd[3], safe_mode

Output: rw_sclk, rw_mosi[3], rw_cs_n[3], rw_miso[3], mtq_pwm[3], rw_fault[3]

CS8: ADCS Mode FSM & Health Monitor   [Management]

Requirements: CS-ADCS-010, CS-ADCS-011, CS-ADCS-012  |  Clock Domain: sys_clk  |  BRAM: 3,072 B  |  DSP48: 0  |  Timing: <1 ms FSM + <1 ms logging

Functional Description:

Mode FSM: 5-state machine BOOT → DETUMBLE → COARSE_POINT → FINE_POINT → SAFE. Any FAULT transitions to SAFE within 2 ms. Health Monitor: continuously monitors SPI/I2C timeout >5ms, IMU rate >500°/s, quaternion norm outside 1±0.01 for >3 cycles. Data Logging: 256×96B circular BRAM buffer @ 100 Hz, dual-port for simultaneous telemetry read.

Existing RTL Files:

- edge_detect.sv — State transition edge detection

- synchronizer.sv — CDC for fault signals

- debouncer.sv — Fault signal debouncing

New RTL to Create:

- adcs_fsm_controller.sv — Main FSM orchestrator

- adcs_state_machine.sv — State transition logic, timer counters

- fault_detector.sv — Multi-fault detection: timeout, overflow, divergence

- fault_logger.sv — FIFO fault log: code + timestamp

- adcs_health_monitor.sv — Aggregates health signals

- bram_circular_buffer.sv — Dual-port 256×96B circular log

Interface Signals:

Input:  sys_clk, rst_n, ce_100hz, imu_valid, mag_valid, ekf_valid, q_norm, omega_max, fault_trigger[8]

Output: adcs_mode[3], mode_valid, health_flag, fault_flags[8], bram_log_addr, bram_log_data

CS9: Orbit Propagator (SGP4-Lite)   [Estimation]

Requirements: CS-ORB-001 through CS-ORB-007  |  Clock Domain: sys_clk  |  BRAM: 1,536 B  |  DSP48: 8  |  Timing: 50ms propagation + 10ms LVLH + 5ms ground track = 65ms

Functional Description:

Full 7-requirement orbit stack. TLE Input: accepts 69-char TLE lines via AXI4-Lite, validates checksums, epoch parsed to MJD. SGP4-Lite Engine: computes ECI position + velocity @ 1 Hz, ≤5 km accuracy over 24h vs STK. LVLH Conversion: R_ECI_to_LVLH matrix, classical orbital elements, WGS84 altitude. MET Counter: 32-bit @ 1 Hz, GPS-syncable. Relative Position: ΔR_ij, ΔV_ij per ISL. Ground Track: geodetic lat/lon ±0.1°, AOS prediction ≤30s.

Existing RTL Files:

- cordic.sv — Sine/cosine for orbital mechanics

- sqrt.sv — Square root for distance calculations

- fp_divider.sv — Fixed-point division for orbital elements

New RTL to Create:

- tle_parser.sv — TLE validation, checksum, epoch extraction

- sgp4_propagator_core.sv — SGP4-Lite engine: ECI propagation

- lvlh_converter.sv — ECI → LVLH frame transformation

- ground_track_calculator.sv — WGS84 lat/lon computation

- contact_window_predictor.sv — AOS/LOS prediction, elevation check

- orbit_health_monitor.sv — Validity flags, overflow detection

- orbit_state_manager.sv — BRAM output bank, MET counter, epoch manager

Interface Signals:

Input:  sys_clk, rst_n, ce_1hz, tle_input[138], tle_write, met_load_value, gps_lla_input[3]

Output: eci_pos[3], eci_vel[3], orbital_elements[6], altitude_m, latitude_rad, longitude_rad, eclipse_flag, propagator_valid

CS10: Laser Pointing FSM   [Management]

Requirements: CS-LSR-001 through CS-LSR-010  |  Clock Domain: sys_clk (100 Hz), clk_100mhz (modulator)  |  BRAM: 512 B  |  DSP48: 2  |  Timing: 10 ms per 100 Hz cycle

Functional Description:

6-state laser acquisition and tracking FSM: IDLE → SEARCH (boustrophedon raster ±10°az×±5°el, ≤0.5° step) → ACQUIRE (spiral ±2°, peak-hold, convergence <0.1° in ≤5s) → TRACK (PD @ 100Hz, RMS <0.05°) → COMM (modulator enabled, ISL 8-bit @ 100MHz) → FAULT. Gimbal stepper: 2-axis STEP/DIR, 400 steps/°, 200 kHz max, trapezoidal acceleration. Signal monitoring: 10 kHz ADC, 12-bit, rolling 16-sample average.

Existing RTL Files:

- pid_controller.sv — PD for TRACK/ACQUIRE pointing control

- pwm_gen.sv — Laser modulator PWM

- stepper_driver.sv — Gimbal STEP/DIR pulses

- lpf.sv — Low-pass filter on signal strength input

New RTL to Create:

- laser_fsm.sv — 6-state machine top-level orchestrator

- gimbal_controller.sv — Gimbal command generation, position calc

- laser_state_machine.sv — State transitions, guards, timers

- raster_scan_engine.sv — SEARCH boustrophedon raster generator

- spiral_refinement.sv — ACQUIRE spiral pattern, peak-hold

- peak_hold_detector.sv — Signal strength peak tracking

- gimbal_stepper_control.sv — STEP/DIR pulse generation, homing

- signal_monitor.sv — 10 kHz signal sampling, rolling average

- laser_modulator.sv — Modulator enable, ISL data mux

- laser_fault_handler.sv — Fault detection and FAULT state handler

Interface Signals:

Input:  sys_clk, clk_100mhz, rst_n, ce_100hz, signal_strength_adc, gimbal_cmd_abs, gimbal_cmd_valid, safe_mode

Output: laser_fsm_state[3], gimbal_step[2], gimbal_dir[2], laser_modulator_en, isl_data[8], signal_strength, fault_code

CS11: Telemetry Encoder & Multiplexer   [Communication]

Requirements: CS-TLM-001 through CS-TLM-008  |  Clock Domain: sys_clk  |  BRAM: 768 B  |  DSP48: 0  |  Timing: <100 ms per frame transmission

Functional Description:

Full CCSDS-aligned frame builder: SYNC(4B)|APID(2B)|SEQ(2B)|LEN(2B)|TS(4B)|PAYLOAD(≤128B)|CRC16(2B). UART TX: 9600–115200 bps, 8N1, configurable inter-frame gap. 1-second superframe scheduler: HK@0ms, ADCS@100ms, ORBIT@200ms, LASER@300ms. ADCS packet 44B (APID 0x101), Orbit packet 47B (APID 0x102), Laser packet 20B (APID 0x103), HK packet 18B (APID 0x100). Command reception: UART RX → CRC check → route by APID to target subsystem via AXI4-Lite.

Existing RTL Files:

- uart_controller.sv — UART TX/RX @ configurable baud

- crc_calc.sv — CRC-16/CCITT-FALSE

- mavlink_parser.sv — MAVLink command parsing

- async_fifo.sv — UART RX → sys_clk FIFO, CDC

New RTL to Create:

- tlm_frame_builder.sv — Frame assembly: SYNC + APID + CRC

- tlm_arbiter.sv — 1 Hz superframe scheduling, slot arbiter

- tlm_encoder_adcs.sv — ADCS packet mux from log buffer

- tlm_encoder_orbit.sv — Orbit packet mux from propagator

- tlm_encoder_laser.sv — Laser packet mux from FSM

- tlm_encoder_hk.sv — HK packet: XADC, uptime, health

- command_decoder.sv — Command frame parsing, CRC check

- uart_tx_scheduler.sv — Baud rate FSM, inter-frame gap

- command_dispatcher.sv — Route commands to target subsystems

Interface Signals:

Input:  sys_clk, rst_n, ce_1hz, uart_rx, adcs_tlm_data[44], orbit_tlm_data[47], laser_tlm_data[20]

Output: uart_tx, tlm_valid, tlm_apid, tlm_seq, cmd_valid, cmd_apid, cmd_code, cmd_data[16]

CS12: System Integration & Clock Distribution   [System]

Requirements: System-level (all subsystems)  |  Clock Domain: sys_clk, clk_100mhz  |  BRAM: 256 B  |  DSP48: 0  |  Timing: N/A (infrastructure layer)

Functional Description:

Central clock manager: derives sys_clk, clk_100mhz, ce_1hz, ce_100hz, ce_1khz from 100 MHz reference. AXI4-Lite slave: full register map 0x0000–0x0BFF across all 12 subsystems. Reset controller: POR sequencing, configurable warm/cold soft reset (cold clears BRAM, warm preserves TLE/MET). XADC system monitor: 1 Hz temperature, VCCINT, VCCO readings; thermal throttle at >110°C. Top-level top_cubesat_mvp.sv instantiates all CS1–CS11 and all interconnects.

Existing RTL Files:

- clk_divider.sv — Frequency division for clock generation

- tick_gen.sv — CE strobe generation at multiple rates

- synchronizer.sv — CDC for cross-domain coordination

New RTL to Create:

- clk_manager.sv — Central clock tree, multiplexer, gating

- axi4_lite_slave.sv — AXI4-Lite protocol handler for all registers

- reset_controller.sv — POR sequencing, soft/cold/warm reset logic

- system_monitor.sv — XADC controller, temperature/voltage tracking

- power_monitor.sv — Per-subsystem power budget tracking, alarms

- resource_arbiter.sv — BRAM/DSP48 access coordination

- top_cubesat_mvp.sv — Top-level instantiation + interconnect

Interface Signals:

Input:  clk_ref_100mhz, rst_pbl_n, axi_clk, axi_rst_n, axi_awaddr, axi_wdata, axi_araddr

Output: sys_clk, clk_100mhz, ce_1hz, ce_100hz, ce_1khz, fpga_temp, vccint, vcco, thermal_alert

# PART 3: Development Workflow & Integration Checklist

## Phase 1: Per-Subsystem Development Workflow

### 1.1 Specification Review

- Read corresponding requirement(s) from specification files

- Extract key metrics: timing budget, accuracy, resource limits

- Identify input/output signals and protocols

- Document clock domains and CDC requirements

- List existing RTL files to reuse

### 1.2 RTL Architecture Design

- Create subsystem-level block diagram

- Define internal module interfaces (signals, bus widths)

- Allocate BRAM and DSP48 resources

- Identify testable boundaries

- Document fixed-point number formats

### 1.3 RTL Implementation

- Create new RTL files (list per subsystem)

- Implement each module with synthesizable SystemVerilog

- Add comments: timing constraints, state diagrams, algorithms

- Ensure consistent naming: subsystem_function.sv

- Include internal assertions for debugging

### 1.4 Simulation & Unit Testing

- Write standalone testbench for each new module

- Run functional simulation (ModelSim/VCS)

- Verify timing: setup/hold, clock-to-out

- Check fixed-point accuracy against golden reference

- Generate waveforms for manual inspection

### 1.5 Integration into CS12

- Add subsystem instantiation in top_cubesat_mvp.sv

- Connect clock/reset signals

- Connect AXI4-Lite register interface

- Route data signals to/from neighboring subsystems

- Add CDC synchronizers where needed

### 1.6 Synthesis & Place & Route

- Synthesize subsystem in isolation (Vivado)

- Check resource utilization

- Verify timing closure (worst-case path)

- Inspect synthesized RTL for optimization issues

- Generate utilization report

### 1.7 System-Level Simulation

- Instantiate full top module with all CS1–CS11

- Apply realistic input stimuli (sensor data, commands)

- Run for functional cycles (100–1000 steps)

- Verify data flow between subsystems

- Check for timing violations in cross-module paths

### 1.8 Documentation

- Create subsystem README: purpose, interfaces, usage

- Document register map (AXI4-Lite addresses)

- Add performance metrics: latency, throughput, resource usage

- Include troubleshooting guide for common issues

## Phase 2: Integration Checklist (System-Level)

### 2.1 Pre-Integration Review

- All 12 subsystems (CS1–CS11) synthesize individually without errors

- No timing violations in isolation

- BRAM utilization: sum of all subsystems < total available

- DSP48 utilization: sum of all subsystems < total available

- All external I/O (SPI, I2C, UART, PWM) mapped to FPGA pins

### 2.2 Clock Domain Integration

- Clock tree synthesizes without glitches

- All CDC paths protected by synchronizers

- Convergence engine strobes (1 Hz, 100 Hz, 1 kHz) verified

- Jitter on ce_1hz, ce_100hz measured <5% of cycle period

- Timing closure achieved on sys_clk and clk_100mhz

### 2.3 Interconnect Verification

- Data pathways: sensors → estimation → control → actuators verified

- Bus arbitration (if any shared buses) validated

- No combinatorial loops in high-speed paths

- All required handshake signals (valid/ready) implemented

- Backpressure handling tested (e.g., UART FIFO full)

### 2.4 AXI4-Lite Register Map Validation

- Register addresses assigned without conflicts

- All subsystems accessible via AXI4-Lite

- Read/write permissions correctly enforced

- Register reset values set appropriately

- Ground station software uses correct address offsets

### 2.5 Memory (BRAM) Allocation Verification

- Total BRAM used < device capacity (Xilinx Artix-7 XC7A35T = 45 KB available)

- Dual-port configurations for simultaneous access correctly instantiated

- Circular buffers (logging) overflow flags tested

- Write/read collisions prevented by CDC or arbitration

### 2.6 Arithmetic & Precision Validation

- Fixed-point overflow/underflow cases identified

- Q-format (signed/fractional bits) consistent across subsystems

- Saturation behavior documented and tested

- CORDIC / SQRT / DIV outputs compared against golden reference

### 2.7 Fault Injection & Recovery Testing

- Inject sensor faults (timeout, bad CRC) → verify FSM transitions to SAFE

- Inject command errors → verify error flags set, no system hang

- Power supply brown-out → verify graceful degradation

- Thermal threshold → verify throttling or shutdown

- Recovery procedures tested

### 2.8 Telemetry & Logging Validation

- All packets transmitted at correct intervals

- CRC-16 verified on ground

- Sequence counter increments properly

- Data alignment (no byte shifts)

- Ground station receives all packets without gaps

### 2.9 End-to-End Functional Test

- Apply simulated mission profile: pre-flight → orbital insertion → science ops

- Monitor all telemetry packets; verify no anomalies

- Inject simulated events: solar eclipse, contact window, laser acquisition

- Verify responses: mode transitions, control law outputs, ISL data

- Total simulation runtime ≥1 simulated mission orbit (~90 minutes)

### 2.10 Resource & Performance Report

- Measure worst-case execution time per subsystem

- Verify no deadline misses (100 Hz ADCS, 1 kHz control)

- Report BRAM utilization as % of total

- Report DSP48 utilization as % of total

- Identify performance bottlenecks for future optimization

# PART 4: Module Quick Reference Table

| CS ID | New Files | Key New File | Existing Files | Timing | BRAM (B) | DSP48 | Interfaces |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS1 | 3 | imu_controller.sv | 3 | 20µs | 256 | 0 | SPI 10MHz |
| CS2 | 3 | i2c_mag_controller.sv | 3 | 150µs | 128 | 0 | I2C 400kHz |
| CS3 | 3 | sun_sensor_adc.sv | 3 | 10ms | 64 | 0 | SPI 10MHz |
| CS4 | 3 | quat_propagator.sv | 3 | 5ms | 96 | 3 | 100Hz strobe |
| CS5 | 5 | ekf_core.sv | 4 | 8ms | 512 | 6 | 100Hz strobe |
| CS6 | 3 | pd_controller_quaternion.sv | 1 | 1ms | 64 | 2 | 1kHz strobe |
| CS7 | 4 | rw_spi_driver.sv | 2 | 1ms | 128 | 1 | SPI, PWM |
| CS8 | 6 | adcs_fsm_controller.sv | 3 | 1ms | 3,072 | 0 | 100Hz strobe |
| CS9 | 7 | sgp4_propagator_core.sv | 3 | 50ms | 1,536 | 8 | 1Hz strobe |
| CS10 | 10 | laser_fsm.sv | 4 | 10ms | 512 | 2 | 100Hz strobe |
| CS11 | 9 | tlm_frame_builder.sv | 4 | — | 768 | 0 | UART, AXI4-Lite |
| CS12 | 7 | top_cubesat_mvp.sv | 3 | — | 256 | 0 | Global interconnect |
| TOTAL |  | 63 new files | 20 | 7,392 B |  | 22 |  |

# PART 5: Implementation Priority & Timeline

## Phase A: Sensor & Estimation Layer (Weeks 1–4)

Foundation for all ADCS functionality. Sensor inputs and state estimation must be in place before control can execute.

- CS1 — IMU SPI: Foundation for all ADCS (gyro, accel, mag)

- CS2 — MAG I2C: Parallel to CS1

- CS3 — SUN ADC: Parallel to CS1/CS2

- CS4 — Quaternion Propagator: After CS1 (depends on gyro data)

- CS9 — Orbit Propagator: Independent of ADCS, can start in Phase A

- CS10 — Laser FSM: Independent, can start in Phase A

- CS5 — EKF: After CS2/CS3/CS4

## Phase B: Control & Actuation Layer (Weeks 5–6)

- CS6 — PD Control Law: After CS5 (uses EKF state estimate)

- CS7 — RW & MTQ Actuators: After CS6

## Phase C: Management & Science (Weeks 7–8)

- CS8 — ADCS FSM & Health Monitor: After CS1–CS7

## Phase D: Communication & Integration (Weeks 9–10)

- CS11 — Telemetry Encoder & Multiplexer: After CS1–CS10 ready

- CS12 — System Top-Level Integration: After all subsystems ready

# PART 6: Success Criteria

- All 37 requirements mapped to unique subsystems

- All 63 new RTL files created and synthesized

- All 20 existing RTL files reused without modification

- Total BRAM ≤ 8 KB (target: XC7A35T = 45 KB available)

- Total DSP48 ≤ 50 (XC7A35T = 90 available)

- All timing constraints met (100 Hz ADCS, 1 kHz control, 1 Hz telemetry)

- End-to-end system simulation passes for 1 mission orbit (~90 min simulated)

- Register map complete and ground station compatible

- All modules documented with testbenches and README files

END OF DOCUMENT

Generated for: Ani2204/FPGA-Convergence  |  Date: 2026-04-02  |  Status: Ready for Implementation  |  Revision: 1.0