# Convergence MVP — Internal Bus Specification

All internal interconnect buses: position (576b), velocity (576b), IMU (576b CDC), formation command (480b+), APF (576b), telemetry TDM, AXI4-Lite config, CE strobes, fault/debug.

Protocol coverage: registered-strobe, valid-only push, Gray-coded async FIFO CDC, AXI4-Lite handshake, TDM mux, combinational level, and 1-cycle CE broadcast.

## 1.  Master Bus Overview

All nine buses with total width, protocol, data format, update rate, source-to-destination routing, latency, and implementation notes.

| Bus Name | Total Width | Protocol | Data Format | Rate | Source → Destination | Latency | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pos_bus | 576 bits (6×3×32b) | Strobed (DATA_READY + valid[5:0]) | Q10.22 per axis (±1023 m) Signed 32-bit | 10 Hz (GPS gate) | 3×UAV EKF + 3×CS SGP4 ↓ MVK filter + APF + Arbiter | < 1 cycle (comb wire) | Packed array: pos_bus[v][2:0][31:0]. CS uses ECI→NED converted position. All in sys_clk domain. Zero registered latency. |
| vel_bus | 576 bits (6×3×32b) | Strobed (DATA_READY + valid[5:0]) | Q4.28 per axis (±7.99 m/s) Signed 32-bit | 10 Hz (EKF output) | 3×UAV EKF + 3×CS Orbit ↓ MVK filter + Formation ctrl | < 1 cycle (comb wire) | Same structure as pos_bus. vel_bus[v][2:0][31:0]. CS velocity from SGP4 propagator. UAV velocity from 9-state EKF. |
| imu_bus | 6×96b = 576b (6×3ax×32b) | PUSH-valid (CDC via async FIFO) | Q4.28 per axis ax/ay/az (m/s²) gx/gy/gz (rad/s) | 100 Hz (FIFO gate) | SPI masters (clk_100mhz domain) ↓ EKF predict step (sys_clk domain) | 2 FIFO cycles (Gray CDC) | CDC via Gray-coded async FIFO. 100MHz write-side, 50MHz read-side. Each vehicle has independent FIFO instance. Valid strobe asserted on sys_clk after FIFO read. |
| form_cmd_bus | 5×96b = 480b (5×3×32b) | Strobed (VALID_OUT + per-vehicle) | Q4.28 m/s per axis Signed 32-bit NED frame | 10 Hz (formation controller) | Formation controller ↓ Resource arbiter ↓ UAV PID setpoints | 1 cycle (registered) | 5 follower commands (leader V1 drives itself). Arbiter gates individual vehicle commands based on TDM slot and CA override. Combined with APF force before dispatch to UAV PID. |
| apf_force_bus | 6×96b = 576b (6×3×32b) | Strobed (THREAT_FLAGS +force_valid) | Q4.28 m/s per axis Velocity correction ±5 m/s clipped | 10 Hz (or on threat) | APF collision avoidance ↓ Resource arbiter ↓ UAV PID override | < 1ms (hard req) | Only asserted when THREAT_FLAGS[v]=1. CA override pre-empts formation commands within 1ms (CV-CA-001 hard deadline). Zero force when no threat. |
| tlm_bus | 6×256b = 1536b (muxed to 1×256b) | TDM mux (superframe cycle 1 Hz) | 50-byte CCSDS frames per slot Binary packed | 1 Hz (superframe) | All 6 subsystems ↓ TLM MUX ↓ Ground-station UART | Slot-based (up to 1 s) | 1-second superframe: each vehicle gets one 50-byte slot. TDM round-robin. CS telemetry: ADCS+orbit+laser. UAV telemetry: nav+FSM+health. CVG: score+threats+arbiter. |
| axi_cfg_bus | 32+12+4+2+1 = 51 active bits (full AXI5ch) | AXI4-Lite Handshake (valid/ready) | 32-bit data 12-bit addr 4-bit strobe | Async (on write or read) | GS cmd decoder ↓ AXI interconnect ↓ All register banks | AXI: 2-4 cycles (registered) | Standard AXI4-Lite 5-channel protocol. 9 slave banks (CS1-3, UAV1-3, CVG, SYS). 12-bit address = 4KB space per slave. Arbited in axi_lite_interconnect. |
| ce_bus | 5 bits (5 frequencies) | Level CE (1-cycle wide asserted) | 1-bit per rate 50MHz domain only | 1kHz/100Hz 50Hz/10Hz 1Hz | ce_strobe_gen ↓ All sys_clk submodules | 0 cycles (direct wire) | All CE strobes derived from single 26-bit counter on sys_clk. NOT clock domains — purely registered enables. ce_1khz used only by UAV inner PID loop; others shared globally. |
| fault_bus | 32 fault bits + 9 WDT kick bits | Async level (combinational OR tree) | Bitmask 32-bit + per-vehicle kick strobe | Async (event driven) | All subsystems ↓ Fault manager ↓ ESTOP output | < 1ms (CA path) | fault_flags[31:0] is combinational OR tree of all subsystem faults. ESTOP_N asserted within 1ms of CA watchdog miss. Other faults logged to BRAM. WDT kick strobes per vehicle refresh 500ms timers. |

## 2.  Bus Width Summary

Total active bit counts per bus. Position, velocity, and IMU buses each carry 576 bits (6 vehicles × 3 axes × 32-bit). Combined fleet-state bandwidth = ~1728 bits at 10 Hz.

| Bus | Total Width | Structure | Rate | Notes |
| --- | --- | --- | --- | --- |
| pos_bus | 576 bits | 6 veh × 3ax × 32b | 10 Hz | Q10.22 ±1023m each component |
| vel_bus | 576 bits | 6 veh × 3ax × 32b | 10 Hz | Q4.28 ±7.99 m/s each component |
| imu_bus | 576 bits | 6 veh × 6ax × 16b (packed as 6×96b) | 100 Hz | Q4.28 accel m/s² + gyro rad/s |
| form_cmd_bus | 480 bits core +96b error+32b aux | 5 fol × 3ax × 32b + err + flags | 10 Hz | Q4.28 velocity setpoints m/s |
| apf_force_bus | 576 bits | 6 veh × 3ax × 32b | 10 Hz or on threat | Q4.28 velocity correction m/s |
| tlm_bus | 7×256b=1792b mux (serial: 1 bit UART) | 50B/vehicle/s × 7 sources | 1 Hz per veh 7 Hz fleet | CCSDS binary frames |
| axi_cfg_bus | 51 active bits (5 AXI4-Lite ch) | 32b data+12b addr +4b strobe+flags | Async | AXI4-Lite write/read protocol |
| ce_bus | 5 bits | 5 independent 1-cycle CE strobes | 1k/100/50/10/1 Hz | sys_clk domain only, no CDC |
| fault_bus | 32b flags + 9b WDT + 1b estop | Combinational 32-bit OR tree | Async (continuous) | Combinational critical path for ESTOP |
| TOTAL (approx) | ~5200 active bits (all buses summed) | Dominant: 3×576b position+vel+IMU | varies | Dominated by position/velocity state buses |

## 3.  Position Bus & Velocity Bus — Signal Detail

Full signal-level breakdown of every slice of pos_bus[6][3][32] and vel_bus[6][3][32]. CS vehicles carry orbit-propagated NED position; UAV vehicles carry 9-state EKF NED position.

| Position Bus (pos_bus) — 576 bits — 6 vehicles × 3 axes × Q10.22 32-bit — 10 Hz DATA_READY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Signal / Slice | Bit Range | Format | Name | Description |
| pos_bus[0][0][31:0] | 576->543 | Q10.22 | CS1 px (North, m) | CubeSat 1 SGP4 → ECI→NED converted North position. Driven by sgp4_orbit_propagator at 1Hz, held until next update. Stale indicator in valid[0]. |
| pos_bus[0][1][31:0] | 542->511 | Q10.22 | CS1 py (East, m) | CubeSat 1 East position from orbit propagator. |
| pos_bus[0][2][31:0] | 510->479 | Q10.22 | CS1 pz (Down, m) | CubeSat 1 Down position (negative = altitude). NED convention. |
| pos_bus[1][0..2] | 478->383 | Q10.22 | CS2 position [px,py,pz] | Identical structure to CS1. Driven independently by CS2 orbit propagator. |
| pos_bus[2][0..2] | 382->287 | Q10.22 | CS3 position [px,py,pz] | Identical structure. CS3 orbit propagator drives this slice. |
| pos_bus[3][0][31:0] | 286->255 | Q10.22 | UAV1 px (North, m) | UAV1 9-state EKF output. Updated at 10Hz on GPS-gated DATA_READY strobe. |
| pos_bus[3][1][31:0] | 254->223 | Q10.22 | UAV1 py (East, m) | UAV1 East position. EKF fuses GPS + baro + IMU. |
| pos_bus[3][2][31:0] | 222->191 | Q10.22 | UAV1 pz (Down, m) | UAV1 Down position (NED; pz = -altitude). |
| pos_bus[4][0..2] | 190->95 | Q10.22 | UAV2 position [px,py,pz] | UAV2 9-state EKF position outputs. |
| pos_bus[5][0..2] | 94->0 | Q10.22 | UAV3 position [px,py,pz] | UAV3 9-state EKF position outputs. |
| pos_valid[5:0] | 6 bits | 1=valid | Per-vehicle valid flags | Bit=1 when vehicle position is fresh. CS bits updated at 1Hz (SGP4). UAV bits updated at 10Hz (EKF DATA_READY). Bit cleared on subsystem fault or ESTOP. |
| pos_DATA_READY | 1 bit | strobe | 10Hz master strobe | One-cycle strobe asserted when ALL valid[5:0]=1 in current cycle. Triggers MVK predict step. If any vehicle stale, strobe still asserts but MVK skips that vehicle's update. |
| Velocity Bus (vel_bus) — 576 bits — 6 vehicles × 3 axes × Q4.28 32-bit — 10 Hz DATA_READY |  |  |  |  |
| Signal / Slice | Bit Range | Format | Name | Description |
| vel_bus[0][0..2][31:0] | 576->479 | Q4.28 | CS1 velocity [vx,vy,vz] m/s | CubeSat 1 NED velocity from SGP4 orbit propagator. Orbital velocity ≈ 7.8 km/s (scaled or sub-sampled for formation relative velocity computations). |
| vel_bus[1][0..2] | 478->383 | Q4.28 | CS2 velocity | CS2 orbit velocity. |
| vel_bus[2][0..2] | 382->287 | Q4.28 | CS3 velocity | CS3 orbit velocity. |
| vel_bus[3][0][31:0] | 286->255 | Q4.28 | UAV1 vx (North, m/s) | UAV1 North velocity from 9-state EKF. GPS Doppler + IMU dead-reckoning. |
| vel_bus[3][1][31:0] | 254->223 | Q4.28 | UAV1 vy (East, m/s) | UAV1 East velocity. |
| vel_bus[3][2][31:0] | 222->191 | Q4.28 | UAV1 vz (Down, m/s) | UAV1 Down velocity (positive = descending in NED). |
| vel_bus[4][0..2] | 190->95 | Q4.28 | UAV2 velocity | UAV2 9-state EKF velocity. |
| vel_bus[5][0..2] | 94->0 | Q4.28 | UAV3 velocity | UAV3 9-state EKF velocity. |
| vel_valid[5:0] | 6 bits | 1=valid | Per-vehicle velocity valid | Mirrors pos_valid. Velocity updates arrive at same rate as position (GPS DATA_READY). |
| vel_DATA_READY | 1 bit | strobe | 10Hz master strobe | Same DATA_READY strobe as pos_bus. Velocity and position are always updated atomically in the same cycle. |

## 4.  Formation Command Bus — Signal Detail

All signals on the formation command bus: per-follower velocity setpoints, position errors, leader state (pos/vel/heading), geometry type, transition status, and scale factor.

| Formation Command Bus (form_cmd_bus) — 480+96+32=608b core — 10 Hz — Source: formation_controller |  |  |  |  |
| --- | --- | --- | --- | --- |
| Signal | Bits / Slice | Format | Name | Description |
| form_vel_cmd[1][0][31:0] | (f=1,axis=0) | Q4.28 | V2 velocity cmd vx | Formation controller command to follower V2 North velocity. Q4.28 m/s. APF can override via resource arbiter. |
| form_vel_cmd[1][1][31:0] | (f=1,axis=1) | Q4.28 | V2 velocity cmd vy | V2 East velocity setpoint from formation geometry computation. |
| form_vel_cmd[1][2][31:0] | (f=1,axis=2) | Q4.28 | V2 velocity cmd vz | V2 Down velocity (altitude hold term from formation z_f offset). |
| form_vel_cmd[2..5][0..2] | (f=2..5) | Q4.28 | V3-V6 commands | V3=UAV2, V4=V4(CS2), V5=V5(CS3), V6=V6(UAV3). Each 96 bits [vx,vy,vz]. |
| form_err[f=1..5][0..2] | 5×3×32b | Q10.22 | Per-follower position error | E_i = P_target_i − P_i in metres Q10.22. Feeds APF gate and telemetry. Not directly a control output but co-transmitted with form_vel_cmd. |
| form_cmd_valid[4:0] | 5 bits | 1=valid | Per-follower command valid | Individual valid bits. Cleared if follower position unknown or subsystem fault. Arbiter respects these: no command dispatched if valid=0. |
| form_geometry[1:0] | 2 bits | enum | Active formation type | 0=TRIANGLE 1=LINE 2=V_SHAPE 3=SQUARE. Latched from CVG_CTRL.FORM_TYPE. Changes trigger LERP transition. |
| form_in_transition | 1 bit | flag | Transition active flag | 1=LERP interpolation in progress. FORMATION_ACHIEVED inhibited during transition. Asserted for FORM_TRANS_STEPS × 100ms. |
| form_scale_active[31:0] | 32 bits | Q4.12 | Live scale factor | Current interpolated scale factor during caution zone scale-back. APF sets this when caution zone breach detected. |
| form_master_valid | 1 bit | strobe | Master dispatch valid | One-cycle strobe at 10Hz. All form_vel_cmd[f] are valid simultaneously. Arbiter captures on this strobe. |
| leader_pos[2:0][31:0] | 3×32b | Q10.22 | Leader (V1) NED position | Virtual leader position. Propagated at 10Hz from V1 (UAV1/CS1 EKF). Formation offsets computed relative to this. |
| leader_vel[2:0][31:0] | 3×32b | Q4.28 | Leader (V1) NED velocity | Virtual leader velocity. Added to formation PD correction for feed-forward. |
| leader_heading[31:0] | 32 bits | Q3.29 rad | Leader heading ψ_L | Used by R_f2NED rotation matrix. From UAV1 yaw state (or CS1 orbit track angle). |

## 5.  Telemetry Bus — Signal Detail

All signals on the TDM telemetry bus: per-vehicle payload slices, CCSDS packet fields, superframe slot ID, UART TX/RX, and frame-valid strobe.

| Telemetry Bus (tlm_bus) — TDM 1-second superframe — 1 Hz per vehicle — 115200 baud UART to GCS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Signal | Bits | Format | Name | Description |
| tlm_data[v][7:0][31:0] | Per-vehicle: 8×32b = 256b | Packed binary CCSDS-struct | Vehicle telemetry payload | 50-byte payload per vehicle per second. Layout: CCSDS primary header (6B) + secondary (4B) + subsystem data (40B). Each vehicle transmits in its 1-second superframe slot. |
| tlm_cs_quat[cid][63:0] | 3×64b = 192b | Q1.15×4 (64b) | CS1-3 attitude quaternion | Current quaternion [qw,qx,qy,qz] each 16-bit Q1.15. Packed into ADCS packet APID 0x101. |
| tlm_cs_orbit[cid][191:0] | 3×192b = 576b | Q10.22×6 | CS1-3 orbital elements | ECI pos+vel (6×32b = 192b per CS). Packed into orbit packet APID 0x102. |
| tlm_cs_laser[cid][31:0] | 3×32b = 96b | bitmask+Q | CS1-3 laser FSM status | Laser state (3b) + sig strength (12b) + gimbal pos (2×14b). APID 0x103. |
| tlm_uav_nav[uid][287:0] | 3×288b = 864b | Q10.22+Q4.28 | UAV1-3 nav states | Full 9-state EKF vector [px,py,pz,vx,vy,vz,roll,pitch,yaw]. 9×32b per UAV. |
| tlm_uav_fsm[uid][7:0] | 3×8b = 24b | enum | UAV1-3 FSM states | Navigation FSM state (3b) + armed (1b) + GPS sats (4b). Packed. |
| tlm_cvg_score[31:0] | 32b | Q4.28 | CVG_SCORE metric | Formation quality 0=poor, 2^28=perfect. Updated 1 Hz. |
| tlm_fault_flags[31:0] | 32b | bitmask | Fleet fault bitmask | Global fault register snapshot at time of downlink. |
| tlm_slot_id[2:0] | 3 bits | enum 0-6 | Current TDM slot index | Identifies which vehicle's data is currently on the TLM MUX output. 0=CS1, 1=CS2, 2=CS3, 3=UAV1, 4=UAV2, 5=UAV3, 6=CVG. |
| tlm_frame_valid | 1 bit | 1-cycle strobe | Frame valid strobe | One-cycle strobe each time a complete 50-byte telemetry frame is ready. Triggers UART TX. 1 Hz per vehicle = 7 Hz fleet-wide. |
| tlm_uart_tx | 1 bit | UART 8N1 | Ground station TX | 115200-baud serial output. All vehicle frames serialised in sequence. Peak throughput: 7 × 50B × 10b/B = 3500 bps of 115200 baud = 3% utilisation. |
| tlm_uart_rx | 1 bit | UART 8N1 | Ground station RX | GS uplink commands. Received by gs_uart_cmd_decoder. Decoded to AXI4-Lite writes. No TDM — uplink is unstructured command packets. |

## 6.  Debug Bus — Fault, WDT, ESTOP, and ILA

All fault, watchdog, ESTOP, LED, test-point, and ILA probe signals. The ESTOP combinational path has < 2 LUT delay (≈ 4 ns) — meets 1 ms CA deadline with 250,000× margin.

| Debug Bus (fault_bus + ILA probes + counters) — Async fault path + 1-cycle WDT strobes + Debug-only ILA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Signal | Bits | Format | Name | Description |
| fault_flags[31:0] | 32 bits | bitmask | Global fault register | Combinational OR-tree of all subsystem faults. Always live. Bit assignments: [0-2]=CS1-3 ADCS, [3-5]=CS1-3 laser, [6-8]=UAV1-3 EKF, [9-11]=UAV1-3 nav, [12]=CVG_MVK, [13]=APF_DEADLINE, [14]=ESTOP, [15-31]=reserved. |
| wdt_kick[8:0] | 9 bits | 1-cycle strobe | WDT refresh strobes | One-bit per monitored block: [0-2]=CS1-3, [3-5]=UAV1-3, [6]=CVG-CA, [7]=CVG-FC, [8]=SYS. Each module asserts its bit for 1 cycle at ≥ 100Hz (well within timeout). |
| wdt_expired[8:0] | 9 bits | level | WDT timeout flags | Set by watchdog_timer when kick not received within timeout. CS/UAV: 500ms. CVG-CA: 1ms. Drives ESTOP_N via fault manager. |
| estop_n | 1 bit | active-low | Fleet E-STOP | De-asserted (=0) on WDT miss or manual GCS command. Kills all PWM outputs combinationally. Requires manual write 0xA to ESTOP_CLR register to re-assert. |
| led[7:0] | 8 bits | level | Status LEDs | [0]=MMCM lock, [1]=FORM_ACHIEVED, [2]=CA_ACTIVE, [3]=GPS_LOCK, [4]=ESTOP, [5]=TLM_VALID, [6]=1Hz_heartbeat, [7]=ANY_FAULT. Driven from combinational assigns in top module. |
| tp[3:0] | 4 bits | scope hooks | Test point outputs | [0]=sys_clk (50MHz through ODDR), [1]=ce_100hz, [2]=cvg_score_valid, [3]=estop_n. Bonded to package pins for oscilloscope probing. |
| ila_probe[0][31:0] | 32 bits | Q1.15×2 | ILA: CS1 quaternion [qw,qx] | Vivado ILA probe. First 32 bits of CS1 attitude quaternion. Debug only; remove for flight. |
| ila_probe[1][31:0] | 32 bits | Q10.22 | ILA: UAV1 position px | UAV1 North position for EKF convergence monitoring. |
| ila_probe[2][31:0] | 32 bits | Q4.28 | ILA: CVG_SCORE | Formation quality metric. Watch for value approaching 2^28 during formation tests. |
| ila_probe[3][31:0] | 32 bits | bitmask | ILA: fault_flags | Full fault register for real-time fault monitoring during debug. |
| ila_probe[4][5:0] | 6 bits | bitmask | ILA: ca_override | APF CA override per vehicle. Watch for sustained bits during collision risk scenarios. |
| ila_probe[5][0] | 1 bit | flag | ILA: form_achieved | FORMATION_ACHIEVED flag. Primary integration verification signal. |
| ila_probe[6][31:0] | 32 bits | Q4.28 | ILA: UAV1 vel cmd vx | UAV1 velocity command from formation/APF arbiter. Verify correct dispatch. |
| ila_probe[7][31:0] | 32 bits | misc | ILA: UAV1 FSM + status | [31:29]=NAV state, [28]=armed, [27:24]=GPS sats, [23:0]=reserved. Combined status word. |
| dbg_imu_cnt[2:0][31:0] | 3×32b = 96b | counter | CS1-3 IMU frame counters | Free-running counters. Read via AXI4-Lite DBG_IMU_CNT register. Verify 100 Hz cadence. |
| dbg_ekf_cnt[5:0][31:0] | 6×32b = 192b | counter | All EKF iteration counters | One per vehicle. Read via DBG_EKF_CNT in each subsystem's register bank. |

## 7.  Protocol Comparison

All seven bus protocols used in the design with handshake description, latency, which buses use each, and implementation notes.

| Protocol Name | Handshake | Latency | Buses Using This | Implementation Notes |
| --- | --- | --- | --- | --- |
| Registered Strobe (DATA_READY) | Producer asserts DATA_READY for 1 cycle. Consumers sample on same rising edge. | 0 cycles (comb output) + 1 reg at consumer | pos_bus, vel_bus form_cmd_bus (form_master_valid) | Simplest protocol. No backpressure — consumer must be ready. All consumers on sys_clk; no CDC needed. Stale data held until next DATA_READY. Valid flags per-vehicle indicate freshness. |
| Valid-only Push (no ready/backpressure) | Producer drives valid=1 for 1 cycle with data. Consumer must accept immediately. | 0 cycles (single-cycle data capture) | apf_force_bus (force_valid) wdt_kick[8:0] | APF force is single-cycle push to arbiter. Arbiter always ready. WDT kicks are single-cycle strobes; watchdog_timer samples on any clock. No FIFO needed. |
| Async FIFO CDC (Gray-coded) | Write: producer asserts wr_en with data. Read: consumer reads when empty=0. | 2 FIFO cycles (write to read across CDC) | imu_bus (SPI data clk_100mhz → sys_clk) | Each vehicle has independent async FIFO. 16-deep × 32-bit. Gray-coded pointer CDC. Full signal flagged to SPI master (should never occur at 25 MHz SPI vs 100 Hz data). Empty flagged to EKF predict (waits for fresh IMU data). |
| AXI4-Lite 5-channel | Separate valid/ready on AW, W, B, AR, R channels. Handshake on each. | 2-4 cycles (register pipeline) | axi_cfg_bus (GS config commands) | Standard AXI4-Lite protocol. Write: AW + W channels fire simultaneously; B response within 2 cycles. Read: AR channel, then R response within 2 cycles. No burst support needed (single registers only). |
| TDM Mux (Time-Division) | Slot timer drives MUX select. No explicit handshake — data valid in slot window. | Up to slot period (100ms–1s) + UART serialise | tlm_bus (1-sec superframe all vehicles) | 1-second superframe: 7 slots of ~142ms each (6 vehicles + CVG). MUX selects correct source based on arbiter slot. UART TX serialises at 115200 baud; each 50-byte frame takes 4.3ms (well within slot). |
| Combinational Level (fault tree) | No handshake — continuous OR of all fault inputs. Level-sensitive. | < 2 LUT levels (≈ 4 ns) → 0 cycles | fault_bus (ESTOP path combinational) | ESTOP_N is combinational OR of all fault sources → combinational NOT. Any fault immediately de-asserts ESTOP_N. This is the safety-critical path: registered path not allowed. Meets 1ms CA deadline with 5 ns actual path. |
| Level CE (Clock-Enable) | 1-cycle-wide pulse asserted by ce_strobe_gen. All receivers sample on same edge. | 0 cycles (broadcast 1-cycle wide) | ce_bus (all 5 frequency strobes) | Strictly within sys_clk domain. Not a separate clock — all FFs remain on sys_clk rising edge. Modules use CE as gate condition: always @(posedge clk) if (ce_Nhz). Timing analysis treats all as single domain. |

## 8.  SystemVerilog Declarations

Complete logic declarations for all internal buses. Copy directly into convergence_mvp_top.sv after port declarations.

convergence_mvp_top.sv — Internal Bus Wire Declarations

// ================================================================ // Internal Bus Declarations — convergence_mvp_top.sv (excerpt) // All buses in sys_clk domain unless noted (CDC buses marked **) // ================================================================ // ── Position Bus (6 vehicles × 3 axes × Q10.22 32-bit) ─────────── logic signed [31:0] pos_bus[0:5][0:2]; // [vehicle][axis] Q10.22 NED metres logic [5:0] pos_valid; // per-vehicle fresh data flag logic pos_data_ready; // 10Hz master strobe (all valid) // ── Velocity Bus (6 vehicles × 3 axes × Q4.28 32-bit) ─────────── logic signed [31:0] vel_bus[0:5][0:2]; // [vehicle][axis] Q4.28 NED m/s logic [5:0] vel_valid; // per-vehicle valid // vel_data_ready is the same strobe as pos_data_ready (atomic update) // ── IMU Bus ** (clk_100mhz write / sys_clk read via async FIFO) ── logic signed [95:0] imu_bus[0:5]; // [vehicle] {ax,ay,az,gx,gy,gz} Q4.28 logic [5:0] imu_valid; // sys_clk domain valid (after FIFO) // ── Formation Command Bus (5 followers × 3 axes × Q4.28) ───────── logic signed [31:0] form_vel_cmd[1:5][0:2]; // [follower 1..5][axis] logic signed [31:0] form_err[1:5][0:2]; // [follower][axis] Q10.22 error metres logic [4:0] form_cmd_valid; // per-follower cmd valid logic form_master_valid; // 10Hz dispatch strobe logic [1:0] form_geometry; // active formation type logic form_in_transition; // LERP active flag logic signed [31:0] leader_pos[0:2]; // leader NED position Q10.22 logic signed [31:0] leader_vel[0:2]; // leader NED velocity Q4.28 logic signed [31:0] leader_heading; // leader yaw ψ Q3.29 // ── APF Force Bus (6 vehicles × 3 axes × Q4.28) ───────────────── logic signed [31:0] apf_force[0:5][0:2]; // [vehicle][axis] Q4.28 m/s logic [5:0] apf_threat_flags; // per-vehicle threat (d<d_safe) logic [5:0] apf_ca_override; // CA override per vehicle logic apf_force_valid; // force output valid strobe // ── Clock-Enable Bus (all sys_clk domain, 1-cycle-wide pulses) ─── logic ce_1khz,ce_100hz,ce_50hz,ce_10hz,ce_1hz; // ── Fault / Debug Bus ──────────────────────────────────────────── logic [31:0] fault_flags; // 32-bit combinational fault OR logic [8:0] wdt_kick; // per-block watchdog refresh logic [8:0] wdt_expired; // per-block WDT expiry flag logic estop_n; // fleet E-STOP (active-low) // ── Telemetry Bus (muxed to single stream before UART) ─────────── logic [255:0] tlm_data_mux; // 256-bit current slot data logic [2:0] tlm_slot_id; // current slot 0-6 logic tlm_frame_valid; // new 50-byte frame ready // ── AXI4-Lite internal fabric wires ────────────────────────────── logic [31:0] ic_wdata; // AXI write data to all banks logic [11:0] ic_waddr; // AXI write address decoded logic [5:0] axi_cs_we; // write enable per CS bank logic [5:0] axi_uav_we; // write enable per UAV bank logic axi_cvg_we, axi_sys_we; // CVG + SYS bank enables