Convergence MVP  —  FPGA Architecture Block Diagram  (Artix-7 XC7A35T  @  50 MHz)

convergence_mvp_top.v  |  3× CubeSat + 3× UAV + Convergence Engine  |  All internal signal buses annotated

╔══════════════════════════════════════════════════════════════════════════════════════════════════════════════╗ ║ CONVERGENCE MVP — ARTIX-7 XC7A35T (CPG236) @ 50 MHz ║ ║ convergence_mvp_top.v — FPGA Top-Level Module ║ ╠══════════════════════════════════════════════════════════════════════════════════════════════════════════════╣ ║ ║ ║ EXTERNAL PINS ┌─────────────────────────────────────────────────────────────────────────┐ ║ ║ ───────────── │ clk_wizard (MMCM0) │ ║ ║ sys_clk_p/n ──────────────►│ IBUFGDS → MMCM → BUFG │ ║ ║ (50 MHz LVDS) │ ┌──────────────────────────────────────────────────────────────────┐ │ ║ ║ │ │ CLKFBOUT_MULT_F=16 DIVCLK_DIVIDE=1 VCO=800 MHz │ │ ║ ║ │ │ clk_out0: CLKOUT0_DIVIDE_F=8 → clk_100mhz (100 MHz) │ │ ║ ║ │ │ clk_out1: sys_clk passthrough → sys_clk ( 50 MHz) │ │ ║ ║ │ └──────────────────────────────────────────────────────────────────┘ │ ║ ║ │ mmcm_locked ──► global_rst_n (active-low, held until MMCM locks) │ ║ ║ └──────────┬──────────────────────────────────────┬───────────────────────┘ ║ ║ │ sys_clk (50 MHz) │ clk_100mhz (100 MHz) ║ ║ ▼ ▼ ║ ║ ┌─────────────────────────────────────────────────────────────────────────────────────────────────────┐ ║ ║ │ axi_lite_interconnect │ ║ ║ │ ┌─────────────────────────────────────────────────────────────────────────────────────────────┐ │ ║ ║ │ │ AXI4-Lite Crossbar (32-bit addr / 32-bit data / 4-bit strobe) │ │ ║ ║ │ │ ┌─────────────┐ axi_awaddr[31:0] ┌─────────────────────────────────────────────────┐ │ │ ║ ║ │ │ │ GS UART RX │─────────────────────► Register Decode (64 registers × 32-bit) │ │ │ ║ ║ │ │ │ cmd_decoder│ axi_wdata[31:0] │ ┌─────────┬──────────┬──────────┬──────────┐ │ │ │ ║ ║ │ │ └─────────────┘ axi_rdata[31:0]◄──│ │ CS1 │ CS2 │ CS3 │ UAV1-3 │ │ │ │ ║ ║ │ │ │ │ regs │ regs │ regs │ regs │ │ │ │ ║ ║ │ │ │ └────┬────┴────┬─────┴────┬─────┴────┬─────┘ │ │ │ ║ ║ │ └──────────────────────────────────────────────┼─────────┼──────────┼──────────┼─────────┘ │ │ ║ ║ └──────────────────────────────────────────────────┼─────────┼──────────┼──────────┼────────────┘ ║ ║ axi_cs[2:0]──────────────────────────────────┘ │ │ │ ║ ║ axi_uav[2:0]────────────────────────────────────────────┴──────────┘ │ ║ ║ axi_cvg──────────────────────────────────────────────────────────────────────►│ ║ ║ │ ║ ╠══════════════════════════════════════════════════════════════════════════════════════════════════════════════╣ ║ ║ ║ ┌────────────────────────────────────────────────────────────────────────────────────────────────────┐ ║ ║ │ cubesat_subsystem ×3 (CS1 shown; CS2, CS3 identical — separate register banks + pin mapping) │ ║ ║ │ │ ║ ║ │ BANK 34 PINS (CS1) ┌────────────────────┐ ┌──────────────────────────────────────────┐ │ ║ ║ │ spi_sck/mosi/miso/cs_n ──►│ imu_spi_master │──►│ quat_ekf_7state │ │ ║ ║ │ (ICM-42688 / MPU-9250) │ (SPI mode 3) │ │ x = [qw,qx,qy,qz, bx,by,bz] Q1.15 │ │ ║ ║ │ │ 100Hz CE strobe │ │ F(7×7) sparse Jacobian │ │ ║ ║ │ └──────────┬──────────┘ │ 2 DSP48 covariance update │ │ ║ ║ │ │imu[95:0] │ BRAM18 P-matrix double-buf │ │ ║ ║ │ ▼ └────────────────┬─────────────────────────┘ │ ║ ║ │ BANK 34 PINS (CS1) ┌───────────────────┐ │ quat[63:0] Q1.15 │ ║ ║ │ i2c_sda/scl ─────────────►│ mag_i2c_master │ ▼ │ ║ ║ │ (HMC5883L / IST8310) │ (400kHz I2C) │ ┌──────────────────────────────────────────┐ │ ║ ║ │ └─────────┬─────────┘ │ adcs_pd_controller │ │ ║ ║ │ │mag[47:0] │ q_err = q_des ⊗ q_cur* │ │ ║ ║ │ BANK 34 PINS (CS1) ┌──────────▼──────────┐ │ u = Kp*e + Kd*de/dt (Q4.12 fixed) │ │ ║ ║ │ sun_adc_sck/mosi ────────►│ sun_sensor_adc │──►│ 2 DSP48 error multiply │ │ ║ ║ │ (12-bit photodiode ADC) │ (SPI 6-ch MUX) │ │ ┌──────────────┬──────────────────────┐ │ ║ ║ │ └─────────────────────┘ │ │ rw_spi_ctrl │ magnetorquer_pwm │ │ ║ ║ │ │ │ (3-axis RW) │ (3-ch, Q4.12 duty) │ │ ║ ║ │ │ └──────────────┴──────────────────────┘ │ ║ ║ │ └──────────────────────────────────────────┘ │ ║ ║ │ │ ║ ║ │ ┌──────────────────────────────┐ ┌──────────────────────────────────────────────────────────┐ │ ║ ║ │ │ sgp4_orbit_propagator │ │ laser_pointing_fsm │ │ ║ ║ │ │ TLE validate + BRAM store │ │ IDLE → SEARCH → ACQUIRE → TRACK → COMM → FAULT │ │ ║ ║ │ │ SGP4-Lite + J2 (1 Hz) │ │ ┌─────────────┐ ┌─────────────────────────────────┐ │ │ ║ ║ │ │ 3 DSP48 Kepler solve │ │ │stepper_ctrl │ │ signal_adc (12-bit strength) │ │ │ ║ ║ │ │ ECI → LVLH frame conversion │ │ │(STEP/DIR) │ │ closed-loop RMS ≤0.05° TRACK │ │ │ ║ ║ │ │ orbit[191:0] → CVG engine │ │ └─────────────┘ └─────────────────────────────────┘ │ │ ║ ║ │ └──────────────────────────────┘ └──────────────────────────────────────────────────────────┘ │ ║ ║ │ │ ║ ║ │ ┌────────────────────────────────────────────────────────────────────────────────────────────────┐ │ ║ ║ │ │ ccsds_telemetry_encoder │ │ ║ ║ │ │ SYNC 0x1ACFFC1D │ CRC-16/CCITT │ APID 0x101(ADCS) 0x102(Orbit) 0x103(Laser) │ │ ║ ║ │ │ 1-sec superframe arbiter → UART TX → ISL_TX[7:0] parallel to convergence_engine │ │ ║ ║ │ └────────────────────────────────────────────────────────────────────────────────────────────────┘ │ ║ ║ └────────────────────────────────────────────────────────────────────────────────────────────────────┘ ║ ║ ISL_TX_CS1[7:0] ──────────────────────────────────────────────────────────────────────────────► ║ ║ ISL_TX_CS2[7:0] ──────────────────────────────────────────────────────────────────────────────► ║ ║ ISL_TX_CS3[7:0] ──────────────────────────────────────────────────────────────────────────────► ║ ║ ║ ╠══════════════════════════════════════════════════════════════════════════════════════════════════════════════╣ ║ ║ ║ ┌────────────────────────────────────────────────────────────────────────────────────────────────────┐ ║ ║ │ uav_subsystem ×3 (UAV1 shown; UAV2, UAV3 identical — separate register banks + pin assignments) │ ║ ║ │ │ ║ ║ │ BANK 14 PINS (UAV1) ┌────────────────────┐ ┌──────────────────────────────────────────┐ │ ║ ║ │ spi_sck/mosi/miso/cs_n ──►│ imu_spi_master │──►│ ekf_9state_navigator │ │ ║ ║ │ (ICM-42688) │ clk_100mhz domain │ │ x=[px,py,pz,vx,vy,vz,roll,pitch,yaw] │ │ ║ ║ │ async_fifo │ CDC: async_fifo │ │ pos: Q10.22 vel: Q4.28 ang: Q3.29 │ │ ║ ║ │ (Gray-coded, 32×16) │ 100→50 MHz │ │ F Jacobian 9×9 (sparse, 28 nonzeros) │ │ ║ ║ │ └──────────┬──────────┘ │ 3 DSP48 CORDIC R-matrix (9 elements) │ │ ║ ║ │ │imu[95:0] │ BRAM18 P-matrix covariance │ │ ║ ║ │ BANK 14 PINS (UAV1) ┌───────────▼─────────┐ └──────────────────┬───────────────────┘ │ ║ ║ │ gps_uart_rx ─────────────►│ gps_nmea_parser │ │ nav_state[287:0] │ ║ ║ │ (NEO-M9N, 9600 baud) │ GGA + VTG │ ▼ │ ║ ║ │ gps_pps ──────────────────│ 10 Hz fix rate │ ┌──────────────────────────────────────┐ │ ║ ║ │ └─────────────────────┘ │ pid_6dof_controller │ │ ║ ║ │ BANK 14 PINS (UAV1) ┌─────────────────────┐ │ Dual-loop: 1kHz inner / 100Hz outer │ │ ║ ║ │ baro_spi ─────────────────►│ baro_spi_master │──►│ 6 axes: roll/pitch/yaw-rate + │ │ ║ ║ │ (BMP388) │ 50Hz altitude │ │ roll/pitch + altitude + Vx/Vy │ │ ║ ║ │ tof_echo ─────────────────►│ tof_echo_capture │ │ 4 DSP48 (shared PID engine) │ │ ║ ║ │ (below 10m AGL) │ pulse width meas. │ │ BRAM18 gain registers (4 modes) │ │ ║ ║ │ └─────────────────────┘ │ ┌──────────────────────────────────┐ │ ║ ║ │ │ │ esc_pwm × 4 (50–400 Hz, 1µs) │ │ ║ ║ │ │ │ motor_mix 4×4 DSP48 MACC │ │ ║ ║ │ │ └──────────────────────────────────┘ │ ║ ║ │ └────────────────────────────────────────┘ │ ║ ║ │ │ ║ ║ │ ┌──────────────────────────────────┐ ┌──────────────────────────────────────────────────────┐ │ ║ ║ │ │ waypoint_nav_fsm (8-state) │ │ mavlink_v2_stack │ │ ║ ║ │ │ DISARMED→ARMED→TAKEOFF │ │ RX: STX 0xFD SYSID CRC-16/MCRF4XX │ │ ║ ║ │ │ →EN_ROUTE→LOITER→LAND │ │ HEARTBEAT 3s timeout → RTL │ │ ║ ║ │ │ →RTL→EMERGENCY │ │ COMMAND_LONG: ARM/DISARM/TAKEOFF/LAND/RTL │ │ ║ ║ │ │ 32-waypoint BRAM18 mission │ │ MISSION_ITEM_INT upload (32 WPs) │ │ ║ ║ │ │ Geofence cylindrical RTL │ │ TX: HEARTBEAT(1Hz) ATTITUDE(10Hz) │ │ ║ ║ │ │ Watchdog 500ms → EMERGENCY │ │ GLOBAL_POSITION_INT(5Hz) SYS_STATUS(1Hz) │ │ ║ ║ │ └──────────────────────────────────┘ │ uart_tx/rx ↔ GCS (115200 baud, 8N1) │ │ ║ ║ │ └──────────────────────────────────────────────────┘ │ ║ ║ └────────────────────────────────────────────────────────────────────────────────────────────────────┘ ║ ║ nav_state_UAV1[287:0] ─────────────────────────────────────────────────────────────────────────► ║ ║ nav_state_UAV2[287:0] ─────────────────────────────────────────────────────────────────────────► ║ ║ nav_state_UAV3[287:0] ─────────────────────────────────────────────────────────────────────────► ║ ║ vel_cmd_UAV1[95:0] ◄─────────────────────────────────────────────────────────────────────────── ║ ║ vel_cmd_UAV2[95:0] ◄─────────────────────────────────────────────────────────────────────────── ║ ║ vel_cmd_UAV3[95:0] ◄─────────────────────────────────────────────────────────────────────────── ║ ║ ║ ╠══════════════════════════════════════════════════════════════════════════════════════════════════════════════╣ ║ ║ ║ ┌────────────────────────────────────────────────────────────────────────────────────────────────────┐ ║ ║ │ convergence_engine │ ║ ║ │ │ ║ ║ │ ┌───────────────────────────────────────────────────────────────────────────────────────────────┐ │ ║ ║ │ │ multi_vehicle_kalman_filter (36-state, 6 vehicles) │ │ ║ ║ │ │ State: x = [px,py,pz,vx,vy,vz]×6 vehicles (Q10.22 pos, Q4.28 vel) │ │ ║ ║ │ │ ┌───────────────────┐ ┌────────────────────┐ ┌─────────────────────────────────────────┐ │ │ ║ ║ │ │ │ ISL RX ×3 │ │ GPS fusion ×6 │ │ F·x predict + K·innov update │ │ │ ║ ║ │ │ │ ISL_RX[7:0] │ │ 5σ outlier gate │ │ 4 DSP48 TDM predict/update │ │ │ ║ ║ │ │ │ pos[95:0] CS1-3 │ │ Q10.22 positions │ │ 2 BRAM18 P-matrix double-buffer │ │ │ ║ ║ │ │ └─────────┬─────────┘ └──────────┬─────────┘ │ DATA_READY strobe 10 Hz │ │ │ ║ ║ │ │ └──────────────────────────────────────► ΔR_ij[15 pairs×3D] → CVG_SCORE │ │ │ ║ ║ │ │ └─────────────────────────────────────────┘ │ │ ║ ║ │ └───────────────────────────────────────────────────────────────────────────────────────────────┘ │ ║ ║ │ joint_state[575:0] ──────────────────────────────────────────────────────────────────► │ ║ ║ │ │ ║ ║ │ ┌───────────────────────────────────┐ ┌──────────────────────────────────────────────────────┐ │ ║ ║ │ │ formation_controller │ │ apf_collision_avoidance │ │ ║ ║ │ │ 4 geometries (BRAM LUT-ROM): │ │ 15 pairs d²=dx²+dy²+dz² (no sqrt for gate) │ │ ║ ║ │ │ ┌──────────┬──────────────────┐ │ │ F_rep = k_rep*(1/d-1/d_safe)/d²*unit_vec │ │ ║ ║ │ │ │ TRIANGLE │ LINE │ │ │ Threshold: D_SAFE_SQ = 100 m² (exact compare) │ │ ║ ║ │ │ │ 120° off │ 50m spacing │ │ │ NR inv_sqrt (1 iter) Q1.30 precision │ │ ║ ║ │ │ ├──────────┼──────────────────┤ │ │ 6 DSP48 TDM across 15 pairs (2.4µs @ 50MHz) │ │ ║ ║ │ │ │ V-SHAPE │ SQUARE │ │ │ F_total[5:0][95:0] → resource_arbiter │ │ ║ ║ │ │ │ 60° ang │ 50m sides │ │ │ ESTOP within 1ms of 10ms miss (CV-CA-001) │ │ ║ ║ │ │ └──────────┴──────────────────┘ │ └────────────────────────────┬─────────────────────────┘ │ ║ ║ │ │ R_f2NED(ψ_L) 4 DSP48 MACs │ │ F_ca[5:0][95:0] │ ║ ║ │ │ P_target = P_L + R×δ_f (10Hz) │ │ │ ║ ║ │ │ LERP transition (configurable) │ │ │ ║ ║ │ └──────────────────────────────────┘ │ │ ║ ║ │ form_err[5:0][95:0] ──────────────────────────────────────────┘ │ ║ ║ │ │ │ ║ ║ │ ┌──────────────────────────────────────▼──────────────────────────────────────────────────────────┐ │ ║ ║ │ │ resource_arbiter (TDM scheduler) │ │ ║ ║ │ │ 6-slot round-robin (100ms/slot configurable 50–500ms) │ │ ║ ║ │ │ CA pre-emption: APF override within 1ms │ ISL TX/RX 60/40 split │ 2ms guard intervals │ │ ║ ║ │ │ Formation dispatch: last 20ms of slot │ Telemetry: 30ms window │ Watchdog: 2×period │ │ ║ ║ │ │ Priority: P0=ESTOP > P1=CA > P2=formation > P3=telemetry > P4=config │ │ ║ ║ │ │ ┌──────────────────────────────────────────────────────────────────────────────────────────┐ │ │ ║ ║ │ │ │ vel_cmd_out[5:0][95:0] → UAV1-3 PID setpoints (Q4.28 m/s) │ │ │ ║ ║ │ │ │ attitude_cmd[5:0][63:0] → CS1-3 ADCS setpoints (Q1.15 quaternion) │ │ │ ║ ║ │ │ │ isl_enable[5:0] → ISL TX gate per vehicle slot │ │ │ ║ ║ │ │ │ ca_override[5:0] → individual vehicle CA flag │ │ │ ║ ║ │ │ │ cvg_score[31:0] → CVG_SCORE metric (Q4.28) + FORMATION_ACHIEVED flag │ │ │ ║ ║ │ │ └──────────────────────────────────────────────────────────────────────────────────────────┘ │ │ ║ ║ │ └────────────────────────────────────────────────────────────────────────────────────────────────┘ │ ║ ║ └────────────────────────────────────────────────────────────────────────────────────────────────────┘ ║ ║ ║ ╠══════════════════════════════════════════════════════════════════════════════════════════════════════════════╣ ║ ║ ║ ┌──────────────────────────────────────────────────────────────────────────────────────────────────────┐ ║ ║ │ global_fault_manager + system_health │ ║ ║ │ xadc_monitor: VCCINT / VCCAUX / temp (°C) fault_flags[31:0] → BRAM18 fault log (timestamped) │ ║ ║ │ watchdog_timers[8:0]: CS1-3 + UAV1-3 + CVG → ESTOP if expired (CA WDT: 1ms, others: 500ms) │ ║ ║ │ ce_strobe_gen: master 26-bit counter → 1kHz / 100Hz / 50Hz / 10Hz / 1Hz CE decode │ ║ ║ └──────────────────────────────────────────────────────────────────────────────────────────────────────┘ ║ ║ ║ ╠══════════════════════════════════════════════════════════════════════════════════════════════════════════════╣ ║ ║ ║ KEY BUS LEGEND ║ ║ ───────────────────────────────────────────────────────────────────────────────────────────────────────── ║ ║ Bus Name Width Format Source → Destination Rate ║ ║ ───────────────────────────────────────────────────────────────────────────────────────────────────────── ║ ║ imu[95:0] 96-bit Q4.28×6 imu_spi_master → EKF input FIFO 100 Hz (sync) ║ ║ quat[63:0] 64-bit Q1.15×4 quat_ekf → adcs_pd_controller 100 Hz ║ ║ nav_state[287:0] 288-bit Q10.22×9 uav_ekf → CVG multi-vehicle KF 10 Hz (GPS gate) ║ ║ joint_state[575:0] 576-bit Q10.22×18 multi_veh_KF → formation / APF 10 Hz ║ ║ vel_cmd[95:0] 96-bit Q4.28×3 resource_arbiter → UAV PID setpoints 10 Hz ║ ║ attitude_cmd[63:0] 64-bit Q1.15×4 resource_arbiter → CS ADCS setpoints 10 Hz ║ ║ form_err[5:0][95:0] 96-bit Q10.22×3 formation_ctrl → APF + arbiter 10 Hz ║ ║ F_ca[5:0][95:0] 96-bit Q4.28×3 APF → resource_arbiter 10 Hz (on threat) ║ ║ ISL_TX[7:0] 8-bit parallel CS telemetry → CVG ISL RX 10 Mbps ODDR ║ ║ axi_awaddr[31:0] 32-bit AXI4-Lite GS cmd decoder → register bank async ║ ║ cvg_score[31:0] 32-bit Q4.28 geometry_score → downlink telemetry 1 Hz ║ ║ fault_flags[31:0] 32-bit bitmask global_fault_mgr → BRAM log async (on event) ║ ║ mmcm_locked 1-bit flag MMCM0 → global_rst_n hold startup ║ ║ ca_override[5:0] 6-bit per-vehicle APF → arbiter + UAV PID < 1 ms latency ║ ║ ║ ╠══════════════════════════════════════════════════════════════════════════════════════════════════════════════╣ ║ ║ ║ RESOURCE SUMMARY Total Used / Available (Artix-7 XC7A35T) ║ ║ ───────────────────────────────────────────────────────────────────────────────────────────────────────── ║ ║ LUTs : 9,862 / 20,800 (47.4%) ████████████████████████░░░░░░░░░░░░░░░░░░░░ PASS < 50% ║ ║ FFs : 14,180 / 41,600 (34.1%) ████████████████░░░░░░░░░░░░░░░░░░░░░░░░░░░░ PASS < 50% ║ ║ DSP48 : 44 / 90 (48.9%) ████████████████████████░░░░░░░░░░░░░░░░░░░░ PASS < 50% ║ ║ BRAM18 : 24 / 50 (48.0%) ███████████████████████░░░░░░░░░░░░░░░░░░░░░ PASS < 50% ║ ║ ║ ║ CLOCK DOMAINS: sys_clk 50MHz (most logic) │ clk_100mhz 100MHz (SPI, async CDC only) ║ ║ CDC CROSSINGS: 3×async_fifo (SPI→sys) + spi_busy/fault via 2-FF sync + req/ack handshake ║ ║ TIMING BUDGET: All paths analysed in Vivado; EKF-05 cov. update MANDATORY pipeline reg (see timing spec) ║ ║ ║ ╚══════════════════════════════════════════════════════════════════════════════════════════════════════════════╝

# Signal Bus Index

All annotated buses and control signals: name, bit-width, fixed-point format, data direction, update rate, and description.

| Signal Name | Width | Format | Direction | Rate | Description |
| --- | --- | --- | --- | --- | --- |
| sys_clk | 1b | 50 MHz | EXT → clk_wizard | — | LVDS input pair sys_clk_p/n. IBUFGDS primitive. |
| clk_100mhz | 1b | 100 MHz | clk_wizard → SPI+CDC | CW only | MMCM CLKOUT0. Used by SPI masters and async FIFO write-side. |
| global_rst_n | 1b | active-L | MMCM locked → all modules | startup | Held low until MMCM locked. Multi-stage sync per domain. |
| imu[95:0] | 96b | Q4.28×6 | SPI → EKF FIFO | 100 Hz | ax,ay,az (m/s²) + gx,gy,gz (rad/s) each Q4.28 32-bit. |
| quat[63:0] | 64b | Q1.15×4 | CS EKF → ADCS PD ctrl | 100 Hz | Unit quaternion [qw,qx,qy,qz] each 16-bit Q1.15. |
| orbit[191:0] | 192b | Q10.22 | CS SGP4 → CVG engine | 1 Hz | ECI pos (3×32b) + vel (3×32b) + orbital elements. |
| ISL_TX[7:0] | 8b | parallel | CS CCSDS → CVG ISL RX | 10 Mbps | 8-bit parallel optical bus. ODDR output regs. Bank 35. |
| nav_state[287:0] | 288b | Q10.22/Q4.28 | UAV EKF → CVG MVK | 10 Hz | 9-state vector × 32b per element = 288 bits per UAV. |
| vel_cmd[95:0] | 96b | Q4.28×3 | CVG arbiter → UAV PID | 10 Hz | NED velocity setpoints [vx,vy,vz] Q4.28 32-bit each. |
| attitude_cmd[63:0] | 64b | Q1.15×4 | CVG arbiter → CS ADCS | 10 Hz | Target quaternion for each CS. Same Q1.15 format as EKF output. |
| joint_state[575:0] | 576b | Q10.22×18 | MVK → form ctrl + APF | 10 Hz | All 6 vehicles × 9-state × 32b. Double-buffered BRAM18 read. |
| form_err[95:0] | 96b | Q10.22×3 | formation ctrl → APF | 10 Hz | Per-vehicle 3D error vector [Ex,Ey,Ez]. Feeds APF gate. |
| F_ca[95:0] | 96b | Q4.28×3 | APF → resource arbiter | < 1ms | Repulsive velocity correction per vehicle. Clipped ±5 m/s. |
| ca_override[5:0] | 6b | bitmask | APF → UAV PID + arbiter | < 1ms | One bit per vehicle. High = formation cmd suppressed by APF. |
| cvg_score[31:0] | 32b | Q4.28 | geo_score → TLM downlink | 1 Hz | CVG_SCORE geometry metric. 0=poor, 1=perfect formation. |
| fault_flags[31:0] | 32b | bitmask | fault_mgr → BRAM log + TLM | async | Per-subsystem fault bits. Triggers ESTOP if CA watchdog bit set. |
| axi_awaddr[31:0] | 32b | AXI4-Lite | GS decoder → reg bank | async | AXI4-Lite 32-bit address. Write address + data in same cycle. |
| axi_wdata[31:0] | 32b | AXI4-Lite | GS decoder → reg bank | async | PID gains / formation params / APF k_rep via AXI4-Lite write. |
| axi_rdata[31:0] | 32b | AXI4-Lite | reg bank → GS TLM | async | Readback of any register. Status + health + EKF state snippet. |
| estop_n | 1b | active-L | fault_mgr → all actuators | < 1ms | Kills all PWM outputs. Cannot be cleared by software alone. |

## Clock Domain Summary

sys_clk (50 MHz)  —  Primary domain. All EKF, PID, formation, APF, AXI, telemetry, and CE strobe logic.

clk_100mhz (100 MHz)  —  SPI masters (ICM-42688 data capture) + async FIFO write-side (CDC boundary).

CDC crossings: (1) 3× async FIFO Gray-coded for SPI IMU data. (2) spi_busy / spi_fault: 2-FF synchroniser + set_false_path. (3) spi_start / cmd: 4-phase req/ack handshake.

All CE strobes (1kHz/100Hz/50Hz/10Hz/1Hz) are registered enables within sys_clk — NOT separate clock domains. No additional CDC required.