convergence_mvp_top.sv — Synthesizable SystemVerilog Skeleton

914 lines  |  N_SATS=3  N_UAVS=3  DATA_W=32  ADDR_W=12  |  Artix-7 XC7A35T  |  Vivado 2023.2  |  SystemVerilog-2012

// ============================================================================= // File : convergence_mvp_top.sv // Project : Convergence MVP — Multi-Vehicle Formation Flying FPGA // Device : Xilinx Artix-7 XC7A35T (CPG236) // Toolchain : Vivado 2023.2 ¦ SystemVerilog-2012 // Author : Convergence Team // Description : Top-level skeleton — port declarations, wire declarations, // and sub-module instantiations ONLY. // No combinational or sequential logic in this file. // All functional implementation lives in child modules. // ============================================================================= // INSTANCE MAP // clk_wizard_inst : clk_wiz_0 (Vivado IP — MMCM) // axi_ic_inst : axi_lite_interconnect (AXI4-Lite crossbar) // fault_mgr_inst : global_fault_manager (fault / ESTOP / watchdog) // ce_gen_inst : ce_strobe_gen (clock-enable dividers) // cs_inst[0..2] : cubesat_subsystem (CS1 / CS2 / CS3 — generate) // uav_inst[0..2] : uav_subsystem (UAV1 / UAV2 / UAV3 — generate) // conv_inst : convergence_engine (MVK + Formation + APF + Arbiter) // pid_shared_inst : pid_engine (shared DSP48 PID block) // cordic_inst : cordic_sincos (shared CORDIC sin/cos) // tlm_mux_inst : telemetry_downlink_mux (GS downlink superframe) // debug_inst : debug_ila_wrapper (ILA — remove for flight) // ============================================================================= `timescale 1ns / 1ps `default_nettype none // ───────────────────────────────────────────────────────────────────────────── // MODULE DECLARATION // ───────────────────────────────────────────────────────────────────────────── module convergence_mvp_top #( // ── Fleet sizing ────────────────────────────────────────────────────────── parameter integer N_SATS = 3, // Number of CubeSat subsystems parameter integer N_UAVS = 3, // Number of UAV subsystems parameter integer N_VEHICLES = N_SATS + N_UAVS, // Total fleet size (6) // ── Data widths ─────────────────────────────────────────────────────────── parameter integer DATA_W = 32, // AXI4-Lite + internal data width parameter integer ADDR_W = 12, // AXI4-Lite address width (4 KB) // ── Fixed-point formats (for documentation / downstream parameter passing) ─ parameter integer Q_POS_INT = 10, // Q10.22 position integer bits parameter integer Q_POS_FRAC = 22, // Q10.22 position fraction bits parameter integer Q_VEL_INT = 4, // Q4.28 velocity integer bits parameter integer Q_VEL_FRAC = 28, // Q4.28 velocity fraction bits parameter integer Q_ANG_INT = 3, // Q3.29 angle integer bits parameter integer Q_ANG_FRAC = 29, // Q3.29 angle fraction bits parameter integer Q_PID_INT = 4, // Q4.12 PID gain integer bits parameter integer Q_PID_FRAC = 12, // Q4.12 PID gain fraction bits // ── SPI/I2C/UART clock rates ────────────────────────────────────────────── parameter integer SYS_CLK_HZ = 50_000_000, // Primary clock frequency parameter integer FAST_CLK_HZ= 100_000_000,// SPI / CDC clock frequency parameter integer GS_BAUD = 115_200, // Ground station UART baud rate parameter integer GPS_BAUD = 9_600, // GPS NMEA UART baud rate // ── Formation / APF ─────────────────────────────────────────────────────── parameter integer BASELINE_M = 50, // Formation baseline spacing (m) parameter integer D_SAFE_M = 10, // APF safety radius (m) // ── Debug ───────────────────────────────────────────────────────────────── parameter integer DEBUG_EN = 1 // 1=include ILA, 0=flight build ) ( // ========================================================================= // CLOCK & RESET // ========================================================================= input wire sys_clk_p, // LVDS clock positive — Bank 35, E3 input wire sys_clk_n, // LVDS clock negative — Bank 35, E4 input wire rst_n, // Active-low async reset (push-button S1) // ========================================================================= // CUBESAT SPI/I2C/PWM PORTS — per-CS index [N_SATS-1:0] // Generated externally via packed arrays; each array bit maps to one CS. // ========================================================================= // ── Gyro / IMU SPI (ICM-42688 / MPU-9250) — Bank 34 ──────────────────── output wire [N_SATS-1:0] cs_gyro_sck, // SPI clock to IMU output wire [N_SATS-1:0] cs_gyro_mosi, // SPI MOSI to IMU input wire [N_SATS-1:0] cs_gyro_miso, // SPI MISO from IMU output wire [N_SATS-1:0] cs_gyro_cs_n, // SPI chip-select (active-low) // ── Magnetometer I2C (HMC5883L / IST8310) — Bank 34 ──────────────────── inout wire [N_SATS-1:0] cs_mag_sda, // I2C data (open-drain, bidir) output wire [N_SATS-1:0] cs_mag_scl, // I2C clock (open-drain output) // ── Sun sensor ADC SPI (6-channel photodiode MUX) — Bank 34 ───────────── output wire [N_SATS-1:0] cs_sun_sck, // Sun sensor ADC SPI clock output wire [N_SATS-1:0] cs_sun_mosi, // Sun sensor ADC SPI MOSI input wire [N_SATS-1:0] cs_sun_miso, // Sun sensor ADC SPI MISO output wire [N_SATS-1:0] cs_sun_cs_n, // Sun sensor ADC chip-select // ── Reaction wheel SPI (3-axis torque command) — Bank 34 ───────────────── output wire [N_SATS-1:0] cs_rw_sck, // Reaction wheel SPI clock output wire [N_SATS-1:0] cs_rw_mosi, // Reaction wheel SPI MOSI input wire [N_SATS-1:0] cs_rw_miso, // Reaction wheel SPI MISO output wire [N_SATS-1:0] cs_rw_cs_n, // Reaction wheel SPI CS // ── Magnetorquer PWM (3 axes per CubeSat) — Bank 34 ────────────────────── output wire [N_SATS*3-1:0] cs_mtq_pwm, // Magnetorquer PWM [CS][axis] // cs_mtq_pwm[i*3+0] = X axis CS_i // cs_mtq_pwm[i*3+1] = Y axis CS_i // cs_mtq_pwm[i*3+2] = Z axis CS_i // ── Laser pointing gimbal — stepper STEP/DIR (2-axis per CS) — Bank 35 ── output wire [N_SATS-1:0] cs_laser_az_step, // Azimuth stepper STEP pulse output wire [N_SATS-1:0] cs_laser_az_dir, // Azimuth stepper DIR output wire [N_SATS-1:0] cs_laser_el_step, // Elevation stepper STEP output wire [N_SATS-1:0] cs_laser_el_dir, // Elevation stepper DIR output wire [N_SATS-1:0] cs_laser_en_n, // Laser driver enable (active-low) // ── ISL parallel optical link (8-bit TX) — Bank 35, ODDR ───────────────── output wire [N_SATS*8-1:0] cs_isl_tx_data, // Parallel ISL TX [CS][bit] output wire [N_SATS-1:0] cs_isl_tx_clk, // ISL TX DDR clock (ODDR) output wire [N_SATS-1:0] cs_isl_tx_valid, // ISL TX data valid strobe input wire [N_SATS*8-1:0] cs_isl_rx_data, // Parallel ISL RX [CS][bit] input wire [N_SATS-1:0] cs_isl_rx_clk, // ISL RX clock (IDDR) input wire [N_SATS-1:0] cs_isl_rx_valid, // ISL RX valid strobe // ========================================================================= // UAV SPI/UART/I2C/PWM PORTS — per-UAV index [N_UAVS-1:0] // ========================================================================= // ── IMU SPI (ICM-42688) — Bank 14 ──────────────────────────────────────── output wire [N_UAVS-1:0] uav_imu_sck, // IMU SPI clock output wire [N_UAVS-1:0] uav_imu_mosi, // IMU SPI MOSI input wire [N_UAVS-1:0] uav_imu_miso, // IMU SPI MISO output wire [N_UAVS-1:0] uav_imu_cs_n, // IMU SPI chip-select // ── GPS UART (NEO-M9N, NMEA) — Bank 14 ─────────────────────────────────── input wire [N_UAVS-1:0] uav_gps_rx, // GPS UART RX (NMEA sentences) output wire [N_UAVS-1:0] uav_gps_tx, // GPS UART TX (config commands) input wire [N_UAVS-1:0] uav_gps_pps, // GPS 1-PPS timing reference // ── Barometer SPI (BMP388) — Bank 14 ───────────────────────────────────── output wire [N_UAVS-1:0] uav_baro_sck, // Baro SPI clock output wire [N_UAVS-1:0] uav_baro_mosi, // Baro SPI MOSI input wire [N_UAVS-1:0] uav_baro_miso, // Baro SPI MISO output wire [N_UAVS-1:0] uav_baro_cs_n, // Baro SPI chip-select // ── Barometer I2C (alternate interface path) — Bank 14 ─────────────────── inout wire [N_UAVS-1:0] uav_baro_sda, // Baro I2C data (bidir) output wire [N_UAVS-1:0] uav_baro_scl, // Baro I2C clock // ── Time-of-Flight sensor — Bank 14 ────────────────────────────────────── output wire [N_UAVS-1:0] uav_tof_trig, // ToF trigger pulse output input wire [N_UAVS-1:0] uav_tof_echo, // ToF echo pulse input (width = range) // ── ESC PWM (4 motors per UAV) — Bank 14 ───────────────────────────────── output wire [N_UAVS*4-1:0] uav_motor_pwm, // Motor ESC PWM [UAV][motor] // uav_motor_pwm[i*4+m] = motor m of UAV_i // ── Servo PWM (optional payload) — Bank 14 ─────────────────────────────── output wire [N_UAVS*2-1:0] uav_servo_pwm, // Servo PWM [UAV][servo] // ── MAVLink UART (GCS interface per UAV) — Bank 14 ─────────────────────── output wire [N_UAVS-1:0] uav_mav_tx, // MAVLink UART TX to GCS relay input wire [N_UAVS-1:0] uav_mav_rx, // MAVLink UART RX from GCS relay // ========================================================================= // AXI4-LITE SLAVE INTERFACE (for configuration from external processor) // ========================================================================= input wire s_axi_aclk, // AXI clock (tie to sys_clk) input wire s_axi_aresetn, // AXI active-low reset // Write address channel input wire [ADDR_W-1:0] s_axi_awaddr, // Write address input wire [2:0] s_axi_awprot, // Write protection type input wire s_axi_awvalid, // Write address valid output wire s_axi_awready, // Write address ready // Write data channel input wire [DATA_W-1:0] s_axi_wdata, // Write data input wire [DATA_W/8-1:0] s_axi_wstrb, // Write byte enables input wire s_axi_wvalid, // Write data valid output wire s_axi_wready, // Write data ready // Write response channel output wire [1:0] s_axi_bresp, // Write response (00=OKAY) output wire s_axi_bvalid, // Write response valid input wire s_axi_bready, // Write response ready // Read address channel input wire [ADDR_W-1:0] s_axi_araddr, // Read address input wire [2:0] s_axi_arprot, // Read protection type input wire s_axi_arvalid, // Read address valid output wire s_axi_arready, // Read address ready // Read data channel output wire [DATA_W-1:0] s_axi_rdata, // Read data output wire [1:0] s_axi_rresp, // Read response (00=OKAY) output wire s_axi_rvalid, // Read data valid input wire s_axi_rready, // Read data ready // ========================================================================= // GROUND STATION UART (shared downlink + uplink to GCS) // ========================================================================= output wire gnd_uart_tx, // GS downlink TX (telemetry) input wire gnd_uart_rx, // GS uplink RX (commands) // ========================================================================= // DEBUG / STATUS OUTPUTS // ========================================================================= output wire [7:0] led, // Status LEDs (active-high) // [0] = MMCM locked // [1] = Formation achieved // [2] = CA active (any threat) // [3] = GPS lock (any UAV) // [4] = ESTOP asserted // [5] = Spare // [6] = Heartbeat 1Hz // [7] = Fault (any) output wire [3:0] tp // Test points (scope probe hooks) // [0] = sys_clk (50 MHz through ODDR) // [1] = ce_100hz strobe // [2] = cvg_score_valid pulse // [3] = estop_n ); // ============================================================================= // INTERNAL CLOCK AND RESET SIGNALS // ============================================================================= // ── Generated clocks ────────────────────────────────────────────────────────── logic sys_clk; // 50 MHz primary clock — all main logic logic clk_100mhz; // 100 MHz — SPI masters + async FIFO write-side logic mmcm_locked; // MMCM lock flag — drives global_rst_n hold // ── Synchronous reset (active-low, deasserted after MMCM locked) ────────────── logic global_rst_n; // Synchronised reset for sys_clk domain logic fast_rst_n; // Synchronised reset for clk_100mhz domain // ── Clock-enable strobes (generated from sys_clk master counter) ───────────── logic ce_1khz; // 1 kHz strobe — UAV inner PID loop logic ce_100hz; // 100 Hz strobe — IMU EKF predict, ADCS control logic ce_50hz; // 50 Hz strobe — Barometer update logic ce_10hz; // 10 Hz strobe — GPS update, formation, APF logic ce_1hz; // 1 Hz strobe — Orbit propagator, CVG_SCORE, downlink // ============================================================================= // AXI4-LITE INTERCONNECT INTERNAL WIRES // ============================================================================= // AXI4-Lite master-side from GS command decoder → register bank arbiter logic [ADDR_W-1:0] ic_awaddr; // Internal AXI write address (post-decode) logic [DATA_W-1:0] ic_wdata; // Internal AXI write data logic [DATA_W/8-1:0] ic_wstrb; // Internal AXI write byte strobe logic ic_awvalid; // Internal AXI write address valid logic ic_wvalid; // Internal AXI write data valid logic [DATA_W-1:0] ic_rdata; // Internal AXI read data (muxed from slaves) logic ic_arvalid; // Internal AXI read address valid logic [ADDR_W-1:0] ic_araddr; // Internal AXI read address // Per-slave AXI4-Lite write-enable strobes (decoded from ic_awaddr) logic [N_SATS-1:0] axi_cs_we; // Write enable per CubeSat register bank logic [N_UAVS-1:0] axi_uav_we; // Write enable per UAV register bank logic axi_cvg_we; // Write enable for CVG register bank logic axi_sys_we; // Write enable for system register bank // ============================================================================= // CUBESAT SUBSYSTEM INTERCONNECTS — packed arrays [N_SATS-1:0] // ============================================================================= // ── IMU data (after async FIFO, sys_clk domain) ────────────────────────────── logic [N_SATS-1:0][95:0] cs_imu_data; // [ax,ay,az,gx,gy,gz] Q4.28 per CS logic [N_SATS-1:0] cs_imu_valid; // IMU data valid (sys_clk domain) // ── Quaternion EKF outputs ──────────────────────────────────────────────────── logic [N_SATS-1:0][63:0] cs_quat; // Attitude quaternion Q1.15 [qw,qx,qy,qz] logic [N_SATS-1:0] cs_quat_valid; // Quaternion valid flag (100 Hz) // ── Orbit propagator outputs ────────────────────────────────────────────────── logic [N_SATS-1:0][191:0] cs_orbit; // Orbital state [ECI pos 96b + vel 96b] logic [N_SATS-1:0] cs_orbit_valid;// Orbit valid (1 Hz) // ── ADCS control outputs ───────────────────────────────────────────────────── logic [N_SATS-1:0][63:0] cs_attitude_cmd; // ADCS quaternion setpoint from CVG logic [N_SATS-1:0][4:0] cs_adcs_state; // ADCS FSM state (5 states) logic [N_SATS-1:0] cs_adcs_fault; // ADCS fault flag // ── Laser FSM status ───────────────────────────────────────────────────────── logic [N_SATS-1:0][2:0] cs_laser_state; // Laser FSM state (6-state encoded) logic [N_SATS-1:0] cs_laser_track; // 1 = TRACK state active logic [N_SATS-1:0] cs_laser_comm; // 1 = COMM state active // ── AXI configuration inputs to CS ─────────────────────────────────────────── logic [N_SATS-1:0][DATA_W-1:0] cs_kp_reg; // Proportional gain register Q4.12 logic [N_SATS-1:0][DATA_W-1:0] cs_kd_reg; // Derivative gain register Q4.12 logic [N_SATS-1:0][DATA_W-1:0] cs_ki_reg; // Integral gain register Q4.12 // ============================================================================= // UAV SUBSYSTEM INTERCONNECTS — packed arrays [N_UAVS-1:0] // ============================================================================= // ── Navigation state (9-state EKF output) ──────────────────────────────────── // 9 states × 32b = 288 bits per UAV // [0:95] = [px,py,pz] Q10.22 ¦ [96:191] = [vx,vy,vz] Q4.28 // [192:287] = [roll,pitch,yaw] Q3.29 logic [N_UAVS-1:0][287:0] uav_nav_state; // Full 9-state EKF output logic [N_UAVS-1:0] uav_nav_valid; // EKF output valid (10 Hz GPS gate) // ── Velocity command from convergence engine → UAV PID ─────────────────────── logic [N_UAVS-1:0][95:0] uav_vel_cmd; // [vx_cmd,vy_cmd,vz_cmd] Q4.28 logic [N_UAVS-1:0] uav_vel_valid; // Velocity command valid // ── Navigation FSM status ───────────────────────────────────────────────────── logic [N_UAVS-1:0][2:0] uav_fsm_state; // Waypoint FSM state (8-state) logic [N_UAVS-1:0] uav_armed; // 1 = UAV is armed logic [N_UAVS-1:0] uav_gps_lock; // 1 = GPS lock acquired // ── CA override from convergence engine → UAV PID ──────────────────────────── logic [N_UAVS-1:0] uav_ca_override;// 1 = APF force replaces formation cmd // ── AXI configuration inputs to UAV ────────────────────────────────────────── logic [N_UAVS-1:0][DATA_W-1:0] uav_pid_kp; // UAV PID Kp register logic [N_UAVS-1:0][DATA_W-1:0] uav_pid_ki; // UAV PID Ki register logic [N_UAVS-1:0][DATA_W-1:0] uav_pid_kd; // UAV PID Kd register // ============================================================================= // CONVERGENCE ENGINE INTERCONNECTS // ============================================================================= // ── Joint fleet state (all 6 vehicles) ─────────────────────────────────────── // CS nav state concatenated with UAV nav state — fed to Multi-Vehicle KF logic [N_SATS*191:0] cvg_cs_orbits; // CS orbit states (ECI) logic [N_UAVS*287:0] cvg_uav_states;// UAV nav states (9-state) // ── Formation controller outputs ───────────────────────────────────────────── // Per-follower velocity correction commands (5 followers) logic [(N_VEHICLES-1)*95:0] cvg_form_vel_cmd; // Formation velocity commands logic cvg_form_valid; // Formation output valid (10 Hz) // ── APF collision avoidance outputs ────────────────────────────────────────── logic [N_VEHICLES*95:0] cvg_apf_force; // APF repulsion forces [veh][xyz] logic [N_VEHICLES-1:0] cvg_threat_flags; // Per-vehicle threat indicator logic [N_VEHICLES-1:0] cvg_ca_override; // CA override per vehicle logic cvg_estop; // Fleet E-STOP (any threat miss) // ── Resource arbiter outputs ────────────────────────────────────────────────── logic [N_UAVS*95:0] cvg_uav_vel_out; // Final UAV velocity commands logic [N_SATS*63:0] cvg_cs_att_cmd; // Final CS attitude setpoints logic [N_VEHICLES-1:0] cvg_isl_enable; // ISL TX gate per vehicle slot // ── Geometry score ──────────────────────────────────────────────────────────── logic [DATA_W-1:0] cvg_score; // CVG_SCORE metric Q4.28 [0..1] logic cvg_score_valid; // Score valid (1 Hz) logic cvg_achieved; // FORMATION_ACHIEVED flag // ── AXI configuration for convergence engine ───────────────────────────────── logic [DATA_W-1:0] cvg_krep_reg; // APF k_rep gain register logic [DATA_W-1:0] cvg_kdamp_reg; // APF k_damp gain register logic [DATA_W-1:0] cvg_dsafe_reg; // APF d_safe threshold register logic [1:0] cvg_form_type; // Formation type: 0=TRI,1=LINE,2=V,3=SQ logic [DATA_W-1:0] cvg_tdm_slot_ms; // TDM slot duration register // ============================================================================= // SHARED MODULE INTERCONNECTS // ============================================================================= // ── Shared CORDIC outputs (time-multiplexed to all EKF / ADCS) ────────────── logic [DATA_W-1:0] cordic_sin_out; // CORDIC sin output Q1.15 logic [DATA_W-1:0] cordic_cos_out; // CORDIC cos output Q1.15 logic [DATA_W-1:0] cordic_theta_in; // CORDIC angle input Q1.15 logic cordic_valid_out; // CORDIC output valid (17 cycles) // ── Shared PID engine (TDM across N_UAVS) ──────────────────────────────────── logic [DATA_W-1:0] pid_error_in; // PID error input Q4.12 logic [DATA_W-1:0] pid_kp_in; // PID Kp gain Q4.12 logic [DATA_W-1:0] pid_ki_in; // PID Ki gain Q4.12 logic [DATA_W-1:0] pid_kd_in; // PID Kd gain Q4.12 logic [DATA_W-1:0] pid_out; // PID output Q4.12 logic [1:0] pid_sat_flags; // {sat_i, sat_p} saturation flags // ============================================================================= // FAULT / ESTOP INTERCONNECTS // ============================================================================= logic [31:0] fault_flags; // Global fault bitmask logic estop_n; // Fleet E-STOP (active-low) logic [N_VEHICLES-1:0] veh_wdt_kick; // Per-vehicle watchdog kick logic [N_VEHICLES-1:0] veh_wdt_expired; // Per-vehicle watchdog expired // ============================================================================= // TELEMETRY MUX INTERCONNECTS // ============================================================================= logic [DATA_W-1:0] tlm_cs_data [N_SATS]; // CS telemetry payload logic [DATA_W-1:0] tlm_uav_data[N_UAVS]; // UAV telemetry payload logic [DATA_W-1:0] tlm_cvg_data; // CVG telemetry payload logic tlm_frame_valid; // Downlink frame valid // ============================================================================= // DEBUG / LED WIRES // ============================================================================= logic heartbeat_1hz; // 1 Hz heartbeat from CE divider logic any_fault; // OR of all fault_flags bits logic any_gps_lock; // OR of uav_gps_lock[N_UAVS-1:0] logic any_threat; // OR of cvg_threat_flags // ============================================================================= // SUBMODULE INSTANTIATIONS // ============================================================================= // ───────────────────────────────────────────────────────────────────────────── // clk_wizard_inst : clk_wiz_0 // Vivado IP — MMCM0. // Generates sys_clk (50 MHz) and clk_100mhz (100 MHz) from LVDS input. // (* DONT_TOUCH *) prevents Vivado from merging or optimising BUFG primitives. // ───────────────────────────────────────────────────────────────────────────── (* DONT_TOUCH = "yes" *) clk_wiz_0 clk_wizard_inst ( // Differential input from board oscillator .clk_in1_p (sys_clk_p), // LVDS clock positive (50 MHz) .clk_in1_n (sys_clk_n), // LVDS clock negative (50 MHz) // Generated clock outputs .clk_out1 (sys_clk), // 50 MHz — primary domain .clk_out2 (clk_100mhz), // 100 MHz — SPI / CDC domain // Status .locked (mmcm_locked), // High when both PLLs are locked // Reset (active-high for Vivado IP) .reset (~rst_n) // Drive high to reset MMCM ); // ───────────────────────────────────────────────────────────────────────────── // ce_gen_inst : ce_strobe_gen // Master 26-bit counter on sys_clk generates one-cycle-wide CE strobes. // All downstream logic gates on CE rather than using divided clocks. // ───────────────────────────────────────────────────────────────────────────── ce_strobe_gen #( .CLK_HZ (SYS_CLK_HZ) // Reference clock frequency ) ce_gen_inst ( .clk (sys_clk), // 50 MHz primary clock .rst_n (global_rst_n), // Synchronous active-low reset // One-cycle-wide clock-enable outputs .ce_1khz (ce_1khz), // 1 kHz — UAV inner PID .ce_100hz (ce_100hz), // 100 Hz — EKF predict, ADCS .ce_50hz (ce_50hz), // 50 Hz — barometer .ce_10hz (ce_10hz), // 10 Hz — GPS update, formation, APF .ce_1hz (ce_1hz) // 1 Hz — orbit, CVG_SCORE, telemetry ); // ───────────────────────────────────────────────────────────────────────────── // fault_mgr_inst : global_fault_manager // Aggregates per-vehicle fault flags, runs watchdog timers, controls ESTOP. // ───────────────────────────────────────────────────────────────────────────── global_fault_manager #( .N_VEHICLES (N_VEHICLES), // Number of vehicles to monitor .CLK_HZ (SYS_CLK_HZ), // For watchdog timer calculation .WDT_CA_MS (1), // CA watchdog timeout — 1 ms (hard deadline) .WDT_VEH_MS (500) // Per-vehicle watchdog timeout — 500 ms ) fault_mgr_inst ( .clk (sys_clk), .rst_n (global_rst_n), // Watchdog kick inputs (each module must periodically assert for 1 cycle) .wdt_kick (veh_wdt_kick), // [N_VEHICLES-1:0] per-vehicle kick .ca_wdt_kick (cvg_form_valid), // CVG must update within 1 ms // XADC health inputs .xadc_temp_alarm (1'b0), // Temperature alarm from xadc_monitor .xadc_volt_alarm (1'b0), // Voltage alarm from xadc_monitor // Fault status outputs .fault_flags (fault_flags), // 32-bit fault bitmask .wdt_expired (veh_wdt_expired), // Per-vehicle watchdog expired .estop_n (estop_n) // Active-low fleet E-STOP ); // ───────────────────────────────────────────────────────────────────────────── // axi_ic_inst : axi_lite_interconnect // Routes AXI4-Lite slave port to per-subsystem register banks. // Decodes ADDR_W-bit address into CS / UAV / CVG / SYS bank selects. // ───────────────────────────────────────────────────────────────────────────── axi_lite_interconnect #( .DATA_W (DATA_W), // Data width (32-bit) .ADDR_W (ADDR_W), // Address width (12-bit = 4 KB) .N_SLAVES (N_SATS + N_UAVS + 2) // CS + UAV + CVG + SYS banks ) axi_ic_inst ( .clk (sys_clk), .rst_n (global_rst_n), // AXI4-Lite slave port (from external processor or internal cmd decoder) .s_awaddr (s_axi_awaddr), .s_awprot (s_axi_awprot), .s_awvalid (s_axi_awvalid), .s_awready (s_axi_awready), .s_wdata (s_axi_wdata), .s_wstrb (s_axi_wstrb), .s_wvalid (s_axi_wvalid), .s_wready (s_axi_wready), .s_bresp (s_axi_bresp), .s_bvalid (s_axi_bvalid), .s_bready (s_axi_bready), .s_araddr (s_axi_araddr), .s_arprot (s_axi_arprot), .s_arvalid (s_axi_arvalid), .s_arready (s_axi_arready), .s_rdata (s_axi_rdata), .s_rresp (s_axi_rresp), .s_rvalid (s_axi_rvalid), .s_rready (s_axi_rready), // Decoded slave write-enable strobes .cs_we (axi_cs_we), // [N_SATS-1:0] per CubeSat .uav_we (axi_uav_we), // [N_UAVS-1:0] per UAV .cvg_we (axi_cvg_we), // Convergence engine registers .sys_we (axi_sys_we), // System registers (watchdogs, fault cfg) // Internal master outputs to register banks .ic_wdata (ic_wdata), .ic_wstrb (ic_wstrb), .ic_rdata (ic_rdata) ); // ───────────────────────────────────────────────────────────────────────────── // cs_inst[N_SATS-1:0] : cubesat_subsystem // Generate block instantiates one cubesat_subsystem per satellite. // Each instance shares the clocks / CE strobes but has independent I/O. // ───────────────────────────────────────────────────────────────────────────── genvar cs_i; generate for (cs_i = 0; cs_i < N_SATS; cs_i = cs_i + 1) begin : cs_gen cubesat_subsystem #( .CS_ID (cs_i), // 0=CS1, 1=CS2, 2=CS3 .POS_FMT_INT (Q_POS_INT), // Q10.22 position integer bits .POS_FMT_FRAC (Q_POS_FRAC), // Q10.22 position fraction bits .ANG_FMT_INT (Q_ANG_INT), // Q3.29 angle integer bits .ANG_FMT_FRAC (Q_ANG_FRAC), // Q3.29 angle fraction bits .PID_FMT_INT (Q_PID_INT), // Q4.12 gain integer bits .PID_FMT_FRAC (Q_PID_FRAC), // Q4.12 gain fraction bits .CLK_HZ (SYS_CLK_HZ), // Clock frequency for timing .FAST_CLK_HZ (FAST_CLK_HZ), // SPI clock domain frequency .EKF_STATE_N (7), // EKF state count [qw,qx,qy,qz,bx,by,bz] .SCAN_H_STEPS (64), // Laser raster scan horizontal steps .SCAN_V_STEPS (32) // Laser raster scan vertical steps ) cs_inst ( // ── Clocks and resets ──────────────────────────────────────────── .sys_clk (sys_clk), // 50 MHz primary clock .clk_100mhz (clk_100mhz), // 100 MHz SPI + CDC clock .rst_n (global_rst_n), // Synchronised active-low reset .fast_rst_n (fast_rst_n), // 100 MHz domain reset // ── Clock-enable strobes ───────────────────────────────────────── .ce_100hz (ce_100hz), // ADCS + EKF predict rate .ce_1hz (ce_1hz), // Orbit propagator update rate // ── IMU SPI (gyro + accel) ─────────────────────────────────────── .gyro_sck (cs_gyro_sck [cs_i]), .gyro_mosi (cs_gyro_mosi [cs_i]), .gyro_miso (cs_gyro_miso [cs_i]), .gyro_cs_n (cs_gyro_cs_n [cs_i]), // ── Magnetometer I2C ───────────────────────────────────────────── .mag_sda (cs_mag_sda [cs_i]), .mag_scl (cs_mag_scl [cs_i]), // ── Sun sensor ADC SPI ─────────────────────────────────────────── .sun_sck (cs_sun_sck [cs_i]), .sun_mosi (cs_sun_mosi [cs_i]), .sun_miso (cs_sun_miso [cs_i]), .sun_cs_n (cs_sun_cs_n [cs_i]), // ── Reaction wheel SPI ─────────────────────────────────────────── .rw_sck (cs_rw_sck [cs_i]), .rw_mosi (cs_rw_mosi [cs_i]), .rw_miso (cs_rw_miso [cs_i]), .rw_cs_n (cs_rw_cs_n [cs_i]), // ── Magnetorquer PWM (3-axis) ──────────────────────────────────── .mtq_pwm (cs_mtq_pwm [cs_i*3 +: 3]), // 3-bit slice // ── Laser pointing gimbal ──────────────────────────────────────── .laser_az_step (cs_laser_az_step [cs_i]), .laser_az_dir (cs_laser_az_dir [cs_i]), .laser_el_step (cs_laser_el_step [cs_i]), .laser_el_dir (cs_laser_el_dir [cs_i]), .laser_en_n (cs_laser_en_n [cs_i]), // ── ISL parallel optical link ───────────────────────────────────── .isl_tx_data (cs_isl_tx_data [cs_i*8 +: 8]), .isl_tx_clk (cs_isl_tx_clk [cs_i]), .isl_tx_valid (cs_isl_tx_valid [cs_i]), // ── Internal data outputs ──────────────────────────────────────── .imu_data_out (cs_imu_data [cs_i]), // [95:0] Q4.28 IMU data .imu_valid (cs_imu_valid [cs_i]), // IMU data valid .quat_out (cs_quat [cs_i]), // [63:0] Q1.15 quaternion .quat_valid (cs_quat_valid [cs_i]), // Quat valid .orbit_out (cs_orbit [cs_i]), // [191:0] ECI orbit state .orbit_valid (cs_orbit_valid [cs_i]), // Orbit valid .adcs_state_out (cs_adcs_state [cs_i]), // [4:0] ADCS FSM state .adcs_fault (cs_adcs_fault [cs_i]), // ADCS fault flag .laser_state (cs_laser_state [cs_i]), // [2:0] Laser FSM state .laser_track (cs_laser_track [cs_i]), // Laser in TRACK state .wdt_kick (veh_wdt_kick [cs_i]), // Watchdog kick output // ── Commands from convergence engine ───────────────────────────── .attitude_cmd (cvg_cs_att_cmd [cs_i*64 +: 64]), // Q1.15 quat cmd .isl_enable (cvg_isl_enable [cs_i]), // ISL TX slot enable // ── AXI4-Lite configuration ────────────────────────────────────── .axi_we (axi_cs_we [cs_i]), // Write enable from AXI IC .axi_wdata (ic_wdata), // Configuration data .axi_kp (cs_kp_reg [cs_i]), // Proportional gain .axi_ki (cs_ki_reg [cs_i]), // Integral gain .axi_kd (cs_kd_reg [cs_i]), // Derivative gain // ── Fault interface ─────────────────────────────────────────────── .estop_n (estop_n) // Fleet E-STOP input ); end // for cs_i endgenerate // ───────────────────────────────────────────────────────────────────────────── // uav_inst[N_UAVS-1:0] : uav_subsystem // Generate block instantiates one uav_subsystem per drone. // Each instance has independent I/O but shares clocks, CE strobes, PID engine. // ───────────────────────────────────────────────────────────────────────────── genvar uav_i; generate for (uav_i = 0; uav_i < N_UAVS; uav_i = uav_i + 1) begin : uav_gen uav_subsystem #( .UAV_ID (uav_i), // 0=UAV1, 1=UAV2, 2=UAV3 .POS_FMT_INT (Q_POS_INT), // Q10.22 position integer bits .POS_FMT_FRAC (Q_POS_FRAC), // Q10.22 position fraction bits .VEL_FMT_INT (Q_VEL_INT), // Q4.28 velocity integer bits .VEL_FMT_FRAC (Q_VEL_FRAC), // Q4.28 velocity fraction bits .ANG_FMT_INT (Q_ANG_INT), // Q3.29 angle integer bits .ANG_FMT_FRAC (Q_ANG_FRAC), // Q3.29 angle fraction bits .PID_FMT_INT (Q_PID_INT), // Q4.12 PID gain integer bits .PID_FMT_FRAC (Q_PID_FRAC), // Q4.12 PID gain fraction bits .CLK_HZ (SYS_CLK_HZ), // Primary clock frequency .FAST_CLK_HZ (FAST_CLK_HZ), // SPI clock domain frequency .GPS_BAUD (GPS_BAUD), // GPS NMEA UART baud rate .GCS_BAUD (GS_BAUD), // MAVLink / GCS UART baud rate .EKF_STATE_N (9), // EKF states [px,py,pz,vx,vy,vz,r,p,y] .MAX_WAYPOINTS (32) // Maximum mission waypoints in BRAM ) uav_inst ( // ── Clocks and resets ──────────────────────────────────────────── .sys_clk (sys_clk), // 50 MHz primary clock .clk_100mhz (clk_100mhz), // 100 MHz SPI + CDC clock .rst_n (global_rst_n), .fast_rst_n (fast_rst_n), // ── Clock-enable strobes ───────────────────────────────────────── .ce_1khz (ce_1khz), // Inner PID loop (1 kHz) .ce_100hz (ce_100hz), // IMU EKF predict (100 Hz) .ce_50hz (ce_50hz), // Barometer update (50 Hz) .ce_10hz (ce_10hz), // GPS update (10 Hz) // ── IMU SPI (ICM-42688) ────────────────────────────────────────── .imu_sck (uav_imu_sck [uav_i]), .imu_mosi (uav_imu_mosi [uav_i]), .imu_miso (uav_imu_miso [uav_i]), .imu_cs_n (uav_imu_cs_n [uav_i]), // ── GPS UART (NEO-M9N) ─────────────────────────────────────────── .gps_rx (uav_gps_rx [uav_i]), .gps_tx (uav_gps_tx [uav_i]), .gps_pps (uav_gps_pps [uav_i]), // ── Barometer SPI (BMP388) ─────────────────────────────────────── .baro_sck (uav_baro_sck [uav_i]), .baro_mosi (uav_baro_mosi [uav_i]), .baro_miso (uav_baro_miso [uav_i]), .baro_cs_n (uav_baro_cs_n [uav_i]), // ── ToF echo sensor ────────────────────────────────────────────── .tof_trig (uav_tof_trig [uav_i]), .tof_echo (uav_tof_echo [uav_i]), // ── ESC PWM (4 motors) ─────────────────────────────────────────── .motor_pwm (uav_motor_pwm [uav_i*4 +: 4]), // 4-bit slice // ── Servo PWM ──────────────────────────────────────────────────── .servo_pwm (uav_servo_pwm [uav_i*2 +: 2]), // 2-bit slice // ── MAVLink UART ───────────────────────────────────────────────── .mav_tx (uav_mav_tx [uav_i]), .mav_rx (uav_mav_rx [uav_i]), // ── Navigation state outputs ───────────────────────────────────── .nav_state_out (uav_nav_state [uav_i]), // [287:0] 9-state EKF .nav_valid (uav_nav_valid [uav_i]), // EKF valid .fsm_state_out (uav_fsm_state [uav_i]), // [2:0] Nav FSM state .armed (uav_armed [uav_i]), // Armed status .gps_lock (uav_gps_lock [uav_i]), // GPS lock status .wdt_kick (veh_wdt_kick [N_SATS + uav_i]), // WDT kick // ── Commands from convergence engine ───────────────────────────── .vel_cmd_in (cvg_uav_vel_out [uav_i*96 +: 96]), // Q4.28 velocity .vel_cmd_valid (cvg_form_valid), // Command valid .ca_override (cvg_ca_override [N_SATS + uav_i]), // APF override // ── AXI4-Lite configuration ────────────────────────────────────── .axi_we (axi_uav_we [uav_i]), .axi_wdata (ic_wdata), .axi_pid_kp (uav_pid_kp [uav_i]), .axi_pid_ki (uav_pid_ki [uav_i]), .axi_pid_kd (uav_pid_kd [uav_i]), // ── Shared DSP resources ───────────────────────────────────────── // PID engine TDM — UAV asserts request; arbitrated at top level .pid_error_out (pid_error_in), // Error to shared PID DSP .pid_kp_out (pid_kp_in), // Kp to shared PID DSP .pid_ki_out (pid_ki_in), // Ki to shared PID DSP .pid_kd_out (pid_kd_in), // Kd to shared PID DSP .pid_result_in (pid_out), // Result from shared PID DSP // ── Fault interface ─────────────────────────────────────────────── .estop_n (estop_n) ); end // for uav_i endgenerate // ───────────────────────────────────────────────────────────────────────────── // conv_inst : convergence_engine // Multi-Vehicle Kalman Filter + Formation Controller + APF + Resource Arbiter. // Processes all N_VEHICLES position/velocity states at 10 Hz. // ───────────────────────────────────────────────────────────────────────────── convergence_engine #( .N_SATS (N_SATS), // Number of CubeSats .N_UAVS (N_UAVS), // Number of UAVs .N_VEHICLES (N_VEHICLES), // Total fleet size .POS_FMT_INT (Q_POS_INT), // Q10.22 position format .POS_FMT_FRAC (Q_POS_FRAC), .VEL_FMT_INT (Q_VEL_INT), // Q4.28 velocity format .VEL_FMT_FRAC (Q_VEL_FRAC), .BASELINE_M (BASELINE_M), // Formation baseline (50 m) .D_SAFE_M (D_SAFE_M), // APF safety radius (10 m) .N_PAIRS (N_VEHICLES*(N_VEHICLES-1)/2), // 15 pairs for 6 vehicles .MVK_STATE_N (6), // Per-vehicle KF state size [px,py,pz,vx,vy,vz] .TDM_SLOTS (N_VEHICLES) // Resource arbiter TDM slots ) conv_inst ( // ── Clocks and resets ──────────────────────────────────────────────────── .clk (sys_clk), .rst_n (global_rst_n), // ── Clock-enable strobes ───────────────────────────────────────────────── .ce_10hz (ce_10hz), // Formation + APF + MVK update rate .ce_1hz (ce_1hz), // CVG_SCORE + orbit epoch update // ── CubeSat inputs (orbit states + ISL received data) ──────────────────── .cs_orbit_in (cs_orbit), // [N_SATS][191:0] ECI orbit states .cs_orbit_valid (cs_orbit_valid), // [N_SATS-1:0] orbit valid flags .cs_isl_rx_data (cs_isl_rx_data), // [N_SATS*8-1:0] ISL parallel RX .cs_isl_rx_clk (cs_isl_rx_clk), // [N_SATS-1:0] ISL RX clock .cs_isl_rx_valid(cs_isl_rx_valid), // [N_SATS-1:0] ISL RX valid // ── UAV inputs (9-state EKF navigation states) ──────────────────────────── .uav_nav_in (uav_nav_state), // [N_UAVS][287:0] navigation states .uav_nav_valid (uav_nav_valid), // [N_UAVS-1:0] navigation valid flags // ── Formation control outputs → UAVs ────────────────────────────────────── .uav_vel_cmd_out (cvg_uav_vel_out), // [N_UAVS*96-1:0] velocity commands .form_cmd_valid (cvg_form_valid), // Formation command valid // ── ADCS attitude commands → CubeSats ──────────────────────────────────── .cs_att_cmd_out (cvg_cs_att_cmd), // [N_SATS*64-1:0] attitude setpoints .isl_enable_out (cvg_isl_enable), // [N_VEHICLES-1:0] ISL TX slot enable // ── Collision avoidance outputs ─────────────────────────────────────────── .ca_override_out (cvg_ca_override), // [N_VEHICLES-1:0] per-vehicle CA flag .threat_flags_out(cvg_threat_flags),// [N_VEHICLES-1:0] threat detected flags .apf_force_out (cvg_apf_force), // [N_VEHICLES*96-1:0] APF force vectors .estop_out (cvg_estop), // CVG-generated E-STOP (CA deadline miss) // ── Geometry score ──────────────────────────────────────────────────────── .cvg_score_out (cvg_score), // Q4.28 formation quality metric .cvg_score_valid (cvg_score_valid), // Score valid (1 Hz) .cvg_achieved (cvg_achieved), // FORMATION_ACHIEVED flag // ── AXI4-Lite configuration ─────────────────────────────────────────────── .axi_we (axi_cvg_we), .axi_wdata (ic_wdata), .krep_reg (cvg_krep_reg), // APF repulsion gain .kdamp_reg (cvg_kdamp_reg), // APF damping gain .dsafe_reg (cvg_dsafe_reg), // APF safety distance .form_type_reg (cvg_form_type), // 2-bit geometry select .tdm_slot_reg (cvg_tdm_slot_ms), // TDM slot duration (ms) // ── Fault interface ─────────────────────────────────────────────────────── .estop_n_in (estop_n), // Fleet E-STOP input .wdt_kick (veh_wdt_kick[N_VEHICLES-1]) // CVG watchdog kick ); // ───────────────────────────────────────────────────────────────────────────── // pid_shared_inst : pid_engine // Single shared Q4.12 PID engine time-multiplexed across all N_UAVS. // Contains 4 DSP48E1 slices: P-term, I-term ×2 (MACC), D-term. // ───────────────────────────────────────────────────────────────────────────── pid_engine #( .FMT_INT (Q_PID_INT), // Q4.12 integer bits .FMT_FRAC (Q_PID_FRAC), // Q4.12 fractional bits .N_AXES (6), // 6-DOF (roll/pitch/yaw rate + angles + alt) .SHARED (1), // 1 = time-multiplexed TDM mode .TDM_SLOTS (N_UAVS) // One slot per UAV ) pid_shared_inst ( .clk (sys_clk), .rst_n (global_rst_n), .ce_1khz (ce_1khz), // Inner-loop clock enable .ce_100hz (ce_100hz), // Outer-loop clock enable // Time-multiplexed data bus (driven by UAV subsystems via arbitration) .error_in (pid_error_in), // Q4.12 error input .kp_in (pid_kp_in), // Q4.12 Kp gain .ki_in (pid_ki_in), // Q4.12 Ki gain (×Ts absorbed) .kd_in (pid_kd_in), // Q4.12 Kd gain (/Ts absorbed) // Output .pid_out (pid_out), // Q4.12 PID control output .sat_flags (pid_sat_flags) // {sat_i, sat_p} overflow flags ); // ───────────────────────────────────────────────────────────────────────────── // cordic_inst : cordic_sincos // 16-stage pipelined CORDIC for sin/cos (rotation mode) and atan2 (vectoring). // Zero DSP48E1. Shared across all EKF rotation-matrix computations. // Throughput: 1 result per cycle; latency: 17 cycles. // ───────────────────────────────────────────────────────────────────────────── cordic_sincos #( .N_ITER (16), // 16 iterations → < 1 LSB Q1.15 error .FMT (1), // 1 = Q1.15 I/O format .PIPELINE (1) // 1 = fully pipelined (1 result/cycle) ) cordic_inst ( .clk (sys_clk), .rst_n (global_rst_n), // Rotation mode inputs .theta_in (cordic_theta_in), // Q1.15 angle (radians/π × 32768) // Outputs .sin_out (cordic_sin_out), // Q1.15 sin(theta) .cos_out (cordic_cos_out), // Q1.15 cos(theta) .valid_out (cordic_valid_out) // Output valid (asserts 17 cycles after input) ); // ───────────────────────────────────────────────────────────────────────────── // tlm_mux_inst : telemetry_downlink_mux // Aggregates CS + UAV + CVG telemetry streams into 1-second superframe. // Outputs to ground station UART (115200 baud). // ───────────────────────────────────────────────────────────────────────────── telemetry_downlink_mux #( .N_CS (N_SATS), .N_UAV (N_UAVS), .BAUD (GS_BAUD), .CLK_HZ (SYS_CLK_HZ), .FRAME_BYTES (50) // 50-byte telemetry packet per source ) tlm_mux_inst ( .clk (sys_clk), .rst_n (global_rst_n), .ce_1hz (ce_1hz), // Superframe epoch // CS telemetry inputs .cs_quat_in (cs_quat), // [N_SATS][63:0] quaternion .cs_orbit_in (cs_orbit), // [N_SATS][191:0] orbit .cs_state_in (cs_adcs_state), // [N_SATS][4:0] ADCS state // UAV telemetry inputs .uav_nav_in (uav_nav_state), // [N_UAVS][287:0] nav state .uav_fsm_in (uav_fsm_state), // [N_UAVS][2:0] FSM state // CVG telemetry inputs .cvg_score_in (cvg_score), // Q4.28 CVG_SCORE .cvg_form_type(cvg_form_type), // Current formation type .fault_in (fault_flags), // 32-bit fault bitmask // UART output .uart_tx (gnd_uart_tx), // GS downlink TX .uart_rx (gnd_uart_rx), // GS uplink RX (pass-through to AXI IC) .frame_valid (tlm_frame_valid) // Downlink frame valid pulse ); // ───────────────────────────────────────────────────────────────────────────── // debug_inst : debug_ila_wrapper // Vivado Integrated Logic Analyser. Instantiated when DEBUG_EN=1. // (* DONT_TOUCH *) prevents removal during synthesis optimisation passes. // REMOVE for flight build: set DEBUG_EN=0 or exclude from synthesis scripts. // ───────────────────────────────────────────────────────────────────────────── generate if (DEBUG_EN) begin : debug_gen (* DONT_TOUCH = "yes" *) debug_ila_wrapper #( .N_PROBES (8), // Number of ILA probe signals .PROBE_W (DATA_W), // Probe width (32-bit) .DEPTH (1024) // Capture depth (1024 samples) ) debug_inst ( .clk (sys_clk), // ILA probe inputs — key signals for integration verification .probe0 (cs_quat[0]), // CS1 attitude quaternion .probe1 (uav_nav_state[0][95:64]), // UAV1 position px .probe2 (cvg_score), // Formation quality metric .probe3 (fault_flags), // Global fault register .probe4 ({28'b0, cvg_ca_override}), // CA override flags .probe5 ({31'b0, cvg_achieved}), // Formation achieved flag .probe6 (cvg_uav_vel_out[0*96 +: 32]), // UAV1 vx command .probe7 ({28'b0, uav_fsm_state[0], uav_armed[0]}) // UAV1 FSM ); end // debug_gen endgenerate // ============================================================================= // LED AND TEST-POINT ASSIGNMENTS // (Continuous combinational assignments — no logic in module body) // ============================================================================= assign led[0] = mmcm_locked; // PLL locked indicator assign led[1] = cvg_achieved; // Formation achieved assign led[2] = ¦cvg_threat_flags; // Any CA threat active assign led[3] = ¦uav_gps_lock; // Any GPS lock assign led[4] = ~estop_n; // E-STOP asserted (active) assign led[5] = tlm_frame_valid; // Downlink activity assign led[6] = heartbeat_1hz; // 1 Hz blink (from CE) assign led[7] = ¦fault_flags; // Any fault present assign tp[0] = sys_clk; // 50 MHz scope reference assign tp[1] = ce_100hz; // 100 Hz CE strobe assign tp[2] = cvg_score_valid; // CVG score update pulse assign tp[3] = estop_n; // E-STOP monitor // ============================================================================= // DERIVED INTERNAL COMBINATIONAL SIGNALS // ============================================================================= // Global reset holds until MMCM is locked. // Must be synchronised into each clock domain; assign here for sys_clk domain. assign global_rst_n = rst_n & mmcm_locked; assign fast_rst_n = rst_n & mmcm_locked; // Same lock; domain sync in submodules // 1 Hz heartbeat (directly from CE strobe — latches in child module if needed) assign heartbeat_1hz = ce_1hz; endmodule // convergence_mvp_top `default_nettype wire // ============================================================================= // END OF FILE — convergence_mvp_top.sv // =============================================================================