# Convergence MVP — Complete Module Hierarchy

convergence_mvp_top.v  |  Artix-7 XC7A35T  |  Full elaborated hierarchy with file paths, instance counts, parameters, and resource estimates.

convergence_mvp_top ── Complete Module Hierarchy Tree Format: module_name ×N (instance count if >1) [PRIM]=primitive [IP]=Vivado IP convergence_mvp_top [TOP] rtl/top/convergence_mvp_top.v [×1] LUT:9862 FF:14180 DSP:44 BRAM:24 ├── clk_wiz_0 [IP] │ ip/clk_wiz_0/clk_wiz_0.xci [×1] LUT:8 FF:16 DSP:0 BRAM:0 │ ├── MMCM_ADV [PRIM] │ │ (Xilinx primitive) [×1] LUT:0 FF:0 DSP:0 BRAM:0 │ ├── IBUFGDS [PRIM] │ │ (Xilinx primitive) [×1] LUT:0 FF:0 DSP:0 BRAM:0 │ └── BUFG ×2 [PRIM] │ (Xilinx primitive) [×2] LUT:0 FF:0 DSP:0 BRAM:0 ├── axi_lite_interconnect │ rtl/interconnect/axi_lite_interconnect.v [×1] LUT:128 FF:160 DSP:0 BRAM:0 │ ├── gs_uart_cmd_decoder │ │ rtl/interconnect/gs_uart_cmd_decoder.v [×1] LUT:48 FF:64 DSP:0 BRAM:0 │ └── axi_register_bank ×9 │ rtl/interconnect/axi_register_bank.v [×9] LUT:16 FF:20 DSP:0 BRAM:0 ├── ce_strobe_gen │ rtl/system/ce_strobe_gen.v [×1] LUT:48 FF:64 DSP:0 BRAM:0 ├── global_fault_manager │ rtl/system/global_fault_manager.v [×1] LUT:96 FF:128 DSP:0 BRAM:1 │ ├── xadc_monitor │ │ rtl/system/xadc_monitor.v [×1] LUT:64 FF:80 DSP:0 BRAM:0 │ │ └── XADC [PRIM] │ │ (Xilinx primitive) [×1] LUT:0 FF:0 DSP:0 BRAM:0 │ └── watchdog_timer ×9 │ rtl/system/watchdog_timer.v [×9] LUT:18 FF:26 DSP:0 BRAM:0 ├── cubesat_subsystem ×3 │ rtl/cubesat/cubesat_subsystem.v [×3] LUT:1180 FF:1600 DSP:6 BRAM:1 │ ├── spi_master │ │ rtl/shared/spi_master.v [×1] LUT:20 FF:32 DSP:0 BRAM:0 │ │ └── spi_shift_reg │ │ rtl/shared/spi_shift_reg.v [×1] LUT:10 FF:18 DSP:0 BRAM:0 │ ├── async_fifo │ │ rtl/shared/async_fifo.v [×1] LUT:48 FF:96 DSP:0 BRAM:0 │ │ ├── gray_encoder ×2 │ │ │ rtl/shared/gray_encoder.v [×2] LUT:4 FF:4 DSP:0 BRAM:0 │ │ └── sync_2ff ×2 │ │ rtl/shared/sync_2ff.v [×2] LUT:0 FF:8 DSP:0 BRAM:0 │ ├── i2c_master │ │ rtl/shared/i2c_master.v [×1] LUT:16 FF:20 DSP:0 BRAM:0 │ ├── sun_sensor_adc │ │ rtl/cubesat/sun_sensor_adc.v [×1] LUT:24 FF:36 DSP:0 BRAM:0 │ ├── quat_ekf_7state │ │ rtl/cubesat/quat_ekf_7state.v [×1] LUT:380 FF:520 DSP:1 BRAM:1 │ │ ├── quat_multiply ×2 │ │ │ rtl/shared/quat_multiply.v [×2] LUT:48 FF:64 DSP:0 BRAM:0 │ │ ├── quat_conjugate ×2 │ │ │ rtl/shared/quat_conjugate.v [×2] LUT:4 FF:8 DSP:0 BRAM:0 │ │ ├── quat_normalize │ │ │ rtl/shared/quat_normalize.v [×1] LUT:60 FF:80 DSP:1 BRAM:0 │ │ └── ekf_covariance │ │ rtl/shared/ekf_covariance.v [×1] LUT:120 FF:160 DSP:0 BRAM:1 │ ├── adcs_controller │ │ rtl/cubesat/adcs_controller.v [×1] LUT:200 FF:280 DSP:2 BRAM:0 │ │ ├── adcs_fsm │ │ │ rtl/cubesat/adcs_fsm.v [×1] LUT:40 FF:60 DSP:0 BRAM:0 │ │ ├── quat_pd_law │ │ │ rtl/cubesat/quat_pd_law.v [×1] LUT:80 FF:100 DSP:2 BRAM:0 │ │ ├── rw_spi_ctrl │ │ │ rtl/cubesat/rw_spi_ctrl.v [×1] LUT:48 FF:64 DSP:0 BRAM:0 │ │ └── magnetorquer_pwm │ │ rtl/cubesat/magnetorquer_pwm.v [×1] LUT:24 FF:36 DSP:0 BRAM:0 │ ├── sgp4_orbit_propagator │ │ rtl/cubesat/sgp4_orbit_propagator.v [×1] LUT:280 FF:360 DSP:3 BRAM:0 │ │ ├── sgp4_kepler_solve │ │ │ rtl/cubesat/sgp4_kepler_solve.v [×1] LUT:120 FF:160 DSP:3 BRAM:0 │ │ ├── eci_to_lvlh │ │ │ rtl/cubesat/eci_to_lvlh.v [×1] LUT:80 FF:100 DSP:0 BRAM:0 │ │ └── tle_bram_store │ │ rtl/cubesat/tle_bram_store.v [×1] LUT:20 FF:28 DSP:0 BRAM:0 │ ├── laser_pointing_fsm │ │ rtl/cubesat/laser_pointing_fsm.v [×1] LUT:200 FF:240 DSP:0 BRAM:0 │ │ ├── gimbal_stepper │ │ │ rtl/cubesat/gimbal_stepper.v [×1] LUT:60 FF:80 DSP:0 BRAM:0 │ │ ├── signal_strength_adc │ │ │ rtl/cubesat/signal_strength_adc.v [×1] LUT:24 FF:32 DSP:0 BRAM:0 │ │ └── raster_scan_ctrl │ │ rtl/cubesat/raster_scan_ctrl.v [×1] LUT:48 FF:60 DSP:0 BRAM:0 │ └── ccsds_telemetry_encoder │ rtl/cubesat/ccsds_telemetry_encoder.v [×1] LUT:120 FF:160 DSP:0 BRAM:0 │ ├── crc16_ccitt │ │ rtl/shared/crc16_ccitt.v [×1] LUT:32 FF:16 DSP:0 BRAM:0 │ ├── superframe_arbiter │ │ rtl/cubesat/superframe_arbiter.v [×1] LUT:32 FF:40 DSP:0 BRAM:0 │ └── isl_parallel_tx │ rtl/cubesat/isl_parallel_tx.v [×1] LUT:16 FF:24 DSP:0 BRAM:0 ├── uav_subsystem ×3 │ rtl/uav/uav_subsystem.v [×3] LUT:860 FF:1180 DSP:3 BRAM:3 │ ├── spi_master │ │ rtl/shared/spi_master.v [×1] LUT:20 FF:32 DSP:0 BRAM:0 │ │ └── spi_shift_reg │ │ rtl/shared/spi_shift_reg.v [×1] LUT:10 FF:18 DSP:0 BRAM:0 │ ├── async_fifo │ │ rtl/shared/async_fifo.v [×1] LUT:48 FF:96 DSP:0 BRAM:0 │ │ ├── gray_encoder ×2 │ │ │ rtl/shared/gray_encoder.v [×2] LUT:4 FF:4 DSP:0 BRAM:0 │ │ └── sync_2ff ×2 │ │ rtl/shared/sync_2ff.v [×2] LUT:0 FF:8 DSP:0 BRAM:0 │ ├── ekf_9state_navigator │ │ rtl/uav/ekf_9state_navigator.v [×1] LUT:360 FF:480 DSP:3 BRAM:1 │ │ ├── cordic_sincos │ │ │ rtl/shared/cordic_sincos.v [×1] LUT:200 FF:1056 DSP:0 BRAM:0 │ │ ├── rotation_matrix_build │ │ │ rtl/uav/rotation_matrix_build.v [×1] LUT:36 FF:48 DSP:0 BRAM:0 │ │ ├── ekf_predict_step │ │ │ rtl/uav/ekf_predict_step.v [×1] LUT:80 FF:110 DSP:3 BRAM:0 │ │ ├── ekf_gps_update │ │ │ rtl/uav/ekf_gps_update.v [×1] LUT:60 FF:80 DSP:0 BRAM:0 │ │ ├── ekf_baro_update │ │ │ rtl/uav/ekf_baro_update.v [×1] LUT:24 FF:32 DSP:0 BRAM:0 │ │ └── ekf_covariance │ │ rtl/shared/ekf_covariance.v [×1] LUT:140 FF:180 DSP:0 BRAM:1 │ ├── gps_nmea_parser │ │ rtl/shared/gps_nmea_parser.v [×1] LUT:50 FF:68 DSP:0 BRAM:0 │ │ ├── uart_rx │ │ │ rtl/shared/uart_rx.v [×1] LUT:16 FF:24 DSP:0 BRAM:0 │ │ └── nmea_sentence_parser │ │ rtl/shared/nmea_sentence_parser.v [×1] LUT:32 FF:40 DSP:0 BRAM:0 │ ├── baro_spi_master │ │ rtl/uav/baro_spi_master.v [×1] LUT:24 FF:36 DSP:0 BRAM:0 │ ├── tof_echo_capture │ │ rtl/uav/tof_echo_capture.v [×1] LUT:16 FF:24 DSP:0 BRAM:0 │ ├── pid_6dof_controller │ │ rtl/uav/pid_6dof_controller.v [×1] LUT:100 FF:140 DSP:0 BRAM:1 │ │ ├── pid_engine │ │ │ rtl/shared/pid_engine.v [×1] LUT:40 FF:56 DSP:0 BRAM:0 │ │ ├── motor_mix_matrix │ │ │ rtl/uav/motor_mix_matrix.v [×1] LUT:24 FF:32 DSP:0 BRAM:0 │ │ ├── esc_pwm_gen ×4 │ │ │ rtl/uav/esc_pwm_gen.v [×4] LUT:8 FF:12 DSP:0 BRAM:0 │ │ └── rate_limiter ×6 │ │ rtl/shared/rate_limiter.v [×6] LUT:6 FF:8 DSP:0 BRAM:0 │ ├── waypoint_nav_fsm │ │ rtl/uav/waypoint_nav_fsm.v [×1] LUT:200 FF:280 DSP:0 BRAM:1 │ │ ├── nav_fsm_core │ │ │ rtl/uav/nav_fsm_core.v [×1] LUT:80 FF:100 DSP:0 BRAM:0 │ │ ├── waypoint_store │ │ │ rtl/uav/waypoint_store.v [×1] LUT:20 FF:28 DSP:0 BRAM:1 │ │ ├── geofence_check │ │ │ rtl/uav/geofence_check.v [×1] LUT:48 FF:60 DSP:0 BRAM:0 │ │ └── preflight_check │ │ rtl/uav/preflight_check.v [×1] LUT:32 FF:40 DSP:0 BRAM:0 │ └── mavlink_v2_stack │ rtl/uav/mavlink_v2_stack.v [×1] LUT:200 FF:280 DSP:0 BRAM:1 │ ├── mavlink_rx_parser │ │ rtl/uav/mavlink_rx_parser.v [×1] LUT:60 FF:80 DSP:0 BRAM:0 │ ├── mavlink_tx_builder │ │ rtl/uav/mavlink_tx_builder.v [×1] LUT:48 FF:64 DSP:0 BRAM:0 │ ├── uart_rx │ │ rtl/shared/uart_rx.v [×1] LUT:16 FF:24 DSP:0 BRAM:0 │ ├── uart_tx │ │ rtl/shared/uart_tx.v [×1] LUT:16 FF:24 DSP:0 BRAM:0 │ ├── crc16_mcrf4xx ×2 │ │ rtl/shared/crc16_mcrf4xx.v [×2] LUT:32 FF:16 DSP:0 BRAM:0 │ └── mavlink_fifo │ rtl/uav/mavlink_fifo.v [×1] LUT:20 FF:28 DSP:0 BRAM:1 ├── convergence_engine │ rtl/convergence/convergence_engine.v [×1] LUT:1400 FF:1820 DSP:13 BRAM:4 │ ├── isl_parallel_rx ×3 │ │ rtl/convergence/isl_parallel_rx.v [×3] LUT:24 FF:36 DSP:0 BRAM:0 │ │ └── IDDR ×8 [PRIM] │ │ (Xilinx primitive) [×8] LUT:0 FF:0 DSP:0 BRAM:0 │ ├── multi_vehicle_kalman │ │ rtl/convergence/multi_vehicle_kalman.v [×1] LUT:480 FF:640 DSP:4 BRAM:2 │ │ ├── mvk_predict_step │ │ │ rtl/convergence/mvk_predict_step.v [×1] LUT:120 FF:160 DSP:4 BRAM:0 │ │ ├── mvk_isl_fusion │ │ │ rtl/convergence/mvk_isl_fusion.v [×1] LUT:80 FF:100 DSP:0 BRAM:0 │ │ ├── mvk_covariance │ │ │ rtl/convergence/mvk_covariance.v [×1] LUT:160 FF:200 DSP:0 BRAM:2 │ │ └── delta_r_compute │ │ rtl/convergence/delta_r_compute.v [×1] LUT:80 FF:100 DSP:0 BRAM:0 │ ├── formation_controller │ │ rtl/convergence/formation_controller.v [×1] LUT:300 FF:380 DSP:2 BRAM:1 │ │ ├── formation_lut_rom │ │ │ rtl/convergence/formation_lut_rom.v [×1] LUT:60 FF:80 DSP:0 BRAM:0 │ │ ├── r_f2ned_rotate │ │ │ rtl/convergence/r_f2ned_rotate.v [×1] LUT:40 FF:56 DSP:2 BRAM:0 │ │ ├── lerp_interpolator │ │ │ rtl/convergence/lerp_interpolator.v [×1] LUT:60 FF:80 DSP:0 BRAM:0 │ │ └── formation_error_mon │ │ rtl/convergence/formation_error_mon.v [×1] LUT:80 FF:100 DSP:0 BRAM:0 │ ├── apf_collision_avoidance │ │ rtl/convergence/apf_collision_avoidance.v [×1] LUT:320 FF:420 DSP:6 BRAM:1 │ │ ├── apf_pair_force │ │ │ rtl/convergence/apf_pair_force.v [×1] LUT:120 FF:160 DSP:6 BRAM:0 │ │ │ └── nr_inv_sqrt │ │ │ rtl/shared/nr_inv_sqrt.v [×1] LUT:48 FF:64 DSP:3 BRAM:0 │ │ ├── apf_force_summer │ │ │ rtl/convergence/apf_force_summer.v [×1] LUT:48 FF:60 DSP:0 BRAM:0 │ │ ├── apf_damping │ │ │ rtl/convergence/apf_damping.v [×1] LUT:36 FF:48 DSP:0 BRAM:0 │ │ └── inv_sqrt_lut │ │ rtl/shared/inv_sqrt_lut.v [×1] LUT:24 FF:0 DSP:0 BRAM:1 │ ├── resource_arbiter │ │ rtl/convergence/resource_arbiter.v [×1] LUT:160 FF:200 DSP:0 BRAM:1 │ │ ├── tdm_fsm │ │ │ rtl/convergence/tdm_fsm.v [×1] LUT:48 FF:60 DSP:0 BRAM:0 │ │ ├── ca_preempt_logic │ │ │ rtl/convergence/ca_preempt_logic.v [×1] LUT:32 FF:40 DSP:0 BRAM:0 │ │ └── arbiter_log_bram │ │ rtl/convergence/arbiter_log_bram.v [×1] LUT:20 FF:28 DSP:0 BRAM:1 │ └── geometry_score │ rtl/convergence/geometry_score.v [×1] LUT:60 FF:80 DSP:1 BRAM:0 ├── pid_engine_shared │ rtl/shared/pid_engine.v [×1] LUT:120 FF:160 DSP:4 BRAM:0 │ ├── pid_p_term │ │ rtl/shared/pid_p_term.v [×1] LUT:20 FF:32 DSP:1 BRAM:0 │ ├── pid_i_term │ │ rtl/shared/pid_i_term.v [×1] LUT:24 FF:40 DSP:2 BRAM:0 │ └── pid_d_term │ rtl/shared/pid_d_term.v [×1] LUT:20 FF:32 DSP:1 BRAM:0 ├── cordic_sincos_shared │ rtl/shared/cordic_sincos.v [×1] LUT:200 FF:1056 DSP:0 BRAM:0 │ └── cordic_atan2 │ rtl/shared/cordic_atan2.v [×1] LUT:200 FF:1056 DSP:0 BRAM:0 ├── telemetry_downlink_mux │ rtl/system/telemetry_downlink_mux.v [×1] LUT:100 FF:140 DSP:0 BRAM:1 │ ├── uart_tx │ │ rtl/shared/uart_tx.v [×1] LUT:16 FF:24 DSP:0 BRAM:0 │ └── tlm_frame_builder │ rtl/system/tlm_frame_builder.v [×1] LUT:48 FF:60 DSP:0 BRAM:0 └── debug_ila_wrapper rtl/debug/debug_ila_wrapper.v [×1] LUT:100 FF:200 DSP:0 BRAM:1 └── ila_0 [IP] ip/ila_0/ila_0.xci [×1] LUT:100 FF:200 DSP:0 BRAM:1

## Module List — All Synthesisable Modules

Every module in the design with source file path, instance count (red = multiply-instantiated), key parameters, and per-instance resource estimates. Xilinx primitives excluded.

| Module Name | Source File | ×N | Key Parameters | LUTs | FFs | DSPs | BRAM18 | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| convergence_mvp_top | rtl/top/convergence_mvp_top.v | 1 | NONE (top level) | 9862 | 14180 | 44 | 24 | Design root. Instantiates clock wizard, 3×CS, 3×UAV, convergence engine, AXI interconnect, fault manager. |
| clk_wiz_0 | ip/clk_wiz_0/clk_wiz_0.xci | 1 | CLKFBOUT_MULT_F=16 ¦ DIVCLK=1 ¦ CLKOUT0_DIV=8 | 8 | 16 | — | — | Vivado IP: MMCM0. Generates 50 MHz (sys_clk) and 100 MHz (clk_100mhz). Lock → global_rst_n. |
| axi_lite_interconnect | rtl/interconnect/axi_lite_interconnect.v | 1 | ADDR_WIDTH=32 ¦ DATA_WIDTH=32 ¦ NUM_SLAVES=9 | 128 | 160 | — | — | AXI4-Lite crossbar. GS UART RX → register decode. 9 slave banks (CS1-3, UAV1-3, CVG, SYS). |
| gs_uart_cmd_decoder | rtl/interconnect/gs_uart_cmd_decoder.v | 1 | BAUD=115200 ¦ CLK_HZ=50000000 | 48 | 64 | — | — | Receives GS uplink commands. Validates CCSDS header+CRC. Drives AXI4-Lite write channel. |
| axi_register_bank | rtl/interconnect/axi_register_bank.v | 9 | NUM_REGS=8 ¦ ADDR_BASE=per_bank | 16 | 20 | — | — | Per-subsystem 8-register bank. CS1/2/3: PID gains, EKF noise, CS config. UAV: gains+nav. CVG: k_rep, TDM timing. |
| ce_strobe_gen | rtl/system/ce_strobe_gen.v | 1 | CLK_HZ=50_000_000 | 48 | 64 | — | — | 26-bit master counter decodes 1kHz/100Hz/50Hz/10Hz/1Hz CE strobes. All sys_clk domain. |
| global_fault_manager | rtl/system/global_fault_manager.v | 1 | N_VEHICLES=6 ¦ WDT_BITS=26 | 96 | 128 | — | 1 | Cross-subsystem fault OR tree. CA watchdog ≤1ms. ESTOP latch (manual clear only). BRAM18 event log. |
| xadc_monitor | rtl/system/xadc_monitor.v | 1 | TEMP_THRESH=85 ¦ VCCINT_THRESH=1100 | 64 | 80 | — | — | Artix-7 XADC wrapper. Samples Vccint/Vccaux/temp. Raises fault_flags[THERMAL] on threshold. |
| watchdog_timer | rtl/system/watchdog_timer.v | 9 | TIMEOUT_CYCLES=par ¦ CLK_HZ=50_000_000 | 18 | 26 | — | — | 9 independent WDTs: 3×CS (500ms), 3×UAV (500ms), 1×CVG-CA (1ms), 1×CVG-FC (150ms), 1×system. |
| cubesat_subsystem | rtl/cubesat/cubesat_subsystem.v | 3 | CS_ID=0/1/2 ¦ BANK=34/15/35 | 1180 | 1600 | 6 | 1 | Instantiated as cs_inst[0], cs_inst[1], cs_inst[2]. Each a self-contained ADCS+orbit+laser+telemetry block. |
| spi_master | rtl/shared/spi_master.v | 1 | CLK_DIV=4 ¦ CPOL=1 ¦ CPHA=1 ¦ DATA_W=8 | 20 | 32 | — | — | ICM-42688 / MPU-9250 SPI interface. clk_100mhz domain. clk_100mhz/4 = 25 MHz SCK. |
| spi_shift_reg | rtl/shared/spi_shift_reg.v | 1 | WIDTH=8 | 10 | 18 | — | — | 8-bit MSB-first shift register. CPOL=1 CPHA=1 (mode 3). |
| async_fifo | rtl/shared/async_fifo.v | 1 | DATA_W=32 ¦ DEPTH=16 ¦ ADDR_W=4 | 48 | 96 | — | — | Gray-coded async FIFO. SPI clk_100mhz write-side → sys_clk read-side. Used for IMU CDC. |
| gray_encoder | rtl/shared/gray_encoder.v | 2 | WIDTH=4 | 4 | 4 | — | — | Gray-code encode for write/read pointer CDC. |
| sync_2ff | rtl/shared/sync_2ff.v | 2 | WIDTH=4 | 0 | 8 | — | — | 2-FF synchroniser for pointer crossing. |
| i2c_master | rtl/shared/i2c_master.v | 1 | CLK_DIV=125 ¦ ADDR_W=7 | 16 | 20 | — | — | HMC5883L / IST8310 400kHz I2C. Open-drain SDA/SCL. 100Hz read cycle. |
| sun_sensor_adc | rtl/cubesat/sun_sensor_adc.v | 1 | N_CH=6 ¦ BITS=12 | 24 | 36 | — | — | 6-channel photodiode ADC SPI read. 12-bit per channel. Feeds ADCS with sun-vector. |
| quat_ekf_7state | rtl/cubesat/quat_ekf_7state.v | 1 | Q_FMT=Q1.15 ¦ N_STATE=7 ¦ TS_HZ=100 | 380 | 520 | 1 | 1 | 7-state quat EKF: [qw,qx,qy,qz, bx,by,bz]. Q1.15 quaternion. 1 DSP48 covariance. BRAM18 P-matrix. |
| quat_multiply | rtl/shared/quat_multiply.v | 2 | FMT=Q1.15 | 48 | 64 | — | — | Hamilton product q1⊗q2. Q2.30 intermediate → >>15 → Q1.15. |
| quat_conjugate | rtl/shared/quat_conjugate.v | 2 | FMT=Q1.15 | 4 | 8 | — | — | q*=[qw,-qx,-qy,-qz]. Saturation on -(-1.0) edge. |
| quat_normalize | rtl/shared/quat_normalize.v | 1 | N_ITER=4 | 60 | 80 | 1 | — | Newton-Raphson 4-iter normalization. BRAM LUT y0 seed. |
| ekf_covariance | rtl/shared/ekf_covariance.v | 1 | N=7 | 120 | 160 | — | 1 | P predict + Joseph-form update. BRAM18 P-matrix store. |
| adcs_controller | rtl/cubesat/adcs_controller.v | 1 | CTRL_HZ=100 ¦ GAINS_FMT=Q4.12 | 200 | 280 | 2 | — | 5-state FSM (BOOT/DETUMBLE/COARSE/FINE/SAFE). Quat PD law. 2 DSP48 error multiply. |
| adcs_fsm | rtl/cubesat/adcs_fsm.v | 1 | N_STATES=5 | 40 | 60 | — | — | BOOT→DETUMBLE→COARSE_POINT→FINE_POINT→SAFE state machine. |
| quat_pd_law | rtl/cubesat/quat_pd_law.v | 1 | FMT=Q4.12 | 80 | 100 | 2 | — | u=Kp*q_err_vec + Kd*omega. 2 DSP48 gain multiply. |
| rw_spi_ctrl | rtl/cubesat/rw_spi_ctrl.v | 1 | N_AXES=3 | 48 | 64 | — | — | 3-axis reaction wheel torque command SPI TX. |
| magnetorquer_pwm | rtl/cubesat/magnetorquer_pwm.v | 1 | N_CH=3 ¦ PWM_BITS=12 | 24 | 36 | — | — | 3-ch PWM for magnetorquer coils. Q4.12 duty. |
| sgp4_orbit_propagator | rtl/cubesat/sgp4_orbit_propagator.v | 1 | UPDATE_HZ=1 ¦ COORD=ECI ¦ FMT=Q10.22 | 280 | 360 | 3 | — | SGP4-Lite with J2 perturbation. 3 DSP48 MACC Kepler solve. ECI→LVLH transform. |
| sgp4_kepler_solve | rtl/cubesat/sgp4_kepler_solve.v | 1 | ITER=8 | 120 | 160 | 3 | — | Eccentric anomaly Newton-Raphson. 3 DSP48. |
| eci_to_lvlh | rtl/cubesat/eci_to_lvlh.v | 1 | FMT=Q10.22 | 80 | 100 | — | — | ECI to LVLH frame rotation using CORDIC outputs. |
| tle_bram_store | rtl/cubesat/tle_bram_store.v | 1 | DEPTH=64 ¦ WIDTH=32 | 20 | 28 | — | — | TLE coefficient storage in LUT-RAM (< 2 Kb). Updatable via AXI. |
| laser_pointing_fsm | rtl/cubesat/laser_pointing_fsm.v | 1 | N_STATES=6 ¦ SCAN_BITS=16 | 200 | 240 | — | — | IDLE/SEARCH/ACQUIRE/TRACK/COMM/FAULT. ±10°×5° raster scan, spiral refinement ≤0.1°. |
| gimbal_stepper | rtl/cubesat/gimbal_stepper.v | 1 | N_AXES=2 ¦ MICRO=16 | 60 | 80 | — | — | 2-axis STEP/DIR trapezoidal accel. 16-microstep. |
| signal_strength_adc | rtl/cubesat/signal_strength_adc.v | 1 | BITS=12 | 24 | 32 | — | — | 12-bit received signal strength via SPI ADC. Feeds closed-loop TRACK. |
| raster_scan_ctrl | rtl/cubesat/raster_scan_ctrl.v | 1 | H_STEPS=64 ¦ V_STEPS=32 | 48 | 60 | — | — | SEARCH scan pattern generator. Transitions to ACQUIRE on signal > threshold. |
| ccsds_telemetry_encoder | rtl/cubesat/ccsds_telemetry_encoder.v | 1 | SYNC=0x1ACFFC1D ¦ CRC_POLY=0x1021 | 120 | 160 | — | — | CCSDS primary/secondary header + CRC-16/CCITT. APID 0x101-0x103. ISL+UART TX. |
| crc16_ccitt | rtl/shared/crc16_ccitt.v | 1 | POLY=0x1021 ¦ INIT=0xFFFF | 32 | 16 | — | — | Parallel CRC-16/CCITT (bit-reversed). 1-cycle per byte. |
| superframe_arbiter | rtl/cubesat/superframe_arbiter.v | 1 | PERIOD_HZ=1 | 32 | 40 | — | — | 1-sec superframe scheduler for ADCS/orbit/laser packets. |
| isl_parallel_tx | rtl/cubesat/isl_parallel_tx.v | 1 | DATA_W=8 ¦ USE_ODDR=1 | 16 | 24 | — | — | 8-bit parallel optical ISL TX via ODDR. Bank 35 pins. |
| uav_subsystem | rtl/uav/uav_subsystem.v | 3 | UAV_ID=0/1/2 ¦ BANK=14 | 860 | 1180 | 3 | 3 | Instantiated as uav_inst[0], uav_inst[1], uav_inst[2]. Full flight-controller: EKF+PID+NAV+MAVLink. |
| spi_master | rtl/shared/spi_master.v | 1 | CLK_DIV=4 ¦ CPOL=1 ¦ CPHA=1 | 20 | 32 | — | — | ICM-42688 SPI. clk_100mhz/4 = 25 MHz SCK. Same module as CS SPI. |
| spi_shift_reg | rtl/shared/spi_shift_reg.v | 1 | WIDTH=8 | 10 | 18 | — | — | 8-bit SPI shift register. |
| async_fifo | rtl/shared/async_fifo.v | 1 | DATA_W=32 ¦ DEPTH=16 ¦ ADDR_W=4 | 48 | 96 | — | — | IMU data CDC: clk_100mhz (SPI) → sys_clk (EKF). Gray-coded pointers. |
| gray_encoder | rtl/shared/gray_encoder.v | 2 | WIDTH=4 | 4 | 4 | — | — | Gray encoder for pointer CDC. |
| sync_2ff | rtl/shared/sync_2ff.v | 2 | WIDTH=4 | 0 | 8 | — | — | 2-FF pointer synchroniser. |
| ekf_9state_navigator | rtl/uav/ekf_9state_navigator.v | 1 | POS_FMT=Q10.22 ¦ VEL_FMT=Q4.28 ¦ ANG_FMT=Q3.29 ¦ TS_HZ=100 | 360 | 480 | 3 | 1 | 9-state: [px,py,pz,vx,vy,vz,roll,pitch,yaw]. IMU+GPS+Baro fusion. 3 DSP48. BRAM18 P-matrix. |
| cordic_sincos | rtl/shared/cordic_sincos.v | 1 | N_ITER=16 ¦ FMT=Q1.15 | 200 | 1056 | — | — | 16-stage CORDIC pipeline. 0 DSP48. For R-matrix sin/cos. |
| rotation_matrix_build | rtl/uav/rotation_matrix_build.v | 1 | FMT=Q1.30 | 36 | 48 | — | — | Builds 3×3 R from [roll,pitch,yaw] CORDIC outputs. |
| ekf_predict_step | rtl/uav/ekf_predict_step.v | 1 | N=9 ¦ DSP=3 | 80 | 110 | 3 | — | x_pred=x+f(x,u)*Ts. R*a_body NED accel. 3 DSP48 MACs. |
| ekf_gps_update | rtl/uav/ekf_gps_update.v | 1 | M_GPS=5 ¦ GATE_SIGMA=3 | 60 | 80 | — | — | GPS 5-measurement update. Innovation gate 5σ. H_gps wire slice. |
| ekf_baro_update | rtl/uav/ekf_baro_update.v | 1 | M_BARO=1 | 24 | 32 | — | — | Scalar baro update. S=P[2,2]+R_baro. Rank-1 P update. |
| ekf_covariance | rtl/shared/ekf_covariance.v | 1 | N=9 | 140 | 180 | — | 1 | 9×9 P predict+update. Joseph form. BRAM18 covariance store. |
| gps_nmea_parser | rtl/shared/gps_nmea_parser.v | 1 | BAUD=9600 ¦ FMT=Q10.22 | 50 | 68 | — | — | NEO-M9N NMEA GGA+VTG parse. LUT-RAM sentence buffer. 10 Hz fix rate. |
| uart_rx | rtl/shared/uart_rx.v | 1 | BAUD=9600 | 16 | 24 | — | — | UART RX. Oversampling×16. 8N1. |
| nmea_sentence_parser | rtl/shared/nmea_sentence_parser.v | 1 | BUF_DEPTH=128 | 32 | 40 | — | — | GGA: lat/lon/alt. VTG: vN/vE. Checksum validate. |
| baro_spi_master | rtl/uav/baro_spi_master.v | 1 | DEVICE=BMP388 ¦ RATE_HZ=50 | 24 | 36 | — | — | BMP388 barometer SPI. 50 Hz altitude. QNH ground-ref compensation. |
| tof_echo_capture | rtl/uav/tof_echo_capture.v | 1 | CLK_HZ=50_000_000 ¦ RANGE_M=10 | 16 | 24 | — | — | ToF pulse-width capture. Active below 10m AGL. Feeds EKF alt update. |
| pid_6dof_controller | rtl/uav/pid_6dof_controller.v | 1 | INNER_HZ=1000 ¦ OUTER_HZ=100 ¦ GAINS_FMT=Q4.12 | 100 | 140 | — | 1 | Dual-loop 6-DOF PID. 0 dedicated DSP48 (uses shared PID engine). BRAM18 gains (4 flight modes). |
| pid_engine | rtl/shared/pid_engine.v | 1 | FMT=Q4.12 ¦ N_AXES=6 ¦ SHARED=1 | 40 | 56 | — | — | Q4.12 P+I+D. Uses BASE-level shared DSP48 block (4 DSPs). Anti-windup. |
| motor_mix_matrix | rtl/uav/motor_mix_matrix.v | 1 | N_MOTORS=4 ¦ FMT=Q4.12 | 24 | 32 | — | — | 4×4 mixing matrix [roll,pitch,yaw,thrust]→[M1,M2,M3,M4]. |
| esc_pwm_gen | rtl/uav/esc_pwm_gen.v | 4 | FREQ_HZ=400 ¦ BITS=16 | 8 | 12 | — | — | 4 ESC PWM generators. 50–400 Hz configurable. 1µs resolution. |
| rate_limiter | rtl/shared/rate_limiter.v | 6 | FMT=Q4.12 | 6 | 8 | — | — | Per-axis output rate limiting. Prevents step commands to ESCs. |
| waypoint_nav_fsm | rtl/uav/waypoint_nav_fsm.v | 1 | N_STATES=8 ¦ MAX_WP=32 ¦ BRAM_WP=1 | 200 | 280 | — | 1 | 8-state: DISARMED→ARMED→TAKEOFF→EN_ROUTE→LOITER→LAND→RTL→EMERGENCY. Geofence cylindrical. |
| nav_fsm_core | rtl/uav/nav_fsm_core.v | 1 | N_STATES=8 | 80 | 100 | — | — | State machine transitions. Illegal-state trap→EMERGENCY. |
| waypoint_store | rtl/uav/waypoint_store.v | 1 | DEPTH=32 ¦ WIDTH=96 | 20 | 28 | — | 1 | 32-waypoint BRAM18 store. [lat,lon,alt,speed] per WP. MAVLink upload. |
| geofence_check | rtl/uav/geofence_check.v | 1 | SHAPE=CYLINDRICAL | 48 | 60 | — | — | Cylindrical geofence. d² check (no sqrt). Breach → RTL command. |
| preflight_check | rtl/uav/preflight_check.v | 1 | N_CHECKS=12 | 32 | 40 | — | — | 12-item pre-flight checklist (GPS lock, IMU health, battery, etc.). |
| mavlink_v2_stack | rtl/uav/mavlink_v2_stack.v | 1 | SYSID=1/2/3 ¦ COMPID=1 ¦ BAUD=115200 | 200 | 280 | — | 1 | MAVLink v2. STX 0xFD + CRC-16/MCRF4XX. HEARTBEAT 1Hz, ATTITUDE 10Hz, POSITION 5Hz. |
| mavlink_rx_parser | rtl/uav/mavlink_rx_parser.v | 1 | MAX_LEN=280 | 60 | 80 | — | — | STX sync + CRC validate + APID decode + payload extract. |
| mavlink_tx_builder | rtl/uav/mavlink_tx_builder.v | 1 | N_MSG=8 | 48 | 64 | — | — | Pack HEARTBEAT/ATTITUDE/POSITION/STATUS. Priority TX queue. |
| uart_rx | rtl/shared/uart_rx.v | 1 | BAUD=115200 | 16 | 24 | — | — | 115200 baud UART RX for GCS MAVLink. |
| uart_tx | rtl/shared/uart_tx.v | 1 | BAUD=115200 | 16 | 24 | — | — | 115200 baud UART TX for GCS MAVLink. |
| crc16_mcrf4xx | rtl/shared/crc16_mcrf4xx.v | 2 | INIT=0x0000 | 32 | 16 | — | — | CRC-16/MCRF4XX for MAVLink frame. 2 instances: RX verify + TX generate. |
| mavlink_fifo | rtl/uav/mavlink_fifo.v | 1 | DEPTH=256 ¦ W=8 | 20 | 28 | — | 1 | MAVLink TX FIFO. BRAM18. Priority ordering for ESTOP messages. |
| convergence_engine | rtl/convergence/convergence_engine.v | 1 | N_VEH=6 ¦ N_CS=3 ¦ N_UAV=3 | 1400 | 1820 | 13 | 4 | Multi-vehicle KF + formation controller + APF + resource arbiter + geometry score. |
| isl_parallel_rx | rtl/convergence/isl_parallel_rx.v | 3 | DATA_W=8 ¦ RATE_MBPS=10 | 24 | 36 | — | — | 8-bit parallel optical ISL RX from CS1/2/3. IDDR input regs. CRC check. |
| multi_vehicle_kalman | rtl/convergence/multi_vehicle_kalman.v | 1 | N=6 ¦ STATE_SZ=6 ¦ POS_FMT=Q10.22 ¦ VEL_FMT=Q4.28 | 480 | 640 | 4 | 2 | 36-state joint filter. 6×6-state per vehicle. F×x predict + K×innov update. DATA_READY 10Hz. |
| mvk_predict_step | rtl/convergence/mvk_predict_step.v | 1 | N_VEH=6 ¦ DSP=4 | 120 | 160 | 4 | — | F×x predict for 6 vehicles. Ts=0.1s. 4 DSP48 TDM. ~45 cycles. |
| mvk_isl_fusion | rtl/convergence/mvk_isl_fusion.v | 1 | GATE_SIGMA=5 | 80 | 100 | — | — | ISL inter-satellite position fusion. 5σ Mahalanobis gate. |
| mvk_covariance | rtl/convergence/mvk_covariance.v | 1 | N=6 | 160 | 200 | — | 2 | 6× independent 6×6 covariance. Joseph form. 2 BRAM18 double-buffer. |
| delta_r_compute | rtl/convergence/delta_r_compute.v | 1 | N_PAIRS=15 | 80 | 100 | — | — | Pre-computes all 15 pairwise ΔR_ij vectors. Feeds CVG_SCORE. |
| formation_controller | rtl/convergence/formation_controller.v | 1 | N_GEOM=4 ¦ N_FOLLOWERS=5 ¦ BASELINE_M=50 | 300 | 380 | 2 | 1 | 4 geometries (TRIANGLE/LINE/V/SQUARE). Virtual leader propagation. LERP transitions. |
| formation_lut_rom | rtl/convergence/formation_lut_rom.v | 1 | N_GEOM=4 ¦ N_FOL=5 | 60 | 80 | — | — | Offset vectors distributed LUT-ROM (1920 bytes < BRAM threshold). |
| r_f2ned_rotate | rtl/convergence/r_f2ned_rotate.v | 1 | FMT=Q1.30 | 40 | 56 | 2 | — | R_f2NED(ψ) × δ_f. CORDIC outputs × delta. 2 DSP48 MACs. |
| lerp_interpolator | rtl/convergence/lerp_interpolator.v | 1 | STEPS=20 ¦ FMT=Q10.22 | 60 | 80 | — | — | Linear interpolation for formation transitions. 20-step = 2s at 10Hz. |
| formation_error_mon | rtl/convergence/formation_error_mon.v | 1 | THRESH_M=1 | 80 | 100 | — | — | Per-vehicle 1m threshold check. 3-cycle persistence. RMS error metric. |
| apf_collision_avoidance | rtl/convergence/apf_collision_avoidance.v | 1 | N_PAIRS=15 ¦ D_SAFE_M=10 ¦ K_REP_FMT=Q4.28 | 320 | 420 | 6 | 1 | 15-pair scan. d²=dx²+dy²+dz² gate (no sqrt). NR inv_sqrt. F_rep vector. 6 DSP48 TDM. |
| apf_pair_force | rtl/convergence/apf_pair_force.v | 1 | DSP=6 | 120 | 160 | 6 | — | Single-pair APF unit (TDM'd over 15 pairs). d²→gate→NR→alpha→F_vec. |
| nr_inv_sqrt | rtl/shared/nr_inv_sqrt.v | 1 | N_ITER=1 ¦ FMT=Q1.30 | 48 | 64 | 3 | — | Newton-Raphson 1-iteration inv_sqrt. BRAM LUT y0 seed (256 entries). |
| apf_force_summer | rtl/convergence/apf_force_summer.v | 1 | N_VEH=6 | 48 | 60 | — | — | Accumulates F_rep over all threatening pairs per vehicle. Newton 3rd law. |
| apf_damping | rtl/convergence/apf_damping.v | 1 | K_DAMP_FMT=Q4.28 | 36 | 48 | — | — | Damping term k_damp×v_rel. Activates on OSCILLATION_DETECT flag. |
| inv_sqrt_lut | rtl/shared/inv_sqrt_lut.v | 1 | DEPTH=256 ¦ W=16 | 24 | 0 | — | 1 | 256-entry × 16-bit inv_sqrt seed table. Distributed ROM in BRAM18. |
| resource_arbiter | rtl/convergence/resource_arbiter.v | 1 | N_SLOTS=6 ¦ DEFAULT_MS=100 | 160 | 200 | — | 1 | Round-robin 6-slot TDM. CA pre-emption < 1ms. ISL 60/40 TX/RX split. P0-P4 priority. |
| tdm_fsm | rtl/convergence/tdm_fsm.v | 1 | N_SLOTS=6 | 48 | 60 | — | — | TDM slot controller. Configurable slot duration 50–500ms. |
| ca_preempt_logic | rtl/convergence/ca_preempt_logic.v | 1 | LATENCY_CYCLES=50 | 32 | 40 | — | — | APF override gate. Injects CA velocity correction within 1ms. |
| arbiter_log_bram | rtl/convergence/arbiter_log_bram.v | 1 | DEPTH=512 ¦ W=64 | 20 | 28 | — | 1 | BRAM18 event log. Records slot assignments + CA events for telemetry. |
| geometry_score | rtl/convergence/geometry_score.v | 1 | N_PAIRS=15 ¦ FMT=Q4.28 | 60 | 80 | 1 | — | CVG_SCORE = 1 - mean(¦ΔR_ij - ΔR_target_ij¦)/scale. 1 DSP48 MACC. 1 Hz output. |
| pid_engine_shared | rtl/shared/pid_engine.v | 1 | FMT=Q4.12 ¦ N_AXES=6 ¦ SHARED=1 ¦ TDM_UAV=3 | 120 | 160 | 4 | — | Shared Q4.12 PID DSP block. Time-multiplexed over 3 UAVs. 4 DSP48: P+I+D+motor-mix. |
| pid_p_term | rtl/shared/pid_p_term.v | 1 | FMT=Q4.12 | 20 | 32 | 1 | — | P term: kp×e. Q4.12×Q4.12→Q8.24→>>12→Q4.12. 1 DSP48. |
| pid_i_term | rtl/shared/pid_i_term.v | 1 | FMT=Q4.12 | 24 | 40 | 2 | — | I term with anti-windup. Q8.24 accumulator. 2 DSP48 MACC. |
| pid_d_term | rtl/shared/pid_d_term.v | 1 | FMT=Q4.12 | 20 | 32 | 1 | — | D term: kd_s×(e[n]-e[n-1]). Pre-adder DSP48 path. 1 DSP48. |
| cordic_sincos_shared | rtl/shared/cordic_sincos.v | 1 | N_ITER=16 ¦ FMT=Q1.15 ¦ PIPELINE=1 | 200 | 1056 | — | — | Single shared CORDIC. Time-multiplexed by CS ADCS (×3) and UAV EKF (×3). 0 DSP48. 16-stage. |
| cordic_atan2 | rtl/shared/cordic_atan2.v | 1 | N_ITER=16 | 200 | 1056 | — | — | Vectoring mode atan2 + magnitude. Same hardware, d_i=-sign(y). |
| telemetry_downlink_mux | rtl/system/telemetry_downlink_mux.v | 1 | N_SRC=6 ¦ BAUD=115200 | 100 | 140 | — | 1 | 1-sec superframe TDM for GS downlink. Combines CS+UAV+CVG telemetry streams. BRAM18 buffer. |
| uart_tx | rtl/shared/uart_tx.v | 1 | BAUD=115200 | 16 | 24 | — | — | 115200 baud UART TX to ground station. |
| tlm_frame_builder | rtl/system/tlm_frame_builder.v | 1 | FRAME_SZ=50 | 48 | 60 | — | — | 50-byte telemetry packet per vehicle per second. |
| debug_ila_wrapper | rtl/debug/debug_ila_wrapper.v | 1 | N_PROBES=8 ¦ DEPTH=1024 ¦ REMOVABLE=1 | 100 | 200 | — | 1 | Vivado ILA 8-probe debug capture. Synthesised with DEBUG=1 flag. Remove for flight build. |
| ila_0 | ip/ila_0/ila_0.xci | 1 | C_NUM_OF_PROBES=8 ¦ C_DATA_DEPTH=1024 | 100 | 200 | — | 1 | Vivado ILA IP core. BRAM18 for capture buffer. |

## Hierarchy Statistics

Summary counts for unique definitions, total instances, sharing relationships, and source file organisation.

| Metric | Value | Detail |
| --- | --- | --- |
| Unique module definitions | 100 modules | Verilog source files across rtl/ and ip/ directories |
| Total elaborated instances | 145 instances | After flattening × 3 CS instances + × 3 UAV instances |
| Modules instantiated ×3 (CS) | 5 unique types × 3 = 15 | cubesat_subsystem + 4 internal sub-modules × 3 |
| Modules instantiated ×3 (UAV) | 6 unique types × 3 = 18 | uav_subsystem + 5 internal sub-modules × 3 |
| Shared (reused) modules | axi_register_bank, watchdog_timer, cubesat_subsystem, gray_encoder, sync_2ff, quat_multiply, quat_conjugate, uav_subsystem, gray_encoder, sync_2ff, esc_pwm_gen, rate_limiter, crc16_mcrf4xx, isl_parallel_rx | uart_rx/tx, spi_master, async_fifo, gray_encoder, sync_2ff, crc16, pid_engine, cordic |
| Primitives (not counted) | MMCM_ADV, IBUFGDS, BUFG ×2, IDDR ×8, XADC | Xilinx library cells — no RTL source needed |
| Vivado IP cores | clk_wiz_0, ila_0 (debug only) | .xci generated by Vivado IP Catalog |
| Source file count (RTL) | ~55 .v files | Across rtl/top/ rtl/shared/ rtl/cubesat/ rtl/uav/ rtl/convergence/ rtl/system/ rtl/debug/ |
| Max hierarchy depth | 5 levels | top → subsystem → controller → sub-block → primitive |
| Design partition | convergence_mvp_top.v → 8 major blocks | clk, AXI, 3×CS, 3×UAV, CVG, shared, system, debug |

## Directory Structure

rtl/

├── top/         convergence_mvp_top.v

├── shared/      spi_master, i2c_master, uart_rx/tx, async_fifo, gray_encoder,

│                sync_2ff, pid_engine, pid_p/i/d_term, cordic_sincos, cordic_atan2,

│                quat_multiply, quat_conjugate, quat_normalize, ekf_covariance,

│                crc16_ccitt, crc16_mcrf4xx, gps_nmea_parser, rate_limiter,

│                nr_inv_sqrt, inv_sqrt_lut

├── cubesat/     cubesat_subsystem, quat_ekf_7state, adcs_controller, adcs_fsm,

│                quat_pd_law, rw_spi_ctrl, magnetorquer_pwm, sgp4_orbit_propagator,

│                sgp4_kepler_solve, eci_to_lvlh, tle_bram_store,

│                laser_pointing_fsm, gimbal_stepper, signal_strength_adc,

│                raster_scan_ctrl, ccsds_telemetry_encoder, superframe_arbiter,

│                isl_parallel_tx, sun_sensor_adc

├── uav/         uav_subsystem, ekf_9state_navigator, rotation_matrix_build,

│                ekf_predict_step, ekf_gps_update, ekf_baro_update,

│                baro_spi_master, tof_echo_capture, pid_6dof_controller,

│                motor_mix_matrix, esc_pwm_gen, waypoint_nav_fsm, nav_fsm_core,

│                waypoint_store, geofence_check, preflight_check,

│                mavlink_v2_stack, mavlink_rx_parser, mavlink_tx_builder,

│                mavlink_fifo

├── convergence/ convergence_engine, isl_parallel_rx, multi_vehicle_kalman,

│                mvk_predict_step, mvk_isl_fusion, mvk_covariance, delta_r_compute,

│                formation_controller, formation_lut_rom, r_f2ned_rotate,

│                lerp_interpolator, formation_error_mon, apf_collision_avoidance,

│                apf_pair_force, apf_force_summer, apf_damping, resource_arbiter,

│                tdm_fsm, ca_preempt_logic, arbiter_log_bram, geometry_score

├── system/      ce_strobe_gen, global_fault_manager, xadc_monitor, watchdog_timer,

│                telemetry_downlink_mux, tlm_frame_builder

├── interconnect/ axi_lite_interconnect, gs_uart_cmd_decoder, axi_register_bank

├── debug/       debug_ila_wrapper

ip/

├── clk_wiz_0/   clk_wiz_0.xci  (Vivado IP — MMCM clock wizard)

└── ila_0/       ila_0.xci      (Vivado IP — ILA debug, flight=remove)

tb/

├── top/         tb_convergence_mvp_top.v

├── cubesat/     tb_quat_ekf_7state.v  tb_adcs_controller.v  tb_sgp4.v

├── uav/         tb_ekf_9state.v  tb_pid_6dof.v  tb_mavlink.v

└── convergence/ tb_mvk_filter.v  tb_formation.v  tb_apf.v