# P-2.6: Parameterization Strategy — Convergence MVP FPGA

8 modules parameterized: cubesat_subsystem, uav_subsystem, pid_engine, kalman_filter, spi_master, async_fifo, cordic_sincos, ce_strobe_gen.

Resource impact notation: ΔLUT / ΔFF = change per unit increase in parameter. O(N²) = quadratic scaling. Colored entries = non-zero resource impact.

## 1.  Parameter Resource Scaling Laws

How key parameters scale with resource consumption — from minimum viable configuration to maximum safe configuration on XC7A35T.

| Module | Key Parameter | Scaling Law | Min Config | Default Config | Max Config | Safe Limit | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cubesat_subsystem | EKF_STATE_N | O(N) | N=4: ~30 LUT +40 FF | N=7: ~280+420 | N=12: ~480+720 | 12 (XC7A35T limit) | State covariance is N×N; resource grows as O(N) per stage but O(N²) for full P update. |
| uav_subsystem | EKF_STATE_N | O(N²) covariance | N=6: 200+300 | N=9: 360+480 | N=12: 640+860 | 10 (Joseph form) | 9-state with Joseph form already uses significant LUT. 12-state pushes limits. |
| uav_subsystem | MAX_WAYPOINTS | O(log2 N) addr | 8: 0 BRAM | 32: 1 BRAM | 256: 1 BRAM | 1536 (1 BRAM18 max) | All fit in 1 BRAM18 up to 1536 WPs. Binary search in waypoint BRAM uses log2(N) cycles. |
| pid_engine | N_AXES × TDM_SLOTS | O(A×S) | A=1,S=1: 20+30 | A=6,S=3: 120+180 | A=12,S=8: 960+1440 | Resources from LUT, not DSP | DSP48 count is fixed (4 shared). Mux routing LUT grows with A×S product. |
| kalman_filter | STATE_W² | O(N²) P-matrix | N=4: 64 words P | N=9: 81 words P | N=36: 1296 words P | 36 (1 BRAM18 @4B/word) | N=36 P-matrix = 5184B = 0.28×18KB BRAM18. Block-diagonal structure reduces ops by ~75% for MVK. |
| cordic_sincos | N_ITER × DATA_W | O(N×W) FF-only | N=8,W=16: 384 FF | N=16,W=18: 864 FF | N=20,W=24: 1440 FF | 20 iter at 24-bit | 0 DSP48 always. Pure LUT+FF CORDIC. Latency = N_ITER+1 cycles. |
| async_fifo | DATA_W × DEPTH | O(D×W) FF-only | D=4,W=8: 32 FF | D=16,W=32: 512 FF | D=512,W=128: 65K FF | Use BRAM above 2KB | Below 2KB: FFs cheaper than BRAM. Above: instantiate BRAM-based FIFO instead. |
| spi_master | CLK_DIV (counter) | O(log2 D) FF | D=2: 1 FF | D=4: 2 FF | D=256: 8 FF | 256 max | Nearly free to change. Only the divider counter changes width. |

## 2.  Module Parameter Tables

Complete parameter listing for every module: name, default, type, valid range, and per-resource impact (ΔLUT, ΔFF, ΔDSP, ΔBRAM). Blue values = increased cost, red = decreased when reducing.

| cubesat_subsystem — rtl/cubesat/cubesat_subsystem.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| VEHICLE_ID | 0 | integer | 0..N_SATS-1 (integer) | — | — | — | — | Minimal | Unique CubeSat index. Controls address decode offsets in AXI register bank, ISL TX channel selection (isl_tx_data[VEHICLE_ID*8+:8]), SGP4 TLE BRAM page, and LED assignment. Must be unique per instance. Compile-time constant only. |
| N_SATS | 3 | integer | 1..6 (integer) | — | — | — | — | Minimal | Total fleet size for CS. Used to compute AXI base address offset = VEHICLE_ID × 0x1000 within cs_bank. Propagated to child modules that need fleet size context. |
| CLK_HZ | 50_000_000 | integer | 1M..200M (integer Hz) | — | — | — | — | Minimal | sys_clk frequency in Hz. Used by I2C master, UART baud generators, and watchdog timer to compute cycle counts. Must match actual clock. Affects derived localparams inside all child modules. |
| FAST_CLK_HZ | 100_000_000 | integer | 50M..200M (integer Hz) | — | — | — | — | Minimal | clk_100mhz frequency for SPI masters. SPI SCK = FAST_CLK_HZ / CLK_DIV (in spi_master). ICM-42688 max SCK = 24 MHz; set CLK_DIV ≥ 5 for FAST_CLK_HZ=100M. |
| EKF_STATE_N | 7 | integer | 4..12 (integer) | ±40×ΔN | ±60×ΔN | — | — | Non-zero | Quaternion EKF state vector length. Default 7 = [qw,qx,qy,qz,bx,by,bz]. Setting to 4 removes gyro bias states (less accurate but cheaper). Each additional state adds ~40 LUT + 60 FF for covariance computation. |
| EKF_NR_ITER | 4 | integer | 1..8 (integer) | — | — | — | — | Minimal | Newton-Raphson iterations for quaternion normalization. 4 iterations = < 1 LSB Q1.15 error. Reduce to 2 for lower latency (< 0.01% error from 8-bit LUT seed). No resource impact (iterative reuse of same DSP48). |
| SGP4_ITER | 8 | integer | 4..16 (integer) | ±10×ΔN | ±15×ΔN | — | — | Non-zero | Kepler eccentric-anomaly solver iterations. 8 = sub-meter accuracy. Reduce to 4 for ~5m accuracy at lower LUT cost. Each iteration adds ~10 LUT + 15 FF in the unrolled iteration loop. |
| ISL_DATA_W | 8 | integer | 8 or 16 (integer) | ±32×ΔW | ±48×ΔW | — | — | Non-zero | ISL parallel link width in bits. Default 8 uses 8 ODDR primitives per CS. Setting 16 doubles throughput to 20 Mbps but requires 16 additional Bank 35 I/O pins per CS (not available on XC7A35T CPG236 — use 8 only). |
| POS_FMT_INT | 10 | integer | 4..16 (integer bits) | — | — | — | — | Minimal | Q-format integer bits for NED position. Q10.22 default covers ±1023 m. For longer-range missions increase to Q16.16 (±32767m) — update POS_FMT_FRAC to 16 simultaneously. |
| POS_FMT_FRAC | 22 | integer | 8..28 (integer bits) | — | — | — | — | Minimal | Q-format fractional bits for NED position. POS_FMT_INT + POS_FMT_FRAC must equal 32. Resolution = 2^{-FRAC} metres. |
| ANG_FMT_INT | 3 | integer | 1..8 | — | — | — | — | Minimal | Q-format integer bits for attitude angles. Q3.29 covers ±3.14 rad (full circle). Increase to Q4.28 if angular rates >π rad/s cause overflow in intermediate computations. |
| DEBUG_EN | 1 | boolean | 0 or 1 | ~100 | ~200 | — | 1 | Non-zero | 1 = instantiate Vivado ILA debug block. 0 = flight build (no ILA). Controls `if (DEBUG_EN) begin : debug_gen` generate block. Saves 100 LUT + 200 FF + 1 BRAM18 when 0. |

| uav_subsystem — rtl/uav/uav_subsystem.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| VEHICLE_ID | 0 | integer | 0..N_UAVS-1 | — | — | — | — | Minimal | Unique UAV index. Drives AXI base address = 0x3000 + VEHICLE_ID × 0x1000. Also sets MAVLink SYSID default = 1+VEHICLE_ID (overridable via AXI MAV_SYSID register). APF pair-table index. |
| N_UAVS | 3 | integer | 1..6 | — | — | — | — | Minimal | Total UAV fleet size. Used for APF pair-count computation in child modules and for TDM slot assignment. |
| CLK_HZ | 50_000_000 | integer | 1M..200M | — | — | — | — | Minimal | Primary clock frequency. Same as cubesat_subsystem. Drives all baud generators and watchdog timers. |
| FAST_CLK_HZ | 100_000_000 | integer | 50M..200M | — | — | — | — | Minimal | SPI/CDC clock. ICM-42688 SCK runs at FAST_CLK_HZ/4 = 25 MHz. |
| EKF_STATE_N | 9 | integer | 6..12 | ±55×ΔN | ±80×ΔN | — | — | Non-zero | 9-state EKF: [px,py,pz,vx,vy,vz,roll,pitch,yaw]. Reduce to 6 to drop attitude states (position+velocity only, no orientation EKF). Each added state costs ~55 LUT + 80 FF in Jacobian + covariance update. |
| EKF_GPS_MEAS_N | 5 | integer | 3..7 | ±20×ΔM | ±30×ΔM | — | — | Non-zero | GPS measurement vector dimension. Default 5 = [px,py,pz,vx,vy]. Add 6=+vz, 7=+heading. Each extra measurement adds ~20 LUT + 30 FF for innovation computation. |
| MAX_WAYPOINTS | 32 | integer | 8..256 (power of 2) | — | — | — | ⌈MAX_WP×12B/18KB⌉ | Non-zero | Waypoint BRAM depth. 32 WPs × 12B each = 384B (fits easily in one BRAM18). 256 WPs × 12B = 3072B — still 1 BRAM18. Beyond 1536 WPs would require 2 BRAM18. |
| GPS_BAUD | 9_600 | integer | 4800..115200 | — | ±2×ΔBaud | — | — | Non-zero | GPS UART baud rate. NEO-M9N default 9600. Setting 115200 requires 12 times more NMEA throughput — use for high-update-rate configurations. Only affects uart_rx divider counter width (+2 FF for higher baud). |
| GCS_BAUD | 115_200 | integer | 9600..921600 | — | ±2 | — | — | Non-zero | MAVLink/GCS UART baud. Default 115200 for ground station link. 921600 for short-range USB-UART dongle. Adjust CLK_HZ/GCS_BAUD to get correct divider. |
| PID_FMT_INT | 4 | integer | 2..8 | — | — | — | — | Minimal | PID gain Q-format integer bits. Q4.12 supports gains up to ±15.999. Increase to Q6.10 for higher gains at cost of 2 fractional bits precision. |
| PID_FMT_FRAC | 12 | integer | 8..14 | — | — | — | — | Minimal | PID gain Q-format fractional bits. PID_FMT_INT + PID_FMT_FRAC must equal 16 for Q-notation. Resolution = 1/2^FRAC. |
| N_MOTORS | 4 | integer | 3..8 | ±8×ΔM | ±12×ΔM | — | — | Non-zero | Motor count. 4=quad, 6=hex, 8=octo. Affects motor mixing matrix dimensions and ESC PWM generator count. Each additional motor adds ~8 LUT + 12 FF + 1 PWM counter. |
| PWM_BITS | 16 | integer | 12..20 | ±2×ΔB | ±4×ΔB | — | — | Non-zero | PWM counter resolution. 16-bit gives 1/65536 duty-cycle resolution at 400 Hz. Increase to 20 for ultra-fine throttle control (audio-grade ESCs). Each bit adds ~2 LUT + 4 FF per PWM channel. |
| DEBUG_EN | 1 | boolean | 0 or 1 | ~100 | ~200 | — | 1 | Non-zero | ILA debug enable. Same as cubesat_subsystem. Set 0 for flight build. |

| pid_engine — rtl/shared/pid_engine.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| DATA_W | 32 | integer | 16..48 (integer) | ∝DATA_W | ∝DATA_W | 1 | — | Non-zero | Input/output data width in bits. Default 32 for Q4.12 packed in lower 16 bits (upper 16 zero or sign-extended). Set to 48 if using extended Q8.40 for high-precision outer loops. Each DSP48 A/B port is 18 bits; products internally 36-48 bits. |
| COEFF_W | 16 | integer | 12..18 (integer) | — | ±2×ΔW | — | — | Non-zero | Gain coefficient width in bits. Default 16 matches Q4.12 (16-bit signed). Range limit 18 = DSP48 A-port maximum. Setting to 18 allows Q4.14 gains for finer resolution without extra DSP. |
| GAIN_INT | 4 | integer | 1..8 | — | — | — | — | Minimal | Gain Q-format integer bits. Default 4 (range ±16.0). Increase to 6 for higher gains (±64) needed for fast attitude recovery. GAIN_INT + GAIN_FRAC = COEFF_W. |
| GAIN_FRAC | 12 | integer | 6..16 | — | — | — | — | Minimal | Gain Q-format fractional bits. Resolution = 1/2^GAIN_FRAC. Default 12 → 0.000244 gain resolution. Decrease to GAIN_INT=6, GAIN_FRAC=10 for aggressive UAVs needing higher gains. |
| ACC_W | 32 | integer | 24..48 | ±4×ΔW | ±6×ΔW | — | — | Non-zero | Integral accumulator width in bits. Default 32 matches Q8.24 intermediate for I term. Increase to 40 if very small Ki with long integration prevents accumulator reaching meaningful values. |
| N_AXES | 6 | integer | 1..12 | ±20×ΔA | ±30×ΔA | — | — | Non-zero | Number of PID axes. Default 6 for UAV 6-DOF (roll/pitch/yaw rate + roll/pitch + alt). Each additional axis adds ~20 LUT + 30 FF for mux routing in TDM mode. |
| SHARED | 1 | boolean | 0 or 1 | ±30 | ±40 | — | — | Non-zero | 0 = dedicated PID instance (no TDM); 1 = shared TDM mode with TDM_SLOTS time slots. Shared mode adds ~30 LUT + 40 FF for TDM counter and slot MUX. Always 1 in top module. |
| TDM_SLOTS | 3 | integer | 1..8 | ±10×ΔS | ±16×ΔS | — | — | Non-zero | Time slots for shared TDM mode. Default 3 = one per UAV. Each slot adds ~10 LUT + 16 FF for slot counter decode. At 1kHz inner loop, 3 slots = 333 µs per UAV compute budget (well adequate). |
| ANTI_WINDUP | 1 | boolean | 0 or 1 | 8 | 4 | — | — | Non-zero | Enable integrator anti-windup clamp. 1 = clamp i_accum when ¦u¦ > SAT_LIMIT. Adds 8 LUT + 4 FF for comparator and clamp MUX. Always recommended; disable only for pure P or PD controllers. |
| SAT_BITS | 16 | integer | 12..20 | — | — | — | — | Minimal | Output saturation register width. Default 16 = Q4.12 full range. Comparator against SAT_MAX/SAT_MIN registers. No resource impact — comparator is same width as output. |

| ekf_covariance / kalman_filter — rtl/shared/ekf_covariance.v rtl/uav/ekf_9state_navigator.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| STATE_W | 9 | integer | 3..36 (integer) | ∝N²×4 | ∝N²×6 | ∝N | ⌈N²×4B/18KB⌉ | Non-zero | State vector dimension N. CS EKF: 7. UAV EKF: 9. MVK: 36. P-matrix = N×N symmetric (N(N+1)/2 unique elements). LUT scales as ~4×N² (N² comparators for Jacobian evaluation). BRAM: N²×32b — 9-state P matrix = 324B (1 BRAM18), 36-state = 5184B (1 BRAM18). Critical: increase beyond 20 states needs careful BRAM planning. |
| MEAS_W | 5 | integer | 1..10 (integer) | ∝M×N×8 | ∝M×N×12 | — | — | Non-zero | Measurement vector dimension M. GPS: 5 [px,py,pz,vx,vy]. Baro: 1. ISL: 3. Innovation vector is M×32b. S-matrix (innovation covariance) = M×M. H-matrix = M×N. Each extra measurement adds ~8×N LUT + 12×N FF for innovation row. |
| POS_FMT_INT | 10 | integer | 4..16 | — | — | — | — | Minimal | Position Q-format integer bits for state elements. Propagated to multiplication shift amounts as localparam. Q10.22 default for ±1023m positions. |
| POS_FMT_FRAC | 22 | integer | 8..28 | — | — | — | — | Minimal | Position Q-format fractional bits. POS_INT + POS_FRAC = 32 required. |
| VEL_FMT_INT | 4 | integer | 2..12 | — | — | — | — | Minimal | Velocity Q-format integer bits. Q4.28 for ±7.99 m/s. Change to Q8.24 for faster UAVs. |
| VEL_FMT_FRAC | 28 | integer | 16..30 | — | — | — | — | Minimal | Velocity Q-format fractional bits. VEL_INT + VEL_FRAC = 32. |
| TS_HZ | 100 | integer | 10..1000 | — | — | — | — | Minimal | EKF predict step frequency in Hz. Controls Ts computation: Ts = 1/TS_HZ. Used to compute Ts_fp = round(Ts × 2^FRAC). Note: actual trigger is CE strobe; TS_HZ must match ce_Nhz strobe rate connected to en_predict port. |
| GATE_SIGMA | 5 | integer | 3..10 | 4 | 4 | — | — | Non-zero | Innovation gate threshold (sigma multiples). Reject measurement if ¦innovation¦² > GATE_SIGMA² × S. Default 5σ. Stored as localparam; threshold comparison adds ~4 LUT + 4 FF per measurement axis. |
| JOSEPH_FORM | 1 | boolean | 0 or 1 | ±60×N | ±80×N | — | — | Non-zero | 1 = Joseph form P update P+ = (I-KH)P(I-KH)^T + KRK^T for numerical stability. 0 = simple P+ = (I-KH)P (faster but may lose symmetry over time). Joseph form recommended for all flight-critical EKFs. Cost: ~60×N LUT + 80×N FF additional. |
| DOUBLE_BUF | 1 | boolean | 0 or 1 | — | — | — | 1 extra | Non-zero | 1 = double-buffer P-matrix in BRAM (write new P while old P is read by other modules). Eliminates read-during-write hazard at cost of 1 additional BRAM18. Strongly recommended when P is shared between EKF and telemetry read-back. |

| spi_master — rtl/shared/spi_master.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| CLK_DIV | 4 | integer | 2..256 (power of 2 recommended) | 4 | 4 | — | — | Non-zero | Clock divisor. SCK = input_clk / CLK_DIV. With FAST_CLK_HZ=100 MHz: CLK_DIV=4 → 25 MHz SCK. ICM-42688 max = 24 MHz; CLK_DIV=5 gives 20 MHz (safe). BMP388 max = 10 MHz; CLK_DIV=10 gives 10 MHz. Counter width = log2(CLK_DIV) bits. |
| CPOL | 1 | boolean | 0 or 1 | — | — | — | — | Minimal | SPI clock polarity. 0 = idle low (mode 0 or 1), 1 = idle high (mode 2 or 3). ICM-42688: CPOL=1 CPHA=1 (mode 3). BMP388: CPOL=0 CPHA=0 (mode 0). Affects CPOL register bit in SPI_CTRL output. |
| CPHA | 1 | boolean | 0 or 1 | 2 | 1 | — | — | Non-zero | SPI clock phase. 0 = sample on leading edge, 1 = sample on trailing edge. ICM-42688 requires CPHA=1. Affects shift timing control: adds 1 MUX and 1 FF when CPHA=1. |
| DATA_W | 8 | integer | 8..32 (multiple of 8) | ±2×ΔW | ±3×ΔW | — | — | Non-zero | SPI data frame width in bits. Default 8-bit for byte-at-a-time transfers (all sensors). Use 16 for 16-bit ADC reads without breaking into two 8-bit transactions. Each additional bit adds ~2 LUT + 3 FF. |
| CS_ACTIVE_L | 1 | boolean | 0 or 1 | — | — | — | — | Minimal | Chip-select polarity. 1 = active low (standard SPI). 0 = active high (non-standard). Affects cs_n output inversion — no resource impact (single NOT gate). |
| FIFO_DEPTH | 0 | integer | 0..16 (power of 2) | 8×depth | 12×depth | — | — | Non-zero | Built-in TX FIFO depth. 0 = no FIFO (single-register operation). 4 = 4-entry FIFO for burst writes. For IMU: depth=0 (one-shot triggered by CE strobe). For reaction wheel command: depth=4 preferred to queue 3-axis command atomically. |

| async_fifo — rtl/shared/async_fifo.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| DATA_W | 32 | integer | 8..128 | ∝DATA_W | ∝DATA_W | — | — | Non-zero | FIFO data width in bits. Default 32 carries one 32-bit IMU sample word. Full IMU burst = 6 words × 32b = 192b; pass as 32-bit words with 6 transfers per CE strobe. |
| DEPTH | 16 | integer | 4..512 (power of 2) | ∝log2(DEPTH) | ∝DEPTH | — | — | Non-zero | FIFO depth in entries. 16 = 16 × 32b = 64 bytes. At 100Hz IMU and 50MHz read clock, FIFO is consumed within 1 µs — depth 16 provides 160µs buffer (adequate). Larger depth adds FFs for storage (16 words × 32b = 512 FFs — no BRAM needed up to ~1024b total). |
| ADDR_W | 4 | integer | 2..9 (= log2(DEPTH)) | ∝ADDR_W | ∝ADDR_W | — | — | Non-zero | Pointer address width. MUST equal log2(DEPTH). Default 4 = log2(16). Used for Gray-code encode/decode and empty/full detection. Synthesis error if ADDR_W ≠ log2(DEPTH). |
| SYNC_STAGES | 2 | integer | 2..4 | — | ∝SYNC_STAGES×ADDR_W | — | — | Non-zero | CDC synchroniser chain depth. 2 = standard (MTBF adequate for 100MHz/50MHz pair). 3 = for 200MHz/50MHz crossing (reduced MTBF risk). Each stage adds ADDR_W FFs per crossing direction (2 directions × ADDR_W × SYNC_STAGES). |

| cordic_sincos — rtl/shared/cordic_sincos.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| N_ITER | 16 | integer | 8..20 | ∝N×12 | ∝N×54 | — | — | Non-zero | Number of CORDIC iterations. 16 = < 1 LSB Q1.15 error. Each iteration = one pipelined adder stage (~12 LUT + 54 FF for three 18-bit registers). Reduce to 12 for 14-bit accuracy saving 4×66=264 resources. Minimum 8 for 10-bit accuracy (GPS-only applications). |
| DATA_W | 18 | integer | 16..24 | ∝N×DATA_W/2 | ∝N×DATA_W×3 | — | — | Non-zero | Internal register width in bits. Default 18 for 16-bit Q1.15 output + 2 guard bits preventing intermediate overflow. 20 bits for 18-bit Q1.17 output if higher precision needed. Each extra bit costs ~N×3 FF (three regs per stage). |
| FMT | 1 | boolean | 0 or 1 | 4 | — | — | — | Non-zero | Output format. 1 = Q1.15 (θ expressed as radians/π × 32768 for input; sin/cos output in Q1.15). 0 = Q1.(DATA_W-2) generic. Affects angle scaling constants in LUT initialization. |
| PIPELINE | 1 | boolean | 0 or 1 | ∝N | — | — | — | Non-zero | 1 = fully pipelined (1 result/cycle, N_ITER latency). 0 = iterative (1 result per N_ITER cycles, lower FF count but throughput 1/N_ITER). Always 1 for convergence MVP (shared CORDIC must serve 6 vehicles in parallel). |
| MODE | BOTH | enum | ROT, VEC, or BOTH | 0 or +20 | — | — | — | Non-zero | Operating mode. ROT = rotation only (sin/cos), VEC = vectoring only (atan2/magnitude), BOTH = runtime-selectable mode. BOTH adds ~20 LUT for d_i selection MUX. MVP uses BOTH for rotation matrix and atan2 computation. |

| ce_strobe_gen — rtl/system/ce_strobe_gen.v |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Default | Type | Valid Range | ΔLUT | ΔFF | ΔDSP | ΔBRAM | Width Impact | Description |
| CLK_HZ | 50_000_000 | integer | 1M..200M | — | — | — | — | Minimal | Input clock frequency. Drives localparam computations for comparator constants. All rates computed as: period_cycles = CLK_HZ / rate_Hz. Must be integer-divisible for exact timing. |
| RATE_1KHZ | 1_000 | integer | 500..10_000 | — | — | — | — | Minimal | Inner-loop CE rate in Hz. Default 1000 Hz for UAV PID inner loop. At CLK_HZ=50M, asserts every 50000 cycles. Can increase to 2000 Hz for faster control bandwidth (requires PID computation fits in 25000 cycles = 500µs). |
| RATE_100HZ | 100 | integer | 50..1000 | — | — | — | — | Minimal | EKF predict rate. 100 Hz asserts every 500000 cycles at 50MHz. Matches IMU data rate (ICM-42688 ODR = 100Hz default). Must be divisor of RATE_1KHZ. |
| RATE_10HZ | 10 | integer | 5..100 | — | — | — | — | Minimal | GPS/formation update rate. 10 Hz = GPS fix rate of NEO-M9N. Formation controller and APF both triggered at this rate. |
| RATE_1HZ | 1 | integer | 0.1..10 | — | — | — | — | Minimal | Orbit propagator / telemetry / heartbeat rate. 1 Hz = SGP4 orbit update and 1-second superframe telemetry epoch. |
| CTR_W | 26 | integer | 20..32 (integer bits) | — | =CTR_W | — | — | Non-zero | Master counter width. 26 bits reaches 2^26 / 50MHz = 1.34 seconds — sufficient for 1Hz output. Wider counters waste FF; narrower counters cannot represent 1Hz period. CTR_W = ceil(log2(CLK_HZ / min_rate_Hz)). |

## 3.  Instantiation Examples

Complete SystemVerilog instantiation with parameter values for the Convergence MVP configuration. All examples represent the default N_SATS=3, N_UAVS=3 build.

cubesat_subsystem (rtl/cubesat/cubesat_subsystem.v)

generate for (cs_i = 0; cs_i < N_SATS; cs_i = cs_i + 1) begin : cs_gen cubesat_subsystem #( .VEHICLE_ID (cs_i), // 0,1,2 for CS1,CS2,CS3 .N_SATS (3), // fleet CS count .CLK_HZ (50_000_000), // 50 MHz sys_clk .FAST_CLK_HZ (100_000_000), // 100 MHz SPI domain .EKF_STATE_N (7), // quat + gyro bias .EKF_NR_ITER (4), // NR normalisation iters .SGP4_ITER (8), // Kepler solver iters .ISL_DATA_W (8), // 8-bit parallel ISL .POS_FMT_INT (10), // Q10.22 position .POS_FMT_FRAC (22), // Q10.22 fraction bits .DEBUG_EN (0) ) cs_inst ( .sys_clk (sys_clk), .vehicle_id (cs_i[2:0]), // ... remaining port connections ); end endgenerate

VEHICLE_ID is the only parameter that MUST differ between instances. All others can share the same value. CLK_HZ / FAST_CLK_HZ must be accurate — they drive integer division for baud/cycle calculations.

uav_subsystem (rtl/uav/uav_subsystem.v)

genvar uav_i; generate for (uav_i = 0; uav_i < N_UAVS; uav_i = uav_i + 1) begin : uav_gen uav_subsystem #( .VEHICLE_ID (uav_i), .N_UAVS (3), .CLK_HZ (50_000_000), .EKF_STATE_N (9), // full 9-state .EKF_GPS_MEAS_N (5), // [px,py,pz,vx,vy] .MAX_WAYPOINTS (32), // 384B in BRAM18 .GPS_BAUD (9_600), .GCS_BAUD (115_200), .PID_FMT_INT (4), // Q4.12 gains .PID_FMT_FRAC (12), .N_MOTORS (4), // quadcopter .PWM_BITS (16), // 1µs resolution @400Hz .DEBUG_EN (0) ) uav_inst ( /* ... port connections ... */ ); end endgenerate

EKF_STATE_N and EKF_GPS_MEAS_N are the most resource-impactful parameters. Increasing EKF_STATE_N by 1 requires a 9×9 instead of 8×8 covariance matrix — resources scale as N². N_MOTORS and PWM_BITS are near-free changes.

pid_engine (rtl/shared/pid_engine.v)

// Shared PID engine — one instance serves all N_UAVS via TDM pid_engine #( .DATA_W (32), // 32-bit Q4.12 (16b active) .COEFF_W (16), // 16-bit signed coefficient .GAIN_INT (4), // Q4.xx integer bits .GAIN_FRAC (12), // Q4.12 → Kp max 15.999 .ACC_W (32), // 32-bit integrator .N_AXES (6), // 6-DOF: roll/pitch/yaw + r/p/alt .SHARED (1), // TDM shared across UAVs .TDM_SLOTS (3), // 3 UAVs × 333µs each .ANTI_WINDUP (1), // clamp I term .SAT_BITS (16) ) pid_shared_inst ( .clk (sys_clk), .ce_1khz (ce_1khz), // inner-loop gate .ce_100hz (ce_100hz), // outer-loop gate .error_in (pid_error_in), .kp_in (pid_kp_in), .pid_out (pid_out) );

DATA_W and COEFF_W are the primary resource drivers. For a fixed-gain controller (no runtime gain changes), COEFF_W can be reduced to 12 and gains hardcoded as localparam — eliminates the coefficient register FF cost.

ekf_covariance / kalman_filter (rtl/shared/ekf_covariance.v)

// CS1 7-state quaternion EKF quat_ekf_7state #( .STATE_W (7), // [qw,qx,qy,qz,bx,by,bz] .MEAS_W (3), // magnetometer [mx,my,mz] .TS_HZ (100), // 100Hz predict rate .GATE_SIGMA (5), .JOSEPH_FORM (1), // numerically stable .DOUBLE_BUF (1) ) cs1_ekf_inst ( /* ... */ ); // UAV1 9-state navigation EKF (GPS+Baro+IMU fusion) ekf_9state_navigator #( .STATE_W (9), // [px,py,pz,vx,vy,vz,r,p,y] .EKF_GPS_MEAS_N(5), // GPS 5-measurement update .POS_FMT_INT (10), // Q10.22 position .POS_FMT_FRAC (22), .VEL_FMT_INT (4), // Q4.28 velocity .VEL_FMT_FRAC (28), .TS_HZ (100), // IMU predict at 100Hz .JOSEPH_FORM (1), .DOUBLE_BUF (1) ) uav1_ekf_inst ( /* ... */ ); // Multi-Vehicle KF (36-state, 6 vehicles × 6-state each) multi_vehicle_kalman #( .STATE_W (36), // 6 veh × [px,py,pz,vx,vy,vz] .MEAS_W (3), // ISL position measurement .N_VEHICLES (6), // fleet size .TS_HZ (10), // 10Hz MVK predict rate .GATE_SIGMA (5), .JOSEPH_FORM (0), // simplified (MVK is block-diag) .DOUBLE_BUF (1) ) mvk_inst ( /* ... */ );

STATE_W is the most impactful parameter — resources scale as O(N²). The 9-state UAV EKF with JOSEPH_FORM=1 uses ~3× more LUT than a 7-state with JOSEPH_FORM=0. MVK 36-state uses block-diagonal P (saves 75% of P-matrix operations vs full dense P).

spi_master (rtl/shared/spi_master.v)

// ICM-42688 SPI instance (mode 3, 25 MHz SCK) spi_master #( .CLK_DIV (4), // 100MHz/4 = 25MHz SCK .CPOL (1), // idle high .CPHA (1), // sample on trailing .DATA_W (8), // byte transfers .CS_ACTIVE_L (1), // active-low CS .FIFO_DEPTH (0) ) imu_spi_inst ( .clk (clk_100mhz), // SPI domain .rst_n (fast_rst_n), .sck (gyro_sck), .mosi (gyro_mosi), .miso (gyro_miso), .cs_n (gyro_cs_n) );

CLK_DIV should always be even to ensure symmetric SCK duty cycle. With CPHA=1, CPOL=1 (mode 3), the sampled data changes after the falling edge. Verify with scope: SCK idle should be high, data captured on rising SCK edge.

async_fifo (rtl/shared/async_fifo.v)

// IMU CDC FIFO: 100MHz write (SPI) → 50MHz read (EKF) async_fifo #( .DATA_W (32), // 32-bit IMU word .DEPTH (16), // 16-entry buffer .ADDR_W (4), // log2(16)=4 .SYNC_STAGES (2) ) imu_fifo_inst ( .wr_clk (clk_100mhz), .rd_clk (sys_clk), .wr_rst_n (fast_rst_n), .rd_rst_n (global_rst_n), .wr_data (spi_rx_data), .wr_en (spi_valid), .rd_data (imu_word), .rd_en (ekf_read_req), .empty (imu_fifo_empty), .full (imu_fifo_full) );

ADDR_W = log2(DEPTH) is a strict constraint — add a synthesis check: `if (2**ADDR_W !== DEPTH) $error(...)`. The DEPTH-4 minimum prevents false full/empty under CDC metastability. Never set DEPTH below 4.

cordic_sincos (rtl/shared/cordic_sincos.v)

// Shared 16-stage CORDIC (rotation + vectoring modes) cordic_sincos #( .N_ITER (16), // 16 iterations, <1 LSB Q1.15 .DATA_W (18), // 18-bit internal (2 guard bits) .FMT (1), // Q1.15 angle encoding .PIPELINE (1), // 1 result per cycle .MODE (2'b11) // ROT+VEC both enabled ) cordic_inst ( .clk (sys_clk), .rst_n (global_rst_n), .mode_sel (cordic_mode), // 0=ROT, 1=VEC .theta_in (cordic_theta), .sin_out (cordic_sin), .cos_out (cordic_cos), .valid_out (cordic_valid) );

N_ITER is the dominant resource parameter: FF cost = 3 × DATA_W × N_ITER = 3 × 18 × 16 = 864 FFs. Reducing to N_ITER=12 cuts FF count by 25% while maintaining 14-bit accuracy — acceptable for formation control (1m accuracy needs only ~12-bit angle resolution).

ce_strobe_gen (rtl/system/ce_strobe_gen.v)

ce_strobe_gen #( .CLK_HZ (50_000_000), // 50 MHz .RATE_1KHZ (1_000), // 1000 Hz inner PID .RATE_100HZ (100), // 100 Hz EKF predict .RATE_10HZ (10), // 10 Hz GPS+formation .RATE_1HZ (1), // 1 Hz orbit+TLM .CTR_W (26) ) ce_gen_inst ( .clk (sys_clk), .rst_n (global_rst_n), .ce_1khz (ce_1khz), .ce_100hz (ce_100hz), .ce_10hz (ce_10hz), .ce_1hz (ce_1hz) );

All rates must be integer divisors of each other in the chain: 1Hz divides 10Hz divides 100Hz divides 1kHz. Changing RATE_100HZ to 50Hz (matching barometer) requires adding a ce_50hz output and adjusting CTR_W if needed.

## 4.  Parameterization Rules & Constraints

| Rule | Description & Rationale |
| --- | --- |
| VEHICLE_ID uniqueness | Each cs_inst[i] and uav_inst[i] must have a unique VEHICLE_ID in range [0, N-1]. Duplicate IDs cause AXI address collision and incorrect ISL channel routing. Enforced via: `if (VEHICLE_ID >= N_SATS) $error('VEHICLE_ID out of range')` |
| ADDR_W = log2(DEPTH) | async_fifo strictly requires ADDR_W = log2(DEPTH). Mismatched values produce incorrect empty/full flags. Add synthesis-time assertion: `initial if (2**ADDR_W !== DEPTH) $error('async_fifo: ADDR_W mismatch')` |
| FMT_INT + FMT_FRAC = 32 | All Q-format parameter pairs must sum to exactly 32 (the register width). Mismatched pairs cause silent precision loss from incorrect shift amounts. Check in top-level parameter block. |
| CLK_HZ divisibility | CLK_HZ must be exactly divisible by each rate: CLK_HZ % RATE_1KHZ == 0, etc. Non-integer division produces incorrect CE timing. Validate: `initial if (CLK_HZ % RATE_1KHZ) $error('CLK_HZ not divisible by RATE_1KHZ')` |
| STATE_W × DOUBLE_BUF | If STATE_W > 16 and DOUBLE_BUF=0: P-matrix reads from address that may be written during EKF update. Always set DOUBLE_BUF=1 when STATE_W >= 7 (all flight-critical EKFs). DOUBLE_BUF=0 allowed only for MVK batch-mode testing. |
| N_ITER min for accuracy | CORDIC N_ITER must be ≥ ceil(-log2(required_error_rad)). For Q1.15 output: required error < 2^-15 = N_ITER ≥ 15. Minimum 16 with guard bit. N_ITER=12 acceptable for formation-only applications (14-bit accuracy). |
| DEBUG_EN = 0 for flight | Set DEBUG_EN=0 in all instances for the flight bitstream. ILA blocks consume BRAM18 and inject routing congestion. Controlled via Vivado build target: `make build TARGET=flight` sets `-generic DEBUG_EN=0` globally. |
| TDM_SLOTS = N_UAVS | pid_engine TDM_SLOTS must equal the number of active UAVs (N_UAVS). Too few slots: some UAVs never get PID updates. Too many: wasted CE budget. Parameterized at top level: `TDM_SLOTS(N_UAVS)`. |