# Convergence MVP — Clock Distribution Network Specification

MMCM0 → 2 BUFG → sys_clk (50 MHz) + clk_100mhz (100 MHz).  CE generator → tick_1khz / tick_100hz / tick_10hz / tick_1hz.  2 reset synchronizers.  3 CDC crossings.

## 1.  MMCM0 Configuration  (Artix-7 XC7A35T)

VCO = 800 MHz. CLKOUT0 ÷8 = 100 MHz. CLKOUT1 ÷16 = 50 MHz. Both outputs through (* DONT_TOUCH *) BUFG primitives.

| Parameter | Value | Artix-7 Limit | Margin | Units | Source | Constraint | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input clock (sys_clk_p/n) | 50 MHz | 1–800 MHz | — | MHz | Board OSC | IBUFGDS | LVDS differential pair. Bank 35, E3/E4. LVCMOS33. |
| MMCM primitive | MMCM_ADV | 5 available | 4 remaining | — | UG953 | Vivado IP | Using 1 of 5 available MMCM blocks. |
| CLKFBOUT_MULT_F | 16.000 | 2–64 | — | × | UG953 | integer | VCO = 50 × 16 = 800 MHz. |
| DIVCLK_DIVIDE | 1 | 1–106 | — | ÷ | UG953 | integer | No pre-divide. VCO = 800 MHz. |
| VCO frequency | 800 MHz | 600–1200 MHz | 200 MHz to limit | MHz | UG953 | in range | Centred in Artix-7 band. Minimises jitter. |
| CLKOUT0 (clk_100mhz) | ÷8 = 100 MHz | — | — | MHz | — | CLKOUT0_DIVIDE_F=8 | SPI masters + async FIFO write-side. |
| CLKOUT1 (sys_clk) | ÷16 = 50 MHz | — | — | MHz | — | CLKOUT1_DIVIDE=16 | Primary domain. ~99% of all FFs. |
| Output jitter | < 150 ps pk-pk | — | — | ps | Vivado | — | 50 MHz period = 20 ns. Jitter < 0.75% of period. |
| Lock time | < 100 µs | — | — | µs | UG953 | — | global_rst_n held until mmcm_locked. Typical 50 µs. |
| BUFG count | 2 of 32 | 32 total | 30 remaining | — | XC7A35T | DONT_TOUCH | One per MMCM output. (* DONT_TOUCH *) prevents merge. |

## 2.  Clock-Enable (CE) Tick Specification

Four derived enables from a single 26-bit counter on sys_clk. One-cycle-wide registered pulses — NOT clock signals. No BUFG, no additional clock domain, no CDC.

| CE Signal | Frequency | Period | Counter value (50 MHz cycles) | Counter bits | Duration | Consumer modules & purpose |
| --- | --- | --- | --- | --- | --- | --- |
| tick_1khz | 1,000 Hz | 1.000 ms | 49,999 | 16-bit | 1 cycle | UAV pid_engine inner loop (all 3 UAVs TDM). Rate limiter. Anti-windup update. PWM reference tick. |
| tick_100hz | 100 Hz | 10.00 ms | 499,999 | 19-bit | 1 cycle | CS ADCS quaternion EKF predict. UAV 9-state EKF predict. ADCS PD control. IMU FIFO read trigger. Outer-loop PID CE. |
| tick_10hz | 10 Hz | 100.0 ms | 4,999,999 | 23-bit | 1 cycle | GPS innovation EKF update. Formation controller offset. APF 15-pair scan. MVK state update. CVG_SCORE. Arbiter slot advance. Nav FSM waypoint check. |
| tick_1hz | 1 Hz | 1,000 ms | 49,999,999 | 26-bit | 1 cycle | SGP4 orbit propagator epoch. Telemetry superframe start. MAVLink WDT decrement. System health snapshot. LED heartbeat blink. |

## 3.  Reset Synchronization Strategy

Async-assert / synchronous-deassert pattern. Two independent 4-stage chains. Both sourced from rst_n & mmcm_locked. ESTOP is a separate combinational safety path, not a clock-domain reset.

| Reset Signal | Clock Domain | Source | Sync stages | Polarity | Behaviour & consumers |
| --- | --- | --- | --- | --- | --- |
| rst_raw_n | ASYNC | Board pin S1 | — | Active-low | Raw push-button. Bounced; used only as input to both synchronizers. Never clocks any FF directly. |
| rst_50_n | sys_clk 50 MHz | rst_raw_n & mmcm_locked | 4-FF chain | Active-low | Primary synchronised reset. Assert LOW immediately; de-assert after 4 sys_clk cycles once source is clean. Used by ALL sys_clk-domain FFs: EKF, PID, formation, APF, AXI, CE generator, fault manager, telemetry, UART. |
| rst_100_n | clk_100mhz 100 MHz | rst_raw_n & mmcm_locked | 4-FF chain | Active-low | 100 MHz domain reset. Independent sync chain on clk_100mhz. Same combinational source as rst_50_n. Consumers: spi_master ×6, async_fifo write-ports ×6. |
| mmcm_locked | MMCM (async) | MMCM_ADV locked | — | Active-high | Gates both synchronizer sources: rst_async = rst_raw_n & mmcm_locked. If MMCM loses lock, both domains instantly enter reset. Not used as FF clock or CE. |
| estop_n | ASYNC (comb) | fault_flags OR | 0 (comb) | Active-low | Combinational NOT of fault OR-tree. Not a clock-domain reset — does not clear FFs. Tri-states all PWM outputs within ~4 ns. Separate safety path from clock-domain resets. |

## 4.  Clock & CE Distribution by Subsystem

✓ = signal used by subsystem. Convergence engine uses only sys_clk + tick_10hz + tick_1hz. SPI masters use clk_100mhz exclusively. No subsystem uses all six clock/CE signals.

| Subsystem | sys_clk | clk_100 | tick_1k | tick_100 | tick_10 | tick_1 | Usage detail |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cubesat_subsystem ×3 | ✓ | ✓ | — | ✓ | ✓ | ✓ | sys_clk: ADCS, EKF7, telemetry, laser FSM, SGP4 shell. clk_100mhz: IMU SPI write. tick_100hz: EKF predict + ADCS. tick_10hz: orbit data to CVG bus. tick_1hz: SGP4 epoch + CCSDS superframe. |
| uav_subsystem ×3 | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | sys_clk: EKF9, PID TDM, nav FSM, MAVLink, baro. clk_100mhz: IMU SPI write. tick_1khz: inner PID. tick_100hz: EKF + outer PID. tick_10hz: GPS update + WP check. tick_1hz: MAVLink heartbeat TX + WDT decrement. |
| convergence_engine | ✓ | — | — | — | ✓ | ✓ | sys_clk only (no SPI). tick_10hz: MVK update, formation dispatch, APF scan, arbiter advance. tick_1hz: CVG_SCORE latch for telemetry. tick_1khz and tick_100hz NOT used — max rate 10 Hz. |
| axi_lite_interconnect + reg banks | ✓ | — | — | — | — | — | sys_clk only. AXI reads/writes occur any cycle — no CE gate. Responds within 2 sys_clk cycles (registered handshake). |
| ce_strobe_gen (master) | ✓ | — | — | — | — | — | Consumes sys_clk + rst_50_n only. This IS the CE generator. Produces all four tick outputs. |
| global_fault_manager + WDT ×9 | ✓ | — | — | — | — | ✓ | sys_clk for WDT cycle counters. CA WDT uses raw cycle count (50000 cycles = 1 ms). tick_1hz: MAVLink 3-second heartbeat epoch. estop_n is combinational — no clock needed. |
| spi_master ×6 + async_fifo wr ×6 | — | ✓ | — | — | — | — | clk_100mhz ONLY. No tick signals. SPI is self-timed. async_fifo write-ports in 100MHz domain. Reset by rst_100_n only. |
| telemetry_downlink_mux + UART | ✓ | — | — | — | — | ✓ | sys_clk for UART baud counter (434 cycles/bit at 115200 baud). tick_1hz: superframe epoch + TDM slot advance. |

## 5.  MMCM + BUFG Instantiation

clk_wizard_inst + bufg_sys + bufg_100 + reset combinational source

// (* DONT_TOUCH *) on MMCM + both BUFGs prevents Vivado from // merging buffers or moving BUFG placement (* DONT_TOUCH = "yes" *) clk_wiz_0 clk_wizard_inst ( .clk_in1_p (sys_clk_p), // Bank35 E3 LVDS + .clk_in1_n (sys_clk_n), // Bank35 E4 LVDS - .clk_out1 (clk_100mhz_raw), // CLKOUT0 100MHz (pre-BUFG) .clk_out2 (sys_clk_raw), // CLKOUT1 50MHz (pre-BUFG) .locked (mmcm_locked), .reset (~rst_n) ); // BUFG_0 — sys_clk (50 MHz) (* DONT_TOUCH = "yes" *) BUFG bufg_sys ( .I (sys_clk_raw), .O (sys_clk) ); // BUFG_1 — clk_100mhz (100 MHz) (* DONT_TOUCH = "yes" *) BUFG bufg_100 ( .I (clk_100mhz_raw), .O (clk_100mhz) ); // Combined async reset: both conditions must be satisfied wire rst_async_n = rst_n & mmcm_locked; // Two independent reset synchronizers — one per clock domain reset_sync #(.N_STAGES(4)) u_rst_50 ( .clk(sys_clk), .rst_async_n(rst_async_n), .rst_sync_n(rst_50_n)); reset_sync #(.N_STAGES(4)) u_rst_100 ( .clk(clk_100mhz), .rst_async_n(rst_async_n), .rst_sync_n(rst_100_n));

## 6.  Reset Synchronizer RTL  (reset_sync.sv)

Parameterized N-stage synchronizer used twice — once per clock domain. Async assert is immediate; deassert propagates through N FFs. XDC false-path constraints shown inline.

reset_sync.sv — async-assert / sync-deassert (N_STAGES=4 both domains)

// reset_sync.sv — async-assert / synchronous-deassert // N_STAGES flip-flops; async preset to 0 (= reset active) // Deassert propagates through chain: HIGH after N_STAGES sys_clk cycles module reset_sync #( parameter integer N_STAGES = 4 ) ( input wire clk, input wire rst_async_n, // async source (rst_n & mmcm_locked) output wire rst_sync_n ); logic [N_STAGES-1:0] chain; always_ff @(posedge clk or negedge rst_async_n) begin if (!rst_async_n) chain <= '0; // ASYNC assert else chain <= {chain[N_STAGES-2:0], 1'b1}; // SYNC deassert end assign rst_sync_n = chain[0]; // HIGH after N_STAGES clean cycles endmodule // XDC constraints (add to .xdc file): // set_false_path -from [get_ports rst_n] \ // -to [get_pins u_rst_50/chain_reg*/PRE] // set_false_path -from [get_ports rst_n] \ // -to [get_pins u_rst_100/chain_reg*/PRE] // set_false_path -from [get_pins */LOCKED] \ // -to [get_pins u_rst_50/chain_reg*/PRE] // set_false_path -from [get_pins */LOCKED] \ // -to [get_pins u_rst_100/chain_reg*/PRE]

## 7.  CE Strobe Generator RTL  (ce_strobe_gen.sv)

Single 26-bit counter. Four parallel comparators fire independently — no priority chain. Synthesis-time $error assertions enforce integer divisibility.

ce_strobe_gen.sv — 26-bit counter → tick_1khz / tick_100hz / tick_10hz / tick_1hz

// ce_strobe_gen.sv — all four tick_Nhz from single 26-bit counter // CRITICAL: tick_* are clock-enables — NEVER pass to BUFG or use as FF clock module ce_strobe_gen #( parameter integer CLK_HZ = 50_000_000, parameter integer RATE_1K = 1_000, parameter integer RATE_100 = 100, parameter integer RATE_10 = 10, parameter integer RATE_1 = 1, parameter integer CTR_W = 26 ) ( input wire clk, rst_n, // sys_clk 50MHz + rst_50_n output logic tick_1khz, tick_100hz, tick_10hz, tick_1hz ); // Compile-time constants — synthesis-time exact values localparam integer C1K = CLK_HZ/RATE_1K-1; // 49,999 localparam integer C100 = CLK_HZ/RATE_100-1; // 499,999 localparam integer C10 = CLK_HZ/RATE_10-1; // 4,999,999 localparam integer C1 = CLK_HZ/RATE_1-1; // 49,999,999 // Synthesis-time divisibility checks ($error aborts elaboration) initial begin if (CLK_HZ%RATE_1K!=0) $error("CLK_HZ/RATE_1K not integer"); if (CLK_HZ%RATE_100!=0) $error("CLK_HZ/RATE_100 not integer"); if (CLK_HZ%RATE_10!=0) $error("CLK_HZ/RATE_10 not integer"); if (CLK_HZ%RATE_1!=0) $error("CLK_HZ/RATE_1 not integer"); end logic [CTR_W-1:0] cnt; always_ff @(posedge clk) begin if (!rst_n) begin cnt<='0; tick_1khz<=0; tick_100hz<=0; tick_10hz<=0; tick_1hz<=0; end else begin cnt <= (cnt==C1[CTR_W-1:0])? '0 : cnt+1; // All four comparators fire in parallel — no priority tick_1khz <= (cnt==C1K [CTR_W-1:0]); tick_100hz <= (cnt==C100[CTR_W-1:0]); tick_10hz <= (cnt==C10 [CTR_W-1:0]); tick_1hz <= (cnt==C1 [CTR_W-1:0]); end end endmodule

## 8.  CDC Crossings  (3 instances)

Async FIFO for IMU bulk data. 2-FF synchronizer for spi_busy status. Req/ack toggle for spi_start command. All other inter-module signals are within sys_clk domain — no additional CDC.

3 CDC crossings: async_fifo + 2-FF sync + req/ack toggle handshake

// Three CDC crossings in the Convergence MVP design: // 1. IMU bulk data: clk_100mhz→sys_clk — Gray-coded async FIFO // 2. spi_busy flag: clk_100mhz→sys_clk — 2-FF synchronizer // 3. spi_start req: sys_clk→clk_100mhz — req/ack toggle // ── CDC 1: async FIFO for IMU burst data ───────────────────────── async_fifo #(.DATA_W(32), .DEPTH(16), .ADDR_W(4), .SYNC_STAGES(2)) u_imu_fifo ( .wr_clk (clk_100mhz), .wr_rst_n(rst_100_n), .rd_clk (sys_clk), .rd_rst_n(rst_50_n), .wr_data(spi_rx_byte),.wr_en(spi_valid), .rd_data(imu_data), .rd_en(imu_rd_en), .empty(fifo_empty), .full(fifo_full) ); // XDC: set_max_delay -datapath_only 20.0 \ // -from [get_cells u_imu_fifo/wr_ptr_gray_reg*] \ // -to [get_cells u_imu_fifo/wr_ptr_sync_reg*[0]] // ── CDC 2: spi_busy single-bit 2-FF synchronizer ────────────────── logic [1:0] busy_s; always_ff @(posedge sys_clk) if(!rst_50_n) busy_s<=0; else busy_s<={busy_s[0],spi_busy_async}; wire spi_busy_sync = busy_s[1]; // XDC: set_false_path -from spi_busy_reg -to busy_s_reg[0] // ── CDC 3: spi_start req/ack toggle handshake ───────────────────── // sys_clk domain: toggle req_flag when start needed logic req_flag; logic [1:0] ack_s; always_ff @(posedge sys_clk) begin if(!rst_50_n) {req_flag,ack_s}<='0; else begin if(spi_start_req & (ack_s[1]==req_flag)) req_flag<=~req_flag; ack_s <= {ack_s[0], ack_from_100}; end end // clk_100mhz domain: detect toggle, echo as ack logic [1:0] req_s; logic req_prev; always_ff @(posedge clk_100mhz) req_s<={req_s[0],req_flag}; assign spi_start_100 = req_s[1] ^ req_prev; // edge detect = start pulse assign ack_from_100 = req_s[1]; // toggle echo = ack

## 9.  Correct CE Usage Pattern

tick_Nhz signals must gate always_ff blocks on sys_clk — never used as FF clocks. Vivado treats them as data paths under standard setup/hold analysis.

Correct CE gate pattern + incorrect misuse examples + Vivado multicycle guidance

// ── CORRECT: tick_Nhz as CE inside always_ff on sys_clk ────────── always_ff @(posedge sys_clk) begin if (!rst_50_n) begin ekf_x <= '0; pid_u <= '0; orbit <= '0; end else begin if (tick_1khz ) pid_u <= pid_next; // 1kHz inner PID if (tick_100hz ) ekf_x <= ekf_next; // 100Hz EKF predict if (tick_10hz ) form_v <= form_next; // 10Hz formation if (tick_1hz ) orbit <= orbit_next; // 1Hz SGP4 end end // ── ILLEGAL: tick used as clock — creates unconstrained domain ─── // always_ff @(posedge tick_100hz) // ← NEVER: CDC hazard + glitch risk // always_ff @(posedge tick_1khz) // ← NEVER: Vivado cannot analyse paths // ── CORRECT: SPI trigger on tick CE, with CDC handshake ────────── always_ff @(posedge sys_clk) begin if (!rst_50_n) spi_req<=0; else if (tick_100hz & !spi_busy_sync) spi_req<=1; // request SPI transfer every 10ms else if (req_acked) spi_req<=0; end // ── Vivado timing: tick_Nhz is a DATA signal, not a clock ───────── // Standard setup/hold analysis covers all CE-gated paths automatically. // Only add multicycle path if your gated logic has > 1 cycle of logic depth: // set_multicycle_path -setup -end 2 -to [get_cells pid_u_reg*]

## 10.  XDC Timing Constraints  (convergence_mvp_clocks.xdc)

Complete constraint file: clock creation, set_clock_groups, reset false paths, CDC max-delay/false-path for all 3 crossings, DONT_TOUCH attributes, SPI I/O delays.

convergence_mvp_clocks.xdc — complete Vivado timing constraints

# convergence_mvp_clocks.xdc # Vivado timing constraints — clocks, resets, CDC # Primary 50MHz input create_clock -period 20.000 -name sys_clk_in [get_ports sys_clk_p] # BUFG outputs (design clocks) create_generated_clock -name sys_clk \ -source [get_pins bufg_sys/I] -divide_by 1 [get_pins bufg_sys/O] create_generated_clock -name clk_100mhz \ -source [get_pins bufg_100/I] -divide_by 1 [get_pins bufg_100/O] # Clocks are asynchronous to each other (CDC handled explicitly) set_clock_groups -asynchronous -group sys_clk -group clk_100mhz # Reset false paths (async assert to both domains) set_false_path -from [get_ports rst_n] -to [get_pins u_rst_50/chain_reg*/PRE] set_false_path -from [get_ports rst_n] -to [get_pins u_rst_100/chain_reg*/PRE] set_false_path -from [get_pins */LOCKED] -to [get_pins u_rst_50/chain_reg*/PRE] set_false_path -from [get_pins */LOCKED] -to [get_pins u_rst_100/chain_reg*/PRE] # CDC 1: async FIFO Gray pointer crossing set_max_delay -datapath_only 20.0 \ -from [get_cells u_imu_fifo*/wr_ptr_gray_reg*] \ -to [get_cells u_imu_fifo*/wr_ptr_sync_reg*[0]] # CDC 2: spi_busy 2-FF sync (false path on launch FF) set_false_path -from [get_cells spi_busy_reg] -to [get_cells busy_s_reg[0]] # CDC 3: req/ack toggle flags set_false_path -from [get_cells req_flag_reg] -to [get_cells req_s_reg[0]] set_false_path -from [get_cells ack_from_100_reg] -to [get_cells ack_s_reg[0]] # DONT_TOUCH on BUFGs set_property DONT_TOUCH true [get_cells bufg_sys] set_property DONT_TOUCH true [get_cells bufg_100] # SPI I/O: ICM-42688 tco_max=14ns @25MHz SCK set_input_delay -clock clk_100mhz -max 14.0 [get_ports {*imu_miso*}] set_input_delay -clock clk_100mhz -min 0.0 [get_ports {*imu_miso*}] set_output_delay -clock clk_100mhz -max 2.0 [get_ports {*imu_mosi*}] set_output_delay -clock clk_100mhz -min 0.0 [get_ports {*imu_mosi*}]