# APF Collision Avoidance  —  Fixed-Point Repulsive Force Specification

Algorithm: Artificial Potential Field (APF)  |  6 vehicles  |  15 pairs  |  Safety radius 10m  |  Q10.22 positions  |  8 DSP48E1  |  <3 µs compute @ 50 MHz

Key optimization: d² = dx²+dy²+dz² (sum of squares) eliminates sqrt for threshold check. Newton-Raphson 1-iter only for force magnitude. Zero sqrt for gate decision.

## 1.  Complete Force Equation — Step-by-Step Fixed-Point Derivation

Every step from raw position inputs to final velocity correction output, with exact fixed-point formats, DSP48 cost, and justification for each computational choice.

| Step | Equation (Symbolic) | Equation (Expanded) | Fixed-Point Notes & Rationale |
| --- | --- | --- | --- |
| 1. Delta vector | delta_ij = P_i - P_j [dx, dy, dz] | dx = Px_i - Px_j dy = Py_i - Py_j dz = Pz_i - Pz_j | Q10.22 - Q10.22 = Q10.22 (exact, no overflow if ¦delta¦ < 1023m). 3 fabric subtractions, 0 DSP48. Points FROM threatening vehicle j TOWARD protected vehicle i — direction of repulsion. Compute for all 15 pairs. |
| 2. Sum of squares (no sqrt needed for threshold check) | d²_ij = dx² + dy² + dz² | d²_ij = dx*dx + dy*dy + dz*dz Threshold: THREAT if d²_ij < D_SAFE_SQ D_SAFE_SQ = 100 m² | Q10.22 × Q10.22 = Q20.44 → take bits [43:12] → Q20.32 (64-bit intermediate, 3 DSP48). Sum three squares. D_SAFE_SQ = 100 × 2^{32} = 429,496,729,600 in Q20.44 representation. Comparison is pure unsigned integer compare — ZERO sqrt needed. This is the key optimization: all 15 pair threat checks use only squares. |
| 3. Threat gate (zero force outside safety radius) | if d²_ij >= D_SAFE_SQ: F_rep_ij = [0,0,0] else: compute force | threat_ij = (d²_ij < D_SAFE_SQ) if !threat_ij: skip pair D_SAFE = 10m (configurable) D_SAFE_SQ = 100 m² | Single 64-bit compare per pair. threat_ij bit stored in THREAT_FLAGS register (15 bits, one per pair). Only pairs with threat_ij=1 proceed to force computation. Typical case: 0 threatening pairs → minimal compute. Worst case: 1 pair → one full APF compute. CAUTION zone: CAUTION_SQ = 400 m² (2×d_safe²). |
| 4. Inverse sqrt (Newton-Raphson, 1 iteration) | inv_d = 1/sqrt(d²) y_{n+1} = y_n*(1.5 - 0.5*d²*y_n²) Initial: y_0 from BRAM LUT | y0 = LUT[d²_upper_8_bits] y1 = y0*(1.5 - 0.5*d²*y0*y0) Result: inv_d ≈ 1/d Format: Q1.30 | BRAM LUT: 256 entries × 16-bit covering d² ∈ [0.01, 400] m² (= safety zone). 1 NR iteration gives 14-bit accuracy (vs 8-bit LUT). For force computation, 10-bit accuracy in inv_d is sufficient (force is velocity correction, not navigation). 2 DSP48 for NR. If higher accuracy needed: 2 NR iters (3 DSP48 total). |
| 5. Scalar force factor alpha | alpha = k_rep*(1/d - 1/d_safe)/d² = k_rep*(inv_d - inv_dsafe)*inv_d² | inv_dsafe = 1/10 = 0.1 m^{-1} = fixed constant 0x1999999A (Q4.28) alpha = k_rep * inv_d * (inv_d - 0.1) * inv_d = k_rep * inv_d² * (inv_d - inv_dsafe) | k_rep = 5.0 m³/s² default → Q4.28: 0x50000000. inv_dsafe = 0.1 m^{-1} = constant. inv_d from NR above (Q1.30). Product: Q4.28 × Q2.60 (inv_d² = inv_d×inv_d) → wide intermediate. alpha in Q1.30 after scaling. Sign check: alpha is POSITIVE when d < d_safe (repulsive). NEGATIVE means attraction → clip to 0 (should never occur but protect against NR error). |
| 6. Force vector (per axis) | F_rep_x = alpha * dx F_rep_y = alpha * dy F_rep_z = alpha * dz | F_x = k_rep*(1/d - 1/d_safe)*(1/d²)*dx = alpha * dx F in m/s² (velocity correction scaled by Ts to get m/s command) | Q1.30(alpha) × Q10.22(delta) = Q11.52 → take bits [51:22] → Q11.30 → clip to Q4.28 (m/s²). 3 DSP48 (one per axis). Saturation: ¦F_x¦ clamped to F_MAX (configurable, default 5 m/s²). The force IS velocity: multiply by Ts (0.01s) to convert to m/s position correction per cycle. 3 DSP48. |
| 7. Sum over all threatening pairs | F_total_i = SUM_{j≠i, threat_ij} F_rep_ij (Sum up to 5 terms for 6-vehicle fleet) | F_total_x = F_01x + F_02x + ... (active pairs) F_total_y = ... F_total_z = ... Max 5 threatening neighbors | Fabric adder: Q4.28 + Q4.28 = Q5.27 (one extra bit for sum). Maximum 5 terms → Q7.25 worst case. Use Q6.26 (32-bit) accumulator: handles sum of 5 × F_MAX without overflow if ¦F_MAX¦ < 8 m/s². Saturation of final sum to CA_VEL_MAX (5 m/s default). 0 DSP48 needed. |
| 8. Velocity command addition | v_cmd_i = v_formation_i + F_total_i * Ts (APF overrides if ¦F_total¦ > threshold) | v_cmd_x = v_form_x + F_total_x * 0.01 v_cmd_y = v_form_y + F_total_y * 0.01 v_cmd_z = v_form_z + F_total_z * 0.01 0.01 = Ts = Q1.15 = 328 | F_total in Q6.26 × Ts(Q1.15) → Q7.41 → take bits [40:13] → Q6.28 ≈ Q4.28 velocity correction. Add to formation velocity command. If ¦F_total¦ > 0: CA_OVERRIDE flag active; formation command suppressed (pure APF). If ¦F_total¦ < epsilon: formation command passes through. Override priority: CA > formation controller per CV-CA-001. |
| 9. Damping term (oscillation prevention) | F_damp = -k_damp * v_rel_ij v_rel_ij = V_i - V_j (relative velocity) | F_damp_x = -k_damp*(vx_i - vx_j) F_damp_y = -k_damp*(vy_i - vy_j) Add to F_rep: F_net = F_rep + F_damp | Damping prevents APF oscillation (two vehicles bouncing off each other). k_damp = 0.3 default (Q4.28). v_rel from EKF velocity outputs. Activated only when OSCILLATION_DETECT flag set (relative velocity reverses sign twice within 3 cycles). 3 additional DSP48 multiplies when active. |

## 2.  Fixed-Point Format Reference

All signals from position inputs through delta computation, d² gate, inv_sqrt, alpha scalar, and force vector output.

| Signal | Bits | Q Format | Integer = Real × | Range | Notes |
| --- | --- | --- | --- | --- | --- |
| Px_i, Py_i, Pz_i (positions) | 32s | Q10.22 | 4,194,304 | ±1023 m | EKF position outputs. Same format as formation controller. |
| dx, dy, dz (delta vector) | 32s | Q10.22 | 4,194,304 | ±1023 m | P_i − P_j. Signed subtraction. Max range ±1023m > worst-case separation. |
| dx², dy², dz² (squared deltas) | 64u | Q20.44 | 2^44 = 1.76×10¹³ | 0 to ~1023² m² | DSP48 P-port (48-bit) holds dx² in Q20.44 range = up to 1.76×10¹³. For ¦d¦<1023m, dx²<1.05×10⁶ m² fits in 40 bits. |
| d² = sum of squares | 64u | Q20.44 | 2^44 | 0 to 3×10⁶ m² | Sum of 3 squared terms. Gate threshold D_SAFE_SQ = 100 m² = 4.39×10¹⁵ in Q20.44 representation. |
| D_SAFE_SQ = 100 m² | 64u | Q20.44 | 4,398,046,511,104 | 100 m² | Constant. Pre-computed: 100 × 2^44 = 1.76×10¹⁵. Store as 48-bit constant in BRAM. |
| inv_d = 1/d | 32s | Q1.30 | 1,073,741,824 | 0 to ~10 m^{-1} | From NR. d_min≈1m → inv_d_max=1.0. d_max=10m → inv_d_min=0.1. Q1.30 covers [0,1] exactly. |
| inv_d_safe = 0.1 | 32s | Q4.28 | 268,435,456 | 0.1 m^{-1} | Constant: 0.1×2^28 = 26,843,546. Hardcoded register. |
| alpha scalar | 32s | Q2.30 | 1,073,741,824 | 0 to ~10 | k_rep*(1/d-1/d_safe)*1/d². Dimensionless force per unit distance. Positive for d<d_safe. |
| k_rep | 32s | Q4.28 | 268,435,456 | 0 to 15.99 m³/s² | Default 5.0 m³/s² → 1,342,177,280. Configurable AXI4-Lite register. |
| F_rep (per component) | 32s | Q4.28 | 268,435,456 | ±15.99 m/s² | alpha × delta. Clipped to ±F_MAX (default 5 m/s²). Applied as velocity correction × Ts. |
| F_total (summed) | 32s | Q6.26 | 67,108,864 | ±63.9 m/s² | Sum of up to 5 pair forces. Extra bits prevent overflow. Saturated to ±CA_VEL_MAX = 5 m/s². |
| v_cmd (final) | 32s | Q4.28 | 268,435,456 | ±15.99 m/s | F_total × Ts + v_formation. Dispatched by resource arbiter. |
| k_damp | 32s | Q4.28 | 268,435,456 | 0 to 0.99 s^{-1} | Damping gain. Default 0.3 → 80,530,637. Activates on OSCILLATION_DETECT. |

## 3.  Numerical Examples — Force Values at Key Distances

Worked examples showing exact force magnitudes and directions at boundary (d=10m, force=0), half-radius (d=5m, moderate), close approach (d=1m, near max), and a 3D diagonal scenario.

| Scenario | Inputs (dx,dy,dz m) | Computed Values | Fixed-Point Integers |
| --- | --- | --- | --- |
| d = d_safe (boundary) Force = 0 | dx=6, dy=8, dz=0 d = sqrt(36+64) = 10.0m d² = 100 m² | 1/d = 0.1 m^{-1} 1/d_safe = 0.1 m^{-1} (1/d - 1/d_safe) = 0.0 F_rep = 0 (exactly) | d² = 100 × 2^{32} = 429,496,729,600 D_SAFE_SQ same → NOT less than gate_ij = 0 → force bypassed Result: F_vec = [0, 0, 0] |
| d = 5m (half safety radius) Moderate force | dx=3, dy=4, dz=0 d = sqrt(9+16) = 5.0m d² = 25 m² | 1/d = 0.2 1/d_safe = 0.1 diff = 0.1 m^{-1} alpha = 5.0×0.1/25 = 0.02 Fx=0.02×3=0.060 m/s² Fy=0.02×4=0.080 m/s² ¦F¦=0.10 m/s² | inv_d = round(0.2×2^{30}) = 214,748,365 inv_dsafe = 26,843,546 diff_fp = 187,904,819 k_rep_fp = 1,342,177,280 alpha_fp ≈ 10,737,418 Fx_fp = alpha×dx (Q4.28 output) = 3×10,737,418 → 32,212,254 |
| d = 1m (very close, max force) High repulsion | dx=1, dy=0, dz=0 d = 1.0m d² = 1 m² | 1/d = 1.0 1/d_safe = 0.1 diff = 0.9 m^{-1} alpha = 5.0×0.9/1 = 4.5 m/s²/m Fx=4.5×1=4.5 m/s² ¦F¦=4.5 m/s² (near CA_VEL_MAX = 5 m/s²) | Before saturation: F_x_fp=4.5×2^{28}=1,207,959,552 CA_VEL_MAX = 5 m/s → 5×2^{28}=1,342,177,280 No saturation in this case. Ts correction: 4.5×0.01=0.045 m/s velocity command |
| Diagonal threat (3D example) Mixed axes | dx=0, dy=8, dz=6 d = sqrt(0+64+36) = 10.0m (same distance, different dir) | Same magnitude as example 1 (on boundary) F_rep = 0 exactly If dx=0,dy=5.66,dz=4.24 (d=7m): alpha=5*(1/7-0.1)/49=0.00875 Fy=0.00875×5.66=0.0495 Fz=0.00875×4.24=0.0371 | d²=64+36=100 → gate off (boundary) For d=7m example: d²_fp = 49×2^{32} = 210,453,397,504 gate: less than D_SAFE_SQ → active Force components computed per Step 6 |

## 4.  15-Pair Processing Schedule (Time-Multiplexed)

All 15 unique vehicle pairs processed sequentially using 8 shared DSP48E1 slices. One pair per ~8-cycle slot. Total computation completes in ~128 cycles = 2.56 µs.

| Cycle | Pair (Vi,Vj) | threat_ij check | Force computation |
| --- | --- | --- | --- |
| 1 | V1 ↔ V2 | d²_{12} < D_SAFE_SQ? | Compute F_rep_{12} all 3 axes (8 DSP48 ops) |
| 2 | V1 ↔ V3 | d²_{13} < D_SAFE_SQ? | Compute F_rep_{13} all 3 axes (8 DSP48 ops) |
| 3 | V1 ↔ V4 | d²_{14} < D_SAFE_SQ? | Compute F_rep_{14} all 3 axes (8 DSP48 ops) |
| 4 | V1 ↔ V5 | d²_{15} < D_SAFE_SQ? | [same] — time-multiplexed DSP48 |
| 5 | V1 ↔ V6 | d²_{16} < D_SAFE_SQ? | [same] — time-multiplexed DSP48 |
| 6 | V2 ↔ V3 | d²_{23} < D_SAFE_SQ? | [same] — time-multiplexed DSP48 |
| 7 | V2 ↔ V4 | d²_{24} < D_SAFE_SQ? | [same] |
| 8 | V2 ↔ V5 | d²_{25} < D_SAFE_SQ? | [same] |
| 9 | V2 ↔ V6 | d²_{26} < D_SAFE_SQ? | [same] |
| 10 | V3 ↔ V4 | d²_{34} < D_SAFE_SQ? | [same] |
| 11 | V3 ↔ V5 | d²_{35} < D_SAFE_SQ? | [same] |
| 12 | V3 ↔ V6 | d²_{36} < D_SAFE_SQ? | [same] |
| 13 | V4 ↔ V5 | d²_{45} < D_SAFE_SQ? | [same] |
| 14 | V4 ↔ V6 | d²_{46} < D_SAFE_SQ? | [same] |
| 15 | V5 ↔ V6 | d²_{56} < D_SAFE_SQ? | [same] |
| 16 | — | — | Sum F_rep per vehicle (fabric adder tree). Apply damping if OSCILLATION. Final clamp. |
| TOTAL | 15 pairs + 1 sum | 16 cycles | 16 × 8 ns = 128 ns @ 50 MHz (actually 16 × ~8 cycles = 128 cycles = 2.56 µs) |

## 5.  DSP48E1 Resource Allocation

8 DSP48E1 shared across all 9 computation steps via time-multiplexing. The sqrt (NR) is the most DSP-intensive step; damping adds 3 more ops when oscillation is detected.

| Operation | DSP48 | Cycles | Shared? | Implementation Detail |
| --- | --- | --- | --- | --- |
| d² = dx²+dy²+dz² (3 squarings + 2 adds) | 3 | 6 | Yes (3 reused) | Each: A×B where A=B=delta_component. P-port 48-bit holds result. Add 3 products in fabric adder (3×48-bit). One DSP per component, sequential. |
| d² threshold compare | 0 | 1 | — | 64-bit unsigned compare in fabric. d² vs D_SAFE_SQ constant. Single cycle combinational. |
| LUT lookup for y0 (inv_sqrt initial) | 0 | 2 | — | BRAM read: 2-cycle latency. 256-entry × 16-bit LUT. Index = d²[47:40] (top 8 bits of Q20.44 d²). |
| NR iteration for inv_d (y1 = y0*(1.5-0.5*d²*y0²)) | 3 | 6 | Yes | 3 DSP48: (1) y0*y0, (2) d²*yy, (3) y0*bracket. Sequential reuse of same physical DSPs. Result: inv_d in Q1.30 with ~14-bit accuracy. |
| alpha = k_rep*diff*inv_d² | 2 | 4 | Yes | diff = inv_d - inv_dsafe (fabric subtract). inv_d² = inv_d*inv_d (1 DSP). alpha = k_rep * diff * inv_d_sq (1 DSP with MACC). Result Q2.30. |
| F_vec = alpha * delta (3 axis components) | 3 | 6 | Yes | F_x = alpha * dx, etc. Q2.30 × Q10.22 → Q12.52 → shift 30 → Q4.28. One DSP per axis, sequential. |
| Damping: k_damp * v_rel (3 components) | 3 | 6 | Yes (only when OSCILLATION) | k_damp × (vx_i - vx_j). v_rel = velocity difference from EKF. Same 3 DSPs reused. Only active when OSCILLATION_DETECT flag. |
| F_sum across pairs (fabric adder tree) | 0 | 3 | — | Up to 5 Q4.28 terms added in fabric. 5×32-bit adder tree (4 adders). No DSP needed — adder chain faster than DSP setup at this width. |
| Velocity command blend F_total*Ts + v_form | 1 | 2 | Yes | F_total × Ts (Q6.26 × Q1.15 = Q7.41 → shift → Q4.28). Add v_form (Q4.28). One DSP + 1 fabric add. |
| TOTAL PER VEHICLE | 8 DSP48 (shared TDM) | ~30 cycles per threat pair | All 8 shared | 8 DSP48s time-multiplexed across all 15 pairs. 15 pairs × ~8 DSP ops = ~120 total ops. At 1 op/cycle: 120 cycles = 2.4 µs << 10 ms deadline. |

## 6.  Verilog RTL — Pair Force Compute Unit

Single-pair APF computation: delta → d² → threshold gate → NR inv_sqrt → alpha scalar → F_vec → saturate. 8 DSP48. ~30 cycle pipeline. Threat gate avoids computation when d > d_safe.

apf_pair_force.v — Single-Pair Repulsion Force (8 DSP48, ~30 cycles)

// ================================================================ // APF Repulsion Force — Single Vehicle-Pair Computation // F_rep = k_rep * (1/d - 1/d_safe) / d² * delta_vec // Key optimization: d² computed exactly (no sqrt for threshold) // sqrt only for force magnitude (Newton-Raphson, 1 iteration) // DSP48: 8 ¦ Latency: ~30 cycles ¦ Format: Q10.22 pos, Q4.28 force // ================================================================ module apf_pair_force ( input wire clk, rst_n, en, // Positions of two vehicles (Q10.22) input wire signed [31:0] Pi_x,Pi_y,Pi_z, // vehicle i input wire signed [31:0] Pj_x,Pj_y,Pj_z, // threatening vehicle j // Parameters (configurable) input wire [31:0] k_rep, // Q4.28, default 5.0 m^3/s^2 input wire [47:0] D_SAFE_SQ, // Q20.44, default 100m^2 // Outputs output reg signed [31:0] F_x,F_y,F_z, // Q4.28 m/s^2 output reg threat, // 1 = pair is within safety zone output reg valid ); // ── STAGE 1: Delta vector (Q10.22 subtraction) ───────────────────── wire signed [31:0] dx = $signed(Pi_x) - $signed(Pj_x), // direction away from j dy = $signed(Pi_y) - $signed(Pj_y), dz = $signed(Pi_z) - $signed(Pj_z; // ── STAGE 2: d² = dx²+dy²+dz² (3 DSP48, sum in fabric) ────────────── // Each: Q10.22 × Q10.22 = Q20.44 (48-bit from DSP P-port) wire [47:0] dx2 = $signed(dx) * $signed(dx), // Q20.44, always positive dy2 = $signed(dy) * $signed(dy), dz2 = $signed(dz) * $signed(dz; wire [49:0] d2 = dx2 + dy2 + dz2; // 50-bit for safety (sum of 3) // ── STAGE 3: Threat gate — compare d² vs D_SAFE_SQ (no sqrt!) ──────── wire threat_wire = (d2[49:0] < {2'b0, D_SAFE_SQ}); // zero-extend D_SAFE_SQ to 50-bit // ── STAGE 4: Newton-Raphson inv_sqrt (1 iteration) ─────────────────── // y0 from BRAM LUT indexed by d2[47:40] (top 8 bits) reg [15:0] inv_sqrt_lut[0:255]; initial $readmemh("inv_sqrt_lut.hex", inv_sqrt_lut); wire [7:0] lut_idx = d2[47:40]; // top 8 bits as index wire signed [31:0] y0 = {{16{1'b0}}, inv_sqrt_lut[lut_idx]}; // Q1.30 estimate // NR: y1 = y0 * (THREE_HALF - HALF * d2_scaled * y0 * y0) // THREE_HALF in Q2.13 = 24576 (3.0 * 8192) wire signed [63:0] y0sq = $signed(y0) * $signed(y0); // Q2.60 wire signed [47:0] d2_sc = d2[43:0]; // Q20.24 (right shift 20) wire signed [63:0] ny = $signed(d2_sc) * $signed(y0sq[59:28]); // ns*y^2 term wire signed [15:0] brkt = 16'sd24576 - $signed(ny[42:27]);// (3 - ns*y2) Q2.13 wire signed [47:0] y1w = $signed(y0) * $signed(brkt); // Q3.43 wire signed [31:0] inv_d = y1w[43:13]; // Q1.30 // ── STAGE 5: alpha = k_rep * (inv_d - inv_dsafe) * inv_d^2 ───────── localparam signed [31:0] INV_DSAFE = 32'sd26843546; // 0.1 * 2^28 wire signed [31:0] diff = $signed(inv_d) - $signed(INV_DSAFE); // positive if d<d_safe wire signed [63:0] inv_d2 = $signed(inv_d) * $signed(inv_d); // Q2.60 wire signed [63:0] alpha_w = ($signed(k_rep) * $signed(diff)) * $signed(inv_d2[59:28]); wire signed [31:0] alpha = alpha_w[55:24]; // Q2.30, scale to match // ── STAGE 6: F_vec = alpha * delta_vec (3 DSP48) ──────────────────── wire signed [63:0] Fx_w = $signed(alpha) * $signed(dx), // Q2.30 × Q10.22 = Q12.52 Fy_w = $signed(alpha) * $signed(dy), Fz_w = $signed(alpha) * $signed(dz; // ── STAGE 7: Saturation + output register ──────────────────────────── localparam signed [31:0] F_MAX = 32'sd1342177280; // 5.0 m/s^2 in Q4.28 always @(posedge clk) begin if (!rst_n) beginF_x<=0; F_y<=0; F_z<=0; threat<=0; valid<=0; end else if (en) begin threat <= threat_wire; if (!threat_wire) beginF_x<=0; F_y<=0; F_z<=0; end // outside zone else begin // F_vec: Q12.52 → shift 30 → Q4.28, then saturate wire signed [31:0] fx = Fx_w[57:26]; // >>26: Q12.52→Q4.26≈Q4.28 wire signed [31:0] fy = Fy_w[57:26]; wire signed [31:0] fz = Fz_w[57:26]; F_x <= ($signed(fx) > $signed(F_MAX)) ? F_MAX : ($signed(fx) < -$signed(F_MAX)) ? -$signed(F_MAX) : fx; F_y <= ($signed(fy) > $signed(F_MAX)) ? F_MAX : ($signed(fy) < -$signed(F_MAX)) ? -$signed(F_MAX) : fy; F_z <= ($signed(fz) > $signed(F_MAX)) ? F_MAX : ($signed(fz) < -$signed(F_MAX)) ? -$signed(F_MAX) : fz; end valid <= 1'b1; end end endmodule

## 7.  Verilog RTL — APF Top-Level Controller

Scans all 15 pairs sequentially. Accumulates Newton's-3rd-law forces (pair repulsion adds to Vi and subtracts from Vj symmetrically). Final saturation, threat_flags register, and ca_override assertion.

apf_controller.v — 15-pair Scan + Force Summation (8 DSP48 TDM)

// ================================================================ // APF Top Level: 15-pair scan + per-vehicle force summation // Iterates all 15 pairs, accumulates F_rep per vehicle // Adds damping when oscillation detected // Output: per-vehicle velocity corrections (Q4.28) // ================================================================ module apf_controller ( input wire clk, rst_n, en_10hz, input wire signed [31:0] P[0:5][0:2], // [vehicle][x/y/z] Q10.22 input wire signed [31:0] V[0:5][0:2], // velocities Q4.28 (for damping) input wire [31:0] k_rep,k_damp, output reg signed [31:0] F_ca[0:5][0:2], // CA velocity correction Q4.28 output reg [14:0] threat_flags, // bit per pair (15 pairs) output reg ca_override, // 1 = any threat active output reg [5:0] oscil_detect ); // ── Pair index table (15 pairs from 6 vehicles) ──────────────────── // Pairs: (0,1),(0,2),(0,3),(0,4),(0,5),(1,2),(1,3)... reg [2:0] pi[0:14], pj[0:14]; // pair vehicle indices initial begin // 15 pairs: (0,1),(0,2),(0,3),(0,4),(0,5),(1,2),(1,3),(1,4),(1,5),(2,3),(2,4),(2,5),(3,4),(3,5),(4,5) pi[0]=0;pj[0]=1; pi[1]=0;pj[1]=2; pi[2]=0;pj[2]=3; pi[3]=0;pj[3]=4; pi[4]=0;pj[4]=5; pi[5]=1;pj[5]=2; pi[6]=1;pj[6]=3; pi[7]=1;pj[7]=4; pi[8]=1;pj[8]=5; pi[9]=2;pj[9]=3; pi[10]=2;pj[10]=4; pi[11]=2;pj[11]=5; pi[12]=3;pj[12]=4; pi[13]=3;pj[13]=5; pi[14]=4;pj[14]=5; end // ── Sequential FSM: scan 15 pairs, accumulate forces ────────────── reg [3:0] pair_idx; // 0..14 reg signed [33:0] F_acc[0:5][0:2]; // accumulator Q6.26 reg scanning; wire signed [31:0] apf_Fx,apf_Fy,apf_Fz; wire apf_threat,apf_valid; // Instantiate single APF pair compute unit (time-multiplexed) apf_pair_force u_apf ( .clk(clk), .rst_n(rst_n), .en(scanning), .Pi_x(P[pi[pair_idx]],[0]), .Pi_y(P[pi[pair_idx]],[1]), .Pi_z(P[pi[pair_idx]],[2]), .Pj_x(P[pj[pair_idx]],[0]), .Pj_y(P[pj[pair_idx]],[1]), .Pj_z(P[pj[pair_idx]],[2]), .k_rep(k_rep), .D_SAFE_SQ(48'h63FFFFFFFFF), // 100*2^32 approx .F_x(apf_Fx), .F_y(apf_Fy), .F_z(apf_Fz), .threat(apf_threat), .valid(apf_valid) ); always @(posedge clk) begin if (!rst_n) beginpair_idx<=0; scanning<=0; ca_override<=0; end else if (en_10hz & !scanning) begin // Start new scan: clear accumulators integer v, ax; for (v=0;v<6;v=v+1) for (ax=0;ax<3;ax=ax+1) F_acc[v][ax]<=0; scanning<=1; pair_idx<=0; end else if (scanning) begin if (apf_valid) begin // Accumulate force on BOTH vehicles (Newton's 3rd: equal and opposite) threat_flags[pair_idx] <= apf_threat; if (apf_threat) begin F_acc[pi[pair_idx]],[0] <= $signed(F_acc[pi[pair_idx]],[0]) + $signed(apf_Fx); F_acc[pi[pair_idx]],[1] <= $signed(F_acc[pi[pair_idx]],[1]) + $signed(apf_Fy); F_acc[pi[pair_idx]],[2] <= $signed(F_acc[pi[pair_idx]],[2]) + $signed(apf_Fz); // Vehicle j receives opposite force F_acc[pj[pair_idx]],[0] <= $signed(F_acc[pj[pair_idx]],[0]) - $signed(apf_Fx); F_acc[pj[pair_idx]],[1] <= $signed(F_acc[pj[pair_idx]],[1]) - $signed(apf_Fy); F_acc[pj[pair_idx]],[2] <= $signed(F_acc[pj[pair_idx]],[2]) - $signed(apf_Fz); end pair_idx <= (pair_idx==14) ? 4'd0 : pair_idx+1; if (pair_idx==14) begin scanning<=0; ca_override<=(¦threat_flags); // Saturate and output F_ca (Q4.28, ±5 m/s^2) integer vi,ax; for (vi=0;vi<6;vi=vi+1) for (ax=0;ax<3;ax=ax+1) F_ca[vi][ax] <= ($signed(F_acc[vi][ax]) > $signed(32'sd1342177280)) ? 32'sd1342177280) : ($signed(F_acc[vi][ax]) < -$signed(32'sd1342177280)) ? (-32'sd1342177280) : F_acc[vi][ax][31:0; end end end end endmodule

## 8.  Design Notes

| Issue | Detail & Mitigation |
| --- | --- |
| Sum-of-squares gate (no sqrt) | The most important optimization: d < d_safe ↔ d² < d_safe². Since d_safe=10m, D_SAFE_SQ=100 m². This single comparison replaces sqrt entirely for the 15-pair threat check. Only ~0% of compute time. All 15 gate checks = 15 × 1 cycle = 15 cycles in fabric. |
| NR sqrt accuracy | Newton-Raphson 1 iteration from 8-bit LUT gives ~14-bit accuracy. Force computation requires only ~10-bit accuracy in inv_d (force values are velocity corrections 0–5 m/s², a 3-bit dynamic range). 1 NR iteration is always sufficient. 2 iterations available if k_rep is large. |
| d → 0 singularity | When d approaches 0, F_rep → infinity. Mitigation: (1) hard minimum d_min = 0.5m hardcoded in LUT (LUT[0] = max representable inv_sqrt value), (2) F_MAX saturation clamps output to 5 m/s², (3) collision avoidance at d < 1m triggers EMERGENCY E-STOP regardless of computed force. |
| Newton's 3rd law | The top-level controller applies F_rep to vehicle i AND the equal-opposite −F_rep to vehicle j in the same pair. This ensures the fleet remains in equilibrium: no net momentum created by APF forces. Both vehicles move away from each other symmetrically. |
| Formation override | When CA_OVERRIDE is active for a vehicle, the formation controller velocity command for that vehicle is zeroed. Pure APF drives the motion. This is a hard combinational override: CA force replaces formation command within 1 arbiter cycle (< 10 ms per CV-CA-001). |
| Oscillation detection | If v_rel (relative velocity between threatening pair) changes sign twice within 3 consecutive 10Hz cycles: OSCILLATION_DETECT flag. Activates damping term F_damp = -k_damp × v_rel. Prevents APF ping-pong between two vehicles. k_damp default 0.3 s^{-1}. |