# Clock Architecture Specification

Convergence MVP FPGA  |  Xilinx Artix-7 XC7A35T (CPG236)  |  Multi-Vehicle CubeSat & UAV Integration

Primary: 50 MHz sys_clk (E3)  •  MMCM0: 100 MHz SPI clock  •  5× CE-domain control strobes  •  7 CDC paths

## Color Legend

| Color | Domain / Risk | Description |
| --- | --- | --- |
|  | sys_clk | 50 MHz primary fabric clock — all CE-domain logic lives here |
|  | clk_100mhz | 100 MHz MMCM output — true async clock domain for SPI |
|  | ctrl_1khz (CE) | 1 kHz enable strobe — UAV PID & ESC updates |
|  | ctrl_100hz (CE) | 100 Hz enable strobe — CubeSat ADCS & EKF |
|  | ctrl_10hz (CE) | 10 Hz enable strobe — Convergence engine solver |
|  | ctrl_1hz (CE) | 1 Hz enable strobe — Orbit propagation & housekeeping |
|  | CDC Risk: HIGH | True asynchronous clock crossing — async FIFO required |
|  | CDC Risk: MEDIUM | Single-bit or handshake CDC — two-FF synchronizer |
|  | CDC Risk: LOW | CE-domain boundary — multicycle path constraint sufficient |

## Section 1 — Clock Domain Table

All six clock domains are defined below. Four of the five control clocks (ctrl_1khz through ctrl_1hz) are implemented as synchronous enable (CE) strobes inside the sys_clk domain — they are NOT routed clocks and do NOT require BUFG resources. Only clk_100mhz is a true second clock domain requiring a separate BUFG and full CDC treatment at every interface boundary.

| Name | Frequency | Generation Method | Source Clock | Used By | Jitter Req. | BUFG? | Implementation Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| sys_clk | 50 MHz | External crystal osc → BUFG direct | E3 pin (LVCMOS33) | All logic; MMCM ref; SPI controllers (base); UART baud gen; PWM timers; arbiter; LED drivers | ≤ 200 ps (crystal) | Yes (IBUFG → BUFG) | Primary fabric clock. MMCM0 reference. Use create_clock in XDC. |
| clk_100mhz | 100 MHz | MMCM0 output (CLKOUT0) VCO=800 MHz ÷ 8 M=16, D=1, O=8 | sys_clk (MMCM0) | SPI master controllers (IMU MPU-9250 / ICM-42688 @ 10 MHz SPI clock derived by /10 CE); high-speed ISL parallel output registers (ODDR) | ≤ 100 ps (MMCM) | Yes (MMCM → BUFG) | True clock domain. Must have its own BUFG. SPI SCK = clk_100mhz gated by CE÷10. Async to all ctrl_Nhz strobes. |
| ctrl_1khz | 1 kHz | Enable strobe from sys_clk 50 MHz ÷ 50 000 = 1 kHz (Counter + pulse-enable) No dedicated BUFG path | sys_clk (CE divider) | UAV PID control loops (all 3 UAVs); ESC PWM period update; servo gimbal update; formation controller input rate | ≤ 500 ns (strobe jitter ok for 1 kHz PID) | No (CE strobe in sys_clk domain) | Logic lives in sys_clk domain; 1 kHz enable pulse gates registers. No CDC needed within sys_clk fabric. |
| ctrl_100hz | 100 Hz | Enable strobe from sys_clk 50 MHz ÷ 500 000 = 100 Hz (Counter + pulse-enable) | sys_clk (CE divider) | CubeSat ADCS (attitude determination & control); EKF update step (all 3 CubeSats); magnetorquer & reaction-wheel command refresh; barometer / GPS data digest | ≤ 5 ms (strobe jitter ok for 100 Hz ADCS) | No (CE strobe in sys_clk domain) | Logic lives in sys_clk domain; 100 Hz enable pulse gates registers. CDC required at boundary with ctrl_10hz solver. |
| ctrl_10hz | 10 Hz | Enable strobe from sys_clk 50 MHz ÷ 5 000 000 = 10 Hz (Counter + pulse-enable) | sys_clk (CE divider) | Convergence engine (formation geometry solver); CubeSat–UAV assignment optimizer; laser gimbal pointing command; inter-satellite link framing | ≤ 50 ms (strobe jitter ok for 10 Hz solver) | No (CE strobe in sys_clk domain) | Logic lives in sys_clk domain; 10 Hz enable pulse gates solver. CDC required at input from ctrl_100hz EKF results. |
| ctrl_1hz | 1 Hz | Enable strobe from sys_clk 50 MHz ÷ 50 000 000 = 1 Hz (Counter + pulse-enable) | sys_clk (CE divider) | CubeSat orbit propagator (SGP4 / Kepler solver); telemetry housekeeping frame; ground-station heartbeat; LED blink indicator | ≤ 500 ms (strobe jitter ok for 1 Hz orbit prop) | No (CE strobe in sys_clk domain) | Logic lives in sys_clk domain; 1 Hz enable pulse gates orbit integrator. Slowest domain — handshake CDC on any multi-bit output. |

### MMCM0 Configuration Detail

The MMCM is the only clock generation primitive required. Configure as follows:

CLKIN1_PERIOD   = 20.000 ns  (50 MHz reference)

CLKFBOUT_MULT_F = 16.0       → VCO = 800 MHz  [within 600–1200 MHz Artix-7 range]

DIVCLK_DIVIDE   = 1

CLKOUT0_DIVIDE_F = 8.0       → CLKOUT0 = 100 MHz  (±0.5 MHz with BANDWIDTH=OPTIMIZED)

CLKOUT0_PHASE   = 0.0

REF_JITTER1     = 0.010      → Output jitter ≈ 60–80 ps pk-pk (within 100 ps budget)

The SPI SCLK at 10 MHz is derived from clk_100mhz by a ÷10 clock-enable inside the SPI master — this avoids an additional MMCM output and ensures the SPI clock is always phase-coherent with sample logic.

### CE Strobe Divider Implementation

Each control rate uses a simple up-counter with a terminal count decode. The enable pulse is exactly one sys_clk cycle wide, guaranteeing no glitches:

// 1 kHz CE example (synthesizable Verilog)

reg [15:0] cnt_1k;

wire ce_1khz = (cnt_1k == 16'd49_999);

always @(posedge sys_clk) cnt_1k <= ce_1khz ? 16'd0 : cnt_1k + 1;

All five CE signals (1 kHz, 100 Hz, 10 Hz, 1 Hz) share a single 26-bit master counter to eliminate drift between control rates.

## Section 2 — Clock Domain Crossing (CDC) Table

There are exactly two true asynchronous CDC boundaries: the clk_100mhz ↔ sys_clk interface. All other apparent domain crossings are CE-to-CE boundaries within sys_clk and require only multicycle path constraints, not synchronizers.

| From Domain | To Domain | Signals Crossing | Signal Type | CDC Technique | Vivado Constraint | Risk | Design Rationale |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TRUE ASYNC CDC — clk_100mhz ↔ sys_clk (Requires synchronizers) |  |  |  |  |  |  |  |
| clk_100mhz | sys_clk | IMU SPI rx_data[31:0], rx_valid, rx_error | Multi-bit data + control | Async FIFO (16-deep, 32-bit) + Gray-coded rd/wr pointers (rx_valid drives wr_en) | set_max_delay -datapath_only -from [get_clocks clk_100mhz] -to [get_clocks sys_clk] 2.0 set_false_path on pointer FFs | HIGH | 100 MHz SPI receive data must cross to 50 MHz sys_clk fabric. Async FIFO is mandatory — the 2:1 frequency ratio means back-pressure is possible. Gray-coded pointers ensure metastability-safe full/empty flags. FIFO depth 16 provides 160 ns burst tolerance. |
| sys_clk (1 kHz CE) | sys_clk (formation block) | uavN_pid_output[15:0] ×3, uavN_pid_valid ×3 | Multi-bit data + valid (same clock domain — CE to CE) | Double-register on valid; Snapshot register on data (capture on pid_valid pulse) Note: both sides in sys_clk — only CE boundary, NOT true CDC | No set_false_path needed; use STA with multicycle path: set_multicycle_path -from [get_cells pid_reg*] 50 (50 sys_clk cycles = 1 kHz period) | LOW | Both producer and consumer clocked by sys_clk with CE strobes. True metastability risk is absent; multicycle path constraint tells the timing tool the data is stable for 50 clock cycles. Simple snapshot register suffices. |
| sys_clk (1 kHz CE) | sys_clk (formation block) | uavN_pid_output[15:0] ×3, uavN_pid_valid ×3 | Multi-bit data + valid (same clock domain — CE to CE) | Double-register on valid; Snapshot register on data (capture on pid_valid pulse) Note: both sides in sys_clk — only CE boundary, NOT true CDC | No set_false_path needed; use STA with multicycle path: set_multicycle_path -from [get_cells pid_reg*] 50 (50 sys_clk cycles = 1 kHz period) | LOW | Both producer and consumer clocked by sys_clk with CE strobes. True metastability risk is absent; multicycle path constraint tells the timing tool the data is stable for 50 clock cycles. Simple snapshot register suffices. |
| clk_100mhz | sys_clk | spi_busy (single bit), spi_fault (single bit) | Single-bit control signal | Two-FF synchronizer (standard double-flop): reg spi_busy_s1, spi_busy_s2; always @(posedge sys_clk) {spi_busy_s2,spi_busy_s1} <= {spi_busy_s1,spi_busy} | set_false_path -from [get_cells spi_busy_ff] -to [get_cells spi_busy_s1_ff] | MED | Single control bits from the 100 MHz SPI FSM to sys_clk logic. Two-FF synchronizer is the canonical solution. set_false_path removes impossible timing requirement on the first flop (metastability is the design intent). MTBF >> system lifetime at these frequencies. |
| sys_clk | clk_100mhz | spi_start (single bit), spi_cmd[7:0] | Handshake — req/ack | 4-phase handshake: 1. sys_clk asserts req + stable cmd[7:0] 2. 2FF sync req into clk_100mhz 3. clk_100mhz asserts ack 4. 2FF sync ack back to sys_clk 5. sys_clk de-asserts req (cmd[7:0] stable entire handshake) | set_false_path -from [get_cells req_ff] -to [get_cells req_s1_ff] set_false_path -from [get_cells ack_ff] -to [get_cells ack_s1_ff] set_false_path on cmd bits (stable during handshake) | MED | sys_clk initiates SPI transaction by asserting start + cmd. 4-phase req/ack handshake ensures cmd[7:0] is stable and metastability-safe before the 100 MHz SPI engine begins. cmd bits use set_false_path because they are guaranteed stable during the synchronization window — not glitch-free individually. |
| CE-DOMAIN CROSSINGS — All within sys_clk (Require multicycle path constraints only) |  |  |  |  |  |  |  |
| sys_clk (1 kHz CE) | sys_clk (formation block) | uavN_pid_output[15:0] ×3, uavN_pid_valid ×3 | Multi-bit data + valid (same clock domain — CE to CE) | Double-register on valid; Snapshot register on data (capture on pid_valid pulse) Note: both sides in sys_clk — only CE boundary, NOT true CDC | No set_false_path needed; use STA with multicycle path: set_multicycle_path -from [get_cells pid_reg*] 50 (50 sys_clk cycles = 1 kHz period) | LOW | Both producer and consumer clocked by sys_clk with CE strobes. True metastability risk is absent; multicycle path constraint tells the timing tool the data is stable for 50 clock cycles. Simple snapshot register suffices. |
| sys_clk (100 Hz CE) | sys_clk (formation block) | csN_ekf_position[47:0] ×3, csN_ekf_velocity[47:0] ×3, ekf_valid ×3 | Multi-bit bus + valid (CE-to-CE within sys_clk) | Snapshot register bank: On ekf_valid pulse, latch all ekf_position & velocity vectors into stable holding registers read by formation logic. | set_multicycle_path -from [get_cells ekf_out_reg*] 500 (500 sys_clk cycles = 100 Hz period) | LOW | EKF outputs 6×48-bit state vectors at 100 Hz. Both sides reside in sys_clk domain — CE gating only. Single-cycle valid pulse triggers a snapshot into holding registers. MCP constraint informs STA of the 500-cycle stability window. |
| sys_clk (100 Hz CE) | sys_clk (10 Hz CE) | csN_position[47:0] ×3 (vehicle position to convergence engine) | Multi-bit bus (CE-to-CE within sys_clk, 10:1 ratio) | Double-register + valid handshake: • 100 Hz producer asserts pos_update • 10 Hz consumer samples on its own CE tick • Stable for 10× consumer period — no metastability | set_multicycle_path -from [get_cells pos_hold_reg*] -to [get_cells conv_in_reg*] 5000 (5000 sys_clk cycles = 10 Hz period) | LOW | 10:1 CE ratio — vehicle position produced at 100 Hz is safely stable for the entire 100 ms convergence engine cycle. Snapshot on 10 Hz CE tick. MCP of 5000 sys_clk cycles is conservative and gives STA the correct hold window. |
| sys_clk (10 Hz CE) | sys_clk (100 Hz CE) | formation_cmd[31:0] ×6 (formation commands back to ADCS/PID) | Multi-bit bus (CE-to-CE within sys_clk, 1:10 ratio) | Holding register updated at 10 Hz; 100 Hz consumers read combinatorially. No synchronizer needed — same clock. Optional: cmd_valid strobe double-registered for safety. | set_multicycle_path -from [get_cells form_cmd_reg*] -to [get_cells adcs_cmd_reg*] 500 (500 sys_clk cycles = 100 Hz period — conservative) | LOW | Formation commands updated at 10 Hz; ADCS/PID reads at 100 Hz. Command is valid for 100 ms — well within 10 ms ADCS sampling interval. No glitch risk as both sides are sys_clk registers; MCP relaxes timing. Add cmd_seq[2:0] counter on command for staleness detection in firmware. |

Note: CDC rows 2–5 above show CE-domain crossings. The rows for clk_100mhz↔sys_clk data (async FIFO) and the two control/handshake crossings are the only paths requiring physical synchronizer flip-flops and set_false_path constraints.

## Section 3 — Vivado TCL Timing Constraints

Complete XDC/TCL constraint file. Apply via Vivado constraint editor or include in .xdc file. All cell name patterns must match your synthesized netlist hierarchy — use get_cells -hierarchical with -filter for robustness.

############################################################################## ## Convergence MVP FPGA — Complete Timing Constraints ## Target: Xilinx Artix-7 XC7A35T (CPG236) ## Author: FPGA Timing Architect ## Generated: 2026-03-20 ############################################################################## # ────────────────────────────────────────────────────────────────────────────── # 1. PRIMARY CLOCK # ────────────────────────────────────────────────────────────────────────────── # 50 MHz crystal oscillator on dedicated GCLK pin E3 create_clock \ -name sys_clk \ -period 20.000 \ -waveform {0 10} \ [get_ports sys_clk_50mhz] # Optional: tighten input jitter budget (crystal spec ~100 ps p-p) set_input_jitter [get_clocks sys_clk] 0.100 # ────────────────────────────────────────────────────────────────────────────── # 2. MMCM0 — 100 MHz OUTPUT # VCO = 50 MHz × 16 = 800 MHz (within 600–1200 MHz VCO range for Artix-7) # CLKOUT0: 800 MHz ÷ 8 = 100 MHz # ────────────────────────────────────────────────────────────────────────────── # Declare generated clock on MMCM output buffer # (Replace MMCM0_inst/CLKOUT0 with actual instance path from your netlist) create_generated_clock \ -name clk_100mhz \ -source [get_pins MMCM0_inst/CLKIN1] \ -multiply_by 2 \ -divide_by 1 \ -master_clock sys_clk \ [get_pins MMCM0_inst/CLKOUT0] # Register the BUFG output as the routed clock create_generated_clock \ -name clk_100mhz_bufg \ -source [get_pins BUFG_100mhz/I] \ -divide_by 1 \ [get_pins BUFG_100mhz/O] # ────────────────────────────────────────────────────────────────────────────── # 3. CONTROL STROBES — declared as virtual clocks for MCP constraints # These are enable pulses inside sys_clk domain, NOT routed clocks. # Declare as virtual so set_multicycle_path can reference them. # ────────────────────────────────────────────────────────────────────────────── create_clock -name ctrl_1khz -period 1000000.000 -virtual create_clock -name ctrl_100hz -period 10000000.000 -virtual create_clock -name ctrl_10hz -period 100000000.000 -virtual create_clock -name ctrl_1hz -period 1000000000.000 -virtual # ────────────────────────────────────────────────────────────────────────────── # 4. CLOCK GROUPS — declare asynchronous relationships # ────────────────────────────────────────────────────────────────────────────── # clk_100mhz is asynchronous to all virtual ctrl clocks # (clk_100mhz derives from sys_clk via MMCM but has a phase shift; # treat as async to avoid false STA paths through CDC synchronizers) set_clock_groups \ -asynchronous \ -group { sys_clk } \ -group { clk_100mhz clk_100mhz_bufg } # Virtual ctrl clocks are not real routed clocks — remove inter-group analysis set_clock_groups \ -asynchronous \ -group { sys_clk } \ -group { ctrl_1khz } \ -group { ctrl_100hz } \ -group { ctrl_10hz } \ -group { ctrl_1hz } # ────────────────────────────────────────────────────────────────────────────── # 5. CDC CONSTRAINTS — clk_100mhz ↔ sys_clk # ────────────────────────────────────────────────────────────────────────────── # 5a. SPI rx_data async FIFO — gray-coded pointer crossing # Allow up to 2× min period (sys_clk = 20 ns → 10 ns max delay on pointer) set_max_delay \ -datapath_only \ -from [get_clocks clk_100mhz_bufg] \ -to [get_clocks sys_clk] \ 10.000 set_max_delay \ -datapath_only \ -from [get_clocks sys_clk] \ -to [get_clocks clk_100mhz_bufg] \ 5.000 # 5b. Single-bit control signals: spi_busy, spi_fault (clk_100mhz → sys_clk) # Two-FF synchronizer first stage — remove timing requirement set_false_path \ -from [get_cells -hierarchical -filter {NAME =~ *spi_busy_ff*}] \ -to [get_cells -hierarchical -filter {NAME =~ *spi_busy_s1*}] set_false_path \ -from [get_cells -hierarchical -filter {NAME =~ *spi_fault_ff*}] \ -to [get_cells -hierarchical -filter {NAME =~ *spi_fault_s1*}] # 5c. Handshake req/ack (sys_clk → clk_100mhz, ack return) set_false_path \ -from [get_cells -hierarchical -filter {NAME =~ *spi_req_ff*}] \ -to [get_cells -hierarchical -filter {NAME =~ *spi_req_s1*}] set_false_path \ -from [get_cells -hierarchical -filter {NAME =~ *spi_ack_ff*}] \ -to [get_cells -hierarchical -filter {NAME =~ *spi_ack_s1*}] # 5d. SPI cmd[7:0] — stable during handshake, exempt from timing set_false_path \ -from [get_cells -hierarchical -filter {NAME =~ *spi_cmd_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *spi_cmd_cap*}] # ────────────────────────────────────────────────────────────────────────────── # 6. MULTICYCLE PATH CONSTRAINTS — CE-domain boundaries in sys_clk # ────────────────────────────────────────────────────────────────────────────── # 6a. PID outputs (1 kHz CE) to formation block # Data stable for 50 sys_clk cycles (= 1 ms = 1 kHz period) set_multicycle_path 50 \ -setup \ -from [get_cells -hierarchical -filter {NAME =~ *pid_out_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *formation_pid_in*}] set_multicycle_path 49 \ -hold \ -from [get_cells -hierarchical -filter {NAME =~ *pid_out_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *formation_pid_in*}] # 6b. EKF outputs (100 Hz CE) to formation block # Data stable for 500 sys_clk cycles (= 10 ms = 100 Hz period) set_multicycle_path 500 \ -setup \ -from [get_cells -hierarchical -filter {NAME =~ *ekf_out_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *formation_ekf_in*}] set_multicycle_path 499 \ -hold \ -from [get_cells -hierarchical -filter {NAME =~ *ekf_out_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *formation_ekf_in*}] # 6c. Vehicle position (100 Hz CE) to convergence engine (10 Hz CE) # Data stable for 5000 sys_clk cycles (= 100 ms = 10 Hz period) set_multicycle_path 5000 \ -setup \ -from [get_cells -hierarchical -filter {NAME =~ *pos_hold_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *conv_in_reg*}] set_multicycle_path 4999 \ -hold \ -from [get_cells -hierarchical -filter {NAME =~ *pos_hold_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *conv_in_reg*}] # 6d. Formation commands (10 Hz CE) to ADCS/PID consumers (100 Hz CE) # Commands updated every 100 ms, sampled every 10 ms — always stable set_multicycle_path 500 \ -setup \ -from [get_cells -hierarchical -filter {NAME =~ *form_cmd_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *adcs_cmd_reg*}] set_multicycle_path 499 \ -hold \ -from [get_cells -hierarchical -filter {NAME =~ *form_cmd_reg*}] \ -to [get_cells -hierarchical -filter {NAME =~ *adcs_cmd_reg*}] # ────────────────────────────────────────────────────────────────────────────── # 7. I/O TIMING CONSTRAINTS # ────────────────────────────────────────────────────────────────────────────── # SPI SCLK outputs (10 MHz, driven from clk_100mhz domain ÷10 CE) # Assume PCB trace delay ≤ 5 ns to IMU set_output_delay -clock clk_100mhz_bufg -max 3.0 [get_ports {cs*_imu_sclk uav*_imu_sclk}] set_output_delay -clock clk_100mhz_bufg -min -1.0 [get_ports {cs*_imu_sclk uav*_imu_sclk}] # SPI MOSI outputs set_output_delay -clock clk_100mhz_bufg -max 3.0 [get_ports {cs*_imu_mosi uav*_imu_mosi}] set_output_delay -clock clk_100mhz_bufg -min -1.0 [get_ports {cs*_imu_mosi uav*_imu_mosi}] # SPI MISO inputs (setup 5 ns, hold 1 ns typical for MPU-9250/ICM-42688) set_input_delay -clock clk_100mhz_bufg -max 5.0 [get_ports {cs*_imu_miso uav*_imu_miso}] set_input_delay -clock clk_100mhz_bufg -min 1.0 [get_ports {cs*_imu_miso uav*_imu_miso}] # UART (all UART ports are async to fabric, relax timing) set_false_path -from [get_ports {*uart_rx *uart_rx}] set_false_path -to [get_ports {*uart_tx *uart_tx}] # PWM outputs (sys_clk domain, low frequency — relax) set_output_delay -clock sys_clk -max 10.0 [get_ports {*pwm* *step* *dir*}] set_output_delay -clock sys_clk -min -5.0 [get_ports {*pwm* *step* *dir*}] # LEDs and test points set_false_path -to [get_ports {shared_dbg_led_* shared_tp_*}] # Reset input (async) set_false_path -from [get_ports sys_rst_n] # ────────────────────────────────────────────────────────────────────────────── # 8. PHYSICAL CONSTRAINTS — key pins # ────────────────────────────────────────────────────────────────────────────── set_property PACKAGE_PIN E3 [get_ports sys_clk_50mhz] set_property IOSTANDARD LVCMOS33 [get_ports sys_clk_50mhz] set_property PACKAGE_PIN C2 [get_ports sys_rst_n] set_property IOSTANDARD LVCMOS33 [get_ports sys_rst_n] # ────────────────────────────────────────────────────────────────────────────── # 9. MMCM ATTRIBUTES (set in IP or via attributes in HDL — shown here for ref) # ────────────────────────────────────────────────────────────────────────────── # MMCM0 parameters for 50 MHz in → 100 MHz out: # CLKIN1_PERIOD = 20.000 (50 MHz) # CLKFBOUT_MULT_F = 16.0 # DIVCLK_DIVIDE = 1 # CLKOUT0_DIVIDE_F = 8.0 → 100 MHz # CLKOUT0_PHASE = 0.0 # BANDWIDTH = OPTIMIZED # REF_JITTER1 = 0.010 # STARTUP_WAIT = FALSE ############################################################################## ## END OF CONSTRAINTS FILE ##############################################################################

## Implementation Checklist

Before tape-out or bitstream generation, verify the following:

[ ] MMCM0 locked signal (mmcm_locked) held in reset until asserted — all CE counters must be held in reset while mmcm_locked = 0

[ ] Async FIFO verified for all 3 CubeSat IMU paths and all 3 UAV IMU paths (6 FIFO instances total)

[ ] Two-FF synchronizer ASYNC_REG attribute applied: (* ASYNC_REG = "TRUE" *) on both flops of every 2FF synchronizer

[ ] set_clock_groups -asynchronous verified in timing report — no paths should escape into the async group

[ ] Vivado CDC report (report_cdc) shows zero unresolved crossings after constraints applied

[ ] Timing report: all setup/hold WNS ≥ 0 on both sys_clk and clk_100mhz domains

[ ] MMCM startup: hold sys_rst_n asserted until mmcm_locked AND MMCM output BUFG has settled (use reset sequencer)

[ ] SPI loopback test at 10 MHz confirms async FIFO round-trip latency ≤ 5 µs at worst case

[ ] Formal CDC verification (Xilinx CDC Audit or third-party tool) run on final netlist