# Convergence Logic — Multi-Agent Coordination Requirements Specification

Convergence MVP  |  Modules: Multi-Vehicle KF · Formation Controller · Collision Avoidance · Resource Arbiter  |  Rev 1.0

33 requirements across 4 modules  |  CRITICAL: 17  HIGH: 14  MEDIUM: 2  |  6-vehicle fleet coordination on Artix-7 XC7A35T

## 1.  Document Purpose & Scope

This specification defines functional requirements for the Convergence Logic FPGA firmware coordinating a six-vehicle mixed fleet (3 CubeSats + 3 UAVs). The four modules — a 36-state Multi-Vehicle Kalman Filter, a leader-follower Formation Controller, an APF-based Collision Avoidance system, and a round-robin Resource Arbiter — together implement autonomous multi-agent formation flying with hard real-time safety guarantees on a 50 MHz sys_clk with 10 Hz convergence CE domain.

## 2.  Priority Classification

| Priority | Count | Definition | Representative Requirements |
| --- | --- | --- | --- |
| CRITICAL | 17 | Failure causes fleet collision, loss of vehicle, or inability to maintain safe separation. Zero tolerance. | All CA override paths, E-STOP, CA/arbiter watchdogs, 36-state vector, pairwise monitoring, safety radius |
| HIGH | 14 | Required for coordinated autonomous operation; single-vehicle degradation acceptable without this. | ISL velocity fusion, covariance maintenance, vehicle loss/re-acq, smooth formation transition, ISL TDM alloc |
| MEDIUM | 2 | Operational convenience or ground-awareness enhancement; does not affect flight safety. | Formation scale command, arbiter telemetry scheduling |
| LOW | 0 | Enhancement features not required for primary mission. | (None in this baseline) |

## 3.  Requirements Summary by Module

| Module | Reqs | CRITICAL | HIGH | MED | Coverage |
| --- | --- | --- | --- | --- | --- |
| Multi-Vehicle KF | 9 | 4 | 4 | 0 | 36-state joint tracking, 9 DSP prediction, ISL fusion, covariance ×6, loss/re-acq, geometry score |
| Formation Ctrl | 8 | 3 | 4 | 1 | 4 geometry types, virtual leader, L-F commands, 1 m threshold, smooth transition, scale, health |
| Collision Avoid. | 9 | 6 | 3 | 0 | APF repulsion, 10 m safety radius, 15-pair monitoring, <10 ms override, boundary, E-STOP, watchdog |
| Resource Arbiter | 7 | 2 | 4 | 1 | Round-robin TDM, 100 ms slots, CA pre-emption, ISL bandwidth, cmd dispatch, telemetry, WDT |
| TOTAL: 33 | 33 | 17 | 14 | 2 | Complete Convergence Logic multi-agent coordination specification |

## 4.  Complete Functional Requirements Table

Each requirement includes: unique REQ_ID · color-coded module tag · titled description with rationale · numbered measurable acceptance criteria · priority classification. Collision avoidance requirements are ordered by priority: global override first, then detection, then response, then health.

| REQ_ID | Module | Description | Acceptance Criteria | Priority |
| --- | --- | --- | --- | --- |
| A. Multi-Vehicle Kalman Filter — 36-State Joint Fleet Tracker |  |  |  |  |
| CV-MKF-001 | MKF | 36-State Joint Tracking Vector The firmware shall maintain a 36-state joint state vector x = [x_i, y_i, z_i, vx_i, vy_i, vz_i] for i = 1..6, tracking all six formation vehicles simultaneously in a common ECI-aligned NED frame. | 1. State vector: 36 elements; each vehicle contributes position (x,y,z in m, Q10.22) and velocity (vx,vy,vz in m/s, Q4.28). 2. Full 36-element state stored in BRAM: 36 × 6 bytes = 216 bytes per epoch; double-buffered (432 bytes total). 3. State for each vehicle initialized from that vehicle's on-board EKF output received via ISL on first valid telemetry frame. 4. Uninitialized vehicle slots flagged with VEHICLE_NOT_PRESENT bit in FLEET_VALID register (6-bit, one per vehicle). 5. State vector completely overwritten each 10 Hz convergence CE cycle; no stale state persists beyond 200 ms. | CRITICAL |
| CV-MKF-002 | MKF | Multi-Vehicle Prediction Step The firmware shall propagate all six vehicle states forward using constant-velocity kinematics at each 10 Hz CE strobe, prior to measurement fusion. | 1. Prediction model: x(k+1) = x(k) + vx(k)·Δt; y(k+1) = y(k) + vy(k)·Δt; z(k+1) = z(k) + vz(k)·Δt for each vehicle (Δt = 0.1 s at 10 Hz). 2. Prediction computed for all 6 vehicles in parallel using 6 independent DSP48 pipelines (3 DSP48s per vehicle × 6 = 18 DSP48s allocated). 3. Prediction step completes within 2 ms of 10 Hz CE strobe. 4. Process noise Q_i configurable per vehicle via 6 sets of AXI4-Lite registers (3 diagonal Q values per vehicle: σ²_pos, σ²_vel_horiz, σ²_vel_vert). | CRITICAL |
| CV-MKF-003 | MKF | ISL Measurement Fusion — Position The firmware shall fuse position measurements received via Inter-Satellite/Inter-Vehicle Links for each vehicle into the joint filter at 10 Hz. | 1. ISL position measurement: lat/lon/alt or NED XYZ from each vehicle's on-board EKF, received over 8-bit parallel ISL bus. 2. Measurement accepted within 10 ms window after 10 Hz CE strobe; late measurements deferred to next cycle. 3. Per-vehicle measurement covariance R_i (3×3 diagonal) configurable via AXI4-Lite; default: σ²_horiz=4 m², σ²_vert=1 m². 4. Innovation for each vehicle: ν_i = z_i − H·x_pred_i; H = [I_3 ¦ 0_3] (position-only measurement). 5. Innovation gate: ¦ν_i¦ > 5σ → measurement rejected; MEAS_OUTLIER_i flag asserted; previous state retained. | CRITICAL |
| CV-MKF-004 | MKF | ISL Measurement Fusion — Velocity The firmware shall optionally fuse velocity measurements from vehicles that provide GPS-derived velocity in their ISL telemetry. | 1. Velocity measurement availability flagged per-vehicle in ISL packet header; fusion skipped if unavailable. 2. Velocity measurement covariance R_vel_i (3×3 diagonal) configurable; default σ²_vel = 0.25 m²/s². 3. Full 6-state update (position + velocity) when both available; position-only update otherwise. 4. Velocity innovation gate: ¦ν_vel_i¦ > 3σ → reject; VEL_OUTLIER_i flag asserted. 5. Velocity update improves velocity estimate convergence: expected σ_vel < 0.3 m/s within 5 update cycles from cold start. | HIGH |
| CV-MKF-005 | MKF | Per-Vehicle 6×6 Covariance Tracking The firmware shall maintain individual 6×6 position-velocity covariance matrices P_i for each of the six vehicles, updated each filter cycle. | 1. Six P_i matrices stored in BRAM: 6 vehicles × 36 elements × 4 bytes = 864 bytes; symmetry exploited (21 unique elements × 4 bytes × 6 = 504 bytes). 2. P_i propagation: P_i(k+1¦k) = F·P_i(k¦k)·Fᵀ + Q_i (F = constant-velocity transition matrix). 3. Joseph-form measurement update for numerical stability: P_i(k¦k) = (I − K_i·H)·P_i(k¦k−1)·(I − K_i·H)ᵀ + K_i·R_i·K_iᵀ. 4. Positive-definiteness monitored: if any diagonal < 0, COV_FAULT_i flag → P_i reset to P0_i (configurable initial covariance). 5. Covariance update for all 6 vehicles completes within 5 ms of 10 Hz CE strobe. | HIGH |
| CV-MKF-006 | MKF | Vehicle Loss & Re-Acquisition The firmware shall handle graceful degradation when one or more vehicles lose ISL connectivity, continuing to track remaining vehicles without disruption. | 1. ISL data absence for > 3 consecutive 10 Hz cycles (300 ms) triggers VEHICLE_LOST_i flag for that vehicle. 2. VEHICLE_LOST_i vehicle: state continues to be predicted (dead-reckoning) but no update applied; covariance grows unbounded. 3. Dead-reckoning state uncertainty σ_pos > DR_UNCERTAINTY_THRESH (configurable, default 10 m) triggers TRACK_DEGRADED_i flag; formation controller notified. 4. On ISL re-acquisition: if innovation gate passes, state accepted and VEHICLE_LOST_i cleared; if innovation > 10σ, re-initialize vehicle state from new measurement. 5. Re-acquisition completes within 2 filter cycles (200 ms) of valid ISL data resumption. | HIGH |
| CV-MKF-007 | MKF | Multi-Vehicle State Output Interface The firmware shall publish the filtered 36-state vector and per-vehicle validity flags to a dual-port BRAM output bank accessible by the formation controller and collision avoidance modules. | 1. Output BRAM: 36 states (216 bytes) + 6×P_i diagonal (6×3×4=72 bytes) + FLEET_VALID (4B) = 292 bytes; double-buffered. 2. Output register bank written atomically at end of each 10 Hz filter cycle (single BRAM write burst). 3. Downstream modules (formation controller, collision avoidance) read from the inactive buffer; no write-read collision possible. 4. DATA_READY strobe asserted for 1 sys_clk cycle after buffer swap; triggers formation controller CE. 5. Relative position matrix ΔR_ij (6×6 lower triangle, 15 pairs × 3 axes × 4B = 180 bytes) computed and appended to output BRAM for direct use by collision avoidance. | CRITICAL |
| CV-MKF-008 | MKF | Convergence Geometry Score The firmware shall compute a scalar formation geometry quality score from the joint filter state, indicating how close the fleet is to the target formation. | 1. Score: CVG_SCORE = 1 − (mean(¦ΔR_ij − ΔR_target_ij¦) / FORMATION_SCALE), clamped [0, 1]. 2. FORMATION_SCALE configurable register (default 10 m); represents maximum expected formation error. 3. CVG_SCORE ≥ 0.95 asserts FORMATION_ACHIEVED flag; CVG_SCORE < 0.5 asserts FORMATION_DEGRADED flag. 4. Score computed using DSP48 accumulator within 1 ms of multi-vehicle state output. 5. CVG_SCORE (Q0.16 fixed-point) and both flags available in AXI4-Lite status register and telemetry. | HIGH |
| CV-MKF-009 | MKF | 10 Hz Update Rate Enforcement The firmware shall guarantee the multi-vehicle filter runs at exactly 10 Hz with bounded jitter, completing predict + update + output within the 100 ms budget. | 1. Filter cycle triggered by 10 Hz CE strobe from sys_clk ÷ 5,000,000 counter. 2. Cycle deadline: all processing (predict + fuse + P_update + output) must complete within 90 ms (10 ms margin). 3. Deadline overrun asserts MKF_OVERRUN flag and increments MKF_OVERRUN_COUNTER; cycle still completes using previous prediction. 4. MKF cycle execution time logged as 16-bit µs counter in status register; readable by resource arbiter for TDM scheduling. 5. Maximum measured execution time ≤ 45 ms for 6-vehicle full update (verified in simulation with all 6 ISL streams active). | CRITICAL |
| B. Formation Controller — Multi-Geometry Leader-Follower Engine |  |  |  |  |
| CV-FC-001 | FORM | Formation Type Register & Geometry Definition The firmware shall support four formation geometries — TRIANGLE, LINE, V-SHAPE, and SQUARE — configurable via an AXI4-Lite formation type register. | 1. FORMATION_TYPE register: 2-bit encoding (00=TRIANGLE, 01=LINE, 10=V_SHAPE, 11=SQUARE). 2. Each formation geometry defined by a set of 6 relative offset vectors ΔR_i = [Δx_i, Δy_i, Δz_i] w.r.t. virtual leader (in meters, Q8.8 fixed-point). 3. Formation geometry tables stored in BRAM: 4 types × 6 vehicles × 3 axes × 2 bytes = 144 bytes. 4. Formation type change: new geometry activates at the start of the next 10 Hz FC cycle; no instantaneous jump (smooth transition per CV-FC-006). 5. Custom geometry upload via AXI4-Lite (write to BRAM formation table) supported in STANDBY mode. | CRITICAL |
| CV-FC-002 | FORM | Virtual Leader State Propagation The firmware shall maintain a virtual leader state [x_L, y_L, z_L, vx_L, vy_L, vz_L] propagated at 10 Hz independently of any physical vehicle. | 1. Virtual leader position updated each 10 Hz cycle: x_L(k+1) = x_L(k) + vx_L(k)·Δt + 0.5·ax_cmd·Δt². 2. Leader velocity command: vx_L, vy_L, vz_L loaded from LEADER_VEL_CMD registers (AXI4-Lite or MAVLink waypoint). 3. Leader acceleration command: ax_L, ay_L, az_L limited to configurable LEADER_ACCEL_MAX (default 2 m/s²). 4. Virtual leader state stored in 6 dedicated registers; readable by all downstream modules. 5. LEADER_VALID flag: asserted when leader velocity command received within last 500 ms; cleared otherwise (leader holds last position). | CRITICAL |
| CV-FC-003 | FORM | Leader-Follower Command Generation The firmware shall compute per-vehicle formation position commands by summing the virtual leader position with each vehicle's assigned formation offset. | 1. Per-vehicle target position: P_target_i = P_leader + R_body2NED · ΔR_i (offset rotated by leader heading). 2. Leader heading ψ_L sourced from LEADER_HEADING register (degrees, Q8.8); rotation applied to formation offsets. 3. Formation commands for all 6 vehicles computed within 2 ms of virtual leader update. 4. Per-vehicle target position written to FORMATION_CMD_i registers (6×3×2 bytes=36 bytes BRAM). 5. Formation assignments (which vehicle occupies which slot) stored in VEHICLE_SLOT[6] register; re-assignable via AXI4-Lite. | CRITICAL |
| CV-FC-004 | FORM | Formation Error Computation & Threshold The firmware shall compute per-vehicle 3D formation error vectors and assert FORMATION_ACHIEVED only when all vehicles are within the error threshold. | 1. Per-vehicle error: E_i = P_target_i − P_estimated_i (from multi-vehicle Kalman filter output). 2. Per-axis threshold: ¦E_i_x¦, ¦E_i_y¦, ¦E_i_z¦ all < 1.0 m (configurable FORM_ERR_THRESH register, range 0.1–10 m). 3. VEHICLE_IN_FORMATION_i flag: asserted per vehicle when 3D error magnitude < FORM_ERR_THRESH for ≥ 3 consecutive 10 Hz cycles. 4. FORMATION_ACHIEVED flag: asserted only when all 6 VEHICLE_IN_FORMATION_i flags are simultaneously set. 5. RMS formation error across all vehicles logged as FORM_RMS_ERROR (Q4.12 meters) in status register at each 10 Hz cycle. | CRITICAL |
| CV-FC-005 | FORM | Formation Correction Command Output The firmware shall compute per-vehicle velocity correction commands from formation error using a PD control law and output them to the resource arbiter. | 1. Correction law: v_corr_i = Kp_f · E_i + Kd_f · dE_i/dt (PD on position error). 2. Kp_f and Kd_f: configurable Q4.12 formation gains; Kp default 0.5 s⁻¹, Kd default 0.2 s. 3. Velocity correction clamped to ±V_CORR_MAX (configurable, default 3 m/s per axis). 4. Per-vehicle correction commands written to CMD_VEL_i BRAM registers for arbiter dispatch. 5. Correction command output available within 3 ms of formation error computation. | HIGH |
| CV-FC-006 | FORM | Formation Transition — Smooth Geometry Switch The firmware shall execute smooth transitions between formation geometries using linear interpolation of target offsets over a configurable transition duration. | 1. Transition triggered by FORMATION_TYPE register write; interpolation runs over TRANSITION_TIME cycles (configurable, default 20 cycles = 2 s). 2. Interpolation: ΔR_i(t) = ΔR_old_i + (t/T_trans) · (ΔR_new_i − ΔR_old_i); applied each 10 Hz cycle. 3. During transition, TRANSITION_ACTIVE flag asserted; FORMATION_ACHIEVED flag inhibited. 4. Transition aborted by collision avoidance override (CV-CA-001); formation holds last interpolated position until CA clears. 5. Transition progress (0–100%) available as TRANSITION_PCT register (8-bit). | HIGH |
| CV-FC-007 | FORM | Formation Scale Command The firmware shall support dynamic scaling of the entire formation geometry about the virtual leader, expanding or contracting all inter-vehicle spacings proportionally. | 1. FORMATION_SCALE register: Q4.12 multiplier applied to all ΔR_i offsets; range 0.1 to 5.0 (10% to 500% of nominal). 2. Scale change applied at next 10 Hz cycle; no interpolation required (abrupt change acceptable). 3. Minimum effective inter-vehicle spacing: SCALE × min(¦ΔR_ij¦) must always exceed 1.5 × SAFETY_RADIUS (collision avoidance interlock). 4. Scale command rejected (SCALE_REJECTED flag) if minimum spacing constraint would be violated; current scale retained. | MEDIUM |
| CV-FC-008 | FORM | Formation Controller Health & Watchdog The firmware shall monitor formation controller health and provide status to the resource arbiter and ground station. | 1. FC_HEALTHY flag: asserted when MKF data age < 200 ms, LEADER_VALID, and no vehicle TRACK_DEGRADED flag set. 2. FC watchdog: if FC fails to write CMD_VEL_i registers within 150 ms (1.5 × 10 Hz period), WDT_FC_EXPIRED flag → arbiter stops dispatching FC commands. 3. Per-vehicle command age counter: CMD_AGE_i (ms since last FC write); arbiter uses this to detect FC stall per vehicle. 4. FORM_ITER_COUNT register: cumulative 10 Hz cycle counter; never resets; ground station uses for FC liveness monitoring. | HIGH |
| C. Collision Avoidance — APF Safety Override Module |  |  |  |  |
| CV-CA-001 | C/A | Collision Avoidance Priority Override The collision avoidance module shall have unconditional priority over all other command sources, pre-empting formation controller and arbiter commands when a safety violation is detected. | 1. CA override latency: from THREAT_DETECTED flag to CA_CMD_VALID output ≤ 10 ms (≤ 1 convergence engine cycle). 2. CA override asserted by asserting CA_OVERRIDE_i flag per threatened vehicle; arbiter immediately substitutes CA command for FC command on next dispatch. 3. CA override remains active until: threat clearance confirmed for ≥ 3 consecutive 10 Hz cycles AND minimum separation > 1.5 × SAFETY_RADIUS. 4. During CA override, formation controller commands are held (not discarded); resume on override release. 5. CA override events logged: vehicle pair, duration, max APF force magnitude; stored in 16-entry CA event FIFO in BRAM. | CRITICAL |
| CV-CA-002 | C/A | Safety Radius Configuration The firmware shall enforce a configurable per-vehicle safety radius defining the minimum allowable inter-vehicle separation at all times. | 1. SAFETY_RADIUS register: Q6.10 fixed-point meters; range 2.0–50.0 m; default 10.0 m. 2. SAFETY_RADIUS applies to all 15 vehicle pairs (6 choose 2) in the 6-vehicle fleet. 3. CAUTION_ZONE_MULT register: scalar multiplier for outer warning zone; default 2.0 (caution at 2× safety radius). 4. SAFETY_RADIUS change takes effect within 1 CA cycle (100 ms); new radius immediately applied to all pair checks. 5. SAFETY_RADIUS cannot be reduced below 2.0 m by any command; firmware enforces hard floor. | CRITICAL |
| CV-CA-003 | C/A | Pairwise Separation Monitoring The firmware shall compute and monitor the 3D separation distance for all 15 vehicle pairs at every 10 Hz cycle, asserting threat flags when separation falls below the safety radius. | 1. All 15 pairwise distances computed from multi-vehicle Kalman filter relative position matrix ΔR_ij each 10 Hz cycle. 2. SEPARATION_ij = sqrt(ΔX² + ΔY² + ΔZ²); computed using DSP48 multiply + CORDIC square-root approximation (5% accuracy sufficient for thresholding). 3. CAUTION_ij flag: separation < 2 × SAFETY_RADIUS; THREAT_ij flag: separation < SAFETY_RADIUS. 4. THREAT_ANY flag: OR of all 15 THREAT_ij flags; asserts global CA override when any pair threatened. 5. All 15 separation distances logged (Q6.10 meters) in CA_SEP BRAM register bank; readable by arbiter for telemetry. | CRITICAL |
| CV-CA-004 | C/A | Artificial Potential Field Repulsion The firmware shall compute Artificial Potential Field (APF) repulsive velocity commands for each vehicle threatened by a separation violation. | 1. APF repulsive force: F_rep_i = Σ_j [ K_rep · (1/d_ij − 1/d_safe) · (1/d_ij²) · (P_i − P_j)/d_ij ] for all j where d_ij < d_safe. 2. K_rep: configurable APF gain register (Q4.12, default 5.0 m³/s). 3. APF velocity command: v_rep_i = F_rep_i; clamped to ±CA_VEL_MAX (configurable, default 5 m/s per axis). 4. APF computation uses DSP48 pipeline; all threatened vehicles processed within 5 ms of separation check. 5. APF force vector magnitude and direction logged per event in CA BRAM event log. | CRITICAL |
| CV-CA-005 | C/A | CAUTION Zone Warning The firmware shall emit CAUTION-level alerts when any vehicle pair enters the caution zone (between 1× and 2× safety radius), triggering formation scale-back before a hard violation. | 1. CAUTION_ij flag triggers: CAUTION_ALERT bit in STATUS register; MAVLink STATUSTEXT(WARNING) for affected pair within 1 MAVLink TX cycle. 2. Formation controller receives CAUTION_ij signal and may optionally increase FORMATION_SCALE to increase spacing (FC-CA soft interlock). 3. CAUTION zone does not activate APF repulsion; APF activates only at THREAT_ij (below SAFETY_RADIUS). 4. CAUTION event counted in CAUTION_COUNTER (cumulative); also logged with timestamp to CAUTION event FIFO (16 entries). 5. CAUTION_ij clears automatically when separation > 2.1 × SAFETY_RADIUS (0.1× hysteresis). | HIGH |
| CV-CA-006 | C/A | Boundary Containment — Formation Volume Limit The firmware shall enforce a configurable 3D bounding volume preventing any vehicle from leaving the formation operating area. | 1. Bounding volume: configurable rectangular prism defined by [X_MIN, X_MAX, Y_MIN, Y_MAX, Z_MIN, Z_MAX] AXI4-Lite registers (meters, NED frame). 2. Boundary check performed each 10 Hz CA cycle for all 6 vehicles using multi-vehicle Kalman filter positions. 3. Vehicle within 5 m of any boundary face: BOUNDARY_WARN_i flag; APF boundary repulsion force applied (same K_rep, opposing normal direction). 4. Vehicle predicted (1-step extrapolation) to cross boundary within next cycle: BOUNDARY_IMMINENT_i flag → CA override → hold vehicle at boundary with v_cmd = 0 in exit direction. 5. Bounding volume disabled via BOUNDING_ENABLE register (default enabled). | HIGH |
| CV-CA-007 | C/A | Emergency Stop Command The firmware shall support an emergency stop command that immediately commands all six vehicles to zero velocity and activates collision avoidance hold. | 1. E-STOP triggered by: AXI4-Lite ESTOP register write, MAVLink CMD_COMPONENT_ARM_DISARM emergency flag, or CA watchdog timeout. 2. On E-STOP: CMD_VEL_i = [0,0,0] for all 6 vehicles dispatched on next arbiter cycle; CA_OVERRIDE_i set for all. 3. E-STOP → MAVLink STATUSTEXT(EMERGENCY) for all 6 vehicles within 1 MAVLink TX cycle. 4. E-STOP does not disarm vehicles (vehicles maintain altitude hold on their own FCU); only overrides formation velocity commands. 5. E-STOP cleared only by explicit AXI4-Lite ESTOP_CLEAR write; no autonomous recovery. | CRITICAL |
| CV-CA-008 | C/A | Collision Avoidance Health Monitoring The firmware shall monitor the collision avoidance module health and escalate to E-STOP if the CA engine fails to complete its cycle within the deadline. | 1. CA cycle deadline: 10 ms from 10 Hz CE strobe to CA_CMD_VALID assertion (matching CA override latency requirement). 2. CA watchdog: if CA_CMD_VALID not asserted within 10 ms, CA_WDT_EXPIRED flag → E-STOP (CV-CA-007) within 1 additional ms. 3. CA cycle execution time logged as 16-bit µs counter; peak execution time < 8 ms required (2 ms margin). 4. CA_HEALTHY flag: asserted when: MKF data age < 200 ms, CA cycle met deadline for last 3 cycles, no APF arithmetic overflow. 5. CA_HEALTHY de-assertion reported in arbiter status and telemetry within 1 arbiter slot. | CRITICAL |
| CV-CA-009 | C/A | APF Saturation & Rate Limiting The firmware shall apply velocity saturation and rate limiting to all APF repulsion commands to prevent abrupt actuator over-drive while still responding within the 10 ms latency budget. | 1. Per-vehicle APF velocity command saturated to ±CA_VEL_MAX before dispatch (default 5 m/s). 2. Rate-of-change limiter on APF command: ΔV per CA cycle ≤ CA_DVEL_MAX (configurable, default 2 m/s per cycle at 10 Hz = 20 m/s²). 3. Saturation active flag CA_SAT_i asserted per vehicle when saturation active; logged to CA event FIFO. 4. If APF force direction reverses (oscillation between two vehicles): OSCILLATION_DETECT_i flag; APF damping term added (K_damp × relative velocity). | HIGH |
| D. Resource Arbiter — Round-Robin TDM Bus & Command Scheduler |  |  |  |  |
| CV-RA-001 | ARB | Round-Robin Time-Division Multiplexing The resource arbiter shall allocate convergence engine computation time and ISL bandwidth to the six vehicles using a round-robin TDM scheme with configurable slot widths. | 1. TDM frame: 6 vehicle slots per superframe; each slot nominally 100 ms (configurable SLOT_WIDTH register, range 50–500 ms). 2. Default superframe period: 6 × 100 ms = 600 ms; total superframe period = 6 × SLOT_WIDTH. 3. Within each slot: vehicle receives full ISL uplink bandwidth + formation command dispatch + telemetry downlink allocation. 4. Round-robin order: slot 0→1→2→3→4→5→0; order configurable via SLOT_ORDER[6] register (supports priority re-ordering). 5. CURRENT_SLOT register (0–5) and SLOT_ELAPSED_MS counter readable via AXI4-Lite for monitoring. | CRITICAL |
| CV-RA-002 | ARB | Slot Width Configuration & Dynamic Adjustment The firmware shall support per-vehicle slot width configuration and runtime adjustment without disrupting active slots. | 1. Per-vehicle slot width: SLOT_WIDTH_i registers (6 × 2 bytes); range 50–500 ms per vehicle; sum defines superframe period. 2. New slot widths take effect at superframe boundary (after current slot 5 completes); no mid-frame changes. 3. Minimum total superframe period: 300 ms (50 ms × 6); maximum: 3000 ms (500 ms × 6). 4. Slot width change logged with old/new values and timestamp in ARBITER_EVENT BRAM FIFO (32 entries). 5. If sum of SLOT_WIDTH_i > 3000 ms, write rejected and SLOT_CONFIG_ERROR flag asserted. | HIGH |
| CV-RA-003 | ARB | Priority Pre-emption for Collision Avoidance The resource arbiter shall unconditionally pre-empt any active slot to dispatch collision avoidance commands within the 10 ms latency requirement. | 1. CA_OVERRIDE_i assertion pre-empts current slot within 1 arbiter cycle (1 ms maximum). 2. Pre-empted slot: current transmission/computation is completed if < 2 ms remaining; aborted if > 2 ms remaining (SLOT_PREEMPTED flag). 3. After CA dispatch: arbiter resumes at the pre-empted slot's remaining time (no time penalty to that vehicle). 4. CA pre-emption latency measured by hardware timer: PREEMPT_LATENCY_US register (16-bit µs); must be ≤ 10,000 µs. 5. Pre-emption events counted in PREEMPT_COUNTER; logged with vehicle index and timestamp. | CRITICAL |
| CV-RA-004 | ARB | ISL Bandwidth Allocation per Slot The arbiter shall allocate ISL transmit bandwidth to the active vehicle during its slot, preventing simultaneous ISL transmissions from multiple vehicles. | 1. ISL_ENABLE_i output: single-bit per vehicle; only active vehicle's ISL_ENABLE asserted during its slot. 2. ISL guard interval: 2 ms between slot transitions with all ISL_ENABLE de-asserted (prevents overlap during clock skew). 3. Active vehicle ISL slot: transmit allocation 60% of slot width; receive allocation 40% (ISL_TX_TIME and ISL_RX_TIME registers). 4. ISL throughput per vehicle per superframe: (0.6 × SLOT_WIDTH_i) × 100 Mbps / superframe_period bytes available. 5. ISL_CONTENTION_i flag: asserted if multi-vehicle Kalman filter detects ISL data from vehicle_i outside its allocated slot (data discarded). | HIGH |
| CV-RA-005 | ARB | Formation Command Dispatch Scheduling The arbiter shall schedule formation controller command dispatches to each vehicle within that vehicle's TDM slot, ensuring no vehicle receives stale formation commands. | 1. Formation command dispatch occurs in the last 20 ms of each vehicle's slot (configurable CMD_DISPATCH_OFFSET register). 2. Command age check: if FC CMD_AGE_i > SLOT_WIDTH_i + 50 ms, STALE_CMD_i flag asserted; stale command not dispatched. 3. STALE_CMD_i triggers: hold vehicle at current position (v_cmd = 0); MAVLink STATUSTEXT(WARNING). 4. Command dispatch confirmed when vehicle acknowledges via MAVLink COMMAND_ACK within 300 ms; no ACK → DISPATCH_NACK_i flag. 5. Dispatch statistics: DISPATCH_SUCCESS_COUNT_i and DISPATCH_FAIL_COUNT_i per vehicle; readable via AXI4-Lite. | HIGH |
| CV-RA-006 | ARB | Telemetry Downlink Scheduling The arbiter shall schedule telemetry downlink transmission for each vehicle within its TDM slot, multiplexing 6-vehicle telemetry over the shared ground-station UART. | 1. Telemetry window: first 30 ms of each vehicle's slot allocated for downlink (TELEM_WINDOW register, configurable). 2. Telemetry packet for each vehicle: 50-byte status summary (position, velocity, formation error, CA status, battery) per slot. 3. Aggregate downlink rate: 6 vehicles × 50 bytes / superframe_period; at 600 ms superframe → 500 bytes/s → well within 57600 baud. 4. Telemetry gap flag: if vehicle slot executes but telemetry not transmitted (e.g., UART busy), GAP_COUNT_i incremented. 5. Ground station receives interleaved telemetry from all 6 vehicles without protocol conflicts. | MEDIUM |
| CV-RA-007 | ARB | Arbiter Health & Superframe Watchdog The firmware shall monitor arbiter health with a superframe-level watchdog, triggering E-STOP if the arbiter fails to cycle through all slots within 2× the expected superframe period. | 1. Superframe watchdog: must complete all 6 slots within 2 × (sum of SLOT_WIDTH_i) ms; else ARB_WDT_EXPIRED flag. 2. ARB_WDT_EXPIRED → E-STOP (CV-CA-007) within 1 additional ms. 3. Per-slot execution time logged: SLOT_EXEC_TIME_i (16-bit µs); accessible via AXI4-Lite for ground telemetry. 4. SLOT_OVERRUN_i flag: asserted if slot_i takes > SLOT_WIDTH_i + 10 ms; overrun counter SLOT_OVERRUN_COUNT_i incremented. 5. ARB_HEALTHY flag: asserted when all 6 slots completed on time for last 3 consecutive superframes. | HIGH |

## 5.  Timing Budget Allocation

All four modules execute within a 100 ms / 10 Hz convergence engine cycle. The critical collision avoidance module has a hard 10 ms latency deadline that takes precedence over the full-cycle budget. The table below shows worst-case execution time estimates.

| Module | Trigger | Deadline | Worst-Case Est. | Budget Remaining | Notes |
| --- | --- | --- | --- | --- | --- |
| Multi-Vehicle KF | 10 Hz CE (100 ms cycle) | 90 ms | 45 ms (6 vehicles) | 45 ms | 18 DSP48s parallel; BRAM access ~10 ms |
| Formation Ctrl | MKF DATA_READY strobe | 8 ms (in MKF budget) | 5 ms | 3 ms | PD law + BRAM write; 6-vehicle loop |
| Collision Avoidance | 10 Hz CE + CA_OVERRIDE | 10 ms (hard limit) | 8 ms | 2 ms | CORDIC sqrt x15 pairs; must fit in 8 ms |
| Resource Arbiter | Continuous; CA pre-empt | 1 ms | < 1 ms | N/A | State machine; negligible compute |
| TOTAL 10 Hz cycle | 100 ms | 90 ms (10 ms margin) | < 58 ms | 32 ms | Sequential MKF+FC+CA; arbiter concurrent |

## 6.  Inter-Module Signal & Data Flow Interface

The table below documents all inter-module signals, their formats, rates, and descriptions. All data transfers use dual-port BRAM handshakes except the CA_OVERRIDE signal which is a combinational wire routed directly to the arbiter for minimum latency.

| From | To | Signal / Bus | Rate | Description |
| --- | --- | --- | --- | --- |
| ISL Parser | Multi-Vehicle KF | ISL_POS_i[6] 36B | 10 Hz | Per-vehicle NED position from ISL telemetry BRAM |
| ISL Parser | Multi-Vehicle KF | ISL_VEL_i[6] 36B | 10 Hz | Per-vehicle NED velocity (optional, per header flag) |
| Multi-Vehicle KF | Formation Controller | MKF_STATE[216B]+dR_ij[180B] | 10 Hz DATA_READY | Full 36-state vector + all 15 pairwise relative positions |
| Multi-Vehicle KF | Collision Avoidance | dR_ij[180B]+SEP_ij[60B] | 10 Hz | Relative position matrix + precomputed 15 separation distances |
| Formation Controller | Resource Arbiter | CMD_VEL_i 6x12B=72B | 10 Hz | Per-vehicle 3D velocity correction commands |
| Collision Avoidance | Resource Arbiter | CA_CMD_i 72B + CA_OVERRIDE_i[6b] | < 10 ms | APF repulsion velocity commands + per-vehicle override flags |
| Resource Arbiter | ISL Transmitter | ISL_ENABLE_i[6b]+CMD_PAYLOAD[50B] | Per slot | Vehicle slot enable + dispatched formation command payload |
| Resource Arbiter | MAVLink Engine | TELEM_PACKET_i[50B] | Per slot | Per-vehicle 50-byte downlink telemetry for GCS |
| AXI4-Lite Config | All modules | Config registers (gains, radii, slots) | On-write | Ground-configurable parameters for all 4 modules |

## 7.  Implementation Notes

Collision Avoidance Latency Budget: The 10 ms hard deadline for CV-CA-001 is the most stringent timing requirement in the entire Convergence system. The CORDIC square-root for 15 pairwise distances must complete in < 5 ms; verified by timing closure analysis showing 15 × CORDIC iterations (16 stages each) completes in 240 ns at 50 MHz — well within budget. The dominant latency is BRAM read of relative position matrix (~2 ms).

DSP48 Resource Allocation: The multi-vehicle KF prediction step allocates 18 DSP48E1 slices (3 per vehicle). The APF computation for CA allocates 6 more (1 per vehicle: 3 components of repulsion force). Formation controller PD law uses 12 (2 per vehicle: proportional + derivative). Total: 36 of 90 available DSP48s = 40% utilization. Remaining 54 DSP48s available for individual vehicle EKF instances.

Safety-Critical Priority Chain: The hardware priority chain is implemented as a combinational OR network: CA_OVERRIDE_ANY → pre-empt arbiter → dispatch CA command within 1 clock cycle propagation delay (20 ns at 50 MHz). This is unconditional — no register stage between the CA flag and the arbiter mux output, ensuring the 10 ms budget is dominated by CA compute time, not dispatch latency.