# CORDIC Sin/Cos Specification  —  16-Iteration Fixed-Point Pipeline

Algorithm: CORDIC (Coordinate Rotation Digital Computer)  |  Modes: Rotation (sin/cos) & Vectoring (atan2/magnitude)  |  Iterations: 16  |  Format: Q1.15

Input: θ ∈ (-π, +π) as Q1.15  |  Output: sin(θ), cos(θ) in Q1.15  |  DSP48: 0  |  LUTs: ~200  |  FFs: ~1100  |  Latency: 17 cycles = 340ns @ 50MHz  |  Throughput: 1/cycle

## 1.  Arctan Lookup Table — All 16 Iterations (Complete LUT with Exact Values)

Each entry is α_i = arctan(2^{-i}), the rotation angle for iteration i. Color coding: blue = coarse corrections (i=0-3), green = medium corrections (i=4-7), yellow = fine corrections (i=8-11), gray = sub-LSB (i=12-15).

| Iter i | α_i (radians) | α_i (degrees) | Shift 2^{-i} | Q2.13 (×8192) | Q1.15 (×32768/π) | Cumul. Angle (°) | Hardware Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.7853981634 | 45.000000° | 2^{-0} = 1.0000 | 0x1922 (6434) | 0x2000 (8192) | 45.000° | Largest step = π/4 = 45°. Full-scale first iteration. Pre-scale x0=Kn×2^15=19898. |
| 1 | 0.4636476090 | 26.565051° | 2^{-1} = 0.5000 | 0x0ED6 (3798) | 0x12E4 (4836) | 71.565° | Second step = 26.565°. Captures coarse direction. |
| 2 | 0.2449786631 | 14.036243° | 2^{-2} = 0.2500 | 0x07D7 (2007) | 0x09FB (2555) | 85.601° |  |
| 3 | 0.1243549945 | 7.125016° | 2^{-3} = 0.1250 | 0x03FB (1019) | 0x0511 (1297) | 92.726° |  |
| 4 | 0.0624188100 | 3.576334° | 2^{-4} = 0.062500 | 0x01FF (511) | 0x028B (651) | 96.303° | Steps now < 4°. Fine correction begins. |
| 5 | 0.0312398334 | 1.789911° | 2^{-5} = 0.031250 | 0x0100 (256) | 0x0146 (326) | 98.093° |  |
| 6 | 0.0156237286 | 0.895174° | 2^{-6} = 0.015625 | 0x0080 (128) | 0x00A3 (163) | 98.988° |  |
| 7 | 0.0078123411 | 0.447614° | 2^{-7} = 0.007813 | 0x0040 (64) | 0x0051 (81) | 99.435° |  |
| 8 | 0.0039062301 | 0.223811° | 2^{-8} = 0.003906250 | 0x0020 (32) | 0x0029 (41) | 99.659° | Steps now < 0.25°. Sub-degree precision. |
| 9 | 0.0019531225 | 0.111906° | 2^{-9} = 0.001953125 | 0x0010 (16) | 0x0014 (20) | 99.771° |  |
| 10 | 0.0009765622 | 0.055953° | 2^{-10} = 0.000976563 | 0x0008 (8) | 0x000A (10) | 99.827° |  |
| 11 | 0.0004882812 | 0.027976° | 2^{-11} = 0.000488281 | 0x0004 (4) | 0x0005 (5) | 99.855° |  |
| 12 | 0.0002441406 | 0.013988° | 2^{-12} = 0.000244141 | 0x0002 (2) | 0x0003 (3) | 99.869° | Steps < 0.016°. Sub-arcminute precision. |
| 13 | 0.0001220703 | 0.006994° | 2^{-13} = 0.000122070 | 0x0001 (1) | 0x0001 (1) | 99.876° |  |
| 14 | 0.0000610352 | 0.003497° | 2^{-14} = 0.000061035 | 0x0000 (0) | 0x0001 (1) | 99.879° |  |
| 15 | 0.0000305176 | 0.001749° | 2^{-15} = 0.000030518 | 0x0000 (0) | 0x0000 (0) | 99.881° | Final step = 0.00175°. Output error < 1 LSB Q1.15. |
| TOTAL | Sum = 1.74325610 rad | 99.881° | — | Kn=0.607252935 | Init x0=19898 | — | Kn=0.607252935 ¦ 1/Kn=1.646760258 ¦ x0_Q115=19898 (pre-scale for output = Q1.15 sin/cos directly) |

## 2.  Iteration Equations — Rotation and Vectoring Modes

Side-by-side comparison of the two CORDIC modes. The only difference is the direction decision: Rotation uses d_i = sign(z_i); Vectoring uses d_i = -sign(y_i).

| Stage | Rotation Mode Equations (compute sin/cos from angle z) | Vectoring Mode Equations (compute atan2, magnitude) | Direction Signal d_i | Fixed-Point Notes |
| --- | --- | --- | --- | --- |
| Init | x[0] = Kn_fp = 19898 (Q1.15) y[0] = 0 z[0] = θ_input (Q1.15) Quadrant reduce θ to (-π/2, π/2) first if ¦θ¦ > π/2 | x[0] = x_in (Q1.15) y[0] = y_in (Q1.15) z[0] = 0 Optional: pre-normalize x,y if ¦x¦,¦y¦ > 0.9 to avoid intermediate overflow | — | x,y use Q2.14 internally (sign + 2 int + 14 frac). z uses Q1.15. All pipeline regs 18-bit signed. |
| i=0 | x[1] = x[0] - d₀·y[0]·1 y[1] = y[0] + d₀·x[0]·1 z[1] = z[0] - d₀·α₀ (shift=0 → no shift) | x[1] = x[0] - d₀·y[0] y[1] = y[0] + d₀·x[0] z[1] = 0 + d₀·α₀ | d₀=+1 if z[0]≥0 (Rot) d₀=-1 if y[0]>0 (Vec) | Shift by 2^0 = 1 (no shift). Simple add/subtract. No barrel shifter needed for i=0. |
| i=1 | x[2] = x[1] - d₁·y[1]>>>1 y[2] = y[1] + d₁·x[1]>>>1 z[2] = z[1] - d₁·α₁ | x[2] = x[1] - d₁·(y[1]>>>1) y[2] = y[1] + d₁·(x[1]>>>1) z[2] = z[1] + d₁·α₁ | d₁=sign(z[1]) (Rot) d₁=-sign(y[1]) (Vec) | Arithmetic right shift (>>> preserves sign). Each stage: 2 adds + 2 shifts + 1 add (z update). |
| i=2 | x[3] = x[2] - d₂·y[2]>>>2 y[3] = y[2] + d₂·x[2]>>>2 z[3] = z[2] - d₂·α₂ | ... | d₂=sign(z[2]) (Rot) | Shift 2^{-2}. Same structure. Synthesis: parametric generate loop. |
| i=3..14 | (identical structure) x[i+1] = x[i] - d_i·(y[i]>>>i) y[i+1] = y[i] + d_i·(x[i]>>>i) z[i+1] = z[i] - d_i·α_i | (identical structure) d_i = -sign(y[i]) z accumulates atan contributions | d_i=sign(z[i]) (Rot) d_i=-sign(y[i]) (Vec) | Each stage uses a FIXED right-shift amount (compile-time constant). This makes each stage purely adders + wires — NO barrel shifter, NO multiplier. |
| i=15 (last) | x[16] = x[15] - d₁₅·(y[15]>>>15) y[16] = y[15] + d₁₅·(x[15]>>>15) z[16] = z[15] - d₁₅·α₁₅ | x[16] = final magnitude × Kn y[16] ≈ 0 (converged) z[16] ≈ atan2(y_in, x_in) | d₁₅=sign(z[15]) (Rot) | >>>15 in Q1.15 = arithmetic shift dropping all fractional bits → essentially ±1 ULP correction at final stage. |
| Output (Rot) | sin(θ) = y[16] (Q1.15) cos(θ) = x[16] (Q1.15) No gain correction needed: x[0]=Kn pre-scaled x0=19898 so x[16]≈cos(θ) in Q1.15 | magnitude = x[16] / Kn angle = z[16] (Q1.15 radians) Apply 1/Kn correction to magnitude or pre-scale input by Kn | — | Pre-scaling x0=Kn*32768=19898 means output IS Q1.15 sin/cos directly. Quadrant correction: apply sign flip based on original quadrant after output. |

## 3.  Exact LUT Values (Q2.13 Synthesis Constants)

Precise arctan values for all 16 iterations with exact Q2.13 integer representations. These are the hardcoded ROM values synthesized as distributed LUT-ROM — no BRAM required. Kn pre-scaling value included.

| i | atan(2^{-i}) exact (radians) | Q2.13 value (× 8192) | Q2.13 hex | Q1.15/π (× 32768/π) | Synthesis implementation note |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.78539816339744828 | 6434 | 0x1922 | 10000 | Largest step. Stored in register or hardcoded constant. |
| 1 | 0.46364760900081720 | 3799 | 0x0ED7 | 7286 | Parametric generate; synth infers optimal adder tree. |
| 2 | 0.24497866312686414 | 2008 | 0x07D8 | 3848 | Parametric generate; synth infers optimal adder tree. |
| 3 | 0.12435499454676144 | 1019 | 0x03FB | 1953 | Parametric generate; synth infers optimal adder tree. |
| 4 | 0.06241880999595735 | 511 | 0x01FF | 979 | Parametric generate; synth infers optimal adder tree. |
| 5 | 0.03123983343026828 | 256 | 0x0100 | 490 | Parametric generate; synth infers optimal adder tree. |
| 6 | 0.01562372862047683 | 128 | 0x0080 | 245 | Parametric generate; synth infers optimal adder tree. |
| 7 | 0.00781234155004064 | 64 | 0x0040 | 123 | Parametric generate; synth infers optimal adder tree. |
| 8 | 0.00390623013196697 | 32 | 0x0020 | 61 | Parametric generate; synth infers optimal adder tree. |
| 9 | 0.00195312251647882 | 16 | 0x0010 | 31 | Parametric generate; synth infers optimal adder tree. |
| 10 | 0.00097656218955932 | 8 | 0x0008 | 15 | Parametric generate; synth infers optimal adder tree. |
| 11 | 0.00048828121119490 | 4 | 0x0004 | 8 | Parametric generate; synth infers optimal adder tree. |
| 12 | 0.00024414062014936 | 2 | 0x0002 | 4 | Parametric generate; synth infers optimal adder tree. |
| 13 | 0.00012207031189367 | 1 | 0x0001 | 2 | Parametric generate; synth infers optimal adder tree. |
| 14 | 0.00006103515617421 | 1 | 0x0001 | 1 | Very small; may round to 0 in Q2.13 — use 1 if nonzero needed. |
| 15 | 0.00003051757811553 | 0 | 0x0000 | 0 | Very small; may round to 0 in Q2.13 — use 1 if nonzero needed. |
| Kn | 0.60725293510313938 | 19898 (×32768) | 0x4DBA | 1/Kn=1.64676026 | x0 = 19898 = 0x4DBA loaded at initialization. Compensates CORDIC amplitude gain so output IS Q1.15 sin/cos without post-multiply. |

## 4.  Quadrant Reduction & Sign Correction

CORDIC converges only for |z| ≤ π/2. Inputs outside this range require quadrant pre-reduction and post-rotation sign correction.

| Input θ | Range | Reduction | Pre-rotation | Output Correction |
| --- | --- | --- | --- | --- |
| Q1 (0 to π/2) | 0 to 1.5708 rad | None needed. Corner: θ=0 → cos=1,sin=0 | x0 = Kn = 19898 y0 = 0 z0 = θ | Direct output: sin = y[16], cos = x[16] |
| Q2 (π/2 to π) | 1.5708 to 3.1416 rad | θ' = π - θ (maps to Q1) | x0 = Kn, y0 = 0 z0 = π - θ (positive, in Q1) | cos = -x[16] sin = y[16] (cos negated for Q2) |
| Q3 (-π to -π/2) | -3.1416 to -1.5708 rad | θ' = -π - θ (maps to Q1, negative) | x0 = Kn, y0 = 0 z0 = -(π + θ) (positive) | cos = -x[16] sin = -y[16] (both negated for Q3) |
| Q4 (-π/2 to 0) | -1.5708 to 0 rad | None needed. Negative angle OK for CORDIC | x0 = Kn, y0 = 0 z0 = θ (negative) | Direct output: sin = y[16], cos = x[16] (negative z gives negative sin) |
| Edge: θ = ±π/2 | Exactly ±1.5708 rad | Return hardcoded: θ=+π/2: sin=1, cos=0 θ=-π/2: sin=-1, cos=0 | — | Bypass CORDIC: exact result from comparison register. |
| Edge: θ = 0 | Exactly 0 | Return hardcoded: sin=0, cos=1 | — | Bypass CORDIC: exact result. |

## 5.  Pipeline Stage Map

17-stage pipeline (pre-stage + 16 CORDIC iterations). Fully pipelined — accepts new input every clock cycle. Quadrant correction flags propagate through all stages alongside x,y,z.

| Stage | Clock | x register | y register | z register | Activity |
| --- | --- | --- | --- | --- | --- |
| Pre | clk 0 | Load x0=19898 (Q2.14 = 0x4DA2) | Load y0=0 | Load z0=θ_in (Q1.15 input angle) | Quadrant detection and pre-rotation. Input register to CORDIC core. |
| S0 | clk 1 | x1=x0 ± y0 | y1=y0 ∓ x0 | z1=z0 - d0·α0 | i=0, no shift. d0=sign(z0). α0=0x6488 (Q2.13). |
| S1 | clk 2 | x2=x1 ± (y1>>1) | y2=y1 ∓ (x1>>1) | z2=z1 - d1·α1 | i=1, shift 1. α1=0x3B58. |
| S2 | clk 3 | x3=x2 ± (y2>>2) | y3=y2 ∓ (x2>>2) | z3=z2 - d2·α2 | i=2, shift 2. α2=0x1F5C. |
| S3 | clk 4 | x4... | y4... | z4... | i=3, shift 3. α3=0x0FEE. |
| S4-S11 | clk 5-12 | pipeline | pipeline | pipeline | Stages 4-11: same structure, shift 4-11. Each stage registered. |
| S12 | clk 13 | x13... | y13... | z13... | i=12, shift 12. α12=0x000A. |
| S13 | clk 14 | x14... | y14... | z14... | i=13, shift 13. α13=0x0005. |
| S14 | clk 15 | x15... | y15... | z15... | i=14, shift 14. α14=0x0003. |
| S15 | clk 16 | x16... | y16... | z16=≈0 | i=15, shift 15 (final). z should be near 0. |
| Out | clk 17 | cos(θ)=x16 (Q1.15, valid) | sin(θ)=y16 (Q1.15, valid) | residual z (error signal) | Apply quadrant correction (sign flip). Output valid. Latency = 16 cycles from input. |
| Notes | Fully pipelined: new input every clock. Throughput = 1 result/cycle. 18-bit signed registers. 16+1 = 17 pipeline stages total (including pre-stage). At 50 MHz: 320ns latency, 50M results/sec throughput. |  |  |  |  |

## 6.  Accuracy vs Iterations

Convergence analysis. Each CORDIC iteration adds approximately 1 bit of accuracy. 16 iterations gives sub-LSB accuracy in Q1.15 (15 fractional bits + 1 guard bit).

| N iter | Max Error (radians) | Max Error (Q1.15 LSBs) | Accuracy (bits) | Notes |
| --- | --- | --- | --- | --- |
| 4 | 0.031250 | ~1024 LSB | ~5 bits | Coarse only. Suitable for rough angle detection, not sin/cos. |
| 8 | 0.000977 | ~32 LSB | ~10 bits | Adequate for 10-bit ADC applications. 8-stage pipeline. |
| 12 | 0.000061 | ~2 LSB | ~14 bits | 14-bit accuracy. Adequate for many control applications. |
| 14 | 0.000015 | <1 LSB | ~16 bits | Near full Q1.15 accuracy. 2 guard iterations. |
| 16 | 0.0000038 | <1 LSB | ~18 bits | Full Q1.15 accuracy (15 fractional bits + 1 sign). 2 bits beyond Q1.15 resolution ensures correct rounding. |
| Notes | CORDIC convergence: each iteration adds 1 bit of accuracy. After N iterations, error ≈ 2^{-(N+1)} radians. For Q1.15 output (15 fractional bits), need N≥15. Using N=16 provides 1 guard bit for correct Q1.15 rounding. |  |  |  |

## 7.  Resource Usage

CORDIC uses zero DSP48E1 — all operations are shifts (compile-time wires) and adders (LUT fabric). This is its primary advantage over polynomial-based sin/cos evaluation.

| Resource | Rotation Mode | Vectoring Mode | Both Modes (shared) | Notes |
| --- | --- | --- | --- | --- |
| DSP48E1 | 0 | 0 | 0 | CORDIC is shift-and-add only. Zero multipliers. This is the primary FPGA advantage over direct polynomial evaluation. |
| LUT6 | ~192 (16×12 LUT/adder) | ~192 | ~200 (shared) | 16 pipelined 18-bit adders × ~12 LUTs each. Synthesis may merge sequential adders. |
| Flip-Flops | ~1056 (16×3×18b regs) | ~1056 | ~1100 (with quad) | 16 pipeline stages × 3 registers (x,y,z) × 18 bits + quadrant pipeline = ~1100 FFs. |
| BRAM18 | 0 | 0 | 0 | Arctan LUT is 16 × 16-bit = 256 bits. Implemented as distributed ROM or LUT-ROM, not BRAM. |
| Pipeline Stages | 17 (pre+16) | 17 | 17 | Fully pipelined. New angle every clock. Throughput = 50M results/sec @ 50 MHz. |
| Latency (cycles) | 17 cycles | 17 cycles | 17 cycles | 17 × 20ns = 340ns total from input to valid output. |
| Latency (time) | 340 ns | 340 ns | 340 ns | Well within 100Hz ADCS cycle (10ms). Can compute 29,412 sin/cos pairs per second at 50MHz throughput. |
| Max frequency | ~150+ MHz | ~150+ MHz | ~150+ MHz | Each stage: 1 adder + 1 shift (wire). Meets 50 MHz with large margin. Minimum path: 1 FA delay ≈ 2ns → max ~200 MHz possible. |
| Accuracy (16 iter) | < 1 LSB Q1.15 | < 1 LSB Q1.15 | < 1 LSB Q1.15 | Error < 2^{-16} radians ≈ 1.5e-5 rad after 16 iterations. Rounding to Q1.15 adds at most 0.5 LSB. |
| Comparison vs Taylor poly (sin) | 0 DSP48 | ~4 DSP48 | — | Taylor 5-term sin(x) needs 4 multiplies → 4 DSP48. CORDIC: 0 DSP48. CORDIC saves DSP48 resources at cost of 17 pipeline stages. |

## 8.  Verilog RTL — Rotation Mode (sin/cos)

16-stage pipelined CORDIC. generate loop creates all 16 stages. Parametric shift i is a compile-time constant — synthesizer routes as wires, not shift registers. Zero DSP48E1.

cordic_sincos.v — 16-stage Rotation Mode (0 DSP48E1, 17-cycle latency)

// ================================================================ // CORDIC 16-iteration Pipelined Sin/Cos (Rotation Mode) // Input: theta_in = Q1.15 angle in radians (-pi to +pi) // scaled as: theta_fp = round(theta_rad * 32768 / pi) // Output: sin_out, cos_out in Q1.15 (-32768 to +32767) // Latency: 16 clock cycles (fully pipelined) // DSP48: 0 (shifts and adds only) // LUTs: ~200, FFs: ~1024 // ================================================================ module cordic_sincos ( input wire clk, rst_n, input wire signed [15:0] theta_in, // Q1.15 (radians/pi × 32768) output reg signed [15:0] sin_out, // Q1.15 output reg signed [15:0] cos_out, // Q1.15 output reg valid_out ); // ── CORDIC arctan lookup table: alpha[i] = atan(2^{-i}) in Q2.13 ────── // Q2.13: multiply real value (radians) by 8192 localparam signed [15:0] ATAN_LUT[0:15] = '{ 16'h3244, // i=0: atan(1) = 0.7854 rad 0x3244=12868/8192=1.5708/2→Q2.13=6434? Let me recalc // Proper Q2.13 values: atan(2^-i) × 8192 // i=0: atan(1)=0.785398 × 8192 = 6434 = 0x1922 // i=1: atan(0.5)=0.463648 × 8192 = 3799 = 0x0ED7 // i=2: atan(0.25)=0.244979 × 8192 = 2008 = 0x07D8 // ... (see complete table in Section 2) 16'h1922,16'h0ED7,16'h07D8,16'h03EB, 16'h01FD,16'h00FF,16'h007F,16'h003F, 16'h001F,16'h0010,16'h0008,16'h0004, 16'h0002,16'h0001,16'h0001,16'h0000}; // ── Pre-scale initial x0 = Kn × 32768 = 19898 (0x4DA2) ────────────── // This compensates for CORDIC gain, so outputs are in Q1.15 directly localparam signed [17:0] KN_INIT = 18'sd19898; // Kn × 32768 // ── Quadrant detection and angle reduction ──────────────────────────── // theta_in is in Q1.15 where 32768 = π (full scale) // CORDIC converges for z0 ∈ (-π/2, π/2) = (-16384, +16384) in Q1.15/π wire quad2 = ( theta_in > 16'sd16384); // θ > π/2 wire quad3 = (theta_in < -16'sd16384); // θ < -π/2 wire signed [15:0] theta_red = quad2 ? (16'sd32767 - theta_in) // π - θ (Q2) : quad3 ? (-16'sd32768 - theta_in)// -π - θ (Q3) : theta_in; // ── Pipeline registers: 17 stages (pre + 16 CORDIC) ───────────────── reg signed [17:0] x[0:16], // Q2.15 (extra integer bit for safety) y[0:16], z[0:16]; reg [1:0] quad[0:16]; // pipeline quadrant flag // ── Stage 0 initialization ──────────────────────────────────────────── always @(posedge clk) begin if (!rst_n) begin x[0]<=0; y[0]<=0; z[0]<=0; quad[0]<=0; end else begin x[0] <= KN_INIT; // Pre-scaled Kn y[0] <= 18'sd0; z[0] <= {{2{theta_red[15]}}, theta_red}; // sign-extend to 18-bit quad[0] <= {quad3,quad2}; end end // ── Generate block: 16 CORDIC stages ───────────────────────────────── genvar i; generate for (i=0; i<16; i=i+1) begin : cordic_stage wire d_i = z[i][17]; // d_i = sign bit of z[i] (0=pos, 1=neg) // d_i=0 (z≥0): rotate +, d_i=1 (z<0): rotate - wire signed [17:0] x_sh = $signed(x[i]) >>> i, // shift by i (compile-time constant!) y_sh = $signed(y[i]) >>> i; always @(posedge clk) begin if (!rst_n) begin x[i+1]<=0; y[i+1]<=0; z[i+1]<=0; quad[i+1]<=0; end else begin // Rotation: x_new = x ∓ y>>i, y_new = y ± x>>i // if d=0 (rotate ccw): x-=y>>i, y+=x>>i, z-=alpha // if d=1 (rotate cw): x+=y>>i, y-=x>>i, z+=alpha x[i+1] <= $signed(x[i]) + (d_i ? $signed(y_sh) : -$signed(y_sh)); y[i+1] <= $signed(y[i]) + (d_i ? -$signed(x_sh) : $signed(x_sh)); z[i+1] <= $signed(z[i]) + (d_i ? $signed(ATAN_LUT[i]) : -$signed(ATAN_LUT[i])); quad[i+1] <= quad[i]; // propagate quadrant flag end end end endgenerate // ── Output stage: quadrant correction ──────────────────────────────── always @(posedge clk) begin if (!rst_n) begin sin_out<=0; cos_out<=0; valid_out<=0; end else begin // x[16]=cos(θ'), y[16]=sin(θ') where θ'=reduced angle // Apply quadrant sign correction: wire signed [15:0] cos_r = x[16][15:0]; wire signed [15:0] sin_r = y[16][15:0]; cos_out <= quad[16][0] ? (-cos_r) : cos_r; // Q2 or Q3: negate cos sin_out <= quad[16][1] ? (-sin_r) : sin_r; // Q3 only: negate sin valid_out <= 1'b1; end end endmodule

## 9.  Verilog RTL — Vectoring Mode (atan2)

Same hardware structure as rotation mode, only direction signal d_i changes from sign(z) to -sign(y). Can share LUT ROM with rotation module. Pre-rotation handles x_in < 0 case (extends atan2 range to full ±π).

cordic_atan2.v — 16-stage Vectoring Mode (0 DSP48E1, atan2 + magnitude)

// ================================================================ // CORDIC Vectoring Mode: atan2(y,x) and magnitude // Input: x_in, y_in in Q1.15 (any unit, ¦x¦,¦y¦ ≤ 0.99) // Output: angle = atan2(y_in, x_in) in Q1.15 (radians/π × 32768) // magnitude = sqrt(x²+y²) × Kn in Q1.15 (apply 1/Kn externally) // Drives y to zero; accumulates angle in z // ================================================================ module cordic_atan2 ( input wire clk, rst_n, input wire signed [15:0] x_in,y_in, // Q1.15 inputs output reg signed [15:0] angle_out, // Q1.15 atan2 result (radians/π) output reg signed [15:0] mag_out // Q1.15 magnitude × Kn ); // Same ATAN_LUT as rotation mode (same arctan values) // Vectoring differs only in: d_i = -sign(y[i]) instead of sign(z[i]) reg signed [17:0] xv[0:16], yv[0:16], zv[0:16]; // Initialization: load x=x_in, y=y_in, z=0 // Handle x_in < 0 (y∈(-π, 0) or (π/2, π)): pre-rotate by ±π wire pre_neg = x_in[15]; // x_in < 0 → add ±π to z always @(posedge clk) begin if (!rst_n) beginxv[0]<=0; yv[0]<=0; zv[0]<=0; end else begin xv[0] <= pre_neg ? -{{2{x_in[15]}},x_in} : {{2{x_in[15]}},x_in}; yv[0] <= pre_neg ? -{{2{y_in[15]}},y_in} : {{2{y_in[15]}},y_in}; zv[0] <= pre_neg ? (y_in[15] ? 18'sd32768 : -18'sd32767) : 18'sd0; end end genvar i; generate for (i=0; i<16; i=i+1) begin : vec_stage // KEY DIFFERENCE: d_i = -sign(y) instead of sign(z) wire d_i = !yv[i][17]; // d_i = -sign(y) = 1 if y<0, drive y toward 0 wire signed [17:0] xs = $signed(xv[i]) >>> i, ys = $signed(yv[i]) >>> i; always @(posedge clk) begin if (!rst_n) beginxv[i+1]<=0; yv[i+1]<=0; zv[i+1]<=0; end else begin // Same equations, d_i now based on y sign xv[i+1] <= $signed(xv[i]) + (d_i ? $signed(ys) : -$signed(ys)); yv[i+1] <= $signed(yv[i]) + (d_i ? -$signed(xs) : $signed(xs)); zv[i+1] <= $signed(zv[i]) + (d_i ? $signed(ATAN_LUT[i]) : -$signed(ATAN_LUT[i])); end end end endgenerate always @(posedge clk) begin angle_out <= zv[16][15:0]; // atan2 result mag_out <= xv[16][15:0]; // magnitude × Kn (apply 1/Kn externally) end endmodule

## 10.  Design Notes & Edge Cases

| Issue | Detail & Mitigation |
| --- | --- |
| Angle scaling convention | The input theta_in represents θ radians scaled as: theta_fp = round(θ × 32768 / π). So +32767 = +π, -32768 = -π. This is NOT the same as Q1.15 where +32767 = +0.9999. The file comment says 'radians/π × 32768' for clarity. |
| CORDIC gain pre-scaling | x0 is pre-loaded with Kn×32768 = 19898 (not 32767). This compensates the inherent CORDIC amplitude growth factor Kn=0.60725 so outputs land in Q1.15. If x0=32767 were used, outputs would need post-multiply by Kn which requires a DSP48. |
| i=15 shift overflow | At stage i=15: y[15]>>>15 for 16-bit data = always 0 or -1 (only sign bit remains). This is expected and correct — the final iteration is a tiny ±1 ULP correction. |
| Vectoring range extension | Basic vectoring only works for x_in > 0 (first and fourth quadrant). The code pre-rotates by ±π when x_in < 0, extending coverage to full ±π. Without this, atan2(y,x) with x<0 gives wrong results. |
| Tan(θ) from CORDIC | To compute tan(θ): run rotation mode, get sin and cos, then divide. Division can be done with another CORDIC vectoring call: atan2(sin, cos) returns θ (not tan), but the magnitude converges to 1. True tan requires sin/cos division = 1 more vectoring CORDIC or a Newton-Raphson reciprocal multiply. |
| Pipeline valid signal | The valid_out signal (not fully shown) should be a shift register of 17 bits, with bit 0 set when a valid input is accepted and propagated. At reset, valid_out = 0 for first 17 cycles (pipeline fill). EKF must gate state update on valid_out. |