# CubeSat FPGA Firmware — Functional Requirements Specification

Convergence MVP  |  Subsystems: ADCS · Orbit Propagator · Laser FSM · Telemetry Encoder  |  Baseline Rev 1.0

Document contains 37 requirements across 4 subsystems  |  CRITICAL: 20  HIGH: 15  MEDIUM: 2  LOW: 0

## 1.  Document Purpose & Scope

This document specifies the complete functional requirements for the CubeSat FPGA firmware implementing four core subsystems on the Xilinx Artix-7 XC7A35T target device. Each requirement is identified by a unique REQ_ID, a measurable acceptance criterion, and a priority classification. Requirements are derived from the mission-level system requirements for the Convergence multi-vehicle formation flying demonstration.

## 2.  Priority Classification

| Priority | Count | Definition | Example Requirements |
| --- | --- | --- | --- |
| CRITICAL | 20 | Failure causes mission loss or safety hazard. Must be implemented, tested, and verified before flight qualification. | IMU interface, EKF, PD control law, all FAULT states, telemetry CRC, uplink decode |
| HIGH | 15 | Failure significantly degrades mission capability. Required for primary mission success. | Covariance update, reaction wheel command, magnetorquer PWM, LVLH conversion, pointing arbiter |
| MEDIUM | 2 | Important for full capability but mission can continue at reduced performance without it. | Ground track prediction, HK packet, contact window prediction |
| LOW | 0 | Enhances operations but not required for primary mission objectives. | (None defined in this baseline) |

## 3.  Requirements Summary by Subsystem

| Subsystem | Total Reqs | CRITICAL | HIGH | MEDIUM + LOW | Coverage Areas |
| --- | --- | --- | --- | --- | --- |
| ADCS | 12 | 6 | 4 | 2 | Sensors, EKF, Control law, PWM, FSM, Health monitoring, Logging |
| Orbit | 7 | 2 | 4 | 1 | TLE input, SGP4-Lite, LVLH, Time, Relative pos, Ground track, Health |
| Laser FSM | 10 | 7 | 3 | 0 | 6 FSM states + transitions, Gimbal control, Signal monitoring |
| Telemetry | 8 | 4 | 3 | 1 | Frame format, UART encoding, ADCS/Orbit/Laser packets, Arbiter, HK, Uplink |
| TOTAL: 37 | 37 | 20 | 15 | 2 | Complete CubeSat FPGA firmware functional specification |

## 4.  Complete Functional Requirements Table

The table below contains all requirements. Each row includes: a unique identifier, the subsystem tag (color-coded), a titled description with rationale, measurable acceptance criteria with numbered test points, and priority classification.

| REQ_ID | Sub- system | Description | Acceptance Criteria | Priority |
| --- | --- | --- | --- | --- |
| A. ADCS — Attitude Determination & Control System |  |  |  |  |
| CS-ADCS-001 | ADCS | IMU Sensor Interface — SPI Read The FPGA shall read 3-axis accelerometer, 3-axis gyroscope, and 3-axis magnetometer data from the MPU-9250 IMU over SPI at 10 MHz SCLK. | 1. SPI transaction completes within 20 µs (16-bit word at 10 MHz). 2. CRC/parity check passes on ≥99.9% of frames during 24-hour continuous operation. 3. IMU data registers updated in BRAM within 2 µs of SPI transaction completion. 4. Raw output units: gyro in LSB/°/s (131 LSB/°/s @ ±250°/s), accel in LSB/g (16384 LSB/g @ ±2g). | CRITICAL |
| CS-ADCS-002 | ADCS | Magnetometer Interface — I2C Read The FPGA shall read 3-axis magnetic field data from the on-board magnetometer over I2C at 400 kHz (Fast Mode). | 1. I2C read cycle completes within 50 µs per axis (3 bytes at 400 kHz). 2. ACK received from magnetometer within 1 I2C clock period; NACK triggers fault flag. 3. Magnetic field data available in BRAM within 1 ms of 100 Hz CE strobe. 4. Output units: raw 13-bit ADC counts, scalable to µT via coefficient register. | CRITICAL |
| CS-ADCS-003 | ADCS | Sun Sensor ADC Interface — SPI Read The FPGA shall read 4-channel sun sensor photodiode signals via an SPI-connected 12-bit ADC, sharing the IMU SPI bus. | 1. All 4 ADC channels read within one 100 Hz CE window (≤10 ms total). 2. ADC resolution ≥12 bits; no missing codes across full-scale input range 0–3.3 V. 3. Channel multiplexing via CS_N sequencing; inter-channel crosstalk <0.1% FS. 4. Sun-presence flag asserted when any channel > configurable threshold (default: 10% FS). | CRITICAL |
| CS-ADCS-004 | ADCS | Attitude Quaternion Propagation The firmware shall propagate the spacecraft attitude quaternion using the gyroscope angular rate integration (quaternion kinematics) at the 100 Hz ADCS update rate. | 1. Quaternion norm ¦q¦ maintained within 1±0.001 after normalization step each cycle. 2. Quaternion propagation equation: q(k+1) = q(k) + 0.5×q(k)⊗ω̄×Δt, computed in fixed-point Q2.46 arithmetic. 3. Numerical drift of integrated attitude <0.01° over 60-second free-run interval (no EKF correction). 4. Propagation completes within 5 ms of 100 Hz CE strobe assertion. | CRITICAL |
| CS-ADCS-005 | ADCS | Extended Kalman Filter — State Update The firmware shall implement a 7-state EKF (quaternion q0..q3, gyro bias bx,by,bz) to fuse IMU and magnetometer measurements for attitude estimation. | 1. EKF predict and update steps complete within 8 ms of 100 Hz CE strobe. 2. Steady-state attitude error ≤0.5° (3σ) under simulated LEO magnetic field profile. 3. Gyro bias estimate converges to within ±0.01°/s of injected truth bias within 60 seconds of initialization. 4. EKF divergence watchdog: if innovation exceeds 5σ for 3 consecutive cycles, reset EKF to last-known-good state. | CRITICAL |
| CS-ADCS-006 | ADCS | EKF Covariance Matrix Management The firmware shall maintain the 7×7 error covariance matrix P and process/measurement noise matrices Q, R in BRAM, updated each EKF cycle. | 1. Covariance matrix remains positive-definite throughout all operating conditions; Joseph-form update used. 2. Diagonal elements of P stored in Q2.46 fixed-point; off-diagonal elements in Q1.46. 3. Matrix update computation (P⁺ = (I−KH)P) completes within 3 ms using DSP48 pipeline. 4. Q and R matrices independently loadable via AXI4-Lite config bus without restarting ADCS. | HIGH |
| CS-ADCS-007 | ADCS | PD Attitude Control Law The firmware shall implement a proportional-derivative (PD) control law operating on attitude quaternion error to generate reaction-wheel torque commands. | 1. Control law: τ = −Kp×q_err(1:3) − Kd×ω, computed at 1 kHz CE rate. 2. Output torque command saturated to ±10 mNm per axis; saturation flag asserted when active. 3. Steady-state pointing error <0.5° (3σ) in all three axes under 1e-6 N·m disturbance torque. 4. Kp and Kd gain registers loadable via AXI4-Lite config bus; changes take effect within 1 control cycle. | CRITICAL |
| CS-ADCS-008 | ADCS | Reaction Wheel Motor Driver Command The firmware shall translate control law torque commands into SPI motor driver commands for 3 orthogonal reaction wheels at 1 kHz update rate. | 1. SPI command issued within 1 ms of torque command register update. 2. Speed command range: ±6000 RPM, represented as 16-bit signed integer (1 LSB = 0.2 RPM). 3. All 3 wheel CS_N lines independently driven; no bus collision possible by design. 4. Wheel fault status (overcurrent, overtemp) read back at 10 Hz and stored in telemetry register. | HIGH |
| CS-ADCS-009 | ADCS | Magnetorquer PWM Drive The firmware shall generate 3-axis magnetorquer PWM drive signals at 10 kHz carrier, with duty cycle proportional to commanded magnetic dipole moment. | 1. PWM resolution ≥12 bits (0.024% duty cycle step) across 0–100% range. 2. Carrier frequency 10 kHz ± 1% across full duty cycle range. 3. Cross-axis magnetic coupling <1% of full-scale dipole on orthogonal axes. 4. PWM output blanked (duty=0) within 1 control cycle when ADCS enters SAFE mode. | HIGH |
| CS-ADCS-010 | ADCS | ADCS Mode Management — State Machine The firmware shall implement a 5-state ADCS FSM: BOOT, DETUMBLE, COARSE_POINT, FINE_POINT, and SAFE. | 1. BOOT→DETUMBLE transition when ¦ω¦ > 0.5°/s on any axis; BOOT→COARSE_POINT otherwise. 2. DETUMBLE→COARSE_POINT when ¦ω¦ < 0.1°/s on all axes for 10 consecutive seconds. 3. COARSE_POINT→FINE_POINT when pointing error <5° for 30 consecutive seconds. 4. Any axis FAULT signal→SAFE within 2 control cycles (2 ms). SAFE blanks all actuator outputs. 5. Mode register readable via telemetry; mode transitions logged with timestamp. | CRITICAL |
| CS-ADCS-011 | ADCS | ADCS Health Monitoring & Fault Detection The firmware shall continuously monitor all ADCS sensor and actuator interfaces for faults, asserting fault flags and transitioning to SAFE mode as appropriate. | 1. SPI/I2C timeout >5 ms on any sensor interface asserts corresponding SENSOR_FAULT flag. 2. IMU rate >500°/s on any axis (hardware fault) asserts IMU_OVERFLOW_FAULT. 3. Quaternion norm ¦q¦ outside 1±0.01 for >3 cycles asserts EKF_DIVERGE_FAULT. 4. Any CRITICAL fault transitions ADCS to SAFE mode and logs fault code + timestamp. 5. Fault status register accessible via AXI4-Lite; cleared only by explicit ground command. | CRITICAL |
| CS-ADCS-012 | ADCS | ADCS Data Logging to BRAM The firmware shall maintain a circular BRAM buffer of ADCS state snapshots, writable at 100 Hz and readable by the telemetry subsystem at any time without ADCS interruption. | 1. Buffer depth ≥256 entries; each entry contains timestamp + quaternion + ω + mode + fault flags = 96 bytes. 2. Write pointer advances at each 100 Hz CE cycle; read pointer independently managed by telemetry encoder. 3. No ADCS cycle is ever delayed by buffer write contention; dual-port BRAM used. 4. Buffer overflow flag asserted when read pointer ≤ write pointer + 1 (wraparound warning). | HIGH |
| B. Orbit Propagator — SGP4-Lite Engine |  |  |  |  |
| CS-ORB-001 | ORBIT | TLE Input & Storage The firmware shall accept and store Two-Line Element set (TLE) data via the AXI4-Lite configuration bus, retaining the last valid TLE across soft resets. | 1. TLE Line 1 (69 chars) and Line 2 (69 chars) stored in dedicated BRAM register bank. 2. TLE checksum validated on write; invalid TLE rejected and FAULT_TLE_INVALID flag asserted. 3. TLE data survives soft reset (FPGA reconfigure preserving BRAM contents). 4. TLE epoch parsed to Modified Julian Date (MJD) with resolution ≤1 second. 5. TLE age (current time − epoch) tracked; TLE_STALE flag asserted if age >7 days. | CRITICAL |
| CS-ORB-002 | ORBIT | SGP4-Lite Propagation Engine The firmware shall implement a simplified SGP4 orbit propagator (SGP4-Lite) computing spacecraft ECI position and velocity at 1 Hz update rate. | 1. ECI position output: X, Y, Z in km, Q16.32 fixed-point, range ±8000 km. 2. ECI velocity output: Vx, Vy, Vz in km/s, Q4.44 fixed-point, range ±10 km/s. 3. Position accuracy ≤5 km (3σ) over 24-hour propagation from valid TLE epoch, validated against STK reference trajectory. 4. Propagation step completes within 50 ms of 1 Hz CE strobe (fits within 100 Hz ADCS cycle budget). 5. Perturbation terms: J2 zonal harmonic included; J3 and atmospheric drag optional via config register. | CRITICAL |
| CS-ORB-003 | ORBIT | Position Output Format & LVLH Frame Conversion The firmware shall convert ECI position/velocity to Local Vertical Local Horizontal (LVLH) frame and output orbital elements (a, e, i, RAAN, ω, ν). | 1. LVLH conversion matrix R_ECI_to_LVLH computed each 1 Hz cycle from position + velocity vectors. 2. Classical orbital elements (a, e, i, RAAN, ω, ν) derived and stored in dedicated registers. 3. Altitude above WGS84 ellipsoid computed and stored; altitude range 200–2000 km. 4. All outputs available in BRAM within 5 ms of propagation step completion. 5. Subsolar point and eclipse flag computed each cycle: eclipse flag asserts when spacecraft enters Earth shadow. | HIGH |
| CS-ORB-004 | ORBIT | Time Reference & Epoch Management The firmware shall maintain an on-board mission elapsed time (MET) counter synchronized to the TLE epoch, used as the propagation time input. | 1. MET counter increments at 1 Hz with 32-bit unsigned integer (max 136-year overflow). 2. MET loadable from ground command via AXI4-Lite with ≤1 second latency. 3. MET delta (drift) since last ground sync stored and included in telemetry. 4. GPS UART message (if GPS valid) automatically updates MET when delta >2 seconds. 5. MET roll-over handled gracefully; propagator uses modular epoch arithmetic. | HIGH |
| CS-ORB-005 | ORBIT | Inter-Satellite Relative Position The firmware shall compute relative position vectors between this CubeSat and the other two formation members using received ISL position data. | 1. Relative position ΔR_12, ΔR_13 (ECI) computed from own position minus received peer positions. 2. Relative velocity ΔV_12, ΔV_13 computed similarly. 3. Formation separation distance ¦ΔR¦ output in km with 0.01 km resolution. 4. Relative position registers updated within 100 ms of new ISL position data arrival. 5. FORMATION_GEOMETRY_VALID flag set when all 3 peer positions received within last 5 seconds. | HIGH |
| CS-ORB-006 | ORBIT | Ground Track & Contact Window Prediction The firmware shall compute the sub-satellite ground track latitude/longitude and predict the next ground station contact window. | 1. Geodetic latitude and longitude (WGS84) computed each 1 Hz cycle with ≤0.1° accuracy. 2. Ground station contact prediction uses configurable ground station lat/lon/elevation mask (stored in config registers). 3. Contact AOS (Acquisition of Signal) time predicted with ≤30-second accuracy for next 2 orbits. 4. CONTACT_WINDOW flag asserted when elevation angle > configurable mask (default 5°). 5. Output: time-to-AOS in seconds, stored in telemetry register. | MEDIUM |
| CS-ORB-007 | ORBIT | Orbit Propagator Health & Validity The firmware shall monitor the orbit propagator for numerical health and data validity, providing status flags to the ADCS and telemetry subsystems. | 1. PROPAGATOR_VALID flag asserted only when: TLE not stale, MET synchronized within 60 s, and last propagation step completed without overflow. 2. Propagation arithmetic overflow on any axis asserts PROPAGATOR_OVERFLOW_FAULT and freezes outputs at last-valid values. 3. Orbital period cross-check: if computed period deviates >5% from TLE mean motion, assert TLE_INCONSISTENCY_WARNING. 4. Health status register updated each 1 Hz cycle; readable via AXI4-Lite and telemetry encoder. | HIGH |
| C. Laser Pointing FSM — Inter-Satellite Link |  |  |  |  |
| CS-LSR-001 | LASER | Laser Pointing FSM — State Definition The firmware shall implement a 6-state FSM: IDLE, SEARCH, ACQUIRE, TRACK, COMM, and FAULT, governing the laser gimbal and modulator. | 1. All 6 states explicitly defined in RTL with no unreachable or undefined state encoding. 2. State register width ≥3 bits; one-hot or binary encoding with Gray-code transitions where possible. 3. Current state readable at all times via AXI4-Lite status register and telemetry. 4. Power-on reset always enters IDLE state within 3 clock cycles of sys_rst_n de-assertion. 5. State transition log (last 16 transitions + timestamps) maintained in circular BRAM buffer. | CRITICAL |
| CS-LSR-002 | LASER | IDLE State Behavior In IDLE state, the firmware shall park the laser gimbal at the home position, disable the laser modulator, and await a START command or autonomous trigger. | 1. On IDLE entry: STEP_AZ and STEP_EL outputs driven to home position (0°, 0°) within 5 seconds. 2. Laser modulator enable signal de-asserted within 1 control cycle of IDLE entry. 3. IDLE→SEARCH transition triggered by: (a) explicit AXI4-Lite START command, or (b) FORMATION_GEOMETRY_VALID flag and target acquisition window open. 4. Power consumption in IDLE: gimbal drivers de-energized (DIR outputs held, STEP outputs stopped). | CRITICAL |
| CS-LSR-003 | LASER | SEARCH State Behavior In SEARCH state, the firmware shall execute a raster scan pattern over a configurable angular search volume to locate the target satellite retroreflector. | 1. Raster scan pattern covers ±10° azimuth × ±5° elevation (configurable) in a boustrophedon (S-curve) pattern. 2. Scan step size ≤0.5° (configurable); dwell time per step ≥10 ms (configurable). 3. SEARCH→ACQUIRE transition when received signal strength (analog input threshold) > configurable acquisition threshold for ≥3 consecutive dwell periods. 4. SEARCH→FAULT transition if full scan volume completed twice without detection (configurable timeout; default 60 s). 5. Scan progress (current az/el, completion percentage) stored in status registers. | HIGH |
| CS-LSR-004 | LASER | ACQUIRE State Behavior In ACQUIRE state, the firmware shall drive the gimbal to center the detected signal using a spiral refinement maneuver, achieving bore-sight alignment. | 1. ACQUIRE refinement spiral: ±2° diameter, reducing by 50% each pass until signal peak located. 2. Peak-hold algorithm updates pointing command at 100 Hz; convergence to within 0.1° of peak in ≤5 seconds. 3. ACQUIRE→TRACK transition when pointing error ≤0.1° for ≥500 ms continuously. 4. ACQUIRE→SEARCH transition if signal lost (below threshold) for >200 ms. 5. ACQUIRE→FAULT transition if refinement fails to converge within 30 s (configurable). | CRITICAL |
| CS-LSR-005 | LASER | TRACK State Behavior In TRACK state, the firmware shall maintain closed-loop pointing using a PD controller on the signal centroid error, minimizing pointing jitter. | 1. Pointing control loop runs at 100 Hz; position error fed from signal centroid sensor (analog input). 2. Steady-state pointing RMS error ≤0.05° under nominal vibration environment. 3. TRACK→COMM transition when pointing error <0.02° for ≥1 second continuously. 4. TRACK→ACQUIRE transition if pointing error >0.5° for >50 ms (tracking lost). 5. TRACK→FAULT transition on any gimbal STEP/DIR hardware fault flag. | CRITICAL |
| CS-LSR-006 | LASER | COMM State Behavior In COMM state, the firmware shall enable the laser modulator for inter-satellite optical data transmission while maintaining closed-loop pointing. | 1. Laser modulator enable asserted within 1 control cycle of COMM entry. 2. ISL 8-bit parallel data output driven at up to 100 MHz (configurable) with VALID strobe. 3. Pointing control continues at 100 Hz in background; pointing must remain <0.05° RMS. 4. COMM→TRACK transition if pointing error >0.1° for >20 ms (modulator disabled immediately). 5. COMM→FAULT transition on: laser overtemperature, optical power failure, or modulator fault. 6. Data throughput ≥100 Mbps when link budget margin ≥3 dB; confirmed via loopback test mode. | CRITICAL |
| CS-LSR-007 | LASER | FAULT State Behavior & Recovery In FAULT state, the firmware shall immediately safe all laser outputs, log the fault cause, and await a manual CLEAR command before re-attempting operation. | 1. On FAULT entry: laser modulator disabled within 1 cycle, gimbal STEP outputs stopped within 1 cycle. 2. Fault cause code (8-bit) logged to fault register with MET timestamp; last 8 faults retained in FIFO. 3. FAULT→IDLE transition only via explicit AXI4-Lite FAULT_CLEAR command (no autonomous recovery). 4. FAULT state active LED indicator asserted; visible in telemetry status byte bit 7. 5. Fault codes defined: 0x01=gimbal_hw, 0x02=signal_lost_timeout, 0x04=laser_overtemp, 0x08=acquire_timeout, 0x10=pointing_diverge, 0x20=modulator_fault. | CRITICAL |
| CS-LSR-008 | LASER | Gimbal Stepper Motor Control The firmware shall generate 2-axis (azimuth, elevation) stepper motor STEP and DIR pulse trains to drive the laser gimbal with configurable micro-stepping. | 1. Step pulse width ≥2 µs (configurable); DIR signal stable ≥1 µs before and after STEP pulse. 2. Maximum step rate 200 kHz per axis; acceleration profile used (trapezoidal) to avoid motor stalling. 3. Position tracking: 32-bit step counter per axis, configurable steps-per-degree (default: 400 steps/°). 4. Soft position limits (configurable ±15° az, ±10° el) enforced in firmware; hard-stop FAULT if exceeded. 5. Homing sequence on power-up: drive to end-stop (configurable direction), zero step counter, move to home position. | HIGH |
| CS-LSR-009 | LASER | Gimbal Pointing Command Interface The firmware shall accept pointing commands in absolute degrees (az/el) and relative step counts from both the ADCS and the AXI4-Lite config bus. | 1. Absolute pointing command: 16-bit signed Q8.8 fixed-point degrees; range −180° to +180° az, −90° to +90° el. 2. Relative command: 16-bit signed step delta; applied on top of current position. 3. Command source priority: AXI4-Lite config > TRACK/ACQUIRE FSM > orbit-derived pointing. 4. Command-to-motion latency ≤1 ms (1 control cycle). 5. Pointing command rejected (with flag) if outside soft limits; target position register updated only for valid commands. | HIGH |
| CS-LSR-010 | LASER | Optical Signal Strength Monitoring The firmware shall continuously sample the received optical signal strength analog input and provide a digital value for use by the acquisition and tracking FSM states. | 1. Signal strength ADC sampled at 10 kHz minimum; result stored as 12-bit unsigned value. 2. Configurable acquisition threshold register (default: 512 counts = 12.5% FS). 3. Rolling average over 16 samples used for threshold comparison to suppress noise spikes. 4. Peak-hold value updated only when new sample exceeds current peak; peak resets on state transitions. 5. Signal strength value included in every telemetry frame. | HIGH |
| D. Telemetry Encoder & Command Decoder |  |  |  |  |
| CS-TLM-001 | TLM | Telemetry Frame Format The firmware shall encode all CubeSat housekeeping and mission data into a fixed-length telemetry frame with a defined header, payload, and checksum. | 1. Frame structure: SYNC(4B) ¦ APID(2B) ¦ SEQ_COUNT(2B) ¦ LENGTH(2B) ¦ TIMESTAMP(4B) ¦ PAYLOAD(≤128B) ¦ CRC16(2B). Total max = 144 bytes. 2. SYNC word fixed at 0x1ACFFC1D (CCSDS standard). 3. APID range: 0x100–0x1FF reserved for CubeSat telemetry; configurable via AXI4-Lite. 4. SEQ_COUNT increments monotonically (mod 2^14) each frame; rollover handled without reset. 5. CRC-16/CCITT-FALSE computed over bytes 4 through (LENGTH+4); verified by ground station. | CRITICAL |
| CS-TLM-002 | TLM | Telemetry Encoding — UART Downlink The firmware shall serialize telemetry frames to a UART TX output at configurable baud rates for RF downlink via the telemetry transmitter. | 1. Supported baud rates: 9600, 19200, 38400, 57600, 115200 bps; selectable via AXI4-Lite register. 2. Frame transmission latency ≤ frame_length_bytes × 10 / baud_rate + 500 µs overhead. 3. Start and stop bits: 1 start, 8 data, 1 stop (8N1); no parity (standard for CubeSat UHF links). 4. UART TX FIFO depth ≥256 bytes; FIFO overflow flag asserted and TLM frame dropped (not corrupted) on overflow. 5. Inter-frame gap ≥10 ms (configurable) to allow ground receiver AGC settling. | CRITICAL |
| CS-TLM-003 | TLM | Telemetry Payload Content — ADCS Packet The firmware shall compose a dedicated ADCS telemetry packet (APID 0x101) containing attitude and control data at 1 Hz downlink rate. | 1. ADCS packet payload: quaternion q[4] (4×4B), angular rate ω[3] (3×4B), EKF mode (1B), fault flags (1B), control torque[3] (3×2B), RW speeds[3] (3×2B), magnetorquer duty[3] (3×1B) = 44 bytes. 2. Quaternion encoded as Q1.30 fixed-point; angular rate as Q4.12 fixed-point (°/s). 3. Packet assembled from BRAM ADCS log buffer within 5 ms of 1 Hz telemetry CE strobe. 4. If ADCS data is stale (>500 ms old), STALE_DATA_FLAG bit set in packet header byte. | CRITICAL |
| CS-TLM-004 | TLM | Telemetry Payload Content — Orbit Packet The firmware shall compose a dedicated orbit telemetry packet (APID 0x102) containing position, velocity, and orbital elements at 1 Hz. | 1. Orbit packet payload: ECI position[3] (3×6B Q16.32), ECI velocity[3] (3×4B Q4.28), altitude (2B, meters), latitude (4B, µ°), longitude (4B, µ°), eclipse_flag (1B), contact_window (1B), time_to_AOS (4B) = 47 bytes. 2. All values sourced from orbit propagator BRAM output registers. 3. PROPAGATOR_VALID flag forwarded in packet status byte; invalid orbit data clearly flagged. 4. TLE epoch age (seconds since last TLE upload) included as 4-byte field. | HIGH |
| CS-TLM-005 | TLM | Telemetry Payload Content — Laser/ISL Packet The firmware shall compose a laser/ISL telemetry packet (APID 0x103) at 1 Hz containing FSM state, pointing data, and link statistics. | 1. Laser packet payload: FSM_state (1B), gimbal_az (2B Q8.8 °), gimbal_el (2B Q8.8 °), signal_strength (2B), pointing_error (2B), fault_code (1B), link_bytes_tx (4B), link_bytes_rx (4B), bit_error_count (2B) = 20 bytes. 2. FSM state encoded as per CS-LSR-001 state encoding table. 3. Bit error count computed from 8b/10b disparity errors or PN23 PRBS comparison in loopback test mode. 4. Link throughput statistics reset on each COMM→non-COMM state transition. | HIGH |
| CS-TLM-006 | TLM | Telemetry Multiplexer & Arbiter The firmware shall arbitrate telemetry packet transmission from ADCS, orbit, laser, and housekeeping sources through the shared ground-station UART using a fixed priority schedule. | 1. Transmission schedule (repeating 1-second superframe): HK(0ms), ADCS(100ms), ORBIT(200ms), LASER(300ms), ADCS(500ms), free(600–1000ms). 2. Arbiter grants UART bus in strict schedule order; no packet may pre-empt another in transmission. 3. If a packet is not ready at its scheduled slot, the slot is skipped and GAP_COUNTER incremented. 4. Housekeeping packet (APID 0x100) always transmitted first; contains arbiter GAP_COUNTER and system health. | HIGH |
| CS-TLM-007 | TLM | Housekeeping Telemetry Packet The firmware shall generate a system-level housekeeping packet (APID 0x100) at 1 Hz containing FPGA resource utilization, power monitors, and inter-subsystem health flags. | 1. HK payload: sys_uptime (4B), FPGA_temp_xadc (2B, 0.1°C), VCCINT (2B, mV), VCCO (2B, mV), subsystem_health_flags (1B: bit0=ADCS, bit1=ORB, bit2=LSR, bit3=ISL), last_fault_code (1B), gap_counter (2B), sw_version (2B) = 18 bytes. 2. XADC temperature read via dedicated FPGA primitive at 1 Hz; range −40°C to +125°C with ±4°C accuracy. 3. VCCINT and VCCO voltages read via XADC supply monitor; alarm flag if out of ±5% nominal. 4. sw_version register loaded at synthesis time from a VHDL/Verilog parameter constant. | MEDIUM |
| CS-TLM-008 | TLM | Ground Command Reception & Decoding The firmware shall receive and decode uplink commands via the shared ground-station UART RX port, routing commands to the appropriate subsystem via the AXI4-Lite bus. | 1. Command frame: SYNC(4B) ¦ APID(2B) ¦ CMD_CODE(1B) ¦ CMD_DATA(≤16B) ¦ CRC16(2B); max 25 bytes. 2. SYNC word 0x1ACFFC1D checked; CRC validated before command execution. 3. Invalid SYNC or CRC increments CMD_REJECT_COUNTER; invalid command frame discarded silently. 4. Command routing: APID 0x200–0x2FF dispatched to ADCS; 0x300–0x3FF to orbit; 0x400–0x4FF to laser. 5. Command execution acknowledgment telemetry packet sent within 500 ms of command receipt. | CRITICAL |

## 5.  Requirements-to-Implementation Traceability Matrix (Selected)

The following matrix maps a representative subset of requirements to FPGA design blocks, clock domains, resource usage, and verification methods. A complete traceability matrix covering all 37 requirements shall be maintained in the project configuration management system.

| REQ_ID | FPGA Block | Clock Domain | BRAM? | DSP48? | Interface | Test Method | Sim? | HIL? | Verification Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS-ADCS-001 | spi_master_imu | clk_100mhz | Y | N | SPI 10MHz | Bench + Sim | Y | Y | Inject known SPI frame; verify BRAM contents |
| CS-ADCS-002 | i2c_master_mag | sys_clk | Y | N | I2C 400kHz | Bench + Sim | Y | Y | I2C protocol analyzer verification |
| CS-ADCS-003 | spi_adc_sun | clk_100mhz | N | N | SPI shared | Bench | Y | N | 4-channel ramp input; check ADC linearity |
| CS-ADCS-004 | quat_propagator | sys_clk | N | Y | CE 100Hz | Sim | Y | N | Compare vs MATLAB/Simulink reference for 60s |
| CS-ADCS-005 | ekf_7state | sys_clk | Y | Y | CE 100Hz | Sim + HIL | Y | Y | Monte Carlo 100 runs; verify 3σ ≤0.5° |
| CS-ADCS-007 | pid_ctrl_uav | sys_clk | N | Y | CE 1kHz | Sim + HIL | Y | Y | Step response test; verify settling <0.5° |
| CS-ORB-001 | tle_bram_store | sys_clk | Y | N | AXI4-Lite | Sim | Y | N | Write valid/invalid TLE; verify accept/reject |
| CS-ORB-002 | sgp4_propagator | sys_clk | Y | Y | CE 1Hz | Sim | Y | N | Compare against STK for 10 test orbits |
| CS-LSR-001 | laser_fsm | sys_clk | Y | N | CE 100Hz | Sim + Bench | Y | Y | State transition coverage 100%; no dead states |
| CS-LSR-005 | laser_track_ctrl | sys_clk | N | Y | CE 100Hz | HIL | Y | Y | Closed-loop gimbal + photodiode test rig |
| CS-TLM-001 | tlm_framer | sys_clk | Y | N | UART TX | Bench + Sim | Y | Y | Frame capture; CRC verify on 10,000 frames |
| CS-TLM-008 | cmd_decoder | sys_clk | N | N | UART RX | Bench + Sim | Y | Y | Inject corrupt frames; verify reject behavior |

## 6.  Implementation Notes

Fixed-Point Arithmetic: All requirements specifying Q-format fixed-point (e.g., Q2.46) shall be implemented using the Artix-7 DSP48E1 primitives with explicit overflow detection. The format notation QA.B means A bits integer, B bits fractional.

AXI4-Lite Configuration Bus: References to 'AXI4-Lite config bus' throughout this document refer to the 20-signal internal AXI4-Lite interconnect implemented on-chip (not routed to I/O pins, as specified in the I/O Interface Specification).

CE Strobe Domains: Requirements specifying update rates of 1 Hz, 10 Hz, 100 Hz, or 1 kHz refer to clock-enable (CE) strobe domains within the 50 MHz sys_clk domain, as defined in the Clock Architecture Specification. No new clock domains are created by these requirements.

Acceptance Criterion Verification: Numbered acceptance criteria in each requirement correspond to individual test cases in the Verification & Validation Plan. Criteria marked 'Sim' require HDL simulation; 'Bench' requires hardware-in-loop testing; 'HIL' requires full-system hardware test.