# 9-State EKF Specification  —  UAV Sensor Fusion (Fixed-Point FPGA)

State: [px,py,pz,vx,vy,vz,φ,θ,ψ]  |  IMU 100Hz  |  GPS 10Hz  |  Baro 50Hz  |  Artix-7 XC7A35T @ 50MHz  |  12 DSP48E1  |  4 BRAM18

Positions Q10.22  |  Velocities Q4.28  |  Angles Q3.29  |  NED frame  |  Euler forward integration  |  ~880 cycles = 17.6 µs per full update cycle

## 1.  State Vector, Control Inputs & Fixed-Point Formats

Complete state vector definition, control input (IMU) descriptions, physical units, Q format, representable ranges, and sensor assignments.

| Index | Symbol | Units | Q Format | Bits | Range | LSB | Physical Meaning / Sensor Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x[0] | px | m | Q10.22 | 32-bit s | ±1023 m | 2.39×10⁻⁷ m | North position (NED). GPS update. Origin = takeoff point. |
| x[1] | py | m | Q10.22 | 32-bit s | ±1023 m | 2.39×10⁻⁷ m | East position (NED). GPS update. |
| x[2] | pz | m | Q10.22 | 32-bit s | ±1023 m | 2.39×10⁻⁷ m | Down position, NED (negative = altitude). Baro + GPS update. |
| x[3] | vx | m/s | Q4.28 | 32-bit s | ±7.99 m/s | 3.72×10⁻⁹ m/s | North velocity. GPS Doppler update. IMU dead-reckoning. |
| x[4] | vy | m/s | Q4.28 | 32-bit s | ±7.99 m/s | 3.72×10⁻⁹ m/s | East velocity. GPS Doppler update. |
| x[5] | vz | m/s | Q4.28 | 32-bit s | ±7.99 m/s | 3.72×10⁻⁹ m/s | Down velocity (positive = descending). GPS + Baro rate update. |
| x[6] | φ (roll) | rad | Q3.29 | 32-bit s | ±3.14 rad | 1.86×10⁻⁹ rad | Roll angle. IMU gyro integration. Range ±180°. |
| x[7] | θ (pitch) | rad | Q3.29 | 32-bit s | ±1.57 rad | 1.86×10⁻⁹ rad | Pitch angle. IMU gyro integration. Limited ±90° to avoid singularity. |
| x[8] | ψ (yaw) | rad | Q3.29 | 32-bit s | ±3.14 rad | 1.86×10⁻⁹ rad | Yaw/heading. IMU gyro + magnetometer. Wrapped to [-π, +π]. |
| Control u[0] | ax_b | m/s² | Q4.28 | 32-bit s | ±7.99 | — | Body-frame acceleration X (IMU). Rotated to NED by R(φ,θ,ψ). |
| Control u[1] | ay_b | m/s² | Q4.28 | 32-bit s | ±7.99 | — | Body-frame acceleration Y (IMU). |
| Control u[2] | az_b | m/s² | Q4.28 | 32-bit s | ±7.99 | — | Body-frame acceleration Z (IMU, includes gravity). |
| Control u[3] | p (gx) | rad/s | Q4.28 | 32-bit s | ±7.99 | — | Roll rate from gyroscope. |
| Control u[4] | q (gy) | rad/s | Q4.28 | 32-bit s | ±7.99 | — | Pitch rate from gyroscope. |
| Control u[5] | r (gz) | rad/s | Q4.28 | 32-bit s | ±7.99 | — | Yaw rate from gyroscope. |

## 2.  Linearized Jacobian F_d  (9×9 Discrete State Transition)

F_d = I + (df/dx)|_{x_k} × Ts evaluated at current state each prediction step. Gray = zero. Blue bold = identity diagonal. Orange = Ts coupling. White = nonzero trig derivatives (computed from CORDIC sin/cos and current gyro rates).

| F_d (9×9 Linearized Discrete State Transition Jacobian) = I + (df/dx)¦_{x_k} × Ts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | px | py | pz | vx | vy | vz | φ | θ | ψ |
| px | 1 | 0 | 0 | Ts | 0 | 0 | 0 | 0 | 0 |
| py | 0 | 1 | 0 | 0 | Ts | 0 | 0 | 0 | 0 |
| pz | 0 | 0 | 1 | 0 | 0 | Ts | 0 | 0 | 0 |
| vx | 0 | 0 | 0 | 1 | 0 | 0 | F36*Ts | F37*Ts | 0 |
| vy | 0 | 0 | 0 | 0 | 1 | 0 | F46*Ts | F47*Ts | 0 |
| vz | 0 | 0 | 0 | 0 | 0 | 1 | F56*Ts | F57*Ts | 0 |
| φ | 0 | 0 | 0 | 0 | 0 | 0 | F66*Ts+1 | F67*Ts | 0 |
| θ | 0 | 0 | 0 | 0 | 0 | 0 | F76*Ts | F77*Ts+1 | 0 |
| ψ | 0 | 0 | 0 | 0 | 0 | 0 | F86*Ts | F87*Ts | 1 |
| Non-zero element symbolic keys — computed at each prediction step: F36 = dv̇x/dφ = Ts(2sinφcosφ(sinθcosθ·ay-sinθcosθ·az) + cos²φ·sinθ·az_n_terms) ¦ Simplified: F36 = -Ts·(ay_n·cosφ + az_n_corr·sinφ) F37 = dv̇x/dθ = Ts·(ax_n·cosθ + az_n_corr·sinθ) ¦ F46,F47,F56,F57: similar trig derivatives for vy,vz rows F66 = dφ̇/dφ = Ts·(q·cosφ-r·sinφ)·tanθ ¦ F67 = dφ̇/dθ = Ts·(q·sinφ+r·cosφ)·sec²θ F76 = dθ̇/dφ = Ts·(-q·sinφ-r·cosφ) ¦ F77 = 0 (θ̇ not directly a function of θ in small-angle) F86 = dψ̇/dφ = Ts·(q·cosφ-r·sinφ)/cosθ ¦ F87 = dψ̇/dθ = Ts·(q·sinφ+r·cosφ)·sinθ/cos²θ |  |  |  |  |  |  |  |  |  |

## 3.  Measurement Jacobians H (GPS 5×9) and H (Baro 1×9)

Both H matrices are constant extraction operators (no runtime Jacobian evaluation needed). GPS picks positions + horizontal velocity; barometer picks negative-pz (altitude).

| H_GPS (5×9) — Measures [px,py,pz,vx,vy] from state. 10 Hz. 10 non-zeros. Extraction matrix (no computation). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | px | py | pz | vx | vy | vz | φ | θ | ψ |
| z_px | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| z_py | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| z_pz | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| z_vx | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| z_vy | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |

| H_BARO (1×9) — Measures altitude = -pz (NED convention). 50 Hz. 1 non-zero. z_baro = -pz → H[0,2] = -1. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | px | py | pz | vx | vy | vz | φ | θ | ψ |
| z_alt | 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 |

## 4.  Prediction Equations (IMU-Driven, 100 Hz)

All 9 state predictions and the covariance propagation step, showing continuous-time equations and discrete Euler approximations with fixed-point format annotations.

| State | Continuous f(x,u) | Discrete Update (Ts = 0.01s IMU) | Fixed-Point Notes |
| --- | --- | --- | --- |
| px_pred | vx | px + vx×Ts | Q10.22 += Q4.28×Ts; Ts in Q1.15=328; product Q5.43→shift 21→Q10.22 |
| py_pred | vy | py + vy×Ts | Identical to px. 1 DSP48 each. |
| pz_pred | vz | pz + vz×Ts | Identical. pz negative = altitude in NED. |
| vx_pred | ax_n = R[0]·ax_b - g×R[0,2] | vx + ax_n×Ts | Rotation: 3 DSP48 MACs (R row dot accel). ax_n Q4.28. += Q4.28×Ts. |
| vy_pred | ay_n = R[1]·ax_b - g×R[1,2] | vy + ay_n×Ts | R[1] row. Same DSP chain, different row. |
| vz_pred | az_n - g = R[2]·ax_b - g | vz + (az_n-g)×Ts | Subtract g=9.81 in Q4.28 (g_fp=2.622×10⁹). 3 DSP48 MACs + 1 subtract. |
| phi_pred | p + (q sinφ + r cosφ) tanθ | phi + phi_dot×Ts | CORDIC for sin/cos/tan. phi_dot Q4.28. 4 DSP48 + 2 CORDIC. |
| theta_pred | q cosφ - r sinφ | theta + theta_dot×Ts | 2 CORDIC outputs reused from phi row. 2 DSP48. |
| psi_pred | (q sinφ + r cosφ)/cosθ | psi + psi_dot×Ts | cosθ reciprocal needed: 1 Newton-Raphson or LUT. 2 DSP48 + divide. |
| P_pred | F·P·F^T + Q | P_d = F_d·P·F_d^T + Q (Euler: P += (F·P + P·F^T + Q)×Ts) | 81-element update. Sparse F reduces to 28 real multiplies. 4 DSP48 shared. Q diagonal: 9 adds. |

## 5.  Measurement Update Equations

GPS (10 Hz) full 5-measurement update and barometric altitude (50 Hz) scalar update, including innovation computation, S matrix, Kalman gain, and Joseph-form P update for numerical stability.

| Sensor/Step | Formula | Dimensions | Fixed-Point Notes |
| --- | --- | --- | --- |
| GPS Innovation | v_gps = z_gps - H_gps·x_pred = z_gps - x_pred[0:4] | 5×1 vector (Q10.22 pos, Q4.28 vel) | H_gps×x_pred is wire slice. Innovation = GPS reading minus predicted position/velocity. No multiply needed for H_gps. |
| GPS S matrix | S_gps = H_gps·P·H_gps^T + R_gps | 5×5 matrix | H·P·H^T extracts top-left 5×5 of P (position-velocity block). R_gps diagonal: σ_pos=9m², σ_vel=0.04m²/s². 25 multiply-adds. |
| GPS Kalman Gain | K_gps = P·H_gps^T · S_gps^-1 | 9×5 matrix | P·H^T is 9×5 (columns 0-4 of P). S^-1 is 5×5 matrix inversion (Cholesky preferred: 15 BRAM + 10 DSP48). K_gps stored in BRAM. |
| GPS State Update | x+ = x_pred + K_gps·v_gps | 9×1 state | K_gps(9×5) × v_gps(5×1) = 45 multiply-adds. 9 DSP48 time-multiplexed. Formats: K in Q4.28, v in Q10.22/Q4.28 → mixed multiply. |
| GPS P Update | P+ = (I - K_gps·H_gps)·P (Joseph form for stability) | 9×9 matrix | K·H is 9×9. Joseph form: P+ = (I-KH)P(I-KH)^T + K·R·K^T. 162 multiplies. Use BRAM triple-buffering. 8 DSP48. |
| Baro Innovation | v_baro = z_baro - (-x_pred[2]) = alt_measured + pz_pred | Scalar | One subtraction. z_baro is barometric altitude (positive up). x_pred[2]=pz is NED (negative altitude). Innovation = z_baro + pz_pred. |
| Baro S scalar | S_baro = P[2,2] + R_baro | Scalar | P[2,2] is pz variance. R_baro = σ_alt² = 0.25m². S_baro is a scalar Q16.16. |
| Baro Kalman Gain | K_baro = P[:,2] / S_baro | 9×1 vector | Scalar division: each P[i,2] ÷ S_baro. Use reciprocal multiplication (Newton-Raphson 1/S): 9 DSP48 multiplies. |
| Baro State Update | x+ = x_pred + K_baro·v_baro | 9×1 state | 9 scalar multiplies (K_baro[i] × v_baro). 1 DSP48 time-multiplexed across 9 elements. |
| Baro P Update | P+ = (I - K_baro·H_baro)·P | 9×9 matrix | K(9×1)·H(1×9) is outer product = rank-1 update. 81 multiplies for outer product. Efficient: only 2 DSP48 needed (row-by-scalar). |
| Normalization | Wrap ψ to [-π, +π] Enforce θ ∈ (-π/2, π/2) | Scalar checks | Yaw wrap: if ψ > π, ψ -= 2π (Q3.29 subtraction, 2π_fp = 3.37e9). Pitch singularity guard: ¦θ¦ > 1.5rad → clamp + warning flag. |

## 6.  Fixed-Point Format Reference

All signals with exact Q format, representable range, and overflow risk analysis. RED cells indicate potential overflow at high-performance flight regimes requiring format adjustment.

| Variable | Bits | Q Format | Integer = Real × | Range | Notes / Overflow Risk |
| --- | --- | --- | --- | --- | --- |
| px, py, pz | 32s | Q10.22 | 4,194,304 | ±1023 m | For >1km range: use Q16.16 (±32767m). LSB=0.24µm. GPS accuracy ~1m → needs Q10.22 for meaningful sub-meter precision. |
| vx, vy, vz | 32s | Q4.28 | 268,435,456 | ±7.99 m/s | For fast UAVs (>8 m/s): use Q8.24 (±127 m/s). LSB=3.7nm/s. GPS velocity σ≈0.1m/s → Q4.28 has >20 bits of precision headroom. |
| φ,θ,ψ (angles) | 32s | Q3.29 | 536,870,912 | ±3.14 rad | π_fp = 1,686,629,713 (Q3.29). 2π_fp = 3,373,259,426. Wrap needs 64-bit subtraction then truncate. LSB=1.9nrad. |
| ax,ay,az (body) | 32s | Q4.28 | 268,435,456 | ±7.99 m/s² | Max accel ≈ 3g=29.4m/s². Overflow! Use Q5.27 (±15.9m/s²) or Q6.26 (±31.9m/s²) for aggressive UAV. g_fp(Q4.28)=2,622,718,976. |
| p,q,r (gyro) | 32s | Q4.28 | 268,435,456 | ±7.99 rad/s | Max gyro ≈ 2000°/s = 34.9 rad/s. Overflow! Use Q6.26 (±63.9 rad/s) for aerobatic flight. Standard Q4.28 ok for gentle maneuvers. |
| P diagonal | 32s | Q16.16 | 65,536 | ±32767 | Covariance variance. Initial σ_pos=10m → P_pp=100 → Q16.16 integer=6,553,600. Grows with time: monitor for overflow. |
| P off-diagonal | 32s | Q8.24 | 16,777,216 | ±127 | Cross-covariances are small. Q8.24 sufficient. If P_pv=0.1 → integer=1,677,722. |
| Kalman gain K | 32s | Q4.28 | 268,435,456 | ±7.99 | K = P·H^T·S^-1. Dimensionless for pos/pos; has units for cross terms. K_p ∈ [0,1] for bounded gain. Check: K×innov must not overflow state format. |
| Innovation v_gps | 32s | Q10.22 | 4,194,304 | ±1023 m | Same format as position states. Max innovation = gate threshold (e.g., 10m = 41,943,040 fp). Gate check before update. |
| Innovation v_baro | 32s | Q10.22 | 4,194,304 | ±1023 m | Same. Baro innovation typically <2m. Gate at 5m (20,971,520 fp). |
| S matrix (gps) | 32s | Q16.16 | 65,536 | ±32767 m² | Innovation covariance. S_pp = P_pp + R_pp ≈ 100+9 = 109m². Integer=7,143,424. |
| Jacobian F entries | 32s | Q3.29 | 536,870,912 | ±3.14 | Trig terms (sin,cos,tan): Q3.29. Velocity coupling terms: Q4.28. Ts entries: Q1.15. |
| Rotation matrix R | 32s | Q1.30 | 1,073,741,824 | ±1.0 | Each R[i,j] ∈ [-1,+1]. Products R[i,j]×accel need 64-bit intermediate or DSP48 P-port. |

## 7.  Resource Usage (DSP48, BRAM, Cycles)

Per-step resource breakdown. 12 DSP48E1 total via time-multiplexing. 4 BRAM18 (one per major matrix: P, K_gps, R_matrix, LUT). Total cycle budget 880 cycles = 17.6 µs — 568× margin vs 10ms.

| Step / Module | DSP48 | BRAM18 | Cycles | Notes |
| --- | --- | --- | --- | --- |
| Rotation matrix R build (from φ,θ,ψ via CORDIC) | 6 | 0 | 24 | CORDIC sin/cos for 3 angles. 3 CORDIC instances × 16 iterations each. R[3×3]=9 elements computed. |
| IMU prediction: p_pred += v×Ts | 3 | 0 | 9 | One DSP48 per axis. Ts×v mixed multiply Q1.15×Q4.28 → Q5.43 → shift. |
| IMU prediction: v_pred += R×a×Ts | 9 | 0 | 27 | 3 DSP48 per velocity axis. R row dot-product with a_body. NED accel add subtract g. |
| IMU prediction: angle rates | 6 | 1 | 30 | CORDIC for sin/cos/tan in Euler kinematic equations. 6 DSP48 for multiplies by gyro rates. |
| P predict: F_d·P·F_d^T + Q | 8 | 2 | 160 | Sparse F reduces 9×9×9=729 ops to ~120 real multiplies. Shared 8 DSP48, BRAM double-buffer P. |
| GPS innovation: v = z - H·x | 0 | 0 | 5 | Wire slice + 5 subtractions in fabric. H_gps is extraction matrix. |
| GPS S matrix (5×5) | 5 | 1 | 50 | 25 multiply-adds for P[0:5,0:5] + R_gps. Cholesky decomposition for inversion: 15 extra ops. |
| GPS K computation: P·H^T·S^-1 | 8 | 2 | 180 | 45-element K_gps. S inversion is most expensive. BRAM for intermediate 9×5 matrix. |
| GPS state update: x += K·v | 9 | 1 | 45 | 9×5 MACC. Mixed format K(Q4.28) × v(Q10.22) → 64-bit → shift → Q10.22. |
| GPS P update (Joseph form) | 8 | 2 | 240 | Full 9×9 update: (I-KH)P(I-KH)^T. ~243 multiplies. Most expensive single step. |
| Baro innovation (scalar) | 0 | 0 | 2 | 1 subtract + sign flip. Trivial. |
| Baro K (9×1) = P[:,2]/S_baro | 3 | 0 | 18 | Reciprocal of S_baro via NR (2 iters=4 DSP). Then 9×multiply for K_baro column. Reuse DSP48s. |
| Baro state update: x += K·v | 1 | 0 | 9 | 9 scalar multiplies. Time-multiplex 1 DSP48 across 9 elements. |
| Baro P update (rank-1) | 2 | 1 | 81 | Outer product K_baro(9×1)×H_baro(1×9) = 81 multiplies. Efficient: row = scalar × vector. |
| TOTAL UNIQUE | 12 DSP48 | 4 BRAM18 | ~880 cycles | 880 × 20ns = 17.6µs at 50MHz. Max update rate = 56kHz >> 100Hz. Ample margin. |

## 8.  Timing & Scheduling

Interleaved IMU/GPS/Baro update schedule showing worst-case coincident timing (all three sensors in same 10ms IMU window) still completes in under 20µs.

| Event | Rate | Period | Cycles @ 50MHz | Schedule Notes |
| --- | --- | --- | --- | --- |
| IMU Predict | 100 Hz | 10 ms | ~460 cycles = 9.2 µs | Triggered by 100Hz CE strobe. All 9 states updated. P predict included. Must complete before next IMU frame. |
| Baro Update | 50 Hz | 20 ms | ~110 cycles = 2.2 µs | Interleaved with IMU predict. Occurs at 50Hz CE strobe. Rank-1 P update is fast (81 cycles). |
| GPS Update | 10 Hz | 100 ms | ~520 cycles = 10.4 µs | Full 5-sensor update. GPS S inversion is the bottleneck (~180 cycles for Cholesky). Triggered by NMEA valid flag. |
| CORDIC compute (angles per predict) | 100 Hz (shared) | 10 ms | ~48 cycles per R (3 angles × 16 iters) | CORDIC for sin/cos/tan runs 3× per predict cycle. Pipelined: 16-stage CORDIC × 3 = 48 cycles. Pre-compute R matrix before P predict. |
| Normalization check | 10 Hz | 100 ms | ~6 cycles | Monitor ¦P¦ < threshold. If P grows unbounded, reset to P0. Yaw wrap check every GPS cycle. |
| Total per IMU cycle | 100 Hz | 10 ms | ~500 cycles | 10 µs << 10 ms. Margin: 1000×. Even with GPS coincidence (+ 520 cycles) = 980 cycles = 19.6 µs. Still 510× margin. |

## 9.  Verilog RTL — Prediction Step

IMU-driven predict module. Rotation matrix R built from CORDIC sin/cos. Position and velocity updated via mixed Q-format multiply. Attitude updated via Euler kinematic equations.

ekf9_predict.v — 9-State Prediction (100 Hz, 12 DSP48 shared)

// 9-State EKF Prediction Step (IMU-driven, 100 Hz) // Updates x[0:8] and P[9×9] using ICM-42688 IMU data // Fixed-point: pos=Q10.22, vel=Q4.28, angles=Q3.29 module ekf9_predict ( input wire clk, rst_n, en_100hz, // Current state input wire signed [31:0] px,py,pz, // Q10.22 input wire signed [31:0] vx,vy,vz, // Q4.28 input wire signed [31:0] phi,tht,psi, // Q3.29 // IMU inputs input wire signed [31:0] ax_b,ay_b,az_b,// Q4.28 body accel input wire signed [31:0] gx,gy,gz, // Q4.28 gyro rad/s // Predicted state outputs output reg signed [31:0] px_p,py_p,pz_p, output reg signed [31:0] vx_p,vy_p,vz_p, output reg signed [31:0] phi_p,tht_p,psi_p ); // Ts = 0.01s in Q1.15 = round(0.01 × 32768) = 328 localparam [15:0] TS_Q115 = 16'd328; // g = 9.81 m/s² in Q4.28 = round(9.81 × 2^28) = 2,637,619,814 localparam signed [31:0] G_Q428 = 32'd2637619814; // ── Position prediction: p_pred = p + v × Ts ────────────────── // Q4.28(v) × Q1.15(Ts) = Q5.43; take bits [43:22] → Q5.21 ≈ Q10.22 wire signed [47:0] dpx = $signed(vx) * $signed(TS_Q115); wire signed [47:0] dpy = $signed(vy) * $signed(TS_Q115); wire signed [47:0] dpz = $signed(vz) * $signed(TS_Q115); // ── CORDIC: compute sin/cos of phi, theta, psi ────────────────── // (instantiate cordic_sincos module for each angle — not shown for brevity) wire signed [31:0] s_phi,c_phi,s_tht,c_tht,s_psi,c_psi; // Q1.30 wire signed [31:0] t_tht; // Q3.28: tan(theta) = sin/cos, via Q1.30÷Q1.30 // ── Rotation matrix R (body→NED), Q1.30 entries ────────────────── wire signed [31:0] R00=($signed(c_tht)*$signed(c_psi))>>30, // cosθ cosψ R10=($signed(s_phi)*($signed(s_tht)*$signed(c_psi))>>30) -($signed(c_phi)*$signed(s_psi))>>30, // sinφsinθcosψ-cosφsinψ ... (R20,R01,R11,R21,R02,R12,R22 follow same pattern) // ── Velocity prediction: v_pred = v + R×a_body×Ts - g_vec×Ts ── // NED acceleration: [ax_n; ay_n; az_n] = R * [ax_b; ay_b; az_b] wire signed [63:0] ax_n_wide = ($signed(R00)*$signed(ax_b))>>30 // Q1.30×Q4.28 = Q5.58 → >>30 = Q5.28 + ($signed(R01)*$signed(ay_b))>>30 + ($signed(R02)*$signed(az_b))>>30; // dvx = ax_n × Ts (Q4.28 × Q1.15 → >>15 → Q4.28 increment) wire signed [47:0] dvx = ($signed(ax_n_wide[31:0))) * $signed(TS_Q115); // ... (dvy, dvz similar; dvz subtracts G_Q428*Ts for gravity) // ── Attitude prediction: Euler kinematics ──────────────────────── // phi_dot = gx + (gy*s_phi + gz*c_phi)*t_tht wire signed [63:0] sphi_gy = ($signed(s_phi)*$signed(gy))>>30, // sinφ×q cphi_gz = ($signed(c_phi)*$signed(gz))>>30, // cosφ×r phi_dot = $signed(gx) + (($signed(sphi_gy)+$signed(cphi_gz))*$signed(t_tht))>>28; wire signed [31:0] dphi = ($signed(phi_dot[31:0)*$signed(TS_Q115))>>15; // theta_dot = gy*c_phi - gz*s_phi wire signed [63:0] tht_dot = $signed(sphi_gy) - $signed(cphi_gz); wire signed [31:0] dtht = ($signed(tht_dot[31:0)*$signed(TS_Q115))>>15; always @(posedge clk) begin if (!rst_n) begin px_p<=0;py_p<=0;pz_p<=0;vx_p<=0;vy_p<=0;vz_p<=0;phi_p<=0;tht_p<=0;psi_p<=0; end else if (en_100hz) begin // Position update: add Q10.22 delta (from Q5.43 >> 21) px_p <= px + dpx[43:22]; // bits [43:22] = Q5.21 → sign-extend to Q10.22 py_p <= py + dpy[43:22]; pz_p <= pz + dpz[43:22]; // Velocity update: add Q4.28 delta vx_p <= vx + dvx[42:15]; // bits [42:15] = Q4.27 ≈ Q4.28 (1 LSB error) vy_p <= vy + dvy[42:15]; vz_p <= vz + dvz[42:15] - ($signed(G_Q428) * $signed(TS_Q115))>>15; // Attitude update: add Q3.29 delta phi_p <= phi + dphi; tht_p <= tht + dtht; psi_p <= psi + dpsi; // (dpsi computed from psi_dot, not shown) end end endmodule

## 10.  Verilog RTL — GPS Measurement Update

GPS update with innovation gating. H_gps extraction implemented as wire differences (zero DSP cost). K_gps loaded from BRAM (pre-computed or computed offline). Innovation gate rejects outliers > ±8m.

ekf9_gps_update.v — GPS 5-Measurement Update (10 Hz, 9 DSP48 shared)

// GPS Measurement Update (10 Hz) // z_gps = [px,py,pz,vx,vy] (5 measurements) // H_gps is extraction: no multiply for innovation // K_gps pre-computed or computed via S inversion each cycle module ekf9_gps_update ( input wire clk, rst_n, gps_valid, // Predicted state (9 elements) input wire signed [31:0] x_pred[0:8], // state array // GPS measurements input wire signed [31:0] z_px,z_py,z_pz, // Q10.22 position input wire signed [31:0] z_vx,z_vy, // Q4.28 velocity // Kalman gain K_gps from BRAM (9×5, Q4.28 each = 180 bytes) input wire signed [31:0] K_gps[0:8][0:4], // Updated state output output reg signed [31:0] x_upd[0:8] ); // ── Innovation: v = z - H_gps*x_pred (5 subtractions, no multiply) ── wire signed [31:0] innov_0 = z_px - x_pred[0], // Q10.22 position innovation innov_1 = z_py - x_pred[1], innov_2 = z_pz - x_pred[2], innov_3 = z_vx - x_pred[3], // Q4.28 velocity innovation innov_4 = z_vy - x_pred[4]; // ── Innovation gate: reject if ¦innov¦ > 5σ (configurable threshold) ── wire gate_ok = (innov_0[31:23] == {9{innov_0[22]}}) & // < ±8m threshold (innov_1[31:23] == {9{innov_1[22]}}) & ... ; // ── State update: x_upd = x_pred + K_gps * innov ────────────────── // K_gps[i][j] (Q4.28) × innov[j] (Q10.22/Q4.28) → accumulate 5 terms // For pos rows (i=0,1,2): K(Q4.28) × innov_pos(Q10.22) → Q14.50 → >>28 → Q10.22 // For vel rows (i=3,4,5): K(Q4.28) × innov_vel(Q4.28) → Q8.56 → >>28 → Q4.28 // For angle rows (i=6,7,8): K(Q4.28) × innov_pos(Q10.22) → >>28 → Q3.22 ≈ Q3.29 (scale diff) reg signed [63:0] dx[0:8]; // wide accumulator per state element integer i, j; always @(posedge clk) begin if (!rst_n ¦¦ !gps_valid ¦¦ !gate_ok) begin // No update — pass through predicted state for (i=0; i<9; i=i+1) x_upd[i] <= x_pred[i]; end else begin // Accumulate K[i][0..4] * innov[0..4] for each state row i for (i=0; i<9; i=i+1) begin dx[i] = ($signed(K_gps[i][0])*$signed(innov_0)) + ($signed(K_gps[i][1])*$signed(innov_1)) + ($signed(K_gps[i][2])*$signed(innov_2)) + ($signed(K_gps[i][3])*$signed(innov_3)) + ($signed(K_gps[i][4])*$signed(innov_4)); // Shift right 28 (from mixed Q4.28 × Q10.22 product Q14.50 → Q10.22) x_upd[i] <= $signed(x_pred[i]) + $signed(dx[i][57:28]); // select correct bits end end end endmodule

## 11.  EKF Design Constraints & Numerical Stability

| Issue | Detail & Mitigation |
| --- | --- |
| Pitch singularity θ→±90° | tan(θ) in Euler kinematics diverges at θ=±90°. Fix: clamp ¦θ¦ < 88° (1.535 rad); assert PITCH_SINGULARITY flag; switch to quaternion formulation above 85°. In practice: UAV never exceeds ±60° pitch in normal flight. |
| Gyro bias accumulation | Euler integration accumulates gyro bias as position error. If gyro drift = 0.1°/s, attitude error after 10s = 1°. Fix: add 3 gyro bias states to make 12-state EKF, OR use EKF reset on GPS heading validation. For Ts=0.01s, drift < 0.01°/cycle. |
| P matrix blow-up | GPS loss causes P to grow unbounded. Monitor P[0,0] diagonal: if > MAX_P_THRESH (e.g., 10000 m²) after GPS absent for > 30s, reset P to P0 (initial covariance). Log GPS_TIMEOUT warning. |
| Joseph-form P update | Standard P+ = (I-KH)P can lose symmetry from floating-point errors. Fixed-point version: use Joseph form P+ = (I-KH)P(I-KH)^T + K·R·K^T at every GPS update (doubles computation but guarantees positive-definite P). |
| Mixed Q-format multiply | Different state elements have different Q formats. K_gps[angle_row] × innov_pos requires careful bit-selection to maintain dimensional consistency. Pre-compute and store format conversion shifts per K row in BRAM alongside K values. |
| GPS NMEA latency | NEO-M9N NMEA string arrives ~100ms after GPS fix epoch. EKF must tag GPS measurement with the epoch timestamp, not receive timestamp. Use GPS PPS signal to align timestamps. Time lag = 0.1s × velocity → up to 0.5m position error at 5m/s without correction. |