# Formation Geometry Specification  —  6-Vehicle Coordination

Formations: Triangle · Line · V-Shape · Square  |  6 vehicles (V1=leader, V2-V6=followers)  |  50m baseline  |  NED frame

Frame: formation-body (x_f=forward, y_f=right)  |  Rotation R_f2NED(ψ_L)  |  Error: per-axis 1m threshold  |  Persistence: 3×10Hz cycles

## 1.  Coordinate Frame & Rotation Matrix

All formation offsets are defined in the formation body frame (x_f = forward along leader heading, y_f = right, z_f = up). The rotation matrix R_f2NED(ψ_L) converts these to NED coordinates for position targeting.

Formation Body Frame → NED conversion:

P_target_i = P_leader + R_f2NED(ψ_L) × δ_f_i

R_f2NED(ψ) = [ cosψ  -sinψ  0 ]   where ψ = leader heading (NED, 0=North, CW positive)

[ sinψ   cosψ  0 ]

[  0      0    1 ]   (z-axis pass-through for altitude)

| R_f2NED(psi) Formation Frame to NED Rotation Matrix (psi = leader heading, degrees) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| psi (deg) | R[0,0] cosP | R[0,1] -sinP | R[1,0] sinP | R[1,1] cosP | Notes |
| 0° | 1.000000 | 0.000000 | 0.000000 | 1.000000 | North-facing. x_f=North, y_f=East. |
| 30° | 0.866025 | -0.500000 | 0.500000 | 0.866025 |  |
| 45° | 0.707107 | -0.707107 | 0.707107 | 0.707107 | NE diagonal. Standard 45 deg transit. |
| 60° | 0.500000 | -0.866025 | 0.866025 | 0.500000 |  |
| 90° | 0.000000 | -1.000000 | 1.000000 | 0.000000 | East-facing. x_f=East, y_f=South. |
| 120° | -0.500000 | -0.866025 | 0.866025 | -0.500000 |  |
| 135° | -0.707107 | -0.707107 | 0.707107 | -0.707107 |  |
| 180° | -1.000000 | -0.000000 | 0.000000 | -1.000000 | South-facing. x_f=South, y_f=West. |
| Full 3x3 matrix: R[2,2]=1, R[0,2]=R[1,2]=R[2,0]=R[2,1]=0 (altitude pass-through). Fixed-point: cosP, sinP from CORDIC Q1.30. P_target = P_leader + R*delta. 4 DSP48 MACs per vehicle. |  |  |  |  |  |

## 2.  Triangle Formation — Double Equilateral (6 vehicles)

Two staggered equilateral triangles. Front triangle: V1 (apex), V2 (right base), V3 (left base). Rear triangle: V4 (apex), V5 (right base), V6 (left base). Triangle height = 50×√3/2 = 43.30m.

Triangle Formation — Top View (x_f = forward, y_f = right)

+North (x_f) ^ ¦ V1 (Leader) <-- (0, 0) / \ / 50m \ / \ V3 (-28.87,-25) V2 (-28.87,+25) \ / \ 50m gap / \ / V6 (-86.60,-25) V4 (-57.74, 0) V5 (-86.60,+25) ¦ +East (y_f) --> Legend: Forward (+x_f) = direction of motion Right (+y_f) = starboard side Altitude offset z_f = 0 (coplanar)

| Triangle Formation — Offset Vectors (formation body frame) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle | x_f (m) (forward +) | y_f (m) (right +) | z_f (m) (up +) | 3D Distance from Leader (m) | Notes |
| V1 (Leader) | 0.00 | 0.00 | 0.00 | 0.00 | Formation origin (leader) |
| V2 | -28.87 | 25.00 | 0.00 | 38.19 | Right flank, 120° from V1 |
| V3 | -28.87 | -25.00 | 0.00 | 38.19 | Left flank, 120° from V1 |
| V4 | -57.74 | 0.00 | 0.00 | 57.74 | Rear apex (rear triangle) |
| V5 | -86.60 | 25.00 | 0.00 | 90.14 | Rear right flank |
| V6 | -86.60 | -25.00 | 0.00 | 90.14 | Rear left flank |

## 3.  Line Formation — Column of 6

All six vehicles in single file. V1 leads; V2 through V6 trail at 50m intervals. Total column length 250m. Zero lateral offset. Simplest formation for transit through narrow corridors.

Line Formation — Top View (single file column)

+North (direction of motion) --> V1(0) -- V2(-50) -- V3(-100) -- V4(-150) -- V5(-200) -- V6(-250) Lead Follow1 Follow2 Follow3 Follow4 Follow5 Spacing: 50m between each vehicle Total span: 250m All y_f = 0 (single-file column)

| Line Formation — Offset Vectors (formation body frame) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle | x_f (m) (forward +) | y_f (m) (right +) | z_f (m) (up +) | 3D Distance from Leader (m) | Notes |
| V1 (Leader) | 0.00 | 0.00 | 0.00 | 0.00 | Front of line |
| V2 | -50.00 | 0.00 | 0.00 | 50.00 | 50m behind V1 |
| V3 | -100.00 | 0.00 | 0.00 | 100.00 | 100m behind V1 |
| V4 | -150.00 | 0.00 | 0.00 | 150.00 | 150m behind V1 |
| V5 | -200.00 | 0.00 | 0.00 | 200.00 | 200m behind V1 |
| V6 | -250.00 | 0.00 | 0.00 | 250.00 | 250m behind V1 |

## 4.  V-Shape Formation — 60° Half-Angle

V1 leads at apex. Two arms extend rearward at ±60° from the direction of motion. Left arm: V2, V4, V6. Right arm: V3, V5. Each successive vehicle is 50m further along the arm, giving lateral spread of 86.6m and 130m for the outermost vehicles.

V-Shape Formation — Top View (60° half-angle from centreline)

V1 (0, 0) <-- Leader (apex) / \ / \ -25m,+43m V2 V3 -25m,-43m / \ / 60deg \ -50m,+87m V4 V5 -50m,-87m / / -75,+130m V6 60° half-angle from centreline Left arm: V2, V4, V6 ¦ Right arm: V3, V5 Arm spacing: 50m along arm direction

| V-Shape Formation — Offset Vectors (formation body frame) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle | x_f (m) (forward +) | y_f (m) (right +) | z_f (m) (up +) | 3D Distance from Leader (m) | Notes |
| V1 (Leader) | 0.00 | 0.00 | 0.00 | 0.00 | Formation apex (tip of V) |
| V2 | -25.00 | 43.30 | 0.00 | 50.00 | Left arm, 1st position, 60° left |
| V3 | -25.00 | -43.30 | 0.00 | 50.00 | Right arm, 1st position, 60° right |
| V4 | -50.00 | 86.60 | 0.00 | 100.00 | Left arm, 2nd position |
| V5 | -50.00 | -86.60 | 0.00 | 100.00 | Right arm, 2nd position |
| V6 | -75.00 | 129.90 | 0.00 | 150.00 | Left arm, 3rd position (rear-most) |

## 5.  Square Formation — 2×3 Rectangular Grid

Two rows of three vehicles each. Front row: V1 (center), V2 (right 50m), V3 (left 50m). Rear row: V4 (center 50m behind), V5 (rear-right), V6 (rear-left). Forms a 100m×50m rectangular coverage pattern.

Square Formation — Top View (2 rows × 3 columns, 50m spacing)

+y_f (Right/East) <-- -50 0 +50 --> V3(-50) V1(0) V2(+50) <-- Front row (x_f = 0) V6(-100) V4(-50) V5(0) <-- Rear row (x_f = -50m) Side: 50m ¦ 50m between rows Forms a 2×3 rectangular grid

| Square Formation — Offset Vectors (formation body frame) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle | x_f (m) (forward +) | y_f (m) (right +) | z_f (m) (up +) | 3D Distance from Leader (m) | Notes |
| V1 (Leader) | 0.00 | 0.00 | 0.00 | 0.00 | Front row center |
| V2 | 0.00 | 50.00 | 0.00 | 50.00 | Front row right, 50m starboard |
| V3 | 0.00 | -50.00 | 0.00 | 50.00 | Front row left, 50m port |
| V4 | -50.00 | 0.00 | 0.00 | 50.00 | Rear row center, 50m behind |
| V5 | -50.00 | 50.00 | 0.00 | 70.71 | Rear row right |
| V6 | -50.00 | -50.00 | 0.00 | 70.71 | Rear row left |

## 6.  Error Calculation Methods

Formation error is computed each 10 Hz cycle from the difference between the commanded target position (from formation geometry) and the actual EKF-estimated position of each follower vehicle.

| Error Quantity | Formula | Units / Format | Computation Notes |
| --- | --- | --- | --- |
| 3D position error for vehicle i | E_i = P_target_i - P_i where: P_target_i = P_leader + R_f2NED × delta_f_i P_i = current position from EKF | NED meters [Ex, Ey, Ez] Q10.22 (32-bit) | P_target computed each 10Hz cycle. R_f2NED×delta_f_i = 2 MACs per component (cos×x_f - sin×y_f). E_i = GPS_pos - target. 3 subtractions. |
| Per-axis error magnitude | Ex = P_target_x - P_x Ey = P_target_y - P_y Ez = P_target_z - P_z (separate for PID control) | Signed meters Q10.22 Range ±1023m | Each error term feeds directly into velocity PID as setpoint. Sign determines direction of correction. Ez used for altitude hold separately. |
| Scalar 3D distance error | d_i = sqrt(Ex² + Ey² + Ez²) | Meters (positive) Q10.22 → Q5.11 after CORDIC sqrt | Use CORDIC vectoring mode for sqrt. Or: approximate d_i ≈ ¦Ex¦+¦Ey¦+¦Ez¦ (1-norm) for quick gate check (1-norm ≤ d ≤ 1.73×1-norm). |
| Formation achieved flag | VEHICLE_IN_FORMATION_i = (¦Ex¦<thresh) AND (¦Ey¦<thresh) AND (¦Ez¦<thresh) for 3 consecutive 10Hz cycles | 1-bit Boolean Threshold: 1m (configurable) | Threshold comparison in Q10.22: 1m = 4,194,304 fp units. Each ¦E¦ is absolute value of signed Q10.22. All 3 axes must be within threshold simultaneously for 300ms (3 cycles). |
| Formation RMS error (fleet quality metric) | E_rms = sqrt(mean(d_i²)) = sqrt((1/5) × sum(d_i²) for i=2..6) | Meters Q10.22 | Computed each 10Hz cycle. Sum of 5 squared distances / 5 = mean. sqrt via CORDIC. Reported in telemetry. Threshold E_rms < 2m for FORMATION_ACHIEVED flag. |
| Per-pair separation error | Sep_ij = ¦P_i - P_j¦ (for collision avoidance check) All 15 pairs monitored | Meters Q10.22 → scalar | Used by collision avoidance (not formation controller). If Sep_ij < SAFETY_RADIUS, formation error overridden by APF repulsion. 15 pair distances computed in parallel. |
| Angular bearing error (for yaw alignment) | beta_err_i = atan2(Ey_i, Ex_i) - psi_des_i where psi_des_i = psi_L (follow leader heading) | Radians Q3.29 or Q1.15 | CORDIC vectoring for atan2. Bearing error feeds yaw PID. Wrap result to [-π,+π]. For formation: psi_des_i = psi_L (all vehicles face same direction as leader). |
| Velocity error (feed-forward damping) | V_err_i = V_target_i - V_i V_target_i = V_leader + K_ff × E_i (proportional feed-forward) | m/s Q4.28 | Feed-forward: add proportional error velocity command to leader velocity. K_ff (0.5 s^{-1} default) reduces overshoot during formation transitions. |

## 7.  Formation Transition — Interpolation Strategies

When switching formation type, all follower target offsets are linearly interpolated from old to new geometry over a configurable number of 10 Hz cycles. Transition duration adapts to maximum vehicle displacement.

| Transition | Start Offsets | End Offsets | Interpolation & Timing Notes |
| --- | --- | --- | --- |
| Triangle → Line | V2: (-28.87, +25, 0) V3: (-28.87, -25, 0) V4: (-57.74, 0, 0) V5: (-86.60, +25, 0) V6: (-86.60, -25, 0) | V2: (-50, 0, 0) V3: (-100, 0, 0) V4: (-150, 0, 0) V5: (-200, 0, 0) V6: (-250, 0, 0) | Linear interpolation over 20 cycles (2s). Per cycle: delta = (end-start)/20. Apply each 10Hz CE. Lock formation flag during transition. Max simultaneous movement: V6 moves ~165m over 2s = 82.5 m/s needed — clamp speed to V_MAX and extend transition time if needed. |
| Line → V-shape | V2: (-50, 0, 0) V3: (-100, 0, 0) V4: (-150, 0, 0) V5: (-200, 0, 0) V6: (-250, 0, 0) | V2: (-25, +43.3, 0) V3: (-25, -43.3, 0) V4: (-50, +86.6, 0) V5: (-50, -86.6, 0) V6: (-75, +130, 0) | V6 has largest displacement (~183m lateral). At 5 m/s max correction, needs 36s. Default TRANSITION_TIME=40 cycles (4s) at reduced speed. Scale if slow: TRANSITION_TIME = max_delta / V_MAX_formation. |
| V-shape → Square | V2: (-25, +43.3, 0) ... V6: (-75, +130, 0) | V2: (0, +50, 0) V3: (0, -50, 0) V4: (-50, 0, 0) V5: (-50, +50, 0) V6: (-50, -50, 0) | V6 moves from (+130, -75) region to (-50, -50) — ~190m. Same speed-limited transition. TRANSITION_ACTIVE inhibits FORMATION_ACHIEVED flag. Formation controller sends velocity corrections continuously during interpolation. |
| Square → Triangle | V2: (0, +50, 0) ... V6: (-50, -50, 0) | V2: (-28.87, +25, 0) ... V6: (-86.60, -25, 0) | Modest movement. All vehicles within 75m of final positions. Transition feasible in 2s at 5 m/s. Smooth: no abrupt position jumps. Largest mover: V6 from (-50,-50) to (-86.60,-25) = ~44m. |
| Any → Loiter (in-place hold) | Current formation offsets (any type) | Capture current EKF positions as targets (no offset from current) | LOITER command: set all delta_f_i = 0 (or preserve current NED positions). No interpolation. Vehicles hold position from LOITER entry. Formation controller uses absolute NED position hold, not offset-from-leader. |

## 8.  Verilog RTL — Formation Controller

Computes all 5 follower target positions each 10 Hz CE cycle. Formation offsets stored in distributed LUT-ROM (4 types × 5 vehicles × 3 axes × 32 bits = 1920 bytes). CORDIC computes rotation matrix. 4 DSP48 MACs for R×δ multiply.

formation_controller.v — Target Position Computation (4 DSP48, 10Hz)

// ================================================================ // Formation Controller: compute target positions for all 5 followers // Input: leader position P_L, heading psi_L, formation type (2-bit) // Output: P_target[1..5] = target NED positions for V2..V6 // Clock: 10 Hz CE strobe // Fixed-point: positions Q10.22, angles Q3.29, cos/sin Q1.30 // ================================================================ module formation_controller ( input wire clk, rst_n, en_10hz, input wire signed [31:0] P_L_x,P_L_y,P_L_z, // leader NED Q10.22 input wire signed [31:0] psi_L, // leader heading Q3.29 input wire [1:0] form_type, // 00=TRIANGLE,01=LINE,10=V,11=SQUARE // Per-follower target positions (NED, Q10.22) output reg signed [31:0] Pt_x[1:5],Pt_y[1:5],Pt_z[1:5] ); // ── Formation offset LUT (in Q10.22 = meters × 4194304) ───────────── // [formation_type][vehicle_index 1..5][axis 0=x,1=y,2=z] // All values in meters, stored as Q10.22 fixed-point // D = 50m = 50 × 4194304 = 209715200 // D×cos60 = 25m = 104857600 // D×sin60 = 43.30m = 181552620 // tri_h/1.5 = 28.87m = 121073756 wire signed [31:0] delta_lut[0:3][0:4][0:2] = '{ // TRIANGLE [type=0]: [v_idx 0..4][x,y,z] '{// V2: '{-32'sd121073756, +32'sd104857600,32'sd0}, // (-28.87m, +25m, 0) {-32'sd121073756, -32'sd104857600,32'sd0}, // V3 {-32'sd242147512,32'sd0,32'sd0}, // V4 (-57.74m) {-32'sd363221268, +32'sd104857600,32'sd0}, // V5 {-32'sd363221268, -32'sd104857600,32'sd0} // V6 }, // LINE [type=1]: V2..-V6 at -50,-100,-150,-200,-250m // VSHAPE [type=2], SQUARE [type=3]: (similar structure, abbreviated) ... // (see full LUT in formation_lut.mif BRAM init file) }; // ── CORDIC sin/cos for leader heading ───────────────────────────────── wire signed [31:0] cos_psi,sin_psi; // Q1.30 from CORDIC cordic_sincos u_cordic (.clk(clk),.theta_in(psi_L[29:14]), // Q3.29→Q1.15 .sin_out(sin_psi[15:0]),.cos_out(cos_psi[15:0])); // ── For each follower: rotate offset by R_f2NED, add leader pos ──────── integer v; reg signed [31:0] dx_f,dy_f; // formation-frame offsets wire signed [63:0] dx_n,dy_n; // NED offsets (before shift) // NED offset: dx_n = cosψ×dx_f - sinψ×dy_f (Q1.30 × Q10.22 → Q11.52 → >>30 → Q10.22) assign dx_n = ($signed(cos_psi)*$signed(dx_f)) - ($signed(sin_psi)*$signed(dy_f)); assign dy_n = ($signed(sin_psi)*$signed(dx_f)) + ($signed(cos_psi)*$signed(dy_f)); always @(posedge clk) begin if (!rst_n) begin : reset_targets integer vi; for (vi=1; vi<=5; vi=vi+1) beginPt_x[vi]<=0; Pt_y[vi]<=0; Pt_z[vi]<=0; end end else if (en_10hz) begin for (v=1; v<=5; v=v+1) begin dx_f = delta_lut[form_type][v-1][0]; // x formation offset dy_f = delta_lut[form_type][v-1][1]; // y formation offset // Rotate and translate: P_target = P_leader + R×delta Pt_x[v] <= P_L_x + dx_n[51:30]; // >>30: Q11.52→Q10.22(approx) Pt_y[v] <= P_L_y + dy_n[51:30]; Pt_z[v] <= P_L_z + delta_lut[form_type][v-1][2]; // z passes through end end end endmodule

## 9.  Verilog RTL — Formation Error Calculator

Per-vehicle error with persistence detection. Computes Ex, Ey, Ez for all 5 followers, checks 1m threshold per axis, requires 3 consecutive in-window cycles before asserting VEHICLE_IN_FORMATION flag.

formation_error.v — Error Computation & Formation Flags (0 DSP48)

// Formation Error Calculator // Computes per-vehicle error E_i = P_target_i - P_i // Flags: VEHICLE_IN_FORMATION, FORMATION_ACHIEVED, RMS error module formation_error ( input wire clk, rst_n, en_10hz, // Target positions (from formation_controller) input wire signed [31:0] Pt_x[1:5],Pt_y[1:5],Pt_z[1:5], // Actual positions (from Multi-Vehicle Kalman Filter) input wire signed [31:0] Pv_x[1:5],Pv_y[1:5],Pv_z[1:5], // Error outputs output reg signed [31:0] Ex[1:5],Ey[1:5],Ez[1:5], output reg [4:0] in_formation, // bit per vehicle output reg formation_achieved, output reg signed [31:0] rms_error ); // 1m threshold in Q10.22: 1.0 × 4194304 = 4194304 = 0x400000 localparam [31:0] THRESH = 32'd4194304; // 1.0m in Q10.22 // Persistence counter: must be in formation for 3 consecutive cycles reg [1:0] persist_cnt[1:5]; integer v; wire signed [31:0] ex_wire[1:5],ey_wire[1:5],ez_wire[1:5]; genvar vi; generate for (vi=1; vi<=5; vi=vi+1) begin assign ex_wire[vi] = Pt_x[vi] - Pv_x[vi]; // Q10.22 subtraction assign ey_wire[vi] = Pt_y[vi] - Pv_y[vi]; assign ez_wire[vi] = Pt_z[vi] - Pv_z[vi]; end endgenerate always @(posedge clk) begin if (!rst_n) begin for (v=1;v<=5;v=v+1) beginEx[v]<=0;Ey[v]<=0;Ez[v]<=0;persist_cnt[v]<=0; end in_formation<=0; formation_achieved<=0; rms_error<=0; end else if (en_10hz) begin for (v=1; v<=5; v=v+1) begin Ex[v] <= ex_wire[v]; Ey[v] <= ey_wire[v]; Ez[v] <= ez_wire[v]; // Check per-axis threshold ¦E¦ < 1m wire in_x = ($signed(ex_wire[v]) < $signed(32'sd4194304)) && ($signed(ex_wire[v]) > -$signed(32'sd4194304)); wire in_y = ($signed(ey_wire[v]) < $signed(32'sd4194304)) && ($signed(ey_wire[v]) > -$signed(32'sd4194304)); wire in_z = ($signed(ez_wire[v]) < $signed(32'sd4194304)) && ($signed(ez_wire[v]) > -$signed(32'sd4194304)); // Persistence: 3 consecutive cycles in formation if (in_x && in_y && in_z) begin persist_cnt[v] <= (persist_cnt[v] < 2) ? persist_cnt[v]+1 : 2; in_formation[v-1] <= (persist_cnt[v] == 2); end else begin persist_cnt[v] <= 0; in_formation[v-1] <= 0; end end // FORMATION_ACHIEVED = all 5 followers in formation formation_achieved <= (in_formation == 5'b11111); end end endmodule

## 10.  Summary — Fixed-Point Resources

| Module/Step | DSP48 | Cycles (10Hz) | Notes |
| --- | --- | --- | --- |
| formation_controller (R_f2NED × δ_f per vehicle) | 4 | ~30 | CORDIC 17 cycles + 2 MACs×5 vehicles. Parallel: 1 CORDIC, 4 DSP time-multiplexed. 5×4=20 MAC ops. Q1.30×Q10.22 product Q11.52 → shift 30 → Q10.22. |
| formation_error (E_i = P_target - P_actual) | 0 | ~5 | 5 vehicles × 3 subtractions = 15 Q10.22 subtracts in fabric. Pure LUT arithmetic. |
| Threshold check (¦E¦ < 1m per axis) | 0 | ~3 | 15 comparisons (5 vehicles × 3 axes). Signed 32-bit compare. Persistent counter (3 cycles). No DSP needed. |
| FORMATION_ACHIEVED flag | 0 | ~1 | AND of 5 VEHICLE_IN_FORMATION bits. Single logic level. |
| Formation transition (linear interpolation) | 3 | ~30 | delta = (end-start)/N: 5 vehicles × 3 axes × 1 DSP multiply/cycle = 45 MACs. Shared 3 DSP48. |
| RMS error (sqrt of mean squared error) | 2 | ~20 | 5 square-distance terms + CORDIC sqrt. 2 DSP for squaring, CORDIC for sqrt. |
| TOTAL | 6 DSP48 | ~89 cycles | 89 × 20ns = 1.78µs. Budget: 100ms (10Hz). Margin: 56,000×. |