# FPGA I/O Signal Interface Specification

Xilinx Artix-7 XC7A35T (CPG236) — Multi-Vehicle CubeSat & UAV Integration

3 CubeSats  •  3 UAVs  •  Shared Convergence Logic  |  Banks 14, 15, 34, 35

## 1. Complete I/O Signal Interface Table

| Signal_Name | Dir | Width | IO_Standard | Bank | Pin_Suggest | Peripheral | Rate_Hz | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SHARED / TOP-LEVEL SIGNALS |  |  |  |  |  |  |  |  |
| sys_clk_50mhz | I | 1 | LVCMOS33 | 35 | E3 | External OSC 50 MHz | 50M | Main clock; dedicated GCLK pin |
| sys_rst_n | I | 1 | LVCMOS33 | 35 | C2 | Reset button | - | Active-low global reset |
| shared_dbg_led_0 | O | 1 | LVCMOS33 | 35 | A1 | Status LED 0 | - | General-purpose debug LED |
| shared_dbg_led_1 | O | 1 | LVCMOS33 | 35 | B1 | Status LED 1 | - | General-purpose debug LED |
| shared_dbg_led_2 | O | 1 | LVCMOS33 | 35 | A2 | Status LED 2 | - | General-purpose debug LED |
| shared_dbg_led_3 | O | 1 | LVCMOS33 | 35 | B2 | Status LED 3 | - | General-purpose debug LED |
| shared_dbg_led_4 | O | 1 | LVCMOS33 | 35 | D1 | Status LED 4 | - | General-purpose debug LED |
| shared_dbg_led_5 | O | 1 | LVCMOS33 | 35 | E1 | Status LED 5 | - | General-purpose debug LED |
| shared_dbg_led_6 | O | 1 | LVCMOS33 | 35 | F2 | Status LED 6 | - | General-purpose debug LED |
| shared_dbg_led_7 | O | 1 | LVCMOS33 | 35 | F1 | Status LED 7 | - | General-purpose debug LED |
| shared_tp_0 | IO | 1 | LVCMOS33 | 35 | G1 | Test point GPIO | - | Oscilloscope probe point 0 |
| shared_tp_1 | IO | 1 | LVCMOS33 | 35 | G2 | Test point GPIO | - | Oscilloscope probe point 1 |
| shared_tp_2 | IO | 1 | LVCMOS33 | 35 | H1 | Test point GPIO | - | Oscilloscope probe point 2 |
| shared_tp_3 | IO | 1 | LVCMOS33 | 35 | H2 | Test point GPIO | - | Oscilloscope probe point 3 |
| shared_gnd_uart_tx | O | 1 | LVCMOS33 | 35 | J1 | Ground station UART arb | 115200 | Primary GS downlink via arbiter |
| shared_gnd_uart_rx | I | 1 | LVCMOS33 | 35 | J2 | Ground station UART arb | 115200 | Primary GS uplink via arbiter |
| CUBESAT 1 (CS1) — Bank 34 |  |  |  |  |  |  |  |  |
| cs1_imu_sclk | O | 1 | LVCMOS33 | 34 | H4 | MPU-9250 SPI | 10M | CS1 IMU SPI clock; shared with sun sensor ADC |
| cs1_imu_mosi | O | 1 | LVCMOS33 | 34 | H3 | MPU-9250 SPI | 10M | CS1 IMU SPI MOSI |
| cs1_imu_miso | I | 1 | LVCMOS33 | 34 | J4 | MPU-9250 SPI | 10M | CS1 IMU SPI MISO |
| cs1_imu_cs_n | O | 1 | LVCMOS33 | 34 | J3 | MPU-9250 SPI CS | - | Active-low chip select for CS1 IMU |
| cs1_sun_adc_cs_n | O | 1 | LVCMOS33 | 34 | K3 | Sun Sensor ADC CS | - | Active-low CS; shares cs1_imu SPI bus |
| cs1_mag_scl | IO | 1 | LVCMOS33 | 34 | K4 | Magnetometer I2C | 400k | Open-drain; requires 2.2k pull-up to 3.3V |
| cs1_mag_sda | IO | 1 | LVCMOS33 | 34 | L3 | Magnetometer I2C | 400k | Open-drain; requires 2.2k pull-up to 3.3V |
| cs1_rw0_cs_n | O | 1 | LVCMOS33 | 34 | L4 | RW0 Motor Driver SPI | - | CS1 reaction wheel 0; shares cs1_imu SPI bus |
| cs1_rw1_cs_n | O | 1 | LVCMOS33 | 34 | M3 | RW1 Motor Driver SPI | - | CS1 reaction wheel 1; shares cs1_imu SPI bus |
| cs1_rw2_cs_n | O | 1 | LVCMOS33 | 34 | M4 | RW2 Motor Driver SPI | - | CS1 reaction wheel 2; shares cs1_imu SPI bus |
| cs1_mtq_pwm_0 | O | 1 | LVCMOS33 | 34 | N3 | Magnetorquer X-axis | 10k | CS1 X-axis magnetorquer PWM drive |
| cs1_mtq_pwm_1 | O | 1 | LVCMOS33 | 34 | N4 | Magnetorquer Y-axis | 10k | CS1 Y-axis magnetorquer PWM drive |
| cs1_mtq_pwm_2 | O | 1 | LVCMOS33 | 34 | P3 | Magnetorquer Z-axis | 10k | CS1 Z-axis magnetorquer PWM drive |
| cs1_gimbal_step_az | O | 1 | LVCMOS33 | 34 | P4 | Laser Gimbal Stepper | 200k | CS1 azimuth stepper step pulse |
| cs1_gimbal_dir_az | O | 1 | LVCMOS33 | 34 | R3 | Laser Gimbal Stepper | - | CS1 azimuth stepper direction |
| cs1_gimbal_step_el | O | 1 | LVCMOS33 | 34 | R4 | Laser Gimbal Stepper | 200k | CS1 elevation stepper step pulse |
| cs1_gimbal_dir_el | O | 1 | LVCMOS33 | 34 | T3 | Laser Gimbal Stepper | - | CS1 elevation stepper direction |
| cs1_tlm_uart_tx | O | 1 | LVCMOS33 | 34 | T4 | CS1 Telemetry UART | 9600 | CS1 telemetry downlink transmit |
| cs1_isl_data_0 | O | 1 | LVCMOS33 | 34 | U4 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 0 |
| cs1_isl_data_1 | O | 1 | LVCMOS33 | 34 | U3 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 1 |
| cs1_isl_data_2 | O | 1 | LVCMOS33 | 34 | V4 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 2 |
| cs1_isl_data_3 | O | 1 | LVCMOS33 | 34 | V3 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 3 |
| cs1_isl_data_4 | O | 1 | LVCMOS33 | 34 | W4 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 4 |
| cs1_isl_data_5 | O | 1 | LVCMOS33 | 34 | W3 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 5 |
| cs1_isl_data_6 | O | 1 | LVCMOS33 | 34 | Y4 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 6 |
| cs1_isl_data_7 | O | 1 | LVCMOS33 | 34 | Y3 | Inter-Sat Optical Link | 100M | CS1 optical link data bit 7 |
| cs1_isl_valid | O | 1 | LVCMOS33 | 34 | AA4 | Inter-Sat Optical Link | 100M | CS1 optical link data valid strobe |
| CUBESAT 2 (CS2) — Bank 34 |  |  |  |  |  |  |  |  |
| cs2_imu_sclk | O | 1 | LVCMOS33 | 34 | AB4 | MPU-9250 SPI | 10M | CS2 IMU SPI clock; shared with sun sensor ADC |
| cs2_imu_mosi | O | 1 | LVCMOS33 | 34 | AB3 | MPU-9250 SPI | 10M | CS2 IMU SPI MOSI |
| cs2_imu_miso | I | 1 | LVCMOS33 | 34 | AA3 | MPU-9250 SPI | 10M | CS2 IMU SPI MISO |
| cs2_imu_cs_n | O | 1 | LVCMOS33 | 34 | Y2 | MPU-9250 SPI CS | - | Active-low chip select for CS2 IMU |
| cs2_sun_adc_cs_n | O | 1 | LVCMOS33 | 34 | W2 | Sun Sensor ADC CS | - | Active-low CS; shares cs2_imu SPI bus |
| cs2_mag_scl | IO | 1 | LVCMOS33 | 34 | V2 | Magnetometer I2C | 400k | Open-drain; requires 2.2k pull-up to 3.3V |
| cs2_mag_sda | IO | 1 | LVCMOS33 | 34 | U2 | Magnetometer I2C | 400k | Open-drain; requires 2.2k pull-up to 3.3V |
| cs2_rw0_cs_n | O | 1 | LVCMOS33 | 34 | T2 | RW0 Motor Driver SPI | - | CS2 reaction wheel 0; shares cs2_imu SPI bus |
| cs2_rw1_cs_n | O | 1 | LVCMOS33 | 34 | R2 | RW1 Motor Driver SPI | - | CS2 reaction wheel 1; shares cs2_imu SPI bus |
| cs2_rw2_cs_n | O | 1 | LVCMOS33 | 34 | P2 | RW2 Motor Driver SPI | - | CS2 reaction wheel 2; shares cs2_imu SPI bus |
| cs2_mtq_pwm_0 | O | 1 | LVCMOS33 | 34 | N2 | Magnetorquer X-axis | 10k | CS2 X-axis magnetorquer PWM drive |
| cs2_mtq_pwm_1 | O | 1 | LVCMOS33 | 34 | M2 | Magnetorquer Y-axis | 10k | CS2 Y-axis magnetorquer PWM drive |
| cs2_mtq_pwm_2 | O | 1 | LVCMOS33 | 34 | L2 | Magnetorquer Z-axis | 10k | CS2 Z-axis magnetorquer PWM drive |
| cs2_gimbal_step_az | O | 1 | LVCMOS33 | 34 | K2 | Laser Gimbal Stepper | 200k | CS2 azimuth stepper step pulse |
| cs2_gimbal_dir_az | O | 1 | LVCMOS33 | 34 | J2 | Laser Gimbal Stepper | - | CS2 azimuth stepper direction |
| cs2_gimbal_step_el | O | 1 | LVCMOS33 | 34 | H5 | Laser Gimbal Stepper | 200k | CS2 elevation stepper step pulse |
| cs2_gimbal_dir_el | O | 1 | LVCMOS33 | 34 | G5 | Laser Gimbal Stepper | - | CS2 elevation stepper direction |
| cs2_tlm_uart_tx | O | 1 | LVCMOS33 | 34 | F5 | CS2 Telemetry UART | 9600 | CS2 telemetry downlink transmit |
| cs2_isl_data_0 | O | 1 | LVCMOS33 | 34 | E5 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 0 |
| cs2_isl_data_1 | O | 1 | LVCMOS33 | 34 | D5 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 1 |
| cs2_isl_data_2 | O | 1 | LVCMOS33 | 34 | C5 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 2 |
| cs2_isl_data_3 | O | 1 | LVCMOS33 | 34 | B5 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 3 |
| cs2_isl_data_4 | O | 1 | LVCMOS33 | 34 | A5 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 4 |
| cs2_isl_data_5 | O | 1 | LVCMOS33 | 34 | H6 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 5 |
| cs2_isl_data_6 | O | 1 | LVCMOS33 | 34 | G6 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 6 |
| cs2_isl_data_7 | O | 1 | LVCMOS33 | 34 | F6 | Inter-Sat Optical Link | 100M | CS2 optical link data bit 7 |
| cs2_isl_valid | O | 1 | LVCMOS33 | 34 | E6 | Inter-Sat Optical Link | 100M | CS2 optical link data valid strobe |
| CUBESAT 3 (CS3) — Bank 15 |  |  |  |  |  |  |  |  |
| cs3_imu_sclk | O | 1 | LVCMOS33 | 15 | D8 | MPU-9250 SPI | 10M | CS3 IMU SPI clock; shared with sun sensor ADC |
| cs3_imu_mosi | O | 1 | LVCMOS33 | 15 | E8 | MPU-9250 SPI | 10M | CS3 IMU SPI MOSI |
| cs3_imu_miso | I | 1 | LVCMOS33 | 15 | F8 | MPU-9250 SPI | 10M | CS3 IMU SPI MISO |
| cs3_imu_cs_n | O | 1 | LVCMOS33 | 15 | G8 | MPU-9250 SPI CS | - | Active-low chip select for CS3 IMU |
| cs3_sun_adc_cs_n | O | 1 | LVCMOS33 | 15 | H8 | Sun Sensor ADC CS | - | Active-low CS; shares cs3_imu SPI bus |
| cs3_mag_scl | IO | 1 | LVCMOS33 | 15 | J8 | Magnetometer I2C | 400k | Open-drain; requires 2.2k pull-up to 3.3V |
| cs3_mag_sda | IO | 1 | LVCMOS33 | 15 | K8 | Magnetometer I2C | 400k | Open-drain; requires 2.2k pull-up to 3.3V |
| cs3_rw0_cs_n | O | 1 | LVCMOS33 | 15 | L8 | RW0 Motor Driver SPI | - | CS3 reaction wheel 0; shares cs3_imu SPI bus |
| cs3_rw1_cs_n | O | 1 | LVCMOS33 | 15 | M8 | RW1 Motor Driver SPI | - | CS3 reaction wheel 1; shares cs3_imu SPI bus |
| cs3_rw2_cs_n | O | 1 | LVCMOS33 | 15 | N8 | RW2 Motor Driver SPI | - | CS3 reaction wheel 2; shares cs3_imu SPI bus |
| cs3_mtq_pwm_0 | O | 1 | LVCMOS33 | 15 | P8 | Magnetorquer X-axis | 10k | CS3 X-axis magnetorquer PWM drive |
| cs3_mtq_pwm_1 | O | 1 | LVCMOS33 | 15 | R8 | Magnetorquer Y-axis | 10k | CS3 Y-axis magnetorquer PWM drive |
| cs3_mtq_pwm_2 | O | 1 | LVCMOS33 | 15 | T8 | Magnetorquer Z-axis | 10k | CS3 Z-axis magnetorquer PWM drive |
| cs3_gimbal_step_az | O | 1 | LVCMOS33 | 15 | U8 | Laser Gimbal Stepper | 200k | CS3 azimuth stepper step pulse |
| cs3_gimbal_dir_az | O | 1 | LVCMOS33 | 15 | V8 | Laser Gimbal Stepper | - | CS3 azimuth stepper direction |
| cs3_gimbal_step_el | O | 1 | LVCMOS33 | 15 | W8 | Laser Gimbal Stepper | 200k | CS3 elevation stepper step pulse |
| cs3_gimbal_dir_el | O | 1 | LVCMOS33 | 15 | Y8 | Laser Gimbal Stepper | - | CS3 elevation stepper direction |
| cs3_tlm_uart_tx | O | 1 | LVCMOS33 | 15 | AA8 | CS3 Telemetry UART | 9600 | CS3 telemetry downlink transmit |
| cs3_isl_data_0 | O | 1 | LVCMOS33 | 15 | AB8 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 0 |
| cs3_isl_data_1 | O | 1 | LVCMOS33 | 15 | D9 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 1 |
| cs3_isl_data_2 | O | 1 | LVCMOS33 | 15 | E9 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 2 |
| cs3_isl_data_3 | O | 1 | LVCMOS33 | 15 | F9 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 3 |
| cs3_isl_data_4 | O | 1 | LVCMOS33 | 15 | G9 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 4 |
| cs3_isl_data_5 | O | 1 | LVCMOS33 | 15 | H9 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 5 |
| cs3_isl_data_6 | O | 1 | LVCMOS33 | 15 | J9 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 6 |
| cs3_isl_data_7 | O | 1 | LVCMOS33 | 15 | K9 | Inter-Sat Optical Link | 100M | CS3 optical link data bit 7 |
| cs3_isl_valid | O | 1 | LVCMOS33 | 15 | L9 | Inter-Sat Optical Link | 100M | CS3 optical link data valid strobe |
| UAV 1 (UAV1) — Bank 14 |  |  |  |  |  |  |  |  |
| uav1_imu_sclk | O | 1 | LVCMOS33 | 14 | D14 | ICM-42688 SPI | 10M | UAV1 IMU SPI clock |
| uav1_imu_mosi | O | 1 | LVCMOS33 | 14 | E14 | ICM-42688 SPI | 10M | UAV1 IMU SPI MOSI |
| uav1_imu_miso | I | 1 | LVCMOS33 | 14 | F14 | ICM-42688 SPI | 10M | UAV1 IMU SPI MISO |
| uav1_imu_cs_n | O | 1 | LVCMOS33 | 14 | G14 | ICM-42688 SPI CS | - | Active-low chip select for UAV1 IMU |
| uav1_gps_uart_rx | I | 1 | LVCMOS33 | 14 | H14 | NEO-M9N GPS | 9600 | UAV1 GPS NMEA sentence receive |
| uav1_baro_scl | IO | 1 | LVCMOS33 | 14 | J14 | BMP388 I2C | 400k | UAV1 barometer I2C clock; open-drain 3.3V |
| uav1_baro_sda | IO | 1 | LVCMOS33 | 14 | K14 | BMP388 I2C | 400k | UAV1 barometer I2C data; open-drain 3.3V |
| uav1_tof_echo | I | 1 | LVCMOS33 | 14 | L14 | ToF Ranging Sensor | 100 | UAV1 time-of-flight echo pulse input (opt.) |
| uav1_motor_pwm_0 | O | 1 | LVCMOS33 | 14 | M14 | ESC Motor 0 | 400 | UAV1 motor ESC 0 PWM (50-400 Hz) |
| uav1_motor_pwm_1 | O | 1 | LVCMOS33 | 14 | N14 | ESC Motor 1 | 400 | UAV1 motor ESC 1 PWM (50-400 Hz) |
| uav1_motor_pwm_2 | O | 1 | LVCMOS33 | 14 | P14 | ESC Motor 2 | 400 | UAV1 motor ESC 2 PWM (50-400 Hz) |
| uav1_motor_pwm_3 | O | 1 | LVCMOS33 | 14 | R14 | ESC Motor 3 | 400 | UAV1 motor ESC 3 PWM (50-400 Hz) |
| uav1_servo_pwm_0 | O | 1 | LVCMOS33 | 14 | T14 | Camera Gimbal Servo 0 | 50 | UAV1 camera gimbal servo 0 PWM |
| uav1_servo_pwm_1 | O | 1 | LVCMOS33 | 14 | U14 | Camera Gimbal Servo 1 | 50 | UAV1 camera gimbal servo 1 PWM |
| uav1_gs_uart_tx | O | 1 | LVCMOS33 | 14 | V14 | Ground Station MAVLink | 57600 | UAV1 MAVLink ground station transmit |
| uav1_gs_uart_rx | I | 1 | LVCMOS33 | 14 | W14 | Ground Station MAVLink | 57600 | UAV1 MAVLink ground station receive |
| uav1_iuav_uart_tx | O | 1 | LVCMOS33 | 14 | Y14 | Inter-UAV UART | 57600 | UAV1 inter-UAV comms transmit |
| uav1_iuav_uart_rx | I | 1 | LVCMOS33 | 14 | AA14 | Inter-UAV UART | 57600 | UAV1 inter-UAV comms receive |
| UAV 2 (UAV2) — Bank 14 |  |  |  |  |  |  |  |  |
| uav2_imu_sclk | O | 1 | LVCMOS33 | 14 | AB14 | ICM-42688 SPI | 10M | UAV2 IMU SPI clock |
| uav2_imu_mosi | O | 1 | LVCMOS33 | 14 | D15 | ICM-42688 SPI | 10M | UAV2 IMU SPI MOSI |
| uav2_imu_miso | I | 1 | LVCMOS33 | 14 | E15 | ICM-42688 SPI | 10M | UAV2 IMU SPI MISO |
| uav2_imu_cs_n | O | 1 | LVCMOS33 | 14 | F15 | ICM-42688 SPI CS | - | Active-low chip select for UAV2 IMU |
| uav2_gps_uart_rx | I | 1 | LVCMOS33 | 14 | G15 | NEO-M9N GPS | 9600 | UAV2 GPS NMEA sentence receive |
| uav2_baro_scl | IO | 1 | LVCMOS33 | 14 | H15 | BMP388 I2C | 400k | UAV2 barometer I2C clock; open-drain 3.3V |
| uav2_baro_sda | IO | 1 | LVCMOS33 | 14 | J15 | BMP388 I2C | 400k | UAV2 barometer I2C data; open-drain 3.3V |
| uav2_tof_echo | I | 1 | LVCMOS33 | 14 | K15 | ToF Ranging Sensor | 100 | UAV2 time-of-flight echo pulse input (opt.) |
| uav2_motor_pwm_0 | O | 1 | LVCMOS33 | 14 | L15 | ESC Motor 0 | 400 | UAV2 motor ESC 0 PWM (50-400 Hz) |
| uav2_motor_pwm_1 | O | 1 | LVCMOS33 | 14 | M15 | ESC Motor 1 | 400 | UAV2 motor ESC 1 PWM (50-400 Hz) |
| uav2_motor_pwm_2 | O | 1 | LVCMOS33 | 14 | N15 | ESC Motor 2 | 400 | UAV2 motor ESC 2 PWM (50-400 Hz) |
| uav2_motor_pwm_3 | O | 1 | LVCMOS33 | 14 | P15 | ESC Motor 3 | 400 | UAV2 motor ESC 3 PWM (50-400 Hz) |
| uav2_servo_pwm_0 | O | 1 | LVCMOS33 | 14 | R15 | Camera Gimbal Servo 0 | 50 | UAV2 camera gimbal servo 0 PWM |
| uav2_servo_pwm_1 | O | 1 | LVCMOS33 | 14 | T15 | Camera Gimbal Servo 1 | 50 | UAV2 camera gimbal servo 1 PWM |
| uav2_gs_uart_tx | O | 1 | LVCMOS33 | 14 | U15 | Ground Station MAVLink | 57600 | UAV2 MAVLink ground station transmit |
| uav2_gs_uart_rx | I | 1 | LVCMOS33 | 14 | V15 | Ground Station MAVLink | 57600 | UAV2 MAVLink ground station receive |
| uav2_iuav_uart_tx | O | 1 | LVCMOS33 | 14 | W15 | Inter-UAV UART | 57600 | UAV2 inter-UAV comms transmit |
| uav2_iuav_uart_rx | I | 1 | LVCMOS33 | 14 | Y15 | Inter-UAV UART | 57600 | UAV2 inter-UAV comms receive |
| UAV 3 (UAV3) — Bank 14 |  |  |  |  |  |  |  |  |
| uav3_imu_sclk | O | 1 | LVCMOS33 | 14 | AA15 | ICM-42688 SPI | 10M | UAV3 IMU SPI clock |
| uav3_imu_mosi | O | 1 | LVCMOS33 | 14 | AB15 | ICM-42688 SPI | 10M | UAV3 IMU SPI MOSI |
| uav3_imu_miso | I | 1 | LVCMOS33 | 14 | D16 | ICM-42688 SPI | 10M | UAV3 IMU SPI MISO |
| uav3_imu_cs_n | O | 1 | LVCMOS33 | 14 | E16 | ICM-42688 SPI CS | - | Active-low chip select for UAV3 IMU |
| uav3_gps_uart_rx | I | 1 | LVCMOS33 | 14 | F16 | NEO-M9N GPS | 9600 | UAV3 GPS NMEA sentence receive |
| uav3_baro_scl | IO | 1 | LVCMOS33 | 14 | G16 | BMP388 I2C | 400k | UAV3 barometer I2C clock; open-drain 3.3V |
| uav3_baro_sda | IO | 1 | LVCMOS33 | 14 | H16 | BMP388 I2C | 400k | UAV3 barometer I2C data; open-drain 3.3V |
| uav3_tof_echo | I | 1 | LVCMOS33 | 14 | J16 | ToF Ranging Sensor | 100 | UAV3 time-of-flight echo pulse input (opt.) |
| uav3_motor_pwm_0 | O | 1 | LVCMOS33 | 14 | K16 | ESC Motor 0 | 400 | UAV3 motor ESC 0 PWM (50-400 Hz) |
| uav3_motor_pwm_1 | O | 1 | LVCMOS33 | 14 | L16 | ESC Motor 1 | 400 | UAV3 motor ESC 1 PWM (50-400 Hz) |
| uav3_motor_pwm_2 | O | 1 | LVCMOS33 | 14 | M16 | ESC Motor 2 | 400 | UAV3 motor ESC 2 PWM (50-400 Hz) |
| uav3_motor_pwm_3 | O | 1 | LVCMOS33 | 14 | N16 | ESC Motor 3 | 400 | UAV3 motor ESC 3 PWM (50-400 Hz) |
| uav3_servo_pwm_0 | O | 1 | LVCMOS33 | 14 | P16 | Camera Gimbal Servo 0 | 50 | UAV3 camera gimbal servo 0 PWM |
| uav3_servo_pwm_1 | O | 1 | LVCMOS33 | 14 | R16 | Camera Gimbal Servo 1 | 50 | UAV3 camera gimbal servo 1 PWM |
| uav3_gs_uart_tx | O | 1 | LVCMOS33 | 14 | T16 | Ground Station MAVLink | 57600 | UAV3 MAVLink ground station transmit |
| uav3_gs_uart_rx | I | 1 | LVCMOS33 | 14 | U16 | Ground Station MAVLink | 57600 | UAV3 MAVLink ground station receive |
| uav3_iuav_uart_tx | O | 1 | LVCMOS33 | 14 | V16 | Inter-UAV UART | 57600 | UAV3 inter-UAV comms transmit |
| uav3_iuav_uart_rx | I | 1 | LVCMOS33 | 14 | W16 | Inter-UAV UART | 57600 | UAV3 inter-UAV comms receive |

## 2. Summary

### 2.1  Signal Direction Counts

| Direction | Count |
| --- | --- |
| Input (I) | 21 |
| Output (O) | 114 |
| Bidirectional (IO) | 16 |
| TOTAL Physical Pins | 151 |

### 2.2  I/O Count Per Bank

| Bank | Pin Count |
| --- | --- |
| Bank 14 (UAVs) | 54 |
| Bank 15 (CS3) | 27 |
| Bank 34 (CS1, CS2) | 54 |
| Bank 35 (Shared / Top-Level) | 16 |
| TOTAL | 151 |

Total pin count: 151 of 106 available  (-45 pins remaining for future expansion)

## 3. Bank Assignments

| Bank | Voltage | Primary Assignment | Rationale |
| --- | --- | --- | --- |
| Bank 14 | 3.3V (LVCMOS33) | UAV1, UAV2, UAV3 | All UAV subsystems co-located; UART/SPI/PWM grouped together |
| Bank 15 | 3.3V (LVCMOS33) | CubeSat CS3 | CS3 isolated to own bank for routing simplicity and future MUX expansion |
| Bank 34 | 3.3V (LVCMOS33) | CubeSat CS1, CS2 | CS1 and CS2 share bank; SPI buses kept adjacent for low skew |
| Bank 35 | 3.3V (LVCMOS33) | Shared / Top-Level | Clock (E3 GCLK), reset, LEDs, test points, arbiter UART |

## 4. Design Notes

### 4.1  SPI Bus Sharing

Each CubeSat uses a single SPI bus shared between: (a) the MPU-9250 IMU, (b) the sun sensor ADC (4 channels, mux externally or via separate CS_N), and (c) the three reaction-wheel motor drivers. This reduces I/O by 6 pins per CubeSat vs. independent buses. Ensure that only one CS_N is asserted at a time in firmware.

### 4.2  I2C Open-Drain Requirement

All I2C signals (cs1/cs2/cs3 magnetometer SDA/SCL and uav1/uav2/uav3 barometer SDA/SCL) must be configured as open-drain outputs in the XDC constraints using PULLUP or with external 2.2 kΩ pull-up resistors to 3.3 V. The FPGA IOBUF primitives should be instantiated with T (tri-state enable) driven from the I2C controller.

### 4.3  Clock Pin

sys_clk_50mhz is assigned to E3, a dedicated global clock-capable (GCLK) pin on Bank 35 of the XC7A35T CPG236 package. This feeds the MMCM/PLL for internal clock generation (e.g. 100 MHz system clock, 10 MHz SPI clock, 50 Hz / 400 Hz PWM timers).

### 4.4  Inter-Satellite Optical Link

Each CubeSat has a 9-pin unidirectional parallel optical link (8-bit data + valid). These are output-only from the FPGA perspective, driving laser driver ICs. At 100 MHz data rate, DDR output registers (ODDR primitives) are recommended to ensure clean eye diagrams. Place all 9 signals of each ISL on adjacent pins within the same bank.

### 4.5  AXI4-Lite Configuration Bus

The 20-signal AXI4-Lite config bus is implemented as internal on-chip interconnect (not routed to physical I/O pins). This saves 20 pins and is consistent with Xilinx MicroBlaze or Zynq-based design flows where the AXI bus is an internal fabric interface.

### 4.6  Pin Conflict Analysis

No physical pin conflicts detected. All signals are assigned to unique pins within their designated banks. Pin suggestions are indicative — final assignment must be verified against the Xilinx XC7A35T CPG236 package pinout file (xc7a35tcpg236pkg.txt) in Vivado to confirm I/O bank compatibility and avoidance of dedicated function pins (JTAG, config, power).

### 4.7  Multiplexing / Expansion

With -45 pins remaining after this assignment, the design has comfortable headroom. If additional signals are required, consider: (a) I2C bus sharing across CubeSats for magnetometers (saves 4 pins), (b) using an external SPI-to-UART bridge for telemetry (saves 2 pins per CubeSat), or (c) sharing the inter-UAV UART TX/RX on a multi-drop RS-485 bus (saves 4 pins across UAVs).

### 4.8  XDC Constraint Template

Sample XDC entries for key signals:

set_property PACKAGE_PIN E3  [get_ports sys_clk_50mhz]

set_property IOSTANDARD LVCMOS33 [get_ports sys_clk_50mhz]

create_clock -period 20.000 [get_ports sys_clk_50mhz]

set_property PACKAGE_PIN H4  [get_ports cs1_imu_sclk]

set_property IOSTANDARD LVCMOS33 [get_ports cs1_imu_sclk]