# 6-State Fixed-Point Kalman Filter  —  FPGA Matrix Specification

State: [px,py,pz,vx,vy,vz]  |  Positions Q15.16 (32-bit)  |  Velocities Q8.24 (32-bit)  |  GPS measurement [px,py,pz]  |  100 Hz / 50 MHz Artix-7

Ts=0.01s  |  3 DSP48E1 total  |  1 BRAM18 (144 bytes)  |  Compute: ~215 cycles = 4.3 µs  |  Budget margin: 2330× vs 10ms target

## 1.  Complete Matrix Specification Table

Every matrix: dimensions, non-zero count, fixed-point format, storage bytes, DSP48 usage, MAC count, and detailed multiply chain description.

| Matrix | Dims | Non- Zeros | Fixed-Point Format | Storage | DSP48 | MACs | Notes & Multiply Chain |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | 6×6 | 9 / 36 | Ts entries: Q8.24 (32-bit) Diag = 1 → Q1.0 (trivial copy) Zeros → skipped | 36 entries × 4B = 144B 1 BRAM18 row block Actually: 9 values stored (zeros omitted) = 36B | 3 | 9 | F·x prediction: 6 output states. Rows 0-2 (position): x_new[i] = x[i] + Ts×x[i+3] MAC: x[i] + Ts×x[i+3] (1 multiply + 1 add per row) Rows 3-5 (velocity): x_new[i+3] = x[i+3] Copy only — multiply by 1 is trivial, no DSP needed Total real MACs: 3 multiplies + 6 adds = 9 operations DSP48 usage: 1 per axis (3 axes) for Ts×v multiply Pipeline: 3 DSP48s → 3 cycles per axis → 9 cycles total Format chain: Q8.24(Ts) × Q8.24(vel) = Q16.48 → shift right 24 → Q8.24 |
| H | 3×6 | 3 / 18 | All entries ∈ {0,1} Q1.0 effectively No fixed-point multiply needed | Only 3 non-zero entries No storage needed: H×x = x[0:2] direct slice Optional: 18×1B = 18B | 0 | 0 | H·x innovation extraction: v = z - H·x_pred (innovation vector) Implementation: 3 subtractions only H×x = {x_pred[0], x_pred[1], x_pred[2]} No multiply needed — H is an extraction mask Innovation: innov[i] = z[i] - x_pred[i] i=0,1,2 All in Q15.16: innovation is GPS_pos - predicted_pos DSP48: 0 (subtraction in fabric LUTs) Cycles: 3 subtract + store = ~6 cycles |
| Q | 6×6 diag | 6 / 36 | qp: Q15.32 (64-bit integer) qv: Q8.40 (64-bit integer) Both fit in 64-bit reg for P+Q add | 6 diagonal values × 8B = 48B (64-bit for precision) Or 4B each if accepting truncation = 24B | 0 | 6 | P update: P_new = P_old + Q (diagonal-only addition) No multiplication — only element-wise add of diagonal P[i,i] += Q[i,i] for i=0..5 DSP48: 0 (fabric adder, 64-bit add) Cycles: 6 add operations = ~6 cycles Precision note: Q uses higher fixed-point precision than P because small noise values need > 32-bit representation qp = 0.01 cm² in Q32.32 = round(0.01 × 2^32) = 42,949,673 |
| R | 3×3 diag | 3 / 9 | rp: Q15.16 (32-bit) GPS σ=30cm → rp=900 cm² Represented as integer 900 (scaling in innovation step) | 3 diagonal values × 4B = 12B Shared in BRAM18 with F/Q/K Total matrix BRAM: 1 BRAM18 (<288 bytes total) | 0 | 3 | Innovation covariance: S = H·P·HT + R With H=[I3¦0]: S = P[0:3,0:3] + R S[i,i] = P[i,i] + rp for i=0,1,2 (3 adds) No multiply: S is just top-left 3×3 of P + R diagonal For constant-gain KF: S not computed at runtime — K is pre-computed from offline S inversion DSP48: 0 for constant-gain (S computed offline) Cycles: 0 (constant-gain) or 3 adds (full KF) |
| K | 6×3 | 6 / 18 | kp rows (0-2): Q1.30 (32-bit) kv rows (3-5): Q8.24 (32-bit) Pre-computed: loaded at boot from configuration BRAM page | 6 non-zero values × 4B = 24B Stored as 6×3 full matrix = 18 × 4B = 72B for general K Or 6B for block-diagonal K | 3 | 18 | x_update = x_pred + K × innovation For block-diagonal K: x_new[i] = x_pred[i] + kp × innov[i] i=0,1,2 x_new[i+3] = x_pred[i+3] + kv × innov[i] i=0,1,2 Total: 6 multiplies (3 kp× + 3 kv×) + 6 adds DSP48: 3 (one per axis: compute kp×innov[i] and kv×innov[i] sequentially) Pipeline: 3 axes × 2 cycles = 6 cycles for multiplies Format chain: kp (Q1.30) × innov (Q15.16) → Q16.46 → shift right 30 → Q15.16 correction kv (Q8.24) × innov (Q15.16) → Q23.40 → shift right 24 → Q8.24 correction |
| P | 6×6 sym | 21 unique (full sym) | Q15.32 (64-bit elements) Or Q15.16 (32-bit, less precision) 3 unique for steady-state: Ppp, Pvv, Ppv | Full: 21 × 8B = 168B → BRAM18 Steady-state: 3 × 8B = 24B → registers (no BRAM) Double-buffered: ×2 = 336B | 3 | 42 | P predict: P- = F·P·FT + Q With sparse F: only 3×3 blocks non-trivial P[i,i]- = P[i,i] + 2×Ts×P[i,i+3] + Ts²×P[i+3,i+3] + qp (i=0,1,2) P[i+3,i+3]- = P[i+3,i+3] + qv P[i,i+3]- = P[i,i+3] + Ts×P[i+3,i+3] P update: P+ = (I-KH)P- (constant-gain: skip at runtime) For full KF: P+[0:3,0:3] = (1-kp)P-[0:3,0:3] With constant gain: P converges → store steady-state P only DSP48: 3 for P predict (Ts² and 2×Ts×Ppv terms) Cycles: ~30 cycles for full P predict (6×5 mult/add ops) |

## 2.  Matrix Sparsity Patterns

Visual representation of all six matrices. Dark blue = diagonal/identity. Light blue = non-zero off-diagonal. Gray = zero (skipped in computation).

| F (6×6 State Transition) — Ts=0.01s on velocity-to-position blocks. px; py; pz; vx; vy; vz. px; 1; 0; 0; Ts; 0; 0. py; 0; 1; 0; 0; Ts; 0. pz; 0; 0; 1; 0; 0; Ts. vx; 0; 0; 0; 1; 0; 0. vy; 0; 0; 0; 0; 1; 0. vz; 0; 0; 0; 0; 0; 1. Ts = 0.01 s (100 Hz update). Stored as Q8.24 integer: Ts_fp = round(0.01 × 2^24) = 167,772. Sparsity: 9 non-zero (6 diagonal + 3 off-diagonal Ts), 27 zeros. Only 9 MACs needed (skip zeros). Off-diagonal entries use mixed multiply: Q8.24 × Q8.24(vel) → Q16.48 → shift right 24 → Q8.24.. | H (3×6 Measurement) — Extracts positions [px,py,pz] from state. px; py; pz; vx; vy; vz. z_px; 1; 0; 0; 0; 0; 0. z_py; 0; 1; 0; 0; 0; 0. z_pz; 0; 0; 1; 0; 0; 0. H is an extraction matrix: multiplying H×x simply picks rows 0,1,2 of x. Implementation: no DSP48 needed — H×x is just x[0:2] copied to innovation buffer. Innovation: v = z - H×x_pred = z[0:2] - x_pred[0:2] (3 subtractions only).. K (6×3 Kalman Gain) — Pre-computed constant gain, block-diagonal. z_px; z_py; z_pz. px; kp; 0; 0. py; 0; kp; 0. pz; 0; 0; kp. vx; kv; 0; 0. vy; 0; kv; 0. vz; 0; 0; kv. K is 6×3 block-diagonal with 6 non-zeros (3 kp + 3 kv). kp ≈ 0.8 (dimensionless position gain) → Q1.30 integer: round(0.8 × 2^30) = 858,993,459. kv ≈ 0.05 (velocity gain, cm/s per cm) → Q8.24 integer: round(0.05 × 2^24) = 838,861. Constant-gain: K pre-computed offline; only 6 values stored (6×4 = 24 bytes). K×innov: 6 scalar multiplies (one per state), each maps to 1 DSP48.. |
| --- | --- |

| Q (6×6 Process Noise) — Diagonal only. px; py; pz; vx; vy; vz. px; qp; 0; 0; 0; 0; 0. py; 0; qp; 0; 0; 0; 0. pz; 0; 0; qp; 0; 0; 0. vx; 0; 0; 0; qv; 0; 0. vy; 0; 0; 0; 0; qv; 0. vz; 0; 0; 0; 0; 0; qv. Q is diagonal: only 6 elements stored. qp = 0.01 cm² → Q15.16 integer: round(0.01 × 2^32) = 42,949,673. qv = 0.001 (cm/s)² → Q8.48 integer: round(0.001 × 2^48) ≈ 2.81×10^11. Storage: 6 × 4 bytes = 24 bytes. Used in P update: P+ = P- + Q (diagonal add).. | R (3×3 Measurement Noise) — Diagonal. z_px; rp; 0; 0. z_py; 0; rp; 0. z_pz; 0; 0; rp. GPS σ=30cm → rp=900 cm². Diagonal 3×3. 12 bytes storage.. | P (6×6 Error Covariance) — Block-diagonal by axis. px; py; pz; vx; vy; vz. px; Ppp; 0; 0; Ppv; 0; 0. py; 0; Ppp; 0; 0; Ppv; 0. pz; 0; 0; Ppp; 0; 0; Ppv. vx; Ppv; 0; 0; Pvv; 0; 0. vy; 0; Ppv; 0; 0; Pvv; 0. vz; 0; 0; Ppv; 0; 0; Pvv. P is 6×6 symmetric. Under NED decoupled model, only 3 block-diagonal 2×2 entries matter per axis. Unique elements: Ppp (pos variance), Pvv (vel variance), Ppv (pos-vel cross). For constant-gain filter: P converges to steady state; store 3 scalars only (9 elements by symmetry → 3 unique = 12 bytes). For full KF: 21 unique elements × 4 bytes = 84 bytes in BRAM.. |
| --- | --- | --- |

## 3.  Fixed-Point Format Reference

Every variable with exact bit-width, Q format, integer representation formula, representable range, and design rationale.

| Variable | Bits | Q Format | Integer Repr. | Real Range | Notes |
| --- | --- | --- | --- | --- | --- |
| px, py, pz (state) | 32-bit s | Q15.16 | val = real × 65536 | ±32767.99 cm (±328 m) | Positions in cm. LSB = 1/65536 cm = 0.015 mm. Range covers ±328 m formation diameter easily. |
| vx, vy, vz (state) | 32-bit s | Q8.24 | val = real × 16777216 | ±127.99 cm/s (±1.28 m/s) | Velocities in cm/s. LSB = 5.96×10⁻⁸ cm/s. For faster vehicles increase to Q12.20 (range ±2047 cm/s). |
| GPS measurement z[0:2] | 32-bit s | Q15.16 | same as px | ±32767.99 cm | GPS provides positions directly; same format as state positions. No conversion needed. |
| Innovation innov[0:2] | 32-bit s | Q15.16 | z - x_pred | ±32767.99 cm | Innovation = GPS - predicted. Same format as position. Max innovation bounded by filter residual. |
| Ts (sampling time) | 32-bit u | Q8.24 | 0.01 × 2^24 = 167772 | 0 to 255.9999 s | Ts = 0.01 s at 100 Hz. Stored as 32-bit unsigned Q8.24 constant. Never changes at runtime. |
| F off-diag (Ts entries) | 32-bit u | Q8.24 | 167772 | 0.01 (= 1 cm per 100 Hz) | Ts entries of F matrix. Mixed multiply Q8.24 × Q8.24 → Q16.48 → shift 24 → Q8.24. |
| kp (position gain) | 32-bit s | Q1.30 | 0.8 × 2^30 = 858993459 | ±1.9999 (dimensionless) | Kalman gain for position. Dimensionless (pos update per cm innovation). Q1.30 for fine resolution. |
| kv (velocity gain) | 32-bit s | Q8.24 | 0.05 × 2^24 = 838861 | ±127.9 cm/s per cm | Kalman gain for velocity. Units: (cm/s) per cm error. Q8.24 matches velocity state format. |
| qp (position noise var) | 64-bit u | Q32.32 | 0.01 × 2^32 = 42949673 | 0 to ~4×10⁹ cm² | Process noise for position. 64-bit for sub-LSB precision accumulation in P predict step. |
| qv (velocity noise var) | 64-bit u | Q8.48 | 0.001 × 2^48 ≈ 2.81×10¹¹ | 0 to 255.9 (cm/s)² | Process noise for velocity. High precision needed for small noise values. |
| rp (GPS meas. noise var) | 32-bit u | Q15.0 (int) | 900 (integer) | 0 to 32767 cm² | GPS σ=30cm → rp=900 cm². Stored as plain integer. Used in pre-computed K only; no runtime division. |
| P diagonal (Ppp,Pvv) | 64-bit s | Q15.32 | val = real × 2^32 | ±32767 cm² or (cm/s)² | Covariance diagonal. 64-bit avoids underflow as P converges. Steady-state: 3 values only. |
| Intermediate products (F×x) | 64-bit s | Q16.48 | product of Q8.24 × Q8.24 | ±65535 full range | Held in DSP48 48-bit P register + 16-bit carry. Arithmetic right-shift 24 to return to Q8.24. |
| Intermediate products (K×v) | 64-bit s | Q16.46 | product of Q1.30 × Q15.16 | ±65535 full range | Held in DSP48 48-bit P register. Shift right 30 to return to Q15.16 position correction. |

## 4.  Clock Cycle Budget  (50 MHz — target < 10 ms = 500,000 cycles)

Detailed per-step breakdown from BRAM load through state update. Constant-gain KF requires only ~215 cycles = 4.3 µs, providing 2330× margin vs the 10 ms budget.

| Step | Operation | Cycles | Cumul. | % of 500k (10ms) | Notes / Bottleneck |
| --- | --- | --- | --- | --- | --- |
| 1. BRAM read | Load x_pred, F sparse entries from BRAM into registers | 32 | 32 | 0.006% | 6 state × 4B + 9 F entries: 15 BRAM reads × 2 cycles each = 30 + 2 overhead |
| 2. Predict: F×x | 3 DSP48 MACs (one/axis) Ts×vx, Ts×vy, Ts×vz | 12 | 44 | 0.009% | 3 axes × (2-cycle DSP48 pipeline + 2 cycles shift+add) = 12 cycles; velocity rows trivially copied |
| 3. Predict store | Write x_pred to registers (×6) | 6 | 50 | 0.010% | 6 register writes = 6 cycles; no BRAM needed (state in flip-flops) |
| 4. H×x extract | Innovation: innov = z - x[0:2] | 6 | 56 | 0.011% | 3 subtractions in fabric = 3 cycles; read GPS z into registers = 3 more cycles |
| 5. K×innov | 3 DSP48 MACs (one/axis) kp×innov + kv×innov per axis | 18 | 74 | 0.015% | 3 axes × (2 DSP48 cycles kp + 2 cycles kv + 2 cycles shift) = 18 cycles total |
| 6. State update | x+ = x_pred + K×innov (6 adds) | 6 | 80 | 0.016% | 6 parallel adds in fabric = 6 cycles; one per state element |
| 7. P predict | F·P·FT + Q (sparse F reduces full 6×6×6=216 MACs → 18 MACs) | 60 | 140 | 0.028% | 18 real MACs × 2 DSP48 cycles = 36; + 6 Q diagonal adds + overhead = ~60 cycles total |
| 8. P update | P+ = (I-KH)P- for pos block (constant-gain: skip) | 0 / 24 | 140 / 164 | 0.028% / 0.033% | Constant-gain KF: skip at runtime (P pre-converged). Full KF: ~24 cycles for 3×3 pos block update. |
| 9. BRAM writeback | Store updated x+, P to BRAM for next cycle | 24 | 164 / 188 | 0.033% / 0.038% | 6 state writes + 6 P diagonal + overhead = ~24 cycles |
| 10. Sequencer overhead | FSM state transitions, address generation, pipeline flush | 50 | 214 / 238 | 0.043% / 0.048% | Conservative overhead estimate including pipeline fill/drain and BRAM address latency |
| TOTAL (const-gain) | Predict + Measurement Update (no P update at runtime) | ~215 | 215 | 0.043% | 215 cycles × 20 ns = 4.3 µs << 10 ms target by 2330× |
| TOTAL (full KF) | Includes P predict + P update | ~238 | 238 | 0.048% | 238 cycles × 20 ns = 4.76 µs — still >> 2000× margin vs 10 ms |

## 5.  BRAM Usage Breakdown

All matrix storage mapped to a single BRAM18 (18 KB capacity). Total footprint: 144 bytes for constant-gain KF = 0.78% of one BRAM18.

| Matrix / Data | Elements | Bytes | BRAM18 Usage | Address Range | Notes |
| --- | --- | --- | --- | --- | --- |
| F sparse (9 non-zeros) | 9 × 32-bit | 36 B | 1 BRAM18 shared (total ≤ 288B) | 0x000–0x023 | Skip zeros: only store {val, row, col} tuples or use fixed addressing for known positions |
| H (implicit) | 3 ones | 0 B | None | N/A | H×x = slice x[0:2]; no storage needed in hardware |
| Q diagonal | 6 × 64-bit | 48 B | (shared) | 0x024–0x053 | 64-bit for precision; or 32-bit with 0.02% rounding error |
| R diagonal | 3 × 32-bit | 12 B | (shared) | 0x054–0x05F | GPS noise rp=900 cm² stored as integer (no fixed-point multiply needed for constant K) |
| K gains (block-diag) | 6 × 32-bit | 24 B | (shared) | 0x060–0x077 | kp (Q1.30) and kv (Q8.24) stored as 32-bit integers; 6 values for block-diagonal K |
| P covariance (steady-state) | 3 × 64-bit | 24 B | (shared) | 0x078–0x08F | 3 unique values: Ppp, Pvv, Ppv; full symmetric 6×6 = 21 × 8B = 168B (separate BRAM18) |
| State vector x (double-buf) | 2 × 6 × 32-bit | 48 B | Registers (FF) | N/A | State kept in flip-flop registers for single-cycle read/write; BRAM unnecessary |
| TOTAL (constant-gain KF) |  | 144 B | 1 BRAM18 | 0x000–0x08F | 144 bytes / 18432 bytes = 0.78% of one BRAM18 — extreme efficiency |
| TOTAL (full-P KF) |  | 312 B | 1 BRAM18 (+1 if double-buf P) | 0x000–0x137 | Adds full P matrix: 21 × 8B = 168B more. Still fits in 1 BRAM18 (512B active range) |

## 6.  DSP48E1 Resource Usage

Only 3 DSP48E1 slices used (3.3% of 90 available on XC7A35T), time-multiplexed across predict and update phases.

| Usage | DSP48 Count | A port (18-bit) | B port (18-bit) | Operation | Cycles | Implementation Detail |
| --- | --- | --- | --- | --- | --- | --- |
| F×x: Ts×velocity | 3 | Ts[17:0] = Ts_fp | vel[i][17:0] (i=0,1,2 per DSP) | A×B → P[47:0] then >>> 24 | 3 × 2 = 6 pipelined | One DSP48 per axis. Ts = 167772 (Q8.24). vel in Q8.24. Product Q16.48 → right-shift 24 → Q8.24 position increment. PREG=1 for pipelining. |
| K×innov: kp×innov | 1-3 | kp[17:0] = Kp_fp | innov[i][17:0] (i=0,1,2 mux) | A×B → P[47:0] then >>> 30 | 3 × 2 = 6 sequential | One DSP48 time-multiplexed across 3 innovation components. kp in Q1.30. innov in Q15.16. Product Q16.46 → right-shift 30 → Q15.16 position correction. PREG=1. |
| K×innov: kv×innov | 1-3 | kv[17:0] = Kv_fp | innov[i][17:0] (i=0,1,2 mux) | A×B → P[47:0] then >>> 24 | 3 × 2 = 6 sequential | Same DSP48 as kp× (time-multiplexed): after kp phase, reload A with kv. kv in Q8.24. Product Q16.40 → right-shift 24 → Q8.24 velocity correction. PREG=1. |
| P predict (Ts²×Pvv) | 1 | Ts[17:0] = Ts_fp | Ts[17:0] = Ts_fp (A=B for Ts²) | A×B → Ts² accumulate | 6 (one per row) | Ts² = 0.0001 s². Pre-compute Ts² once and store as constant. Then DSP48 in MACC mode: P[i,i] += 2×Ts×Ppv + Ts²×Pvv + qp. Reuses same physical DSP as K× phases. |
| P predict (Ts×Ppv) | (shared) | Ts[17:0] | Ppv[17:0] | A×B → Ts×Ppv × 2 (shift left 1) | 6 | Shares DSP48 with Ts²×Pvv step. Sequential: first compute Ts×Ppv, then Ts²×Pvv, accumulate into P diagonal. No new DSP needed. |
| Adders (fabric LUTs) | 0 | N/A | N/A | 6 × 32-bit adds (state update) | 6 | x+ = x_pred + correction implemented in fabric. At 50 MHz, 32-bit add fits in one LUT level. No DSP48 required or beneficial. |
| TOTAL | 3 DSP48E1 |  |  |  | ~27 cycles | 3 physical DSP48s time-multiplexed across all MAC phases. 3 of 90 available on XC7A35T = 3.3% DSP utilization. |

## 7.  Verilog RTL — Predict Step

Synthesizable Verilog for F×x: Ts×v multiply using DSP48, sparse F structure exploited. All 6 states updated in a single always block. 3 DSP48 instances.

kf_predict.v — F×x State Prediction (3 DSP48E1)

// ============================================================ // KF PREDICT STEP: x_pred = F * x_curr // Sparse F: only Ts off-diagonals are non-trivial multiplies // State format: pos=Q15.16, vel=Q8.24 // Ts format: Q8.24 = 167772 (= 0.01 × 2^24) // DSP48: 3 (one per axis, time-multiplexed) // ============================================================ module kf_predict ( input wire clk, rst_n, en, input wire signed [31:0] x_px, x_py, x_pz, // Q15.16 positions input wire signed [31:0] x_vx, x_vy, x_vz, // Q8.24 velocities output reg signed [31:0] xp_px, xp_py, xp_pz,// Q15.16 predicted pos output reg signed [31:0] xp_vx, xp_vy, xp_vz ); // Ts = 0.01 s in Q8.24 = 167772 localparam signed [31:0] TS_Q824 = 32'd167772; // DSP48 intermediate: Q8.24 × Q8.24 = Q16.48 (use 48-bit) wire signed [47:0] dx, dy, dz; // Ts × velocity products // DSP48 × 3: Ts × vx, Ts × vy, Ts × vz (parallel) assign dx = $signed(TS_Q824) * $signed(x_vx); // Q16.48 assign dy = $signed(TS_Q824) * $signed(x_vy); assign dz = $signed(TS_Q824) * $signed(x_vz); // Shift right 24: Q16.48 → Q16.24 (take bits [47:24] = position delta in Q16.16) // Note: Ts is Q8.24, vel is Q8.24 → product Q16.48 → >>24 gives Q16.24. // But we want Q15.16 output! So take bits [47:32] after >>16 additional = >>40 total. // Safer: since vel is Q8.24 and Ts=0.01, delta_pos = 0.01×vel. // vel range: max ≈ 128 cm/s → delta_pos = 0.01×128 = 1.28 cm → fits Q15.16 easily. // Implementation: (Ts × vel) >> 24 gives Q8.16 intermediate; sign-extend to Q15.16. wire signed [31:0] delta_x = {{8{dx[47]}}, dx[47:24]}; // Q15.16 wire signed [31:0] delta_y = {{8{dy[47]}}, dy[47:24]}; wire signed [31:0] delta_z = {{8{dz[47]}}, dz[47:24]}; always @(posedge clk) begin if (!rst_n) begin xp_px<=0; xp_py<=0; xp_pz<=0; xp_vx<=0; xp_vy<=0; xp_vz<=0; end else if (en) begin // Positions: p_new = p_old + Ts*v (F rows 0-2) xp_px <= x_px + delta_x; xp_py <= x_py + delta_y; xp_pz <= x_pz + delta_z; // Velocities: v_new = v_old (F rows 3-5 are identity) xp_vx <= x_vx; xp_vy <= x_vy; xp_vz <= x_vz; end end endmodule

## 8.  Verilog RTL — Update Step

Synthesizable Verilog for K×innovation update. H×x is implemented as wire slice (no multiply). K is block-diagonal: 3 DSP48 for kp× and kv× per axis.

kf_update.v — K×innovation State Update (3 DSP48E1 shared)

// ============================================================ // KF UPDATE STEP: x+ = x_pred + K * innovation // innovation = z[0:2] - x_pred[0:2] (H is extraction mask) // K is block-diagonal: kp for positions, kv for velocities // kp format: Q1.30 ¦ kv format: Q8.24 // DSP48: 3 (one per axis, kp then kv sequentially) // ============================================================ module kf_update ( input wire clk, rst_n, en, // Predicted state input wire signed [31:0] xp_px, xp_py, xp_pz, // Q15.16 input wire signed [31:0] xp_vx, xp_vy, xp_vz, // Q8.24 // GPS measurements input wire signed [31:0] z_px, z_py, z_pz, // Q15.16 // Kalman gains (pre-computed, constant) input wire signed [31:0] kp, // Q1.30 position gain input wire signed [31:0] kv, // Q8.24 velocity gain // Updated state outputs output reg signed [31:0] x_px, x_py, x_pz, // Q15.16 output reg signed [31:0] x_vx, x_vy, x_vz ); // Innovation: v = z - H*x_pred = z[0:2] - x_pred[0:2] (no multiply!) wire signed [31:0] innov_x = z_px - xp_px; // Q15.16 wire signed [31:0] innov_y = z_py - xp_py; wire signed [31:0] innov_z = z_pz - xp_pz; // K*innovation: position correction Q1.30 × Q15.16 → Q16.46 → >>30 → Q15.16 wire signed [47:0] kp_vx, kp_vy, kp_vz; assign kp_vx = $signed(kp) * $signed(innov_x); // Q16.46 assign kp_vy = $signed(kp) * $signed(innov_y); assign kp_vz = $signed(kp) * $signed(innov_z); // Velocity correction: Q8.24 × Q15.16 → Q23.40 → >>24 → Q8.16 (pad to Q8.24) wire signed [47:0] kv_vx, kv_vy, kv_vz; assign kv_vx = $signed(kv) * $signed(innov_x); // Q23.40 assign kv_vy = $signed(kv) * $signed(innov_y); assign kv_vz = $signed(kv) * $signed(innov_z); // Corrections: shift right to return to state formats wire signed [31:0] corr_px = kp_vx[47:16]; // >>30 from Q16.46 → Q15.16 wire signed [31:0] corr_py = kp_vy[47:16]; wire signed [31:0] corr_pz = kp_vz[47:16]; wire signed [31:0] corr_vx = {{8{kv_vx[47]}}, kv_vx[47:24]};// >>24 → Q8.24 wire signed [31:0] corr_vy = {{8{kv_vy[47]}}, kv_vy[47:24]}; wire signed [31:0] corr_vz = {{8{kv_vz[47]}}, kv_vz[47:24]}; always @(posedge clk) begin if (!rst_n) begin x_px<=0; x_py<=0; x_pz<=0; x_vx<=0; x_vy<=0; x_vz<=0; end else if (en) begin x_px <= xp_px + corr_px; // position update x_py <= xp_py + corr_py; x_pz <= xp_pz + corr_pz; x_vx <= xp_vx + corr_vx; // velocity update x_vy <= xp_vy + corr_vy; x_vz <= xp_vz + corr_vz; end end endmodule

## 9.  Constant-Gain vs Full-KF Mode

| Aspect | Constant-Gain KF (Recommended) | Full KF (Optional) |
| --- | --- | --- |
| K computation | Pre-computed offline; stored as constants | Computed each cycle: K=P·HT·(HPHTr+R)^-1 — requires matrix inversion |
| P update at runtime | None — P converges to steady-state | P updated every cycle: P+ = (I-KH)P- |
| DSP48 usage | 3 (predict + update only) | 6-9 (predict + P predict + P update + K compute) |
| BRAM usage | 144 bytes (1 BRAM18) | 312 bytes (1 BRAM18) |
| Compute cycles | ~215 cycles (4.3 µs) | ~500 cycles (10 µs) |
| When to use | Fixed GPS noise (σ known), steady-state operation | Variable noise environment, maneuver detection, adaptive KF |
| Accuracy | Identical to full KF at steady-state | Better during transients (GPS outage, noise changes) |