# Fixed-Point PID Specification  —  Q4.12 Format

Discrete Euler PID  |  Ts = 1 ms  |  Q4.12 two's complement  |  Target: Xilinx Artix-7 XC7A35T  |  4 DSP48E1 total

Q4.12: 1 sign + 4 integer + 12 fractional bits  |  Range: -16.0 to +15.9998  |  Resolution: 2^-12 = 0.000244  |  Integer value = real × 4096

## 1.  Per-Term Fixed-Point Specification Matrix

The table below covers all aspects of each PID term: variable formats, intermediate multiplication result widths, required right-shifts to return to Q4.12, overflow detection logic, anti-windup strategy, DSP48 count, and numerical precision analysis.

| Specification Aspect | P Term (Kp × e[n]) | I Term (Ki × Ts × sum(e)) | D Term (Kd × (e[n]-e[n-1]) / Ts) |
| --- | --- | --- | --- |
| Variable formats | kp : Q4.12 (16-bit signed) error : Q4.12 (16-bit signed) p_term : Q4.12 (output) | ki : Q4.12 (16-bit signed) ts_q : Q4.12 = 4 (0.001×4096) i_accum: Q8.24 (32-bit accumulator) i_term : Q4.12 (output) | kd_s : Q4.12 (= Kd/Ts, pre-scaled) err_cur: Q4.12 (16-bit signed) err_prv: Q4.12 (16-bit signed) d_diff : Q5.12 (17-bit, one extra int bit) d_term : Q4.12 (output) |
| Multiplication result format | kp × error = Q4.12 × Q4.12 = Q8.24 (32-bit full product) 32-bit intermediate needed; no overflow if ¦kp×err¦ < 256 | ki × ts_q = Q4.12 × Q4.12 = Q8.24 (scaled gain) Then × i_accum(Q8.24) requires Q16.48 — use two DSP48s or reduce precision; practical: keep Q8.24 acc | kd_s × d_diff = Q4.12 × Q5.12 = Q9.24 (32-bit product) No /Ts needed (absorbed) ¦kd_s×d_diff¦ < 2^9 = 512 overflow if Kd/Ts × 2×emax > 16 |
| Required right shifts to return to Q4.12 | product >> 12 Q8.24 >>> 12 = Q8.12 (take [27:12]) Then saturate [15:0] to Q4.12 Keep bits [27:12], check [31:28] for overflow detection | i_scaled = (ki×error) >>> 12 i_accum += i_scaled (Q8.24 acc) i_term = i_accum >>> 12 Takes [19:0] of 32-bit acc = Q8.12 Saturate final to Q4.12 | product >>> 12 Q9.24 >>> 12 = Q9.12 (take [28:12]) Saturate [16:0] to Q4.12 Check bits [31:29] for overflow Result valid if ¦d_term¦ < 16.0 |
| Overflow detection bits | p_overflow = ¦product[31:28]¦ != 0 OR product[31] != product[28] (sign extension check) If overflow: saturate to +32767 or -32768 (Q4.12 max/min) | i_overflow = ¦i_accum[31:24]¦ != 0 AND sign(i_accum[23]) != sign Anti-windup clamp first: if i_accum > CLAMP_MAX → hold if i_accum < CLAMP_MIN → hold | d_overflow = product[31:29] not all-same as sign bit [28] If ¦Kd_s¦ × 2×emax > 15.999, reduce Kd register or increase integer bits to Q5.12 for d_term |
| Anti-windup / clamping | No integrator → no windup Saturation only on final output: +32767 (max Q4.12) -32768 (min Q4.12) SAT_P flag for telemetry | Integrator clamped BEFORE add: if (i_accum > AW_MAX) i_accum = AW_MAX if (i_accum < AW_MIN) i_accum = AW_MIN AW_MAX/AW_MIN: Q8.24 registers default ±8388608 (±2.0 in Q4.12) SAT_I flag when clamped | No integrator → no windup Saturation on d_term output only after right shift: +32767 / -32768 boundary SAT_D flag for telemetry Rate limit optional (see Sec 5) |
| DSP48 resources | 1 DSP48E1 A×B: kp(18b) × error(18b) P port: 48-bit; take [39:12] Pipeline: 2 reg stages @100MHz within spec | 2 DSP48E1 DSP1: ki × error → scaled DSP2: MACC — accumulate i_accum += scaled_ki_err P=A×B+C mode for MACC 3 reg stages total | 1 DSP48E1 A = kd_s (18b) B = d_diff = err_cur - err_prv PRE-ADD: d_diff computed in DSP DSP A:B pre-adder path P port bits [39:12] = result 2 reg stages |
| Precision analysis | Precision: 2^-12 = 0.000244 Round error ≤ 0.5 LSB = 0.000122 Worst-case Kp gain error: 0.000244 / 1.0 = 0.024% (1 LSB Kp at unity gain) | Ts quantisation: 4/4096=0.000977 vs ideal 0.001000 → -2.3% error Use Ki_scaled = Ki×Ts combined: store Ki×0.001 directly in register eliminating Ts mult entirely | Kd/Ts max = 1000×Kd range: if Kd = 0.010 → Kd_s = 10.0 Q4.12 if Kd = 0.001 → Kd_s = 1.0 Q4.12 Max safe Kd/Ts = 15.999 (Q4.12 max) For larger: use Q8.12 for Kd_s |

## 2.  Signal Bit-Width & Format Table

Every signal in the fixed-point PID pipeline with its exact bit-width, Q format, representable range, and LSB value. All arithmetic is two's complement signed.

| Signal / Variable | Width | Q Format | Real Value Range | 1 LSB | Notes / Rationale |
| --- | --- | --- | --- | --- | --- |
| setpoint / feedback | 16b s | Q4.12 | -16.000 to +15.9998 | 0.000244 | Sensor/command inputs; ±16 covers ±16 rad, m/s, etc. |
| error = sp − fb | 17b s | Q5.12 | -32.000 to +31.9998 | 0.000244 | 17-bit to prevent subtraction overflow; truncate if inputs guaranteed <±8 |
| kp, ki_s, kd_s | 16b s | Q4.12 | 0.000 to +15.999 | 0.000244 | Gain registers; unsigned in practice (positive gains only) |
| p_term intermediate | 32b s | Q8.24 | -256.0 to +255.9999 | 6e-8 | Q4.12×Q4.12=Q8.24; 32-bit DSP48 P-port output |
| p_term output | 16b s | Q4.12 | -16.000 to +15.9998 | 0.000244 | After >>> 12 + saturation of Q8.24 result |
| ki_err (DSP1) | 32b s | Q8.24 | -256.0 to +255.9999 | 6e-8 | ki_s×error product before accumulation |
| i_accum | 32b s | Q8.24 | -128 to +127.9999 | 6e-8 | Accumulator; anti-windup clamp keeps ¦val¦<aw_max/min |
| i_term output | 16b s | Q4.12 | -16.000 to +15.9998 | 0.000244 | i_accum >>> 12 + saturation |
| d_diff = e[n]-e[n-1] | 17b s | Q5.12 | -32.000 to +31.9998 | 0.000244 | Max diff = 2×emax = 2×15.999 = 31.998; needs Q5.12 |
| d_prod intermediate | 32b s | Q9.24 | -512 to +511.9999 | 6e-8 | Q4.12×Q5.12 = Q9.24; use [31:12] after >>>12 |
| d_term output | 16b s | Q4.12 | -16.000 to +15.9998 | 0.000244 | After >>> 12 + saturation; kd_s max = 15.999 |
| sum_wide | 18b s | Q6.12 | -64.000 to +63.9998 | 0.000244 | P+I+D sum; 18-bit prevents triple overflow |
| u_out (final) | 16b s | Q4.12 | -16.000 to +15.9998 | 0.000244 | Final PID output; saturated to Q4.12 limits |
| aw_max / aw_min | 32b s | Q8.24 | ±128.0 typical | 6e-8 | Anti-windup bounds; aw_max = 4×4096×4096 for ±2.0 |

## 3.  DSP48E1 Resource Usage Summary

The complete PID controller uses exactly 4 DSP48E1 slices (4.4% of the 90 available on XC7A35T). All multiplies are registered at 2-cycle pipeline depth, meeting 100 MHz timing closure.

| Block | DSP48 Count | Operation | A Width | B Width | Configuration |
| --- | --- | --- | --- | --- | --- |
| P Term (pid_p_term) | 1 | A×B | 18-bit (kp) | 18-bit (error) | P=A×B; registered; PREG=1; take P[39:12] |
| I Term DSP#1 (pid_i_term) | 1 | A×B | 18-bit (ki_s) | 18-bit (error) | P=A×B; no accumulate; feeds MACC chain |
| I Term DSP#2 (pid_i_term) | 1 | MACC P=A×B+C | 18-bit (ki_s) | 18-bit (error) | C=i_accum; P=A×B+C; implements += |
| D Term (pid_d_term) | 1 | Pre-add then × | 18-bit (kd_s) | 18-bit (d_diff) | D-A pre-adder: d_diff = error-error_prev; then A×B |
| PID Summer (pid_summer) | 0 | Adder tree (fabric) | 18-bit sign-ext | 18-bit sign-ext | 3-input add in LUT fabric; 2 CLB levels; no DSP needed |
| Error subtractor | 0 | Fabric subtractor | 16-bit | 16-bit | sp − fb; LUT fabric; 1 CLB level |
| TOTAL | 4 DSP48E1 |  |  |  | Out of 90 available on XC7A35T = 4.4% utilization |

## 4.  Precision & Quantisation Error Analysis

Each source of fixed-point error is quantified and a mitigation strategy given. The dominant error is Ts quantisation for the integral term, eliminated by absorbing Ts into the ki_s gain register.

| Error Source | Magnitude | Effect on PID | Mitigation Strategy |
| --- | --- | --- | --- |
| Quantisation of Kp, Ki, Kd gains | ±0.5 LSB = ±0.000122 | Gain error < 0.024% at unity gain; < 0.012% at Kp=2.0 | Use Q4.12 — acceptable for flight control. For tighter: Q2.14 gains. |
| Ts = 0.001 s represented as 4/4096 | −2.34% of Ts value | Ki effectively scaled by 0.9766×; corrected by absorbing Ts into ki_s register | Store ki_s = Ki×0.001 in register directly; no run-time Ts multiply needed. |
| Accumulator truncation (Q8.24 >> 12) | ±0.5 LSB = ±0.000122 | Integral output rounding ≤ 1 LSB per update; negligible at 1 kHz | Arithmetic right-shift preserves sign; truncation error bounded by 1 LSB. |
| d_diff subtraction rounding | 0 (exact) | No rounding — integer subtraction of Q4.12 values is exact | Use 17-bit wide subtraction to prevent borrow overflow. |
| kd_s = Kd/Ts quantisation | ±0.000244 | For Kd=0.010, kd_s=10.0 exact; for Kd=0.001, kd_s=1.0 exact | Integer Kd/Ts values → zero error. Non-integer: ±1 LSB gain error < 0.025%. |
| Pipeline latency (3 cycles at 50 MHz) | 60 ns total | At 1 kHz update rate: 0.006% of period — completely negligible | Register final output with valid strobe; latency constant so harmless. |
| DSP48 rounding mode (truncation) | −0.5 LSB systematic | Systematic negative bias on all multiply results; ≈ 0.000122 offset | Add 1'b1 to bit [11] before right-shift for symmetric rounding (optional). |

## 5.  Verilog RTL — P Term

Proportional term: p_term = kp * error. Q4.12 × Q4.12 → Q8.24 multiply, arithmetic right-shift >>12, signed saturation to Q4.12. 2-cycle pipeline (DSP48 PREG + output register).

pid_p_term.v — Proportional Term (1 DSP48E1)

// ============================================================ // P TERM: p_term = Kp * e[n] // Format: Q4.12 x Q4.12 = Q8.24 -> shift >>12 -> Q4.12 // Bits: 32-bit product, take [27:12], saturate to [15:0] // DSP48: 1 (A*B, registered output) // ============================================================ module pid_p_term ( input wire clk, rst_n, input wire signed [15:0] kp, // Q4.12 gain input wire signed [15:0] error, // Q4.12 error output reg signed [15:0] p_term, // Q4.12 result output reg sat_p ); // Stage 1: full-precision multiply (Q8.24, 32-bit) reg signed [31:0] prod; // Q8.24 intermediate reg signed [31:0] prod_r1;// pipeline stage 1 always @(posedge clk) begin if (!rst_n) begin prod_r1 <= 32'd0; end else begin // DSP48 multiply: A[17:0]=kp, B[17:0]=error prod_r1 <= $signed(kp) * $signed(error); end end // Stage 2: arithmetic right-shift >>12 + saturation wire signed [31:0] shifted = prod_r1 >>> 12; // Q8.12 // Overflow: bits [31:16] must all match sign bit [15] wire overflow = (shifted[31:16] != {16{shifted[15]}}); always @(posedge clk) begin if (!rst_n) begin p_term <= 16'd0; sat_p <= 1'b0; end else if (overflow) begin // Saturate to Q4.12 max/min p_term <= shifted[31] ? 16'h8000 : 16'h7FFF; // negative overflow → -16.0 positive → +15.9998 sat_p <= 1'b1; end else begin p_term <= shifted[15:0]; sat_p <= 1'b0; end end endmodule

## 6.  Verilog RTL — I Term

Integral term with anti-windup clamp. ki_s = Ki × Ts is pre-stored in register (eliminates Ts quantisation). Accumulator is Q8.24 (32-bit). Anti-windup clamps BEFORE each accumulate step. 2 DSP48E1 (multiply + MACC).

pid_i_term.v — Integral Term with Anti-Windup (2 DSP48E1)

// ============================================================ // I TERM: i_term = Ki * Ts * sum(e[0..n]) // Strategy: absorb Ts into Ki: ki_s = Ki * 0.001 (pre-stored) // accumulate: i_accum += ki_s * error (each 1ms) // Format: accumulator Q8.24 (32-bit), output Q4.12 // Anti-windup: clamp i_accum BEFORE adding new term // DSP48: 2 (multiply + MACC accumulate) // ============================================================ module pid_i_term ( input wire clk, rst_n, en, // en = 1kHz CE strobe input wire signed [15:0] ki_s, // Ki*Ts in Q4.12 input wire signed [15:0] error, // Q4.12 error input wire signed [31:0] aw_max, // anti-windup upper Q8.24 input wire signed [31:0] aw_min, // anti-windup lower Q8.24 output reg signed [15:0] i_term, // Q4.12 output output reg sat_i ); // Q8.24 integrator accumulator (32-bit signed) reg signed [31:0] i_accum; wire signed [31:0] ki_err_full; // Q8.24 product wire signed [31:0] ki_err_s; // Q4.24 -> scaled wire signed [31:0] i_next; wire clamped; // DSP48 #1: ki_s * error -> Q8.24 product assign ki_err_full = $signed(ki_s) * $signed(error); // No shift needed: ki_s is already ki*Ts, result is Q8.24 // (ki_s[Q4.12] * error[Q4.12] = Q8.24, drop lower 0 bits for accum) // Anti-windup: clamp BEFORE accumulate // i_next = clamp(i_accum + ki_err_full, aw_min, aw_max) assign i_next = i_accum + ki_err_full; assign clamped = (i_next > aw_max) ¦ (i_next < aw_min); always @(posedge clk) begin if (!rst_n) begin i_accum <= 32'd0; i_term <= 16'd0; sat_i <= 1'b0; end else if (en) begin // DSP48 #2: MACC — accumulate with anti-windup if (clamped) begin i_accum <= (i_next > aw_max) ? aw_max : aw_min; sat_i <= 1'b1; end else begin i_accum <= i_next; sat_i <= 1'b0; end // Output: i_accum >> 12 = Q8.24 -> Q8.12, saturate to Q4.12 reg signed [31:0] shifted_i; shifted_i = i_accum >>> 12; i_term <= (¦shifted_i[31:15]) ? (shifted_i[31] ? 16'h8000 : 16'h7FFF) : shifted_i[15:0]; end end endmodule

## 7.  Verilog RTL — D Term

Derivative term: kd_s = Kd/Ts is pre-stored in register (eliminates /Ts overflow). DSP48 pre-adder computes e[n]-e[n-1] inside the DSP48 A-D path, saving one subtract stage. 1 DSP48E1.

pid_d_term.v — Derivative Term (1 DSP48E1, pre-adder)

// ============================================================ // D TERM: d_term = Kd * (e[n] - e[n-1]) / Ts // Strategy: pre-scale kd_s = Kd / Ts (stored in register) // d_diff = error - error_prev (Q5.12, 17-bit) // d_term = kd_s * d_diff >> 12 (Q4.12 output) // DSP48 pre-adder: A-D path computes d_diff inside DSP48 // DSP48: 1 (with pre-adder for subtraction) // ============================================================ module pid_d_term ( input wire clk, rst_n, en, input wire signed [15:0] kd_s, // Kd/Ts in Q4.12 input wire signed [15:0] error, // Q4.12 current error output reg signed [15:0] d_term, // Q4.12 output output reg sat_d ); reg signed [15:0] error_prev; // e[n-1] register wire signed [16:0] d_diff; // Q5.12: may hit ±16 wire signed [31:0] d_prod; // Q9.24 product wire signed [31:0] d_shifted; // Q9.12 after >>12 wire d_overflow; // DSP48 pre-adder path: D port=error, A port=error_prev, op=D-A // Equivalent to: d_diff = error - error_prev (signed 17-bit) assign d_diff = {error[15], error} - {error_prev[15], error_prev}; // sign-extend both to 17 bits before subtract for safety // DSP48 multiply: kd_s * d_diff (Q4.12 x Q5.12 = Q9.24) assign d_prod = $signed(kd_s[15:0]) * $signed(d_diff[16:0); // Right shift 12: Q9.24 -> Q9.12 (take bits [31:12]) assign d_shifted = d_prod >>> 12; // Overflow: result must fit in Q4.12 (bits [31:15] all = sign) assign d_overflow = (d_shifted[31:15] != {17{d_shifted[15]}}); always @(posedge clk) begin if (!rst_n) begin error_prev <= 16'd0; d_term <= 16'd0; sat_d <= 1'b0; end else if (en) begin error_prev <= error; // register e[n-1] if (d_overflow) begin d_term <= d_shifted[31] ? 16'h8000 : 16'h7FFF; sat_d <= 1'b1; end else begin d_term <= d_shifted[15:0]; sat_d <= 1'b0; end end end endmodule

## 8.  Verilog RTL — PID Summer & Saturation

Three-input adder with Q6.12 18-bit intermediate to prevent overflow. Final saturation to Q4.12. All four sat_flags (P, I, D, sum) provided for telemetry monitoring. 0 DSP48E1 (fabric adder tree).

pid_summer.v — P+I+D Adder with Saturation (0 DSP48E1)

// ============================================================ // PID SUMMER: u[n] = p_term + i_term + d_term // All inputs Q4.12; sum may overflow → Q5.12 intermediate // Final saturation to Q4.12 output // DSP48: 0 (adder tree in fabric — fast at 50 MHz) // ============================================================ module pid_summer ( input wire clk, rst_n, input wire signed [15:0] p_term, i_term, d_term, input wire sat_p, sat_i, sat_d, // individual sat flags output reg signed [15:0] u_out, // Q4.12 final output output reg sat_sum ); // Three-input adder: extend to Q6.12 (18-bit) to prevent overflow wire signed [17:0] sum_wide; assign sum_wide = {{2{p_term[15]}}, p_term} + {{2{i_term[15]}}, i_term} + {{2{d_term[15]}}, d_term}; // Overflow check: sum_wide[17:15] must all equal sign bit [15] wire sum_ov = (sum_wide[17:15] != {3{sum_wide[15]}}); always @(posedge clk) begin if (!rst_n) begin u_out <= 16'd0; sat_sum <= 1'b0; end else begin if (sum_ov) begin u_out <= sum_wide[17] ? 16'h8000 : 16'h7FFF; sat_sum <= 1'b1; end else begin u_out <= sum_wide[15:0]; sat_sum <= sat_p ¦ sat_i ¦ sat_d; end end end endmodule

## 9.  Verilog RTL — Top-Level Instantiation

Complete PID controller wrapping all four submodules. Error subtractor in fabric. All gains and anti-windup bounds accessible as input ports (driven from AXI4-Lite gain register bank). Total latency: 3 clock cycles.

pid_controller_q412.v — Top Level (4 DSP48E1 total)

// ============================================================ // TOP-LEVEL PID CONTROLLER (Q4.12 fixed-point) // Pipeline latency: 3 clock cycles (registered DSP48 outputs) // Update rate: 1 kHz (en strobe from 50 MHz / 50000 counter) // DSP48 total: 4 (1 P + 2 I + 1 D) // ============================================================ module pid_controller_q412 ( input wire clk, // 50 MHz sys_clk input wire rst_n, // active-low reset input wire en, // 1 kHz CE strobe input wire signed [15:0] setpoint, // Q4.12 input wire signed [15:0] feedback, // Q4.12 input wire signed [15:0] kp, // Q4.12 proportional gain input wire signed [15:0] ki_s, // Q4.12 Ki*Ts (absorbs Ts) input wire signed [15:0] kd_s, // Q4.12 Kd/Ts (absorbs 1/Ts) input wire signed [31:0] aw_max, // Q8.24 anti-windup upper input wire signed [31:0] aw_min, // Q8.24 anti-windup lower output wire signed [15:0] u_out, // Q4.12 control output output wire [3:0] sat_flags ); wire signed [15:0] error = $signed(setpoint) - $signed(feedback); // Q4.12 subtractor wire signed [15:0] p_term, i_term, d_term; wire sat_p, sat_i, sat_d, sat_sum; pid_p_term u_p (.clk(clk),.rst_n(rst_n),.kp(kp),.error(error), .p_term(p_term),.sat_p(sat_p)); pid_i_term u_i (.clk(clk),.rst_n(rst_n),.en(en),.ki_s(ki_s), .error(error),.aw_max(aw_max),.aw_min(aw_min), .i_term(i_term),.sat_i(sat_i)); pid_d_term u_d (.clk(clk),.rst_n(rst_n),.en(en),.kd_s(kd_s), .error(error),.d_term(d_term),.sat_d(sat_d)); pid_summer u_s (.clk(clk),.rst_n(rst_n),.p_term(p_term), .i_term(i_term),.d_term(d_term),.sat_p(sat_p), .sat_i(sat_i),.sat_d(sat_d),.u_out(u_out),.sat_sum(sat_sum)); assign sat_flags = {sat_sum, sat_d, sat_i, sat_p}; endmodule

## 10.  Gain Register Encoding Reference

Converting real gain values to Q4.12 register integers for AXI4-Lite configuration:

| Parameter | Real Value | Q4.12 Integer | Hex | Notes |
| --- | --- | --- | --- | --- |
| kp (roll) | 1.5 | 6144 | 0x1800 | 1.5 × 4096 = 6144.0 exact |
| kp (alt hold) | 0.5 | 2048 | 0x0800 | 0.5 × 4096 = 2048.0 exact |
| ki_s (= Ki×Ts) | 0.02 × 0.001 = 0.00002 | 0 | 0x0000 | Too small for Q4.12! Use Q4.28 for ki_s |
| ki_s (practical) | 0.1 × 0.001 = 0.0001 | 0 (rounds to 0) | 0x0000 | Warning: ki < 0.244 rounds to 0 in Q4.12 |
| ki_s (min useful) | 0.000244 (= 1 LSB) | 1 | 0x0001 | Minimum non-zero integral gain |
| kd_s (= Kd/Ts) | 0.01 / 0.001 = 10.0 | 40960 | 0xA000 | 10.0 × 4096 = 40960 — fits Q4.12 (max 65535) |
| kd_s (Kd=0.001) | 0.001 / 0.001 = 1.0 | 4096 | 0x1000 | 1.0 × 4096 = 4096 exact |
| aw_max (Q8.24) | 2.0 m/s (example) | 33554432 | 0x2000000 | 2.0 × 4096 × 4096 = 33554432 |

IMPORTANT: For Ki values smaller than 0.000244 (1 Q4.12 LSB) the integral gain rounds to zero. Solution: store ki_s in Q4.28 format (32-bit) and use a dedicated 32-bit multiply path for the I term, or scale the error signal before the PID to increase effective gain resolution.