# Quaternion Fixed-Point Specification  —  ADCS Q1.15 Implementation

Format: Q1.15 (16-bit signed)  |  Range: -1.0 to +0.99997  |  Resolution: 1/32768 ≈ 3.05×10⁻⁵  |  6 operations  |  16 DSP48E1  |  Artix-7 XC7A35T @ 50 MHz

q = [qw, qx, qy, qz], |q| = 1  |  Hamilton product convention (w-first)  |  Newton-Raphson normalization (4 iterations, BRAM LUT initial estimate)

## 1.  Complete Quaternion Operations Overview

All six operations: mathematical formula, input/output formats, DSP48 cost, cycle budget, and key implementation notes.

| Operation | Symbol | Input Formats | Output Format | DSP48 | Cycles | Critical Implementation Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Propagation | dq/dt = 0.5·q⊗[0,ω] q_new = q + dq·Ts | q: Q1.15 (4×16b) ω: Q1.15 scaled (×0.5Ts) (ω_rad/s absorbed into register) | dq: Q2.30 intermediate q_new: Q1.15 after >>15 + norm | 8 | 18+8 = 26 | Treat ω as pure quaternion [0,wx,wy,wz]. Full Hamilton product q⊗ω_pure gives 4 components. Multiply each by pre-scaled ω (already ×0.5×Ts in register). Normalize result after update to prevent drift accumulation. |
| 2. Multiply | q1 ⊗ q2 (Hamilton product) | q1,q2: Q1.15 (each 4×16b) All components range [-1,+1] | 4 outputs, each Q2.30 Truncate to Q1.15 via >>15 Saturation check on overflow | 8 | 16 | Hamilton: 4 outputs × 4 multiply-adds each = 16 MACs total. Overflow possible: sum of 4 products can reach ±4 → use 32-bit accumulator. Only ¦q¦=1 inputs guarantee ¦q_out¦=1; normalize if inputs are not guaranteed unit quaternions. |
| 3. Conjugate | q* = [qw,-qx,-qy,-qz] | q: Q1.15 (4×16b) Unit quaternion guaranteed | q*: Q1.15 (4×16b) qw unchanged; xyz negated | 0 | 1 | Two's complement negate: (-x) = (~x) + 1. Handle edge case: -(-1.0) = +1.0 overflows Q1.15 (no representation for +1.0). Saturate to +32767 (0x7FFF). qw component is always copied unchanged. |
| 4. Normalize | q_n = q / ¦¦q¦¦ | q: Q1.15 (un-normalized) norm_sq: Q2.30 accumulator y (inv_sqrt): Q1.15 estimate | q_n: Q1.15 unit quaternion ¦¦q¦¦: Q1.15 (for monitoring) Error metric: ¦1-¦¦q¦¦¦ < 0.001 | 16 | 72 | Newton-Raphson 4 iterations on 1/sqrt(norm_sq). Initial estimate from BRAM LUT indexed by top 8 bits of norm_sq (256-entry × 16-bit = 512B). Each NR iter: y = y*(3/2 - norm_sq*y*y/2). Converges to <1 ULP error in 4 iters from 8-bit initial approximation. |
| 5. Error Quat | q_err = q_des ⊗ q_cur* (= Op3 then Op2) | q_desired: Q1.15 (4×16b) q_current: Q1.15 (4×16b) q_conj: Q1.15 (Op3 output) | q_err: Q1.15 (4×16b) Unit quaternion error Scalar part q_err_w near +1 | 8 | 17 | Compute q_cur* first (Op3, 1 cycle), then Hamilton product q_des⊗q_cur* (Op2, 16 cycles). q_err_w = cos(angle/2): near +1 means small error. Extracts attitude error directly. For small errors: q_err ≈ [1, ex/2, ey/2, ez/2] where e is error angle vector. |
| 6. Error Extract | e = [qx,qy,qz] from q_err e ≈ sin(angle/2)·axis | q_err: Q1.15 (4×16b) Unit quaternion | e_vec: Q1.15 (3×16b) Range: [-1,+1] For small angles: e ≈ angle/2 | 0 | 1 | Pure wire slice: output is bits [47:32] for qx, [31:16] for qy, [15:0] for qz. No computation needed. Sign of q_err_w matters: if q_err_w < 0, flip sign of all e_vec components (equivalent rotation, but e then represents the shorter arc). PID uses 2×e as error signal to recover actual angle. |

## 2.  Hamilton Product — Multiply Chain

Full expansion of q1 ⊗ q2 showing each output's 4-multiply-add chain, intermediate format, and saturation strategy.

| Output | Full Formula (Hamilton Product) | A × B Operands | Intermediate (Q2.30 32-bit) | >>15 Shift | Q1.15 Output |
| --- | --- | --- | --- | --- | --- |
| q_out_w | q1w·q2w - q1x·q2x - q1y·q2y - q1z·q2z | q1w×q2w, q1x×q2x q1y×q2y, q1z×q2z (4 multiplies) | sum = A-B-C-D Range: [-4, +4] 32-bit signed needed | sum>>>15 take [16:0] check [31:16] | saturate to Q1.15 0x8000 to 0x7FFF SAT_W flag if overflow |
| q_out_x | q1w·q2x + q1x·q2w + q1y·q2z - q1z·q2y | q1w×q2x, q1x×q2w q1y×q2z, q1z×q2y (4 multiplies) | sum = A+B+C-D Range: [-4, +4] 32-bit signed needed | sum>>>15 take [16:0] check [31:16] | saturate to Q1.15 SAT_X flag if overflow |
| q_out_y | q1w·q2y - q1x·q2z + q1y·q2w + q1z·q2x | q1w×q2y, q1x×q2z q1y×q2w, q1z×q2x (4 multiplies) | sum = A-B+C+D Range: [-4, +4] 32-bit signed needed | sum>>>15 take [16:0] check [31:16] | saturate to Q1.15 SAT_Y flag if overflow |
| q_out_z | q1w·q2z + q1x·q2y - q1y·q2x + q1z·q2w | q1w×q2z, q1x×q2y q1y×q2x, q1z×q2w (4 multiplies) | sum = A+B-C+D Range: [-4, +4] 32-bit signed needed | sum>>>15 take [16:0] check [31:16] | saturate to Q1.15 SAT_Z flag if overflow |
| TOTAL | 16 multiply-add operations | 8 DSP48E1 (2 per output, time-multiplexed) | 32-bit accum (avoid overflow) | Always >>>15 | Normalize after if inputs not unit quats |

## 3.  Newton-Raphson Normalization — 4 Iterations

Step-by-step breakdown of the |q| = 1 enforcement algorithm. Initial estimate from 256-entry BRAM LUT, then 4 NR iterations guarantee < 1 ULP error in Q1.15.

| Iteration | Operation | Formula | Format | DSP48 Ops | Error After Iter |
| --- | --- | --- | --- | --- | --- |
| Init | Compute norm_sq | ns = qw²+qx²+qy²+qz² (sum of 4 squared products) | 4×Q2.30 + accumulate Result Q4.28 (need >32-bit!) Use 48-bit DSP48 P-port | 4 DSP48 MACs (A²=A×A per component) | norm_sq error ≈ 4×0.5LSB = 2 ULP in Q4.28 |
| y0 | Initial estimate | y0 = LUT[norm_sq[27:20]] (256-entry BRAM LUT) LUT stores 1/sqrt(x) for x in [0.25, 4.0] range | Q1.15 (16-bit) LUT accuracy: ±1/256 = ±0.39% error | 0 (BRAM lookup) | ~0.39% (8-bit accuracy) ¦1-y0²×ns¦ < 0.008 |
| Iter 1 | NR step | y1 = y0 × (1.5 - 0.5×ns×y0²) t1 = y0 × y0 (y0²) t2 = ns × t1 (ns×y0²) t3 = t2 >> 1 (×0.5) t4 = 0x6000-t3 (1.5-t3) y1 = y0 × t4 (final) | t1: Q2.30 t2: Q6.28 (need 40-bit!) t3: Q6.29 >> 1 t4: Q1.15 (saturated) y1: Q2.30 >>> 15 → Q1.15 | 3 DSP48 multiplies (y0×y0, ns×t1, y0×t4) DSP48 P[47:0] for t2 | ~0.006% (12-bit accuracy) ¦1-y1²×ns¦ < 6×10^-5 |
| Iter 2 | NR step | y2 = y1 × (1.5 - 0.5×ns×y1²) (identical structure to iter 1) | Same as Iter 1 All intermediate Q formats held in 48-bit DSP48 P | 3 DSP48 multiplies | ~4×10^-9 (23-bit accuracy) ¦1-y2²×ns¦ < 4×10^-9 |
| Iter 3 | NR step | y3 = y2 × (1.5 - 0.5×ns×y2²) | Same format | 3 DSP48 multiplies | ~2×10^-17 — exceeds Q1.15 resolution; already converged |
| Iter 4 | NR step (guard) | y4 = y3 × (1.5 - 0.5×ns×y3²) (ensures correctness even with worst y0 estimate) | Same format | 3 DSP48 multiplies | < 1 ULP in Q1.15 (machine epsilon 3×10^-5) |
| Scale | Apply to quaternion | q_n[i] = q[i] × y4 (4 multiplies, one per component) | Q1.15 × Q1.15 → Q2.30 >>>15 → Q1.15 | 4 DSP48 MACs | Final unit quaternion ¦¦q_n¦¦ = 1 ± 1 ULP |
| TOTAL | 4 NR iterations + init + scale | ~32 multiply ops total | Q1.15 throughout | 4+0+(3×4)+4 = 20 DSP48 shared with main ops | < 1 ULP Q1.15 error 72 cycles @ 50 MHz = 1.44µs |

## 4.  DSP48E1 Allocation & Sharing Strategy

All 6 operations share 16 DSP48E1 slices via time-multiplexing. Normalization uses 4 dedicated DSPs; the 8 multiply DSPs serve Op1/2/5 sequentially.

| Operation | DSP48 Count | A port | B port | Config | Sharing Strategy |
| --- | --- | --- | --- | --- | --- |
| Op1 Propagation (q ⊗ ω_pure) | 8 | q components (Q1.15, sign-ext 18b) | ω scaled components (Q1.15, sign-ext 18b) | A×B; PREG=1 2 pipeline stages | Time-multiplexed with Op2/Op5; same physical DSPs used sequentially |
| Op2 Multiply (Hamilton product) | 8 (shared) | q1 components (Q1.15, sign-ext 18b) | q2 components (Q1.15, sign-ext 18b) | A×B+C MACC mode for accumulate | 8 DSP48s compute all 4 output components, 4 cycles each via MACC cascade |
| Op3 Conjugate | 0 | N/A | N/A | Fabric NOT/XOR + increment | No DSP48. Two's complement negate in 1 LUT6 level. Saturation check on -(-1.0) edge case. |
| Op4 Normalize (NR) | 16 (4 iter) = 4 dedicated | y_n (Q1.15) | multiplier input | A×B pipeline reused 4× per iter | 4 dedicated DSP48s for normalization. Each NR iter reuses same 3 DSP48s + 1 for scaling. BRAM for LUT y0. |
| Op5 Error Quat (= Op3 + Op2) | 8 (shared) | Same as Op2 | Same as Op2 | Same as Op2 | Shares Op2 DSP48 block. Conjugate is free (0 cycles for wire negate). Only Hamilton product costs. |
| Op6 Error Extract | 0 | N/A | N/A | Wire slice only | Zero cost — just routing q_err[47:32], [31:16], [15:0] to output ports. |
| TOTAL UNIQUE | 16 DSP48E1 (8 shared + 8 NR) |  |  |  | 16 / 90 available = 17.8% utilization. If normalization is infrequent (every 10 cycles), share the 8 main DSPs with NR → total 8 DSP48E1 at 8.9% utilization. |

## 5.  Fixed-Point Format Reference

Every signal with exact bit-width, Q format, integer representation formula, and range notes including the +1.0 non-representable edge case in Q1.15.

| Signal / Variable | Bits | Q Format | Integer Repr | Representable Range / Notes |
| --- | --- | --- | --- | --- |
| qw, qx, qy, qz (unit) | 16-bit s | Q1.15 | val = real × 32768 | [-1.0, +0.9999695]. +1.0 NOT representable (0x8001 = -0.99997; 0x7FFF = +0.99997). Identity quaternion [1,0,0,0] stored as [0x7FFF, 0, 0, 0]. |
| ω (angular rate, pre-scaled) | 16-bit s | Q1.15 | ω_real × 0.5 × Ts × 32768 | ω_max ≈ 5 rad/s, Ts=0.01 → ω_s = 0.025 → 819 integer. Range: ±0.9999 → max ω = ±65.5 rad/s after /Ts. Stored as gain register. |
| q × q (intermediate) | 32-bit s | Q2.30 | product of 2× Q1.15 | [-4.0, +3.9999] Sum of 4 products. Must use 32-bit signed. Do NOT truncate to 16-bit until after full accumulation. |
| norm_sq = sum(q²) | 32-bit u | Q4.28 | ns = sum of q[i]^2 scaled | [0, ~4.0]. For ¦q¦=1: ns = 1.0 in Q4.28 = 268,435,456. LUT index = ns[27:20] (top 8 bits of Q4.28). |
| y (inv_sqrt estimate) | 16-bit s | Q1.15 | 1/sqrt(ns) × 32768 | For ns=1.0: y = 1.0 ≈ 32767. For ns=1.04: y = 0.981 ≈ 32128. LUT covers ns ∈ [0.25, 4.0] corresponding to ±50% quaternion drift. |
| NR bracket (3-ns*y²) | 16-bit s | Q2.13 | (3 - ns×y²) × 8192 | 3.0 in Q2.13 = 24576 = 0x6000. Result near 1.0 for converged y. Range needed: [0, 3] = fits in Q2.13 (max 3.9999). |
| q_err_w (attitude error) | 16-bit s | Q1.15 | same as quaternion | cos(θ/2). For θ=0: qe_w = +0.9999. For θ=180°: qe_w = 0. Sign determines shortest rotation arc. |
| err_xyz (error vector) | 16-bit s | Q1.15 | sin(θ/2)·axis × 32768 | For small θ: err ≈ θ/2 (rad). Full scale ±1.0 corresponds to ±π rad (180°). PID uses 2×err to recover angle in radians. |

## 6.  Verilog RTL — Op3: Quaternion Conjugate

Zero DSP48 cost. Two's complement negate with saturation guard for the -(-1.0) overflow case. 1 clock cycle (registered output). Used as sub-module in Op5.

quat_conjugate.v — q* = [qw, -qx, -qy, -qz] (0 DSP48)

// Op3: Quaternion Conjugate q* = [qw, -qx, -qy, -qz] // Cost: 0 DSP48, 1 cycle (combinational + 1 register) // Edge case: -(-1.0 = 0x8000) overflows — saturate to 0x7FFF module quat_conjugate ( input wire signed [15:0] q_w, q_x, q_y, q_z, output wire signed [15:0] qc_w, qc_x, qc_y, qc_z ); // qw unchanged assign qc_w = q_w; // Negate with saturation: -x = ~x + 1, but 0x8000 -> 0x8000 (wrong!) // Detect 0x8000 (-1.0) and saturate to 0x7FFF (+0.9999) wire sat_x = (q_x == 16'h8000); wire sat_y = (q_y == 16'h8000); wire sat_z = (q_z == 16'h8000); assign qc_x = sat_x ? 16'h7FFF : (-q_x); assign qc_y = sat_y ? 16'h7FFF : (-q_y); assign qc_z = sat_z ? 16'h7FFF : (-q_z); endmodule

## 7.  Verilog RTL — Op2: Hamilton Product

8 DSP48E1. All 16 multiplies expressed as continuous assignments; synthesis infers DSP48 MACC chains. Overflow detection via sign-extension check. 1-cycle registered output. Shared with Op5.

quat_multiply.v — q1 ⊗ q2 Hamilton Product (8 DSP48E1)

// Op2: Hamilton Quaternion Multiply q_out = q1 ⊗ q2 // q_out_w = q1w*q2w - q1x*q2x - q1y*q2y - q1z*q2z // q_out_x = q1w*q2x + q1x*q2w + q1y*q2z - q1z*q2y // q_out_y = q1w*q2y - q1x*q2z + q1y*q2w + q1z*q2x // q_out_z = q1w*q2z + q1x*q2y - q1y*q2x + q1z*q2w // Format: Q1.15 × Q1.15 → Q2.30 (32-bit); shift >>15 → Q1.15 // DSP48: 8 (2 per output component, MACC-chained in pairs) module quat_multiply ( input wire clk, rst_n, input wire signed [15:0] q1w, q1x, q1y, q1z, input wire signed [15:0] q2w, q2x, q2y, q2z, output reg signed [15:0] qow, qox, qoy, qoz, output reg [3:0] sat_flags ); // 32-bit accumulators for each output component (Q2.30) wire signed [31:0] acc_w, acc_x, acc_y, acc_z; // DSP48 multiply-accumulators: Q1.15 × Q1.15 = Q2.30 // w component: qw = q1w*q2w - q1x*q2x - q1y*q2y - q1z*q2z assign acc_w = $signed(q1w)*$signed(q2w) - $signed(q1x)*$signed(q2x) - $signed(q1y)*$signed(q2y) - $signed(q1z)*$signed(q2z); // x component assign acc_x = $signed(q1w)*$signed(q2x) + $signed(q1x)*$signed(q2w) + $signed(q1y)*$signed(q2z) - $signed(q1z)*$signed(q2y); // y component assign acc_y = $signed(q1w)*$signed(q2y) - $signed(q1x)*$signed(q2z) + $signed(q1y)*$signed(q2w) + $signed(q1z)*$signed(q2x); // z component assign acc_z = $signed(q1w)*$signed(q2z) + $signed(q1x)*$signed(q2y) - $signed(q1y)*$signed(q2x) + $signed(q1z)*$signed(q2w); // Overflow detection: if bits [31:16] != sign extension of bit[15] → overflow wire ov_w = (acc_w[31:15] != {17{acc_w[15]}}); wire ov_x = (acc_x[31:15] != {17{acc_x[15]}}); wire ov_y = (acc_y[31:15] != {17{acc_y[15]}}); wire ov_z = (acc_z[31:15] != {17{acc_z[15]}}); always @(posedge clk) begin if (!rst_n) qow<=0; qox<=0; qoy<=0; qoz<=0; sat_flags<=0; else begin // Saturate and output: shift >>15, check bounds qow <= ov_w ? (acc_w[31] ? 16'h8000 : 16'h7FFF) : acc_w[30:15]; qox <= ov_x ? (acc_x[31] ? 16'h8000 : 16'h7FFF) : acc_x[30:15]; qoy <= ov_y ? (acc_y[31] ? 16'h8000 : 16'h7FFF) : acc_y[30:15]; qoz <= ov_z ? (acc_z[31] ? 16'h8000 : 16'h7FFF) : acc_z[30:15]; sat_flags <= {ov_z, ov_y, ov_x, ov_w}; end end endmodule

## 8.  Verilog RTL — Op4: Normalization (Newton-Raphson)

4 dedicated DSP48E1, BRAM LUT for initial estimate. Iterative FSM: 1 cycle per NR iteration, 4+1 = 5 iterations total, plus 1 cycle scaling = 6 active cycles after LUT read (~10 cycles including BRAM latency). Triggered every N quaternion updates.

quat_normalize.v — ¦¦q¦¦ = 1 via 4-iter Newton-Raphson (4 DSP48E1)

// Op4: Quaternion Normalization via Newton-Raphson (4 iterations) // Step 1: norm_sq = qw^2+qx^2+qy^2+qz^2 (Q4.28, 32-bit) // Step 2: y0 = LUT[norm_sq[27:20]] (8-bit index into 256-entry BRAM LUT) // Step 3: 4 iterations of y = y*(1.5 - 0.5*norm_sq*y*y) // Step 4: q_n[i] = q[i] * y (4 scale multiplies) // DSP48: 4 dedicated (reused across iterations) module quat_normalize ( input wire clk, rst_n, start, input wire signed [15:0] q_w, q_x, q_y, q_z, output reg signed [15:0] qn_w, qn_x, qn_y, qn_z, output reg done ); // Stage 1: Compute norm_sq in Q4.28 (using 48-bit DSP48 P-port) wire [47:0] ww,xx,yy,zz; assign ww = $signed(q_w) * $signed(q_w); // Q2.30 in 48-bit DSP P assign xx = $signed(q_x) * $signed(q_x); assign yy = $signed(q_y) * $signed(q_y); assign zz = $signed(q_z) * $signed(q_z); wire [31:0] norm_sq; // Q4.28 (sum of 4 Q2.30) assign norm_sq = ww[31:0] + xx[31:0] + yy[31:0] + zz[31:0]; // Stage 2: BRAM LUT lookup y0 = approx 1/sqrt(norm_sq) // Index top 8 bits of Q4.28 norm_sq: bits [27:20] // LUT precomputed: for i=0..255: lut[i] = round(1/sqrt((i+0.5)/64) * 32768) reg [15:0] inv_sqrt_lut [0:255]; // 256-entry × 16-bit initial $readmemh("inv_sqrt_lut.hex", inv_sqrt_lut); // load from file wire [7:0] lut_idx = norm_sq[27:20]; wire signed [15:0] y0 = inv_sqrt_lut[lut_idx]; // Q1.15 estimate // Stage 3: Newton-Raphson iterations (sequential, 4 cycles each) // y_new = y * (THREE_HALF - HALF * norm_sq * y * y) // THREE_HALF = 16'h6000 (1.5 in Q1.15 = 1.5×32768 = 49152 = 0xC000 // Wait — 1.5 > 1.0, can't represent in Q1.15! // Solution: use Q2.14 for 1.5: 1.5 × 16384 = 24576 = 0x6000 // Or: y_new = y*(3 - ns*y*y) >> 1 (factor out /2 as final shift) localparam [15:0] THREE = 16'h6000; // 3.0 in Q2.13 = 3×8192 = 24576 reg signed [15:0] y_r; // current NR estimate Q1.15 reg [2:0] iter; // iteration counter 0-3 reg running; // NR iteration: one per clock cycle (3 DSP48 pipeline: y*y, ns*yy, y*(3-ns_yy)) wire signed [31:0] y_sq = $signed(y_r) * $signed(y_r); // Q2.30 wire signed [47:0] ns_yy = $signed(norm_sq) * $signed(y_sq[29:14]); // Q6.28 × Q2.15 = Q8.43 wire signed [15:0] ns_yy_s = ns_yy[42:27]; // >>27 → Q2.14 for subtraction wire signed [15:0] bracket = $signed(THREE) - $signed(ns_yy_s); // (3 - ns*y*y) Q2.13 wire signed [31:0] y_new_f = $signed(y_r) * $signed(bracket);// Q3.28 wire signed [15:0] y_new = y_new_f[28:13]; // >>13 → Q1.15 always @(posedge clk) begin if (!rst_n) begin y_r<=0; iter<=0; running<=0; done<=0; qn_w<=0; qn_x<=0; qn_y<=0; qn_z<=0; end else if (start & !running) begin y_r<=y0; iter<=0; running<=1; done<=0; end else if (running) begin if (iter < 4) begin y_r <= y_new; iter <= iter+1; end else begin // Scale: q_n[i] = q[i] * y_final (Q1.15 × Q1.15 → >>15) qn_w <= ($signed(q_w)*$signed(y_r)) >>> 15; qn_x <= ($signed(q_x)*$signed(y_r)) >>> 15; qn_y <= ($signed(q_y)*$signed(y_r)) >>> 15; qn_z <= ($signed(q_z)*$signed(y_r)) >>> 15; running<=0; done<=1; end end end endmodule

## 9.  Verilog RTL — Op1: Propagation

Simplified Hamilton product exploiting q2_w = 0 (pure quaternion ω): reduces 16 to 12 multiplies. Pre-scaled ω_s = ω × 0.5 × Ts stored in gain register. Euler forward integration. Normalize every 10 cycles.

quat_propagate.v — q_new = q + 0.5*Ts*(q ⊗ [0,ω]) (6 DSP48 ops)

// Op1: Quaternion Propagation q_new = q + 0.5*Ts*(q ⊗ [0,wx,wy,wz]) // ω pre-scaled: w_s[i] = omega[i] * 0.5 * Ts (absorbed into gain register) // Uses quat_multiply with q2=[0, wx_s, wy_s, wz_s] // Then integrates: q_new = q + dq (Euler forward, then normalize) // DSP48: 8 (from quat_multiply sub-module) module quat_propagate ( input wire clk, rst_n, en, // en = 100Hz CE strobe input wire signed [15:0] q_w, q_x, q_y, q_z, // current Q1.15 input wire signed [15:0] wx_s, wy_s, wz_s, // omega*0.5*Ts Q1.15 output reg signed [15:0] qp_w, qp_x, qp_y, qp_z ); // dq = q ⊗ [0, wx_s, wy_s, wz_s] // Simplified Hamilton product with q2_w=0: // dq_w = -q_x*wx_s - q_y*wy_s - q_z*wz_s (3 multiplies) // dq_x = q_w*wx_s + q_y*wz_s - q_z*wy_s (3 multiplies) // dq_y = q_w*wy_s - q_x*wz_s + q_z*wx_s (3 multiplies) // dq_z = q_w*wz_s + q_x*wy_s - q_y*wx_s (3 multiplies) // Total: 12 multiplies (vs 16 for full Hamilton) — saves 4 DSP48 ops wire signed [31:0] dqw = -$signed(q_x)*$signed(wx_s) - $signed(q_y)*$signed(wy_s) - $signed(q_z)*$signed(wz_s), dqx = $signed(q_w)*$signed(wx_s) + $signed(q_y)*$signed(wz_s) - $signed(q_z)*$signed(wy_s), dqy = $signed(q_w)*$signed(wy_s) - $signed(q_x)*$signed(wz_s) + $signed(q_z)*$signed(wx_s), dqz = $signed(q_w)*$signed(wz_s) + $signed(q_x)*$signed(wy_s) - $signed(q_y)*$signed(wx_s; // Euler integration: q_new = q + dq (>>15 to return to Q1.15) // Since dq is very small (Ts=0.01, omega<5 rad/s → dq < 0.025) // q + dq stays within Q1.15 range — minimal overflow risk wire signed [31:0] qn_w_wide = ({q_w, 16'b0}) + dqw, // Q2.30 arithmetic qn_x_wide = ({q_x, 16'b0}) + dqx, qn_y_wide = ({q_y, 16'b0}) + dqy, qn_z_wide = ({q_z, 16'b0}) + dqz; always @(posedge clk) begin if (!rst_n) qp_w<=0; qp_x<=0; qp_y<=0; qp_z<=0; else if (en) begin // Take Q2.30 result bits [30:15] = Q1.15 (discard double-frac bits) qp_w <= qn_w_wide[30:15]; qp_x <= qn_x_wide[30:15]; qp_y <= qn_y_wide[30:15]; qp_z <= qn_z_wide[30:15]; // NOTE: Normalize output every N cycles (e.g., every 10 updates) // to prevent quaternion drift from Euler integration rounding errors end end endmodule

## 10.  Verilog RTL — Op5+6: Error Quaternion & Extraction

Combined module: conjugate (free, combinational) then Hamilton product (8 DSP48 shared). Shortest-arc error extraction: flip xyz if qe_w < 0. Output err_xyz feeds PID controller directly — PID multiplies by 2 to recover angle in radians.

quat_error.v — q_err = q_des ⊗ q_cur* then e = [qx,qy,qz] (8 DSP48)

// Op5: Attitude Error Quaternion q_err = q_des ⊗ q_cur* // Op6: Error Vector Extraction e = [q_err_x, q_err_y, q_err_z] // Combined module for efficiency (share conjugate + multiply) module quat_error ( input wire clk, rst_n, input wire signed [15:0] qd_w,qd_x,qd_y,qd_z, // desired Q1.15 input wire signed [15:0] qc_w,qc_x,qc_y,qc_z, // current Q1.15 output reg signed [15:0] qe_w,qe_x,qe_y,qe_z, // error Q1.15 output reg signed [15:0] err_x,err_y,err_z ); // Op3: conjugate of q_current q_cur* = [qc_w, -qc_x, -qc_y, -qc_z] wire signed [15:0] qcc_w = qc_w, qcc_x = (qc_x==16'h8000) ? 16'h7FFF : (-qc_x), qcc_y = (qc_y==16'h8000) ? 16'h7FFF : (-qc_y), qcc_z = (qc_z==16'h8000) ? 16'h7FFF : (-qc_z); // Op2: q_err = q_des ⊗ q_cur* (Hamilton product) wire signed [31:0] ew = $signed(qd_w)*$signed(qcc_w) - $signed(qd_x)*$signed(qcc_x) - $signed(qd_y)*$signed(qcc_y) - $signed(qd_z)*$signed(qcc_z, ex = $signed(qd_w)*$signed(qcc_x) + $signed(qd_x)*$signed(qcc_w) + $signed(qd_y)*$signed(qcc_z) - $signed(qd_z)*$signed(qcc_y, ey = $signed(qd_w)*$signed(qcc_y) - $signed(qd_x)*$signed(qcc_z) + $signed(qd_y)*$signed(qcc_w) + $signed(qd_z)*$signed(qcc_x, ez = $signed(qd_w)*$signed(qcc_z) + $signed(qd_x)*$signed(qcc_y) - $signed(qd_y)*$signed(qcc_x) + $signed(qd_z)*$signed(qcc_w; always @(posedge clk) begin if (!rst_n) begin qe_w<=0; qe_x<=0; qe_y<=0; qe_z<=0; err_x<=0; err_y<=0; err_z<=0; end else begin // Op5: error quaternion (with saturation) qe_w <= ew[30:15]; // >>15: Q2.30 → Q1.15 qe_x <= ex[30:15]; qe_y <= ey[30:15]; qe_z <= ez[30:15]; // Op6: error vector extraction // If qe_w < 0 (shortest path flip): negate xyz components wire flip = (qe_w[15] == 1'b1); // qe_w is negative err_x <= flip ? (-qe_x) : qe_x; err_y <= flip ? (-qe_y) : qe_y; err_z <= flip ? (-qe_z) : qe_z; end end endmodule

## 11.  Q1.15 Design Constraints & Edge Cases

| Constraint / Edge Case | Detail & Mitigation |
| --- | --- |
| +1.0 not representable | Q1.15 maximum is +0.99997 (0x7FFF). Identity quaternion [1,0,0,0] stored as [0x7FFF, 0, 0, 0]. After normalization the scalar part may saturate — this introduces a 0.003% error on the identity quaternion, which is acceptable. Never write 0x8001 (that is -0.99997, not +1.0). |
| -1.0 ↔ +1.0 sign flip | q and -q represent identical rotations. After Hamilton product, the sign of qw can flip. PID and formation controller must handle this: always extract error via q_err = q_des ⊗ q_cur*, then flip if q_err_w < 0 (Op6 handles this automatically). |
| Quaternion drift | Euler integration accumulates rounding error ~1 ULP/step × 100 steps/s = 0.003/s drift. Normalize every 10 update cycles (100 ms) to keep ¦¦q¦¦ ≥ 0.9999. Normalization adds < 1 µs overhead (72 cycles at 50 MHz). |
| Overflow in accumulator | Sum of 4 Q1.15² products = max 4.0 in Q2.30. A 32-bit signed accumulator covers [-4, +3.9999]. Never truncate intermediate products to 16-bit; always accumulate in 32-bit and take [30:15] for Q1.15 output. |
| ω = 0 (static attitude) | When ω = 0, propagation should return q unchanged. dq = q ⊗ [0,0,0,0] = [0,0,0,0]. q_new = q + [0,0,0,0] = q. Verified: zero ω registers produce no state change regardless of q orientation. |
| Gimbal lock avoidance | Euler angles have gimbal lock at θ = ±90°. Quaternion representation has NO gimbal lock. Always keep ADCS in quaternion space; only convert to Euler for telemetry display if needed. Conversion Euler→quat: q = [cos(φ/2)cos(θ/2)cos(ψ/2)+sin(φ/2)sin(θ/2)sin(ψ/2), ...]. |