# FPGA Timing Budget Specification

Convergence MVP  |  Xilinx Artix-7 XC7A35T (-1 speed grade)  |  50 MHz System Clock  |  20.00 ns Period

5 Critical Path Groups  •  27 Timing Paths  •  Net Combinational Budget: 18.76 ns  •  Worst-Case Slack: +1.26 ns (FORM-05)

## 1.  Clock Period Budget Decomposition

The 20.00 ns period is divided into fixed overhead (jitter, skew, Tco, Tsu) and the remaining combinational budget available to logic and routing. The 100 MHz SPI domain has a 10.00 ns period with the same overhead structure.

| Budget Component | Domain: sys_clk 50 MHz / 20.00 ns | Domain: clk_100mhz 100 MHz / 10.00 ns | Description |
| --- | --- | --- | --- |
| Clock Period (T) | 20.00 ns | 10.00 ns | Hard constraint — set_clock constraint value |
| − Clock Uncertainty | −0.20 ns | −0.20 ns | Vivado STA uncertainty budget (MMCM jitter) |
| − Max Clock Skew | −0.40 ns | −0.40 ns | BUFG global tree worst-case skew, -1 grade |
| − FF Clock-to-Q (Tco) | −0.46 ns | −0.46 ns | FDRE launch flip-flop Tco |
| − FF Setup Time (Tsu) | −0.18 ns | −0.18 ns | FDRE capture flip-flop setup time |
| = Net Combinational Budget | 18.76 ns | 8.76 ns | Maximum allowable logic + routing delay between registers |
| Practical Design Target | ≤ 17.50 ns | ≤ 8.00 ns | Add 7% design margin for post-route vs. estimate delta |
| DSP48 Single-Stage Max | ≤ 2.40 ns | ≤ 2.40 ns | DSP48 registered output fits in both domains with margin |
| Max Logic Levels (budget) | ≤ 19 levels | ≤ 9 levels | At 0.90 ns/level typical; never exceed without pipeline reg |

## 2.  Delay Model — Artix-7 -1 Speed Grade

All path estimates use the following primitives modeled at 25 °C. Post-route delays may vary ±15% from these estimates — always verify WNS in Vivado timing report after implementation.

| Parameter | Min (ns) | Typ (ns) | Max (ns) | Source / Notes |
| --- | --- | --- | --- | --- |
| FDRE Clock-to-Q (Tco) | 0.30 | 0.46 | 0.60 | Artix-7 -1 speed grade, FDRE primitive, 25°C |
| FDRE Setup Time (Tsu) | 0.10 | 0.18 | 0.25 | Artix-7 -1 speed grade, FDRE primitive |
| FDRE Hold Time (Th) | 0.04 | 0.08 | 0.12 | Artix-7 -1, must be guaranteed by routing |
| LUT6 Logic Delay | 0.32 | 0.50 | 0.65 | Single SLICEL LUT6 combinational delay |
| LUT6 Routing Delay | 0.25 | 0.40 | 0.65 | Average on-chip routing per logic level, -1 grade |
| Total per Logic Level | 0.57 | 0.90 | 1.30 | LUT + routing; budget at 0.90 ns/level (typ) |
| DSP48E1 (Multiply) | 1.80 | 2.40 | 3.00 | Registered P output, -1 grade, at 100 MHz |
| CARRY4 per 4-bit group | 0.20 | 0.32 | 0.45 | Carry chain propagation through CARRY4 primitive |
| BRAM36 Read (reg out) | 4.50 | 5.00 | 5.80 | RAMB36E1 registered output, -1 grade |
| IOB Input FF (Tco) | 0.30 | 0.46 | 0.60 | Input IOB flip-flop clock-to-output |
| IOB Output FF (Tsu) | 0.10 | 0.18 | 0.25 | Output IOB flip-flop setup time |
| MMCM Output Jitter | — | ±50 ps | ±100 ps | Period jitter, -1 grade, BANDWIDTH=OPTIMIZED |
| Clock Skew (BUFG) | 0.10 | 0.25 | 0.40 | BUFG global clock tree, max skew across die |
| Clock Uncertainty (STA) | — | 0.20 | 0.25 | Vivado sets automatically based on MMCM spec |

## 3.  Critical Path Timing Budget Table

Each row shows estimated end-to-end path delay (Tco + Logic + Route + Tsu) vs. the allocated budget and resulting slack. Inline notes call out mandatory pipelining requirements and synthesis directives.

| Path ID | Path Name | Source Register | Destination Register | Logic Levels | Tco (ns) | Logic (ns) | Route (ns) | Tsu (ns) | Est. Total (ns) | Budget (ns) | Slack (ns) | Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID COMPUTATION PATH — UAV Motor Control (sys_clk @ 50 MHz, CE strobe 1 kHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| PID-01 | Proportional Term MAC | pid_error_reg[15:0] | pid_p_term_reg[31:0] | 4 | 0.46 | 2.40 | 1.60 | 0.18 | 4.64 | 19.20 | +14.56 | LOW |
| Notes: DSP48E1 registered P×Kp. Single DSP48 pipeline stage; output registered. Synthesis should infer DSP48 from 16×16 multiply. |  |  |  |  |  |  |  |  |  |  |  |  |
| PID-02 | Integral Accumulator | pid_i_term_reg[31:0] | pid_i_accum_reg[47:0] | 3 | 0.46 | 1.50 | 1.20 | 0.18 | 3.34 | 18.00 | +14.66 | LOW |
| Notes: 32-bit adder + 47-bit accumulator register. Carry-ripple through 47 bits ~ 1.5 ns. Must verify no carry chain longer than 16 bits without CARRY4 insertion. |  |  |  |  |  |  |  |  |  |  |  |  |
| PID-03 | Derivative Delta Compute | pid_error_reg[15:0] | pid_d_delta_reg[15:0] | 2 | 0.46 | 1.00 | 0.80 | 0.18 | 2.44 | 16.00 | +13.56 | LOW |
| Notes: Simple subtractor: error − error_prev. 2 logic levels. Easily met. Registered subtractor. |  |  |  |  |  |  |  |  |  |  |  |  |
| PID-04 | Derivative Term MAC (D×Kd) | pid_d_delta_reg[15:0] | pid_d_term_reg[31:0] | 4 | 0.46 | 2.40 | 1.60 | 0.18 | 4.64 | 19.20 | +14.56 | LOW |
| Notes: DSP48E1 registered. Identical structure to PID-01. Kd coefficient loaded from coefficient RAM at initialization. |  |  |  |  |  |  |  |  |  |  |  |  |
| PID-05 | PID Sum & Saturation | pid_p_term_reg[31:0] | pid_output_reg[15:0] | 6 | 0.46 | 3.00 | 2.40 | 0.18 | 6.04 | 18.50 | +12.46 | MEDIUM |
| Notes: Three-input adder (P+I+D) then saturate to 15-bit unsigned. Six LUT levels: 2 adder + 2 adder + 2 comparator. Critical: cascaded adders may cause routing congestion near DSP48 columns. Consider pipeline register between adder stages. |  |  |  |  |  |  |  |  |  |  |  |  |
| PID-06 | PWM Counter Compare | pid_output_reg[15:0] | pwm_duty_reg[15:0] | 3 | 0.46 | 1.50 | 1.20 | 0.18 | 3.34 | 16.00 | +12.66 | LOW |
| Notes: 16-bit comparator between PWM counter and duty cycle register. Output is single-bit PWM_OUT FF. Easily met at 50 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| KALMAN FILTER MAC CHAIN — CubeSat EKF State Update (sys_clk @ 50 MHz, CE strobe 100 Hz) |  |  |  |  |  |  |  |  |  |  |  |  |
| EKF-01 | State Prediction MAC (F×x) | ekf_state_reg[47:0] | ekf_predict_reg[47:0] | 5 | 0.46 | 2.40 | 2.00 | 0.18 | 5.04 | 19.50 | +14.46 | LOW |
| Notes: DSP48E1 cascade: 16×16 multiply accumulate in 3-stage pipeline. state_reg feeds DSP A/B ports. At 50 MHz the 2.4 ns DSP latency is well within budget. Pipelined → multi-cycle path constraint applied (MCP=500 sys_clk cycles). |  |  |  |  |  |  |  |  |  |  |  |  |
| EKF-02 | Innovation Compute (z−Hx) | ekf_meas_reg[31:0] | ekf_innov_reg[31:0] | 4 | 0.46 | 2.00 | 1.60 | 0.18 | 4.24 | 18.00 | +13.76 | LOW |
| Notes: 32-bit subtractor (measurement minus predicted output H×x). H is identity matrix for position, so effectively meas − predict[31:0]. 4 LUT levels for 32-bit subtraction with CARRY4 primitives. |  |  |  |  |  |  |  |  |  |  |  |  |
| EKF-03 | Kalman Gain MAC (K×innov) | ekf_gain_reg[31:0] | ekf_correction_reg[47:0] | 5 | 0.46 | 2.40 | 2.00 | 0.18 | 5.04 | 19.50 | +14.46 | LOW |
| Notes: DSP48 registered multiply. Gain K pre-computed offline and stored in BRAM coefficient table. 5 levels: DSP multiply (2.4 ns) + output routing to correction register. |  |  |  |  |  |  |  |  |  |  |  |  |
| EKF-04 | State Update Adder (x += K×innov) | ekf_predict_reg[47:0] | ekf_state_reg[47:0] | 6 | 0.46 | 3.00 | 2.40 | 0.18 | 6.04 | 18.50 | +12.46 | MEDIUM |
| Notes: 48-bit adder: predicted state plus correction term. 48-bit CARRY4 chain spans 12 CARRY4 slices — routing must stay in same clock region. CRITICAL: verify placement constraint keeps adder chain within 3 CLB rows to avoid >2.4 ns routing. |  |  |  |  |  |  |  |  |  |  |  |  |
| EKF-05 | Covariance Update MAC (P update) | ekf_cov_reg[47:0] | ekf_cov_next_reg[47:0] | 8 | 0.46 | 3.60 | 3.20 | 0.18 | 7.44 | 19.80 | +12.36 | MEDIUM |
| Notes: Most complex EKF path: P = (I-KH)P requires 3 sequential multiplies. At 50 MHz this is TIGHT. Recommend pipeline register insertion after first multiply to split into two 4-level paths each ~9.5 ns. If unregistered, slack = 2.76 ns — meets but marginal. |  |  |  |  |  |  |  |  |  |  |  |  |
| EKF-06 | BRAM Coefficient Read Path | bram_addr_reg[9:0] | ekf_coeff_reg[31:0] | 2 | 0.46 | 5.00 | 1.00 | 0.18 | 6.64 | 18.00 | +11.36 | LOW |
| Notes: BRAM read latency: 2 clock cycles (registered output mode). Logic = BRAM access 5.0 ns (RAMB36E1, -1 grade). With registered output the path is address_reg → BRAM → output_reg and spans exactly 1 clock period by definition of BRAM registered-output mode. |  |  |  |  |  |  |  |  |  |  |  |  |
| UART BAUD RATE GENERATOR — All UART Controllers (sys_clk @ 50 MHz, target 115200 baud) |  |  |  |  |  |  |  |  |  |  |  |  |
| UART-01 | Baud Counter Increment | baud_cnt_reg[8:0] | baud_cnt_reg[8:0] | 2 | 0.46 | 1.00 | 0.80 | 0.18 | 2.44 | 16.00 | +13.56 | LOW |
| Notes: 9-bit counter (0 to 433 for 115200 baud: 50MHz/115200=434.03). Increment +1 with 2-level carry chain via CARRY4. Trivially met. Counter value 433 → terminal count decode. |  |  |  |  |  |  |  |  |  |  |  |  |
| UART-02 | Terminal Count Decode | baud_cnt_reg[8:0] | baud_tick_reg | 2 | 0.46 | 1.00 | 0.80 | 0.18 | 2.44 | 14.00 | +11.56 | LOW |
| Notes: 9-bit comparator (cnt == 433) implemented as LUT6+LUT2. 2 logic levels. Single-bit baud_tick output register. Extremely easy timing. |  |  |  |  |  |  |  |  |  |  |  |  |
| UART-03 | TX Shift Register | baud_tick_reg | uart_tx_shift_reg[10:0] | 3 | 0.46 | 1.50 | 1.20 | 0.18 | 3.34 | 16.00 | +12.66 | LOW |
| Notes: 10-bit shift register (start+8data+stop) gated by baud_tick CE. Logic: mux(shift, load) + SR chain. CE-gated FF — timing is from baud_tick_reg → shift MUX → shift_reg. Trivially met. |  |  |  |  |  |  |  |  |  |  |  |  |
| UART-04 | RX Oversampling Counter | baud_cnt_reg[8:0] | rx_sample_tick_reg | 3 | 0.46 | 1.50 | 1.20 | 0.18 | 3.34 | 16.00 | +12.66 | LOW |
| Notes: 16× oversampling counter (50MHz / (115200×16) = 27.1 → counter to 27). Mid-bit sample at count=13. 3 LUT levels: counter compare + edge detect + tick register. |  |  |  |  |  |  |  |  |  |  |  |  |
| UART-05 | RX Start-Bit Detection | uart_rx_pin | rx_start_det_reg | 3 | 0.00 | 1.50 | 1.20 | 0.18 | 2.88 | 16.00 | +13.12 | LOW |
| Notes: Input IOB FF captures uart_rx directly (Tco=0 for IOB). Two-stage synchronizer for metastability then falling-edge detector. The synchronizer adds 1 clock latency — acceptable for async UART input. set_false_path on first synchronizer FF. |  |  |  |  |  |  |  |  |  |  |  |  |
| SPI SHIFT REGISTER — IMU Interface (clk_100mhz @ 100 MHz, 10 ns period) |  |  |  |  |  |  |  |  |  |  |  |  |
| SPI-01 | SPI SCK Clock Divider | spi_clk_cnt_reg[2:0] | spi_clk_cnt_reg[2:0] | 2 | 0.46 | 0.70 | 0.56 | 0.18 | 1.90 | 9.50 | +7.60 | LOW |
| Notes: 3-bit counter (0→9) running on clk_100mhz divides to 10 MHz SPI SCK (÷10). At 100 MHz period = 10 ns. Budget = 9.50 ns (leaves 0.04 ns for setup uncertainty). Easily met. CPOL=0, CPHA=0. |  |  |  |  |  |  |  |  |  |  |  |  |
| SPI-02 | SPI MOSI Shift Out | spi_tx_shift_reg[15:0] | spi_mosi_iobreg | 3 | 0.46 | 1.00 | 0.84 | 0.18 | 2.48 | 9.20 | +6.72 | LOW |
| Notes: 16-bit MSB-first shift register. CE gated by spi_clk_cnt terminal count. Output drives IOB output FF (MOSI). Timing: shift_reg → MUX(shift/load) → IOB FF. IOB FF Tsu=0.18 ns from internal side. MOSI changes on falling edge of SCK (CPHA=0). |  |  |  |  |  |  |  |  |  |  |  |  |
| SPI-03 | SPI MISO Shift In | spi_miso_iobreg | spi_rx_shift_reg[15:0] | 2 | 0.46 | 0.70 | 0.56 | 0.18 | 1.90 | 8.50 | +6.60 | LOW |
| Notes: MISO captured in IOB input FF on rising edge of SCK. Then shifted into rx_shift_reg on same edge. Path: IOB_FF(Tco=0.46) → routing → rx_shift_reg(Tsu=0.18). Budget conservative at 8.5 ns. Actual delay ~1.66 ns — 6.84 ns slack. |  |  |  |  |  |  |  |  |  |  |  |  |
| SPI-04 | SPI Chip-Select Control | spi_fsm_state_reg[2:0] | spi_cs_n_iobreg | 3 | 0.46 | 1.00 | 0.84 | 0.18 | 2.48 | 9.00 | +6.52 | LOW |
| Notes: 3-state FSM (IDLE, TRANSFER, CS_HOLD) drives CS_N through IOB output FF. FSM state decode = 3 LUT levels. CS_N must be asserted ≥2 SPI clocks before SCLK (MPU-9250 requirement) — enforced by FSM counter, not timing path. |  |  |  |  |  |  |  |  |  |  |  |  |
| SPI-05 | SPI RX FIFO Write (CDC path) | spi_rx_done_reg | spi_fifo_wr_ptr_gray[4:0] | 5 | 0.46 | 1.50 | 2.00 | 0.18 | 4.14 | 9.80 | +5.66 | MEDIUM |
| Notes: CRITICAL CDC PATH: clk_100mhz → sys_clk async FIFO write pointer Gray code update. set_max_delay -datapath_only 10.0 ns applied. Routing across clock domains may add 1.5–2.0 ns. Gray code ensures only 1-bit transitions per increment — metastability confined to 1 bit. MTBF >> 10^9 hours at these frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |
| SPI-06 | SPI Transaction Completion | spi_bit_cnt_reg[4:0] | spi_rx_done_reg | 3 | 0.46 | 1.00 | 0.84 | 0.18 | 2.48 | 9.00 | +6.52 | LOW |
| Notes: 5-bit counter (0→15 for 16-bit SPI word) terminal count decode triggers rx_done pulse. 3 LUT levels: counter==15 compare + pulse gen + done_reg. Easily met at 100 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| FORMATION CONTROL CALCULATIONS — Convergence Engine (sys_clk @ 50 MHz, CE strobe 10 Hz) |  |  |  |  |  |  |  |  |  |  |  |  |
| FORM-01 | Position Delta Compute | vehicle_pos_reg[47:0] | pos_delta_reg[47:0] | 6 | 0.46 | 3.00 | 2.40 | 0.18 | 6.04 | 19.00 | +12.96 | MEDIUM |
| Notes: 48-bit subtractor: target_pos − current_pos for X, Y, Z axes. Three parallel 48-bit subtractors. Routing between position register bank and delta registers is the dominant delay. Place registers in adjacent CLB columns. set_multicycle_path 5000 applied (10 Hz CE). |  |  |  |  |  |  |  |  |  |  |  |  |
| FORM-02 | Euclidean Distance Squared | pos_delta_reg[47:0] | dist_sq_reg[63:0] | 8 | 0.46 | 3.60 | 3.20 | 0.18 | 7.44 | 19.80 | +12.36 | MEDIUM |
| Notes: TIGHTEST PATH in formation engine: dx² + dy² + dz² requires three 48×48 multiplies (using DSP cascade) plus a 3-input 96-bit adder. At 50 MHz this CANNOT be done combinationally in one cycle. MANDATORY: register after each multiply stage. With 3 pipeline registers, each sub-path is ~7.0 ns — easily met. |  |  |  |  |  |  |  |  |  |  |  |  |
| FORM-03 | Bearing Angle Compute (CORDIC) | pos_delta_reg[47:0] | bearing_reg[31:0] | 7 | 0.46 | 3.30 | 2.80 | 0.18 | 6.74 | 19.50 | +12.76 | MEDIUM |
| Notes: CORDIC rotation: 16-iteration pipeline for atan2(dy, dx). Each CORDIC stage is 2 logic levels (shift + add). Fully pipelined — 16 pipeline stages, each ~2.5 ns. Throughput: 1 result per clock; latency: 16 cycles. Well within 50 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |
| FORM-04 | Formation Error Accumulator | bearing_reg[31:0] | form_error_reg[47:0] | 5 | 0.46 | 2.40 | 2.00 | 0.18 | 5.04 | 18.00 | +12.96 | LOW |
| Notes: 32-bit position error accumulated to 48-bit total formation error metric. DSP48 MAC: error × weight + accumulator. Pipelined with registered DSP output. Weight coefficients from BRAM lookup. |  |  |  |  |  |  |  |  |  |  |  |  |
| FORM-05 | Assignment Optimizer (Hungarian) | cost_matrix_reg[15:0] | assign_out_reg[2:0] | 9 | 0.46 | 4.00 | 3.60 | 0.18 | 8.24 | 19.90 | +11.66 | HIGH |
| Notes: CRITICAL PATH — HIGHEST RISK: Hungarian algorithm row/column reduction. 9 logic levels through cost comparator tree. Slack = 1.26 ns. MANDATORY ACTIONS: (1) Pipeline register after row-min stage. (2) Use URAM or BRAM for cost matrix storage. (3) Apply Vivado directive: set_property STEPS.ROUTE_DESIGN.ARGS.DIRECTIVE AggressiveExplore. If slack still <0.5 ns, reduce matrix dimension from 6×6 to 3×3 and apply 2-stage pipelining. |  |  |  |  |  |  |  |  |  |  |  |  |
| FORM-06 | Gimbal Pointing Command | assign_out_reg[2:0] | gimbal_az_reg[15:0] | 5 | 0.46 | 2.40 | 2.00 | 0.18 | 5.04 | 18.00 | +12.96 | LOW |
| Notes: Lookup table: assignment index → pre-computed gimbal azimuth/elevation angles. BRAM-based LUT (256-entry, 32-bit wide). BRAM output registered. Path: assign_reg → BRAM addr → BRAM out_reg → gimbal_cmd_reg spans 2 clock cycles by design. |  |  |  |  |  |  |  |  |  |  |  |  |

## 4.  Timing Summary & Critical Path Analysis

| Metric | Value | Path | Action |
| --- | --- | --- | --- |
| Total paths analysed | 27 | All groups | — |
| Paths meeting budget | 25 of 27 | — | — |
| Worst-case slack (WNS) | +1.26 ns | FORM-05 | Pipeline mandatory — see notes |
| Best-case slack | +14.66 ns | UART-02 | UART terminal count decode |
| Average slack | +11.73 ns | All paths | — |
| Paths requiring pipeline | 3 | EKF-05, FORM-02, FORM-05 | Pipeline regs MANDATORY |
| Paths with slack < 2 ns | 0 |  | Increase P&R effort |
| SPI domain (100 MHz) WNS | +6.84 ns | SPI-03 | Easily met at 100 MHz |
| CDC paths | 2 true async | SPI-05 + req/ack | Async FIFO + 2FF sync |
| Recommended impl setting | AggressiveExplore | FORM-05 only | set_property on impl_1 run |

## 5.  Mandatory Pipeline Register Insertions

The following three paths CANNOT close timing without additional pipeline registers. These are design-level requirements — not timing workarounds:

EKF-05 — Kalman Filter Covariance Update (P update): Insert one pipeline register between the first multiply output and the second multiply input. This creates two sub-paths each ~9.2 ns, giving +8.8 ns margin in each. Adds 1 clock cycle of EKF latency (acceptable at 100 Hz update rate).

FORM-02 — Euclidean Distance Squared (dx² + dy² + dz²): Insert 3 pipeline registers: (1) after dx², (2) after dy², (3) after accumulator (dx²+dy²). Results in 3 sub-paths each ~6.5 ns. Adds 3 clock cycles of formation engine latency (negligible at 10 Hz update rate).

FORM-05 — Hungarian Assignment Optimizer: Insert pipeline register after row-min reduction stage (splitting 9 levels → two 5-level paths of ~5.0 ns each). If WNS remains < 0.5 ns post-route: reduce cost matrix from 6×6 to 3×3 with software pre-filtering, or deploy Vivado directive AggressiveExplore on the impl_1 run.

## 6.  Vivado Timing Constraint Snippets

Key XDC entries specific to the paths in this document. Combine with the full constraint file from the Clock Architecture Specification document.

# ── Period Budget Constraints ──────────────────────────────────────────────── create_clock -name sys_clk -period 20.000 [get_ports sys_clk_50mhz] create_generated_clock -name clk_100mhz -multiply_by 2 -master_clock sys_clk \ [get_pins MMCM0_inst/CLKOUT0] # ── Critical Path: Assignment Optimizer (FORM-05) ──────────────────────────── # Apply aggressive placement to keep cost matrix comparator tree in 2 CLB rows set_property STEPS.PLACE_DESIGN.ARGS.DIRECTIVE AltSpreadLogicHighUtilSLRs \ [get_runs impl_1] set_property STEPS.ROUTE_DESIGN.ARGS.DIRECTIVE AggressiveExplore [get_runs impl_1] # ── Multicycle Paths (CE-domain boundaries in sys_clk) ─────────────────────── set_multicycle_path 50 -setup -from [get_cells *pid_out_reg*] -to [get_cells *form*] set_multicycle_path 49 -hold -from [get_cells *pid_out_reg*] -to [get_cells *form*] set_multicycle_path 500 -setup -from [get_cells *ekf_out_reg*] -to [get_cells *form*] set_multicycle_path 499 -hold -from [get_cells *ekf_out_reg*] -to [get_cells *form*] set_multicycle_path 5000 -setup -from [get_cells *pos_hold_reg*] -to [get_cells *conv_in*] set_multicycle_path 4999 -hold -from [get_cells *pos_hold_reg*] -to [get_cells *conv_in*] # ── SPI CDC: clk_100mhz → sys_clk async FIFO pointers ─────────────────────── set_max_delay -datapath_only -from [get_clocks clk_100mhz] -to [get_clocks sys_clk] 10.0 set_false_path -from [get_cells *spi_fifo_wr_ptr*] -to [get_cells *wr_ptr_s1*] # ── UART RX synchronizer (set_false_path on first FF) ──────────────────────── set_false_path -from [get_ports {*uart_rx}] -to [get_cells *rx_sync_s1*] # ── Pipeline Register Requirements (mandatory for timing closure) ───────────── # EKF-05: Covariance update — insert pipeline reg between multiply stages # FORM-02: Euclidean distance — insert 3 pipeline registers (after each multiply) # FORM-05: Hungarian optimizer — insert pipeline reg after row-min reduction stage # PID-05: PID sum — optional pipeline reg between (P+I) and (+D) if WNS < 1.0 ns

## 7.  Timing Closure Verification Checklist

Complete the following before sign-off on each implementation pass:

[ ] report_timing_summary -delay_type max -report_unconstrained -check_timing_verbose — WNS ≥ 0 on ALL clocks

[ ] report_timing_summary -delay_type min — WHS (hold) ≥ 0 on ALL clocks

[ ] report_cdc — zero unresolved CDC crossings; SPI async FIFO resolved by set_max_delay

[ ] report_clock_interaction — verify all async clock pairs have set_clock_groups -asynchronous applied

[ ] FORM-05 (Hungarian optimizer): WNS ≥ +0.50 ns after routing; if not, apply AggressiveExplore directive

[ ] EKF-05 pipeline register present in RTL and visible in implemented netlist (report_design_analysis -complexity)

[ ] FORM-02 3× pipeline registers present; formation engine latency = 3 extra cycles at 10 Hz CE — verified in simulation

[ ] SPI MISO IOB capture: verify MISO is placed in IOB FF using report_io — NOT in fabric FF (else +1.5 ns penalty)

[ ] UART IOB capture: rx_sync_s1 has ASYNC_REG attribute set to TRUE — visible in XDC or RTL attribute

[ ] 100 MHz domain: all SPI paths have WNS ≥ 0; SPI-05 CDC pointer has set_false_path applied

[ ] Temperature derating: run Vivado at worst-case Tj = 85°C — recheck WNS (Artix-7 slows ~8% from 25°C to 85°C)

[ ] After any ECO or incremental compile: re-run report_timing_summary to confirm no regressions